| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4691 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.STGROUP: clause L.5 – discussion and update proposal for "use case #5: SCTP transport modes" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal for clause L.5 related to "use case #5 "SCTP transport modes".

1	Discussion

There are three stream grouping variants possible around SCTP transport modes. AVD-4730 proposes additional text / editor's notes for the correspondent subclauses in chapter 11 of H.248.SCTP.

Every use case for H.248.STGROUP stream grouping should be complemented by a concrete H.248 signalling example in order to carefully check the content of H.248 descriptors, the mapping, allocation and partial usage of SDP between H.248 component stream and H.248 (de-)aggregation stream level, as well as the identification of potential interactions with existing signalling elements.

There wasn't yet any comprehensive technical discussion about this use case at the last 2015-02 meeting. Thus, there are still some pending questions for clarification around this use case.

Focus of the considered stream grouping examples is on the semantics as indicated by clauses 11.2 and 11.3 in H.248.SCTP.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4680, the input draft for this meeting.

NOTE – Some changes are complemented by electronic comments in order to provide more background information or editorial hints. It's at the discretion of the meeting and/or editor to handle these comments (deletion or conversion to editor note).

Change proposal #1:

[Begin Proposed Correction]

## L.5	Use case #5: SCTP transport modes

### L.5.1	Background

Protocol stacks for valid SCTP transport modes are outlined in clause 6.1/[b-ITU-T H.248.SCTP]. Figure L.9 illustrates the most complex SCTP transport mode, given by "tunneled SCTP via DTLS/L4", and the option of dynamic L4 replacements between UDP and TCP due to potential NAT issues during the lifetime of the communication service.

Figure L.9 – Generic protocol stack model for mapping of SCTP streams to applications
(here at the example of SCTP transport mode "tunneled SCTP via DTLS/L4")

This SCTP transport mode is characterized by

- option of L4 protocol and dynamic replacement(s);

- option of tunneled stack (L4 over L4); and

- option of single- vs multi-stream-SCTP mode.

Such a protocol stack requires usually multiple H.248 streams, but might be served in a specific case by a single H.248 stream only. The next sub-clause tries to evaluate such decision criteria.

### L.5.2	Guidelines for the derivation of H.248 streams per SCTP transport mode

There are theoretically eight combinations taken the assumption that each of the above three conditions leads finally to an "H.248 stream yes/no?" decisions. Figure L.10 summarizes a sample decision logic:

NOTE – The example serial execution of decisions could be parallized as well.

Figure L.10 – Example decision logic for the derivation 
of H.248 streams per SCTP transport mode

Whether there are finally really explicit H.248 streams requested by the MGC for each decision point is basically conditional. The default case would be to minimize the number of H.248 streams in order to reduce "signalling overhead", which means the merging of multiple protocol layers in single H.248 streams, whenever possible. The division in separated H.248 streams would be only done in case of semantical ambiguities, stream-specific functionality and/or control activities with limited scope on streams only.

The SCTP association serves multiple, N SCTP streams in parallel, with a value N between one ("the single-SCTP-stream-mode case") and usually "smaller than ten" for typical, present communication services.

The resulting amount of H.248 streams is between one as minimum and 'N+3' as theoretical maximum.

It has to be noted that the decision logic of Figure L.10 could be actually applied to all known SCTP transport modes. For instance:

- SCTP transport mode "native SCTP": amount of H.248 streams = 'N+1' (because a) without L4-over-L4 tunneling, and b) without DTLS); or

- SCTP transport mode "transport secured SCTP": amount of H.248 streams = '1' (due single-SCTP-stream-mode because DTLS connection is not shared by multiple application protocols).

### L.5.3	Use case #5.1: Domain with "tunneled SCTP via  switched {DTLS/UDP | DTLS/RFC4571/TCP}"

A possible H.248 context model and example H.248 signalling are discussed for the use case between the interworking of the two network domains with different transport modes as illustrated in Figure L.11. The example stream grouping model takes into account multiple application level traffic flows, which are mapped on individual data channels (DC):

Figure L.11 – Use case #5.1: SCTP transport modes – "tunneled SCTP via 
switched {DTLS/UDP | DTLS/RFC4571/TCP}"

Such a fine granular stream group mapping is motivated by following objectives or assumptions in this example:

- UDP is the primary IP transport protocol, but it cannot be excluded that detected NAT problems could lead to a replacement by TCP during the active conferencing phase.

- The SCTP association and DTLS session/connection might be decoupled concerning the directionality of establishment.

- The SCTP association and DTLS session/connection are decoupled concerning the timeline of establishment.

- The DTLS protocol is profiled, primarily due to supported protocol versions and the explicit notification of possible error alerts.

- SCTP association establishment is conditional, dependent on successful DTLS authorization and established DTLS session.

- Thus, SCTP protocol procedures initially blocked. SCTP association establishment is rather triggered in a subsequent step, after confirmation of trusted user.

- H.248 SEP for DTLS is able for both DTLS roles concerning incoming and outgoing DTLS session establishment procedures.

- SCTP protocol failures could lead to a shut-down of the SCTP association, but should not lead to the release of a DTLS session/connection, rather a re-establishment of an SCTP association.

- Performance monitoring: a few protocol specific statistics are requested, which result in stream-level statistics.

The considered context model uses three H.248 SEPs at (de-)aggregation level (T1(S1), T1(S2) and T1(S3)) besides the N H.248 component streams. Hence, this example is related to hierachical stream grouping. Table L.5.1 illustrate example H.248 signalling with regards to the initial creation of the context and the preparation of the H.248 (de-)aggregation streams:

Table L.5.1 – Example H.248 media descriptor – step 1 for initial DTLS establishment

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ TerminationState { mgroup/groupse = ["SCTP-DC K,K+1 ,… ,K+N", "SCTP-TRANSPORT 3", "DTLS-TRANSPORT 2"] ; Semantics "SCTP-TRANSPORT", ; "DTLS-TRANSPORT are not yet specified; }, ; --- STREAM 1 --------------------------------- Stream = 1 { ; L4 connection: UDP or TCP LocalControl { ipdc/realm = <IP_realm>, Mode = SendReceive, mgroup/strdeagg = [2] ; -> Stream 2 'DTLS' ... }, Local { v=0 c=IN IP6 $ m=- $ udp – ; Media type and media format agnostic, as ; this SEP does not need to know anything ; related to UDP payload formats. ; <proto> value "udp" as per RFC 4566. ; Assumption: ICE might also conclude with ; a TCP candicate pair, which would result ; in an SCTP/DTLS/RFC 4571/TCP/IP ; transport protocol stack. a=ice-ufrag:$ a=ice-pwd:$ }, Remote { v=0 c=IN IP6 <UE_IP_data> m=- <UE_port_data> udp – a=ice-ufrag:<UE_ICE_ufrag_data> a=ice-pwd:<UE_ICE_pwd_data> } }, ; --- STREAM 2 --------------------------------- Stream = 2 { ; DTLS connection LocalControl { Mode = SendReceive, tlsbsc/bceb="unblocked", mgroup/strdeagg = [3] ; -> Stream 3 'SCTP' ... }, Local { v=0 c=- - - m=- – DTLS – a=fingerprint:$ $ }, Remote { v=0 c=- - - m=- – DTLS – a=fingerprint:<UE_hash_func_data> <UE_fingerprint_data> } }, ; --- STREAM 3 --------------------------------- Stream = 3 { ; SCTP association LocalControl { Mode = SendReceive, mgroup/strdeagg = [K,K+1 ,… ,K+N] ; deaggreg. ... }, Local { v=0 c=- - - m=application –SCTP webrtc-datachannel a=fmtp:webrtc-datachannel \ max-message-size=$ a=sctp-port=$ ; Assumption: MGC lets MG choose local SCTP ; port and maximal message size. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel \ max-message-size=<UE_SCTP_max_message_size> a=sctp-port=<UE_SCTP_port> } }, ; --- STREAM K, K+1, …-------------------------- Stream = 4 { ; Data application LocalControl { Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP <application usage> … a=dcmap:<SCTP_Stream_ID> subprotocol="…" … }, Remote { v=0 c=- - - m=application – DTLS/SCTP <application usage> … a=dcmap:<SCTP_Stream_ID> subprotocol="…" … } }, | Stream 1: - (L4)-protocol is terminated - (L4)-protocol = 'UDP' (initially) - (L4)-protocol is linked to (L4+)-protocol (here 'DTLS'), which is part of Stream 2 - that deaggregation from the (L4) stream to the DTLS stream is indicated via the strdeagg property Stream 2: - (DTLS)-protocol is terminated - (DTLS)-protocol is linked to (SCTP)-protocol (Stream 3) - that deaggregation from the DTLS stream to the SCTP stream is indicated via the strdeagg property With respect to the establishment of the (DTLS)-bearer-connection: - the SDP "a=setup:" attribute cannot be used due to ambiguity; - the incoming/outgoing establishment procedures are controlled via tlsbsc package elements (not shown in the example) Stream 3: - (SCTP)-protocol is terminated; - SEP represents the (SCTP)-association-endpoint; - (SCTP)-association is linked to (SCTP)-streams (Stream K, K+1, …) - that deaggregation from SCTP association level to SCTP streams is indicated via the strdeagg property With respect to the establishment of the (SCTP)-bearer-connection: - the SDP "a=setup:" attribute cannot be used due to ambiguity; - the incoming/outgoing establishment procedures are controlled via sctpbcc package elements (not shown in the example) Streams K, K+1, etc are just indicated because the intention of this use case is not to emphasize on a particular communication service using this SCTP transport mode. |
| ; other Terminations T2, … |  |

{Editor’s note (2015-06): there are pending questions for clarification and actions in above table, see highlighted text in yellow.}

The example H.248 command requests highlights some specifics related to the "stream deaggregation" characteristic:

a) semantical grouping of SEPs within a termination:

- the example protocol stack is characterized by multiple hierarchical relationships, which are reflect usage of property mgroup/groupse at termination level and property mgroup/strdeagg at stream level; and

- deaggration from a particular SEP to other SEP(s) covers the range from one to many;

b) LD-/RD-embedded SDP media description:

- IP transport address:

- the actual IP transport address information is only subject of the SDP "c="- / "m="-lines of H.248 SEPs which serving as protocol endpoint for the IP and L4 protocol;

- other SEPs which are agnostic for that kind of information using the "-" wildcard for correspondent SDP information elements;

- transport protocol (stack segment) indication:

- the "m="-line <proto> should provide a value for the protocol(s) which are assigned to that SEP;

- this would be a single protocol value only in above example (i.e., the SIP level "DTLS/SCTP" value is mapped to self-contained "DTLS" and "SCTP" values)

- SDP media format attributes: only H.248 SEP relevant attributes are sufficient, not relevant SDP attributes should be omitted in order to avoid redundancy.

{Editor's note (2015-02): above observations are still preliminary …}

When the remote user equipment answers, then the MGC will modify the configuration of context C1. There might be following actions:

Table L.5.2 – Example H.248 media descriptor – step 2 for completion

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| FIXTHIS |  |

[End Proposed Correction]

____________________

E.g., an initial UDP connection might be replace at a later point in time by a TCP connection, triggered by ICE refreshment activities.