| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4695 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.PAURES: H.248 use cases and design considerations (clauses 1 to 7) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes update proposals based on a review of clauses 1 to 7.

# 1	Introduction

We did review the existing Draft with focus on RTP topologies other than the RTP mixer, as well as the MG-internal handling of RTCP control flows. We did also consider the "RTP pause and resume" mechanism as "signalling protocol", inclusive the state machines associated with the signalling protocol endpoints. Such an analysis did lead to a number of review comments and update proposals. The change proposals were mapped on three contributions (AVD-4695, - 4696 & -4697) which address different sections of the Draft, however, some overlappings can't be excluded to relations between some items.

This contribution starts the discussion on the "high level framework". AVD-4696 does review the current protocol solution (i.e., the rempr package) and AVD-4697 provides first results on the difficult question concerning potential interaction issues with other RTCP related services.

# 2	Discussion

## 2.1	"PAUSE-RESUME" as inband signalling protocol

[IETF STREAM PAUSE] defines essentially a signalling protocol (called "P/R-signalling" in this contribution) between the two signalling instances of an RTP sender and RTP receiver with the purpose of temporarily stopping the unidirectional RTP media packet flow in sender-to-receiver direction (Figure 1).

Figure 1 – RTP stream Pause and Resume as feedback-based control loop
(NOTE: the "P/R-signalling"-path is in red)

The syntax of the signalling protocol is based on RTCP feedback messages. Such an RTCP feedback message is either carried as

- compound RTCP packet (= "RFC 3550 default") or

- reduced-size RTCP packet.

The aspects of that introduction are relevant to terminology (see next clause 2.2) and possible interactions with RTP topologies (H.248.88; see AVD-4697), because the applied RTP topology by an H.248 MG impacts the RTCP handling by the MG.

## 2.2	Terminology for RTP entities

[IETF STREAM PAUSE] (and draft H.248.PAURES) use actually mutiple names for the two "P/R"-signalling instances, which isn't necessarily a contradiction, but might be difficult for the reader. The IETF "RTP grouping taxonomy" document (draft-ietf-avtext-rtp-grouping-taxonomy) should help to clarify the terminology situation. Figure 2 illustrates again that functional processing model from that IETF draft. The two "P/R"-signalling instances are represented by the functions media packetizer and media depacketizer in that model.

"P/R"-signalling affects primarily two RTP topology categories: point-to-point and mixer-based.

Conclusions for RTP topology "point-to-point":

- we might equate the terms RTP source, RTP sender, RTP stream sender and media sender with media packetizer; and

- we might equate the terms RTP sink, RTP receiver, RTP stream receiver and media receiver with media packetizer.

Conclusions for RTP topology "mixer":

we'd like to postpone the discussion of expected terminology impact, but already highlight the main differences:

- the RTP sender and RTP receiver in its capacity as RTP source and RTP sink entities are identifiable via a "unique" SSRC value;

- the actual RTP source is synonym with the RTP sender in case of point-to-point topology (because the RTP source is directly coupled with the media source);

- but not in case of mixer: the RTP sender represents now the RTP source as output of a media source mixer instance, leading to the fact of "RFC 3550 mixer" behaviour of SSRC replacement and CSRC insertion.

The "SSRC aspect" is fundamental in case of "P/R"-signalling, keeping in mind that the RTCP feedback message for "P/R"-signalling carries three SSRC types (SSRC of packet sender, SSRC of media source and SSRC of target).

NOTE – We got the additional term of packet sender, which represents the RTP sender or RTP receiver, in its capacity as originating point of a "P/R"-signalling procedure.

Figure 2 – Terminology for RTP – Unidirectional information flow from communication source to communication sink – Model for RTP topology "point-to-point"

Semantic of RTP stream pause and resume:

Clause 6/H.248.PAURES discusses the relation and reuse option of other H.248 sub-series Recommendations, which results essentially in the specific semantic of RTP stream pause and resume. There seems to be two fundamental communication service categories (from RTP perspective):

Table 1 – Application-specific semantic of RTP stream pause and resume

| Aspect | Streaming services | Conversational services |
| --- | --- | --- |
| Semantic of "Pause / Resume": | Stop of RTP sender process. Buffering of unsend RTP packets. RTP packets "ready to send" are not discarded. | Stop of RTP sender process. Discard of RTP packets "ready to send" during "pause state". |
| Impact on "timeline" | Delay effect. | No delay ("realtime nature"). |
| Information loss: | No | Yes, intended. |
| H.248 related technology: | [ITU-T H.248.9], [ITU-T H.248.66] | [ITU-T H.248.PAURES] |
| RTP functional processing model (see Fig. 2); | Function "Media Transport Sender" effectively realizes above protocol semantic. | Function "Media Transport Sender" effectively realizes above protocol semantic, despite the fact that function "Media Packetizer" represents the P/R-signalling endpoint. NOTE – Theoretically function "Media Capture" could be already "paused", however, the requirement for immediate resume (as fast as possible) implies to keep the pipeline stages S:1, S:2, S:3 and S:4 (S:4b) "in service". |

We'd like to suggest a minor update of the last paragraph from clause 6.

## 2.3	Network use cases with H.248 entities

[IETF STREAM PAUSE], clause 3 provides use cases from RTP perspective, without explicit consideration of gateway equipement with RTP support. We'd like to evaluate again these use cases from H.248 perspective.

### 2.3.1	End-to-end RTP topology "Point-to-Point"

Figure 3 starts with the "Point-to-Point" RTP topology:

Figure 3 – Network use cases with H.248 MGs – RTP topology "Point-to-Point"

Observations from "Point-to-Point" RTP topology:

- H.248 MG provides RTP sender and/or RTP receiver role due to its RTP endsystem function (as consequence of an H.248 "RTP to non-RTP" context model) see (1.b);

- H.248 MG provides "transparent "P/R"-signalling forwarding" behaviour due to its role as "middlebox for RTP traffic" (e.g., H.248 MG in H.248.88 RTPTT topology or H.248 MG in RTP agnostic mode), see (1.a);

- H.248 MG provides RTP sender and/or RTP receiver role due to an explicit network demarcation in two separate (RTP) domains, i.e., H.248 MG in H.248.88 B2BRE topology ("the original (RTP)-stream-connection between A and B would be divided in two (RTP)-stream-connections "A-to-MG" and "MG-to-B"; and RTP stream Pause / Resume might be applied at each H.248 RTP termination");

- thus, there are already three resulting use cases with different H.248 MG behaviour related to "P/R"-signalling involvement and MG-internal processing of the underyling RTCP feedback messages.

### 2.3.2	End-to-end RTP topology "RTP mixer in unicast-only"

Figure 4 continues with the "RTP mixer in unicast-only" RTP topology:

Figure 4 – Network use cases with H.248 MGs – RTP topology "RTP mixer in unicast-only"

Observations from "RTP mixer in unicast-only" RTP topology:

- scenario (2.a) doesn't add any new H.248 use case;

- H.248 MG provides RTP sender and/or RTP receiver role due to its RTP mixer function  see (2.b);

- scenario (2.b): there might be a requirement for "transparent "P/R"-signalling forwarding" for the entire RTP mixer;

- scenario (2.b): there might be a mix of "transparent "P/R"-signalling forwarding" behaviour (e.g., due to end-to-end PAU/RES between B and C) and RTP sender and/or RTP receiver role behaviour for the other endpoint combinations.

Hence, for (2.b), there are effectively two types of H.248 context configurations for such an H.248 media server:

- H.248 context in "RTCP FB message"-agnostic mode; or

- H.248 context in "RTCP FB message"-aware mode ("which would inherently serve the all above listed (2.b) use case variations in a single context").

### 2.3.3	End-to-end RTP topology "RTP mixer in Point-to-Multipoint"

Figure 5 continues with the "Point-to-Multipoint" RTP topology:

Figure 5 – Network use cases with H.248 MGs – RTP topology "Point-to-Multipoint"

Observations from "Point-to-Multipoint" RTP topology:

- scenario (3.a), potential H.248 use case, but so far out of scope of existing standardized H.248 profiles;

- scenario (3.b) doesn't add any new H.248 use case.

### 2.3.4	Conclusions for network use cases

The existing draft is touching the use case aspect at the beginning of clause 7. However, the real number of significant use cases is increasing when considering H.248 MGs with different RTP topology models in the media path. We'd like to suggest to add a new Appendix I with a set of example H.248 network scenarios.

## 2.4	State model of P/R-signaling endpoints

### 2.4.1	Models

Clause 6/[IETF STREAM PAUSE] outlines the RTP pause state model at RTP sender side, see Figure 6:

Figure 6 – RTP sender (RTP source): RTP Pause State model 
(copy from [IETF STREAM PAUSE])

Such a state model would be associated to an H.248 stream endpoint which is enabled for processing RTP streams with applied pause/resume capability. The state model as such is important because reflecting the semantics of P/R-signaling behaviour. For instance, the H.248 Signals and Events of the H.248 rempr package relate typically to state transitioning behaviour of such a model. It might be noted that the present model of [IETF STREAM PAUSE] indicates primarily the high level behaviour, but seems to be not (yet) comprehensive with respect to all possible signalling actions. That means the additional consideration of all possible failure and unsuccessful cases beyond the small set of "good cases" only. Figure 7 provides a slightly extended model, but is still incomplete from protocol engineering perspective:

Figure 7 – RTP sender (RTP source): RTP Pause State model – extended model for discussion

### 2.4.2	Conclusions for P/R-signaling models

Following observations could be made from [IETF STREAM PAUSE]:

- the protocol semantics are described (clause 5) and defined (clauses 6 and 8) over multiple places in [IETF STREAM PAUSE];

- the protocol semantics are specified in behavioural language (and not in a formal specification language);

- above two list items making it difficult in checking protocol correctness and whether there are potential inconsistencies;

- the RTP pause state model for the RTP sender might benefit from additional indication of unsuccessful state transitions;

- an RTP pause state model for the RTP receiver is missing, probably because the "media receiver" model looks straightforward; however, that's not the case with the P/R-signaling endpoint at RTP receiver side due to some design considerations (clauses 4.5, 4.6 and 4.7) concerning the supervision of unreliable signalling transport, missing acknowledgements, unsolicited indications etc (i.e., there might be a model with "more than one state" …).

Above observations are related primarily to [IETF STREAM PAUSE] and not (yet) to existing draft H.248.PAURES.

However, parallel AVD-4696 proposes to add a new Appendix (II) with such state models due to valuable means in checking the protocol correctness of the rempr package design.

# 3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD 4678, the input draft for this Q.3/16 meeting (AVD-4678).

Change proposal #1:

[Begin Proposed Correction]

Summary

This Recommendation allows the RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] to be supported on H.248 controlled gateways.

# 1	Scope

This Recommendation describes the support of RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] on H.248 controlled gateways. It describes the mapping of call control SDP related to pause/resume to H.248 SDP and vice versa. It also contains a package that allows several different implementation types to be supported. This allows the Media Gateway (MG) to operate in an autonomous manner by receiving, making pause/resume decisions and sending RTCP level pause/resume messages with minimal MGC involvement. A Media Gateway Controller (MGC) controlled configuration is also supported whereby the reception of pause/request requests/responses is indicated to the MGC for it to make decisions and determine a response. It also allows the MGC to initiate pause/resume requests.

It also contains considerations related to the support of RTP stream Pause/Resume in H.248 controlled gateways.

The Recommendation also details the relationship to other existing H.248.x sub-series Recommendations where pause/resume functions are supported.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.1 (12/2009), Gateway control protocol: Advanced media server packages.

[ITU-T H.248.66]	Draft Recommendation ITU-T H.248.66 (xxxx), Gateway control protocol: Packages for RTSP and H.248 interworking.

[ITU-T H.248.STGROUP]	Recommendation ITU-T H.248.STGROUP (xxxx), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 3264]	IETF RFC 3264 (2002), An Offer/Answer Model with Session Description Protocol (SDP).

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 5104]	IETF RFC 5104 (2008), Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF).

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF BUNDLE]	draft-ietf-mmusic-sdp-bundle-negotiation-16 (xxxx), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[IETF STREAM PAUSE]	IETF draft-ietf-avtext-rtp-stream-pause-07 (xxxx), RTP Stream Pause and Resume.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	RTP stream [b-ITU-T H.248.RTPMUX]: {Editor's note: insert definition according to the agreements for H.248.RTPMUX, see AVD-4684.}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	P/R-signalling: the "RTP pause and resume" signalling as defined by [IETF STREAM PAUSE].

NOTE – The signalling protocol syntax is based on dedicated RTCP "feedback" packet types, the signalling protocol procedures use the associated bidirectional RTCP control flow and the signalling protocol semantics are defined in clauses 6 to 9 in [IETF STREAM PAUSE].

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AVPF | [IETF RFC 5104]: "The extended RTP profile for RTCP-based feedback" |
| --- | --- |
| CCM | Codec Control Message |
| DTMF | Dual Tone Multi Frequency |
| FB | (RTCP) Feedback (message or packet type) |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| P/R | (RTP stream) Pause and Resume |
| RTCP | Real-Time Control Protocol |
| RTCP FB | RTCP Feedback (message / packet type) |
| RTP | Real-Time Protocol |
| SDP | Session Description Protocol |
| SSRC | Synchronisation Source Identifier |
| TMMBN | Temporary Maximum Media Stream Bit Rate Notification [IETF RFC 5104] |
| TMMBR | Temporary Maximum Media Stream Bit Rate Request [IETF RFC 5104] |

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

# 5	Conventions

Elements of the ITU-T H.248 protocol model, e.g., Context, Termination, Stream, Event are represented using the first letter capitalized. Property, Event, Signal and Parameter identities are given in italics.

The suffix ".req" added to an ITU-T H.248 command name stands for a command request, while the suffix ".rep" stands for a command reply. For example "Notify.req" represents a Notify Request.

Capitalised names PAUSE, RESUME, REFUSED, PAUSED refer to Codec Control Messages (CCM) defined in [IETF STREAM PAUSE].

NOTE – The CCMs TMMBR and TMMBN [IETF RFC 5104] could be alternatively used for emulating specific pause/resume behaviour (besides the "native CCMs" [IETF STREAM PAUSE] for RTP stream pause and resume). See also clause 9.

[End Proposed Correction]

Change proposal #6:

[Begin Proposed Correction]

# 6	Relation to other Recommendations in the ITU-T H.248 sub-series

Gateways utilising H.248 currently have several methods to pause the transmission of media.

[ITU-T H.248.1]: Contains the Local Control Descriptor ‘StreamMode’ property that allows the MGC to effectively stop sending a RTP media flow (i.e. change from send/receive to receive only) whilst still maintaining an RTCP control flow. To resume the flow of media the MGC can change the StreamMode back to send/receive. However this is a local MG action rather than a request to a remote endpoint. The action is also triggered from a call control level stimulus.

[ITU-T H.248.1]: The H.248 connection model (including the TopologyDescriptor) allows Terminations/Streams to be isolated from other Terminations/Streams in the Context. This allows a MGC to isolate an H.248 media stream from other input streams effectively stopping the flow of RTP media. The MGC may then move the Termination/Stream back into a Context to resume RTP media flow.

[ITU-T H.248.9]: Contains the “Play Collect” package that enables a user to request (via a DTMF digit) that the playout of media associated with a signal be paused. It also enables the user to resume the RTP media flow via a DTMF digit.

[ITU-T H.248.66]: Contains the “Signal pause package” which is a generic package that allows the MGC to request a pause in the playout of media and later resume RTP media flow. It is suited to scenarios where media playout has been initiated through the use of a H.248 Signals rather than real-time media flows. It again is a local MG action rather than a request to a remote endpoint and the action is also triggered from a call control level stimulus.

{Editor’s note: ITU-T H.248.88 should possibly be considered here as it relates to RTCP handling. The RTP pause / resume mechanism utilises RTCP so there may be a relation.}

None of the above methods match the functionality supported by [IETF STREAM PAUSE]. [ITU-T H.248.1] and [ITU-T H.248.66] do not support user originated requests being terminated at the MG and [ITU-T H.248.9] and [ITU-T H.248.66] do not allow the pause of H.248 media streams not related to the playout of signals.

Pause/resume relates to real-time media flows (as subject of conversational communication services). Therefore the resumption of an RTP media flow is based on the media at the time of the resume request rather than at the pause point. [ITU-T H.248.66] (as typically applied in the context of streaming communication services) assumes that the playout occurs from the pause point unless another point is indicated in the resumption request.

[End Proposed Correction]

Change proposal #7:

[Begin Proposed Correction]

# 7	Design considerations

Clause 3/[IETF STREAM PAUSE] describes the main use cases for RTP stream pause and resume. It lists a number of RTP topologies. A H.248 media gateway may act as a media sender, media receiver, mixer and/or other RTP topologies with involvement in P/R-signalling. Appendix I provides a list of example use cases from perspective of H.248 gateways.  As such the design criteria listed in clause 4/[IETF STREAM PAUSE] should be considered when implementing RTP stream pause/resume on gateways.

{Editor’s note: Further details regarding how the SSRC is determined should be added. For example H.248.71 Source Description package, Call Control Signalling or via the rempr package are possibilities.}

## 7.1	Real-time nature

Given the real-time aspects described in clause 4.1/[IETF STREAM PAUSE] a gateway should ensure prompt handling of resume requests, i.e. a MG and optionally MGC should process and act upon any requests to resume a RTP stream in an expedited manner.

Pause requests are also time-dependent, but not that critical than resume requests.

## 7.2	Message direction

A gateway may initiate a local pause or resume of an RTP stream, request a remote RTP sender to pause or resume an RTP stream or respond to a request from a remote RTP receiver to pause or resume an RTP stream (see also Figure x1).

Figure x1 – RTP stream Pause and Resume as feedback-based control loop
(NOTE: the "P/R-signalling"-path is highlighted in red)

Clause 8 provides functionality for an MG to act autonomously or for it to co-ordinate with the MGC to implement RTP stream pause/resume. Furthermore it allows the specification of which pause CCM messages are used (NOTE: there are configuration options in order to support partial implementations, see clause 8.6.1.1).

## 7.3	Apply to individual sources

Clause 4.3/[IETF STREAM PAUSE] indicates that the pause CCM messages relate to a single RTP stream, i.e. relates to a single synchronization source identifier (SSRC). This usually relates to a single H.248 stream. An RTP session may have multiple RTP streams. A single SDP "m="-line typically describes an RTP session. However where media multiplexing is used (e.g. as described by [IETF BUNDLE]) multiple SDP "m="-lines can describe an RTP session. Thus a H.248 Stream using a SDP "m="-line may describe one or more RTP streams. [IETF RFC 5576] provides attributes for source specific parameters.

This has implications for Signals and Events related to pause and resume on H.248 streams. In order for any Signals and Events to be correctly scoped where a single SDP "m="-line describes multiple sources the Signal or Event must be able to reference the source. Clause 8 allows the SSRC to be associated with pause / resume related Signals and Events. The MGC shall include the ssrc parameter in rempr Package Signals when there are multiple SSRCs in a H.248 Stream to uniquely identify the RTP stream instance.

Alternatively the MGC may consider alternate means (such as through H.248 stream grouping [ITU-T H.248.STGROUP]) to ensure that there is a single RTP stream per H.248 stream.

{Editor’s note: The scope of the CCM messages needs to be clarified. E.g. do they relate to a particular payload type.}

## 7.4	Consensus

Clause 4.4/[IETF STREAM PAUSE] indicates that there must be consensus between RTP receivers before an RTP sender can pause a RTP stream. This requires that all RTP receivers be aware of the current RTP stream state (i.e. paused or active (termed playing in clause 6/[IETF STREAM PAUSE])) and that all receivers be aware of the pause request. Relevant example network use cases with H.248 media gateways are indicated in Appendix I.4. As described in clause 5.2/[IETF STREAM PAUSE] the RTP sender should delay the pausing of a stream for a wait period. As the gateway acts as an RTP sender it should ensure that pausing meets these requirements. If MG autonomous response is indicated, the MG shall be responsible for maintaining the wait period. If MGC controlled mode is indicated, the MGC shall be responsible for maintaining the wait period.

If a new RTP receiver that supports pause messages is added whilst an RTP stream is in the paused state, the gateway shall send it a PAUSED indication as per clause 5.3/[IETF STREAM PAUSE]. The MG is responsible for sending the PAUSED indication. In the case of MGC controlled mode the MG shall only send a PAUSED indication if the MGC had previously sent a “Local Pause” (rempr/lpause) Signal for the particular RTP stream.

## 7.5	Message acknowledgements

As described in clause 4.5/[IETF STREAM PAUSE] the RTP and RTCP do not guarantee reliable data transmission, meaning that the PAUSE, RESUME, PAUSED and REFUSED messages may be lost. H.248 MGCs and MGs should ensure that they follow the procedures in [IETF STREAM PAUSE] with respect to sending PAUSED and REFUSED and detecting paused and resumed states. Clause 8 provides Events for detecting when a RTP media stream has been paused or resumed to allow co-ordination between MGC and MG.

## 7.6	Retransmitting Requests

As per clause 4.6/[IETF STREAM PAUSE], PAUSE and RESUME requests sent from a H.248 gateway (as RTP receiver) to an RTP stream sender may result in no response (either the receipt of a PAUSED or REFUSED, or media action (pause/resume)) being detected from the sender. The gateway may assume that the PAUSE or RESUME was lost and thus may need to retransmit the message. Depending on the configuration the MGC or MG is responsible for the retransmission once the timeout period is met. If MG autonomous response is indicated, the MG shall be responsible for retransmitting the messages. If MGC controlled mode is indicated, the MGC shall be responsible for retransmitting the message (i.e. by sending a new signal to the MG indicating pause or resume).

## 7.7	Sequence Numbering

Clause 4.7/[IETF STREAM PAUSE] indicates that PAUSE request messages have a sequence number associated with them. This number is then referenced by the other pause CCM messages. As the pause identity is updated for each PAUSE request the gateway needs to manage the Pause identities (i.e., parameter PauseID in the Feedback Control Information field). If MG autonomous mode is indicated, then the MG shall be responsible for assigning the identities. If MGC controlled mode is indicated, then the MGC shall be responsible for assigning the identities.

[End Proposed Correction]

Change proposal #8:

[Begin Proposed Correction]

Appendix I

Network use cases with H.248 media gateways in the end-to-end RTP path with Pause/Resume signaling

(This appendix does not form an integral part of this Recommendation.)

## I.1	Introduction

Clause 3/[IETF STREAM PAUSE] describes the main use cases for RTP sessions with usage of RTP stream pause and resume. The example use cases consider the unidirectional RTP traffic flow between two or more RTP endpoints, without or with additional RTP nodes in between. Such interim RTP nodes realize a specific RTP topology function, which basically impact the variation of RTP pause/resume scenarios. This Appendix provides a mapping of the clause 3/[IETF STREAM PAUSE] use cases on H.248 gateways, i.e., when an H.248 media gateway provides an RTP endpoint and an associated RTP topology function. The result of that analysis should be a list of example reference network configurations with its characteristic usage of RTP stream pause/resume in a particular H.248 gateway (profile).

The H.248 use cases might be classified in H.248 context models with a single (clause I.2), two (clause I.3) or more than two (clause I.4) H.248 RTP-enabled terminations.

All use cases illustrate only one traffic direction (of a uni- or bidirectional RTP session) due to the asymmetric nature of RTP stream pause / resume between RTP sender(s) and RTP receiver(s).

## I.2	Single RTP termination

Relates to H.248 connection models with a single RTP-enabled termination per H.248 context. For instance all variants of H.248 physical-to-RTP media gateway types within PSTN emulation subsystems. The H.248 RTP termination represents an RTP endpoint, i.e., all RTCP packet types are terminated (inclusive RTCP FB) and there are not any context-internal P/R-signalling aspects as opposed to subsequent use case discussions of following clauses I.3 and I4.

## I.3	Dual RTP terminations

The notion of "dual RTP terminations" relates to an H.248 context with two H.248 RTP-enabled terminations (which are labelled as T1 and T2) in the diagrams below. Clause 3.1/[IETF STREAM PAUSE] provides use cases for a "Point-to-Point" RTP topology, from an end-to-end communication perspective. Such an end-to-end (RTP)-connection might be routed via an interim H.248 IP-to-IP media gateway, which again could be enabled for multiple different "RTP topology" modes, such as an RTP media translator, RTP transport translator or Back-to-Back RTP endsystem topology (see [b-ITU-T H.248.88]). This clause lists some use cases for such an H.248 connection model, but without discussion of the applied RTP topology by the (T1, T2)-context configuration.

Figure I.1 illustrates two use cases, termed as "Point-to-Point Downstream" and "Point-to-Point Upstream":

NOTE – The legend applies to all Figures in this Appendix.

Figure I.1 – Use-cases #1 "Point-to-Point Downstream" & #2 "Point-to-Point Upstream"

NOTE to the remote RTP nodes (from H.248 MG perspective): Node Y is a pure RTP sink. Node X represents an optional, virtual RTP source (from MG perspective) because the real RTP source X' might be not visible from MG perspective. There are RTCP-based services where the real RTP source X' might become visible, such as in case of RTCP XR based performance monitoring (see [b-ITU-T H.248.48, H.248.87].

Once, the H.248 MG provides support of RTP stream pause/resume only in outgoing (#1) or in incoming (#2) RTP traffic direction. The P/R-signalling related RTCP packet types needs to be terminated by the correspondent H.248 terminations (and not context-internally forwarded).

Use cases #1 and #2 might be requested for parallel support, leading to configurations as outlined by Figure I.2:

Figure I.2 – Use-cases #3 "Point-to-Point Down- and Up-stream"

There are again multiple options: firstly, the P/R-signalling loops might be fully decoupled (#3a) or coupled (#3b). It has to be emphasized that the P/R-signalling loops were intentionally established with a local scope (i.e., X-to-T1 and T2-to-Y) and not as a signal loop with a global scope (i.e., end-to-end loop between X and Y). The decoupled mode (#3a) could lead to situations of a paused RTP source T2 but a still playing RTP source X, which leads to the question whether the MG should buffer or discard RTP packets received at T1? That question is not further elaborated in this Recommendation because conditional, e.g., dependent whether parallel congestion controls would be in use (e.g., such as [b-ITU-T H.248.82]).

Use case #3b reflects the "native RTP stream pause and resume" case where the "RTP sink" requests to stop the RTP send process at all, which means that the actual RTP source should be switched to pause mode (i.e., X or even X' and not T2). Hence, the two P/R-signalling loops needs to be coupled. There are two coupling options: either indirectly via the MGC or directly MG-internally.

{Editor's note (2015-06): the MGC method could be already supported by applying the rempr package at both terminations T1 and T2. The MG method is not yet supported …}

Finally, there might be a single, end-to-end P/R-signalling loop across the H.248 MG, see Figure I.3:

Figure I.3 – Use-cases #4 "Point-to-Point transparent"

The "end-to-end" RTP stream pause and resume scope implies that neither termination T1 nor T2 provides an "RTP sender" or "RTP receiver" role. P/R-signalling information needs to be transparently forwarded by the MG. Such a forwarding requirement for dedicted RTCP packet types needs to be taking into account by a suitable RTP toplogy (see [b-ITU-T H.248.88]).

## I.4	Multiple RTP terminations

Such a use case would be the result when an H.248 media gateway would realize the "RTP mixer" function in all examples of clauses 3.2 to 3.5 in [IETF STREAM PAUSE].

NOTE – Another multipoint configuration is the (RTP) relay topology in [IETF STREAM PAUSE], which represents a "RTP translator with only unicast paths" according to [b-IETF rtp-topologies-update].

Figure I.4 illustrates a general multipoint configuration with enabled P/R-signaling between all RTP endpoints:

Figure I.4 – Use-case #5 "Multipoint-to-Multipoint" (with six RTP participants)

The MG behaviour (in use case #5) with respect to P/R-signalling handling is similar as in use case #3, apart from the additional complexity that the MG-internal P/R-signalling interworking effort correlates with the applied mixer function for a multiparty, multimedia communication service. Three specific multipoint use case examples are discussed in more detail below.

Figure I.5 illustrates a configuration for three RTP participants where the downstream RTP sink Y1 gets the RTP traffic forwarded from one-out-of-two upstream RTP sources X1 or X2:

Figure I.5 – Use-case #6 "Dualpoint-to-Point switching" (with three RTP participants)

Such a use case is e.g. discussed in clause 10.3/[IETF STREAM PAUSE] ("voice activated mixer"). The two self-contained P/R-signalling loops X1-to-T1,1 and X2-to-T1,2 needs to be coupled by the H.248 gateway due to the applied service configuration.

Figure I.6 illustrates a configuration between a sender X1 and two receivers Y1 and Y2 and an interim H.248 MG in RTP transport relay topology (see also Figure 18/[IETF STREAM PAUSE]):

Figure I.6 – Use-case #7 "Point-to-Dualpoint copying" (with three RTP participants) 
and a "P/R-signalling active MG"

Both receivers Y1 and Y2 might request the single sender X1 to pause the RTP playout process, hence there are inherent interactions between the concurrent P/R-signalling loops. Use case #7 represents a network configuration where the H.248 gateway resolves such interactions. Use case #8 (Figure I.7) shows the same end-to-end scenario, but with a "dumb H.248 gateway" because the MG is enforced for "transparently forwarding of all RTCP FB "P/R signalling messages".

Figure I.7 – Use-case #8 "Point-to-Dualpoint copying" (with three RTP participants) 
and a "P/R-signalling passive MG"

In summary, the H.248 MG provides the same RTP topology in use cases #7 and #8 (to be confirmed), but once an H.248 profile with and without H.248 rempr package usage.

[End Proposed Correction]

Change proposal #9:

[Begin Proposed Correction]

Bibliography (placeholder)

[b-ITU-T H.248.82]	###

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

[b-IETF rtp-topologies-update]	IETF RFC ### (###), RTP Topologies.
URL: http://tools.ietf.org/wg/avtcore/draft-ietf-avtcore-rtp-topologies-update/

[End Proposed Correction]

____________________