| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4701 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.CLOUD: new Appendix II related to an example classification of low-level gateway functions |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a new Appendix II related to an example classification of low-level gateway functions.

1	Introduction

This contribution is related to clause 7.2 "Classification of H.248 packet gateway functions", see update proposal by parallel contribution AVD-4700.

2	Discussion

Attached change proposal related to a new Appendix II provides the results of our analysis and an example classification.

How did we came to that proposal? We did firstly consider some existingH.248 profile level work, such as

- 3GPP-defined packet gateways

  - Any useful structure in functional areas at the level of stage 2 ("functional (= signalling) requirements")?

  - Any useful structure at stage 3 level, i.e., the structure of call-dependent procedures in H.248 profiles?

- ITU-T Y.Sup2 "Session border gateways"

  - List of functional areas for the media path of such an SBG

- MultiService Forum "Session border gateway"

  - List of functional areas for data-path SBGs (= H.248 MG level)

  - List of functional areas for signaling-path SBGs (= H.248 MGC level)

  - Separation in various network level functions (customer edge, network edge, network core)

- ETSI TISPAN defined packet gateways ("border gateways")

  - H.248 Ia profiles provides a number of detailed functional areas due to embedded stage 2 type of information before the actual H.248 profile sections.

  - Ex living list of Ia profile version 4 and 5 feature candidates.

That analysis did lead to a proposal (see AVD-4700) with respect to

- classification method

- classification criteria and

- top level functional areas and

- examples list of functions at detailed level (see new proposed Appendix II).

Again, the whole purpose of such an evaluation is to figure out how existing H.248 packet gateway might be impacted by cloudification, i.e., what kind of functional transformations might be considered.

We are tending to following three classification criteria:

- aspect "network topology": criteria CT with values TD and TI (for topology-dependent and topology-independent);

- aspect "traffic aggregate": criteria CF with values values FD and FI (for flow-dependent and flow-independent); and

- aspect "protocol layer": criteria CL with values of type Lx.

There are different weights behind each criteria. The top priority criteria seems to be related to network topology due to its direct relation to cloudification (of networks).

3	Analysis

New proposed Appendix II provides a list of example functions plus a proposed rating of each function according to above three criteria. We'd like to provide a rough analysis of that data here, however, this is for information only due to the subjective character of the underlying proposal at this stage.

Table 1 provides the quantitative results related to the topology aspect. The underlying question again is the following: a pre-cloud H.248 packet gateways supports a set of TD and a set of TI functions, - and what might happen when transforming such a gateway due to cloudification? And one cloudification option might be the separation in TD- and TI-specific H.248 cloud gateway types …

Table 1 – Analysis "Topology aspect"

| Graphical representation | Evaluation: |
| --- | --- |
|  | Comments: |
|  | - the portion of topology-dependent border gateway network functions is relatively high; - however, the "topology aspect" needs to be revisited at a closer look when designing and engineering cloud-based communication services; - thus, an existing "TD" function might be possibly softened to a "TI" property (i.e., "soft TD" / "hard TD"); - consequently, portion TD of ca 60% might be reduced to X%? But X will be not/never zero, or? |
|  | Conclusions, observations, questions: |
|  | - importance of "network protection / demarcation" to cloud operators? - model for "cloud-to-cloud peering"? (see also Y.3511 "Framework of inter-cloud computing") - what are the "topology-top-features"? - security aspect "network protection"? - security aspect "user info hiding"? - aspect of SLA assurance (i.e. QoS /GoS …)? |

Table 2 provides the rough summary of the flow aspect:

Table 2 – Analysis "Flow aspect"

| Graphical representation | Evaluation: |
| --- | --- |
|  | Comments: |
|  | - domination of "FD" is actually no surprise due to selected scope on "H.248 controlled" (i.e., call-/flow-individual) network functions |
|  | Conclusions, observations, questions: |
|  | - flow-independent (FI) functions need to classified in two further categories - one category might remain in an H.248 based entity … - … and one category might be subject of delegation to other cloud entities… |

Table 3 summarize the results of the protocol layer related categorization:

Table 3 – Analysis "Protocol layer aspect"

| Graphical representation | Evaluation: |
| --- | --- |
|  | Comments: |
|  | - layer-independent function: usually located in the "gateway control path", i.e., the service control logic of an MG; - lower-layers (L2 & L3): portion is actually not surprising because an "H.248 border gateway" does not represent legacy Ethernet switch or IP (service) router devices, etc; - upper-layers (L3+): the usual focus of "media aware / transport aware" BGW functionality |
|  | Conclusions, observations, questions: |
|  | - analysis confirms current positioning in (standardized) network solutions - however, there might be a shift when considering H.248 gateway usage options in cloud-based network solutions … (ffs) |

4	Summary

We'd like to suggest to add below new proposed Appendix II.

Above analysis in Tables 1 to 3 provides a rought quantitative orientation from perspective of the selected classification criteria. This is for discussion and still premature, hence where are not yet any concrete change proposals at this stage.

We need to revisit such kind of information when discussing

- concrete cloudification use cases ( clause 8/H.248.CLOUD) and

- possible impact on H.248 "packet gateway" profiles ( clause 10/H.248.CLOUD).

5	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4664 as input draft to this meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix II

Example classification of packet gateway functions

(This appendix does not form an integral part of this Recommendation.)

## II.1	Motivation

This Appendix provides an inventory of self-contained network functions, which might be used as baseline for the cloudification of H.248 packet gateways. The inventory is just an example and reflects the state-of-the-art of H.248 profile elements as defined so far for various packet gateway types. The classification scheme applies the identification criteria as introduced in clause 7.3.2, which covers aspects related to network topology, traffic aggregates and protocol layer/stack.

Any real-world cloudification of H.248 packet gateways would typically consider additional criteria, such as related to commercial aspects ("business values/priorities related to service delivery") or cost factors related to the development, deployment and operation. Such kind of criteria are out of scope of this Recommendation.

# II.2	Inventory of self-contained network functions (example)

The following tables contain three columns related to the three proposed classification criteria (in clause 7.3.2.3):

- aspect "network topology": criteria CT with values TD and TI (for topology-dependent and topology-independent);

- aspect "traffic aggregate": criteria CF with values values FD and FI (for flow-dependent and flow-independent); and

- aspect "protocol layer": criteria CL with values of type Lx.

Disclaimer: the following inventory represents just an example, is non-exhaustive and not obligatory with respect to the proposed ratings.

Clause 7.3.3 introduced a proposal of twelve top level areas, which are covered by individual Tables II.x (with x as index of the functional area in Table 2):

Table II.1 – Functional area "Service quality support"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Ethernet priority tagging/untagging | L2 | TD | FD | H.248.54 vlan package |
| 1.2 | Ethernet priority-based handling (forwarding) | L2 | TD | FD |  |
| 1.3 | IP QoS class tagging | L3 | TD | FD | H.248.52 qos package |
| 1.4 | IP DiffServ pre-marker | L3 | TD | FD | H.248.52 ds package |
| 1.5 | IP DiffServ per-hop behaviour | L3 | TD | FD | - |
| 1.6 | IP DiffServ translator | L3 | TD | FD | H.248.52 ds package |
| 1.7 | Bearer connection admission control | L3 | TD | FD |  |
| 1.8 | IP byterate policing (H.248 flow level) | L3 | TD | FD | H.248.53 tman v2 package |
| 1.9 | IP byterate policing (H.248 stream level) | L3 | TD | FD | H.248.53 tman v1 package |
| 1.10 | IP byterate policing (H.248 termination level) | L3 | TD | FD | H.248.53 tman v1 package |
| 1.11 | IP packet size policing (H.248 ... level) | L3 | TD | FD | H.248.53 pacs package |
| 1.12 | IP byterate shaping | L3 | TD | FD | H.248.Shaper |
| 1.13 | IP packet-rate shaping | L3 | TD | FD | H.248.Shaper |
| 1.14 | IP resource reservation (RSVP support) | L3 | TD | FD | H.248.65 rsvp package |
| 1.15 | Path MTU support (conversion) | L3 | TD | FD |  |
| 1.16 | Excplicit congestion notification (ECN) | L3 | TD | FD | H.248.82 |
| 1.17 | IPDV (jitter) reduction (between two IP domains) | L3 | TD | FD | G.799.3 |
| 1.18 | IPDV (jitter) elimination (between two IP domains) | L3 | TD | FD | G.799.3 |
| 1.19 | RTP PDV (jitter) reduction (between two RTP domains; RTPTT topology) | L4+ | TD | FD | H.248.31 ajb package (tbc) |
| 1.20 | RTP PDV (jitter) elimination (between two RTP domains; B2BRE topology) | L4+ | TD | FD | H.248.31 ajb package (tbc) |
| 1.21 | Rate control | L4+ | TD | FD |  |
| 1.22 | Echo control (acoustic echo suppression) | L7 | TD | FD | Q.115.0 SPNE control package |
| 1.23 | Audio format conversion (transcoding) | L7 | TI | FD |  |
| 1.24 | Audio signal detection | L7 | TI | FD | DTMF, etc |
| 1.25 | Audio signal insertion (e.g.,announcements) | L7 | TI | FD |  |
| 1.26 | Audio signal optimization (e.g., gain control, removal of digits and tones) | L7 | TI | FD | …, H.248.68 |
| 1.27 | Video format conversion (transrating, transsizing, transorientation, transcoding, ...) | L7 | TI | FD |  |
| 1.28 | Text format conversion (transcoding) | L7 | TI | FD |  |
| 1.29 | Text transport translation | L4+ | TI | FD | Text-over-T.140/RTP to other |
| 1.30 | Voiceband data services support (generic) | L7 | TI | FD | V.152 |
| 1.31 | Fax: packet relay (T.38) to pass-through (V.152) | L7 | TI | FD | V.153 |
| 1.32 | Emergency service tagging | — | TI | FD | H.248 ContextAttribute |
| 1.33 | Service priority | — | TI | FD | H.248 ContextAttribute |
| 1.34 | Service precedence and pre-emption | — | TI | FD | H.248.44 prectn package |
| 1.35 | Pull mode of "QoS policy rules" | L3 | TI | FD | H.248.55 plm package |
| 1.36 | QoS-guaranteed connectivity | L3 | TD | FD |  |
| 1.37 | … |  |  |  |  |

Table II.2 – Functional area "End-to-end connectivity support"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 2.1 | IP domain indication | L3 | TD | FD | H.248.41 ipdc package |
| 2.2 | MPLS LSP support | L2 | TD | FD | H.248.54 mpls package |
| 2.3 | RTCP port allocation (methods) | L4 | TD | FD | H.248.57 rtcph package |
| 2.4 | L3/L4 NAT-T: Latching remote source transport address | L3 | TD | FD | H.248.37 ipnapt package |
| 2.5 | L3/L4 NAT-T: TCP merge mode | L4 | TD | FD | H.248.84 |
| 2.6 | L4+ NAT-T: B-ALG (Bearer level Application Gateway) | L7 | TD | FD | H.248.78 |
| 2.7 | L3/L4 NAT-T: ICE/STUN/TURN support | L3 | TD | FD | H.248.50 |
| 2.8 | L3/L4 transport connection: keep alive | L3 | TD | FD | H.248.50 |
| 2.9 | RTP topology control / RTCP handling | L4+ | TD | FD | H.248.88 |
| 2.10 | Interworking tunneled-to-untunneled user traffic | L4+ | TD | FD |  |
| 2.11 | Mux/Demux of user traffic (transport level) | L4 | TD | FD | H.248.STGROUP |
| 2.12 | Mux/Demux of user traffic (L4+ level) | L4+ | TD | FD | H.248.RTPMUX |
| 2.13 | Internet Control Message Protocol (ICMP) | L3 | TD | FD |  |
| 2.14 | Local IP router support | L3 | TD | FI |  |
| 2.15 | … |  |  |  |  |

Table II.3 – Functional area "End user information hiding / Topology hiding"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 3.1 | Virtual LAN (VLAN) IDentifier (VID) translation | L2 | TD | FD |  |
| 3.2 | NAT44 | L3 | TD | FD |  |
| 3.3 | NAT46 | L3 | TD | FD |  |
| 3.4 | NPT66 | L3 | TD | FD |  |
| 3.5 | IP router mode ("topology unhiding") | L3 | TD | FD | H.248.64 |
| 3.6 | Topology hiding for signalling path | L3 | TD | FD |  |
| 3.7 | … |  |  |  |  |

Table II.4 – Functional area "Special communication services (supplementary, regulated, operational)"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 4.1 | Lawful interception | L3 | TD | FD | H.248.60 ("TD in past, TI in future?") |
| 4.2 | GETS | — | TI | FD |  |
| 4.3 | Emergency | — | TI | FD |  |
| 4.4 | Supplementary (e.g., call transfer, etc) | — | TI | FD |  |
| 4.5 | … |  |  |  |  |

Table II.5 – Functional area "Decomposition of application-specific network nodes"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 5.1 | Media server / Conference server | — | TI | FD | H.248.9, H.248.73, H.248.74 |
| 5.2 | Streaming server (RTSP based) | — | TI | FD | H.248.66 |
| 5.3 | Instant messaging server (MSRP switch) | — | TI | FD | H.248.69 |
| 5.4 | WebRTC gateway | — | TI | FD | H.248.WebRTC |
| 5.5 | Session independent policing | — | TD | FI |  |
| 5.6 | Access transfer gateway (mobile SRVCC) | — | TD | FD |  |
| 5.7 | … |  |  |  |  |

Table II.6 – Functional area " Performance monitoring "

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 6.1 | L3/L4 NAT-T: latching statistics | L3 | TD | FD | H.248.37 lstat package |
| 6.2 | IP traffic policing | L3 | TD | FD | H.248.53 tmanr package |
| 6.3 | Network-level transport capacity usage (MP) | L3 | TI | FD | Hundreds of H.248 statistics; H.248.30 ("premature RTCP XR"); H.248.48 ("general RTCP XR"); H.248.58; H.248.61; H.248.71; H.248.87 ("Montoring framework for RTP traffic") |
| 6.4 | Network-level GoS (Grade of Service) (MP) | L3 | TD | FD |  |
| 6.5 | Application-level transport capacity usage (MP) | L7 | TI | FD |  |
| 6.6 | Application-level GoS (Grade of Service) (MP) | L7 | TD | FD |  |
| 6.7 | Network-level transport capacity usage (RP_RTCP) | L4+ | TI | FD |  |
| 6.8 | Network-level GoS (Grade of Service) (RP_RTCP) | L4+ | TI | FD |  |
| 6.9 | Application-level transport capacity usage (RP_RTCP) | L4+ | TI | FD |  |
| 6.10 | Application-level GoS (Grade of Service) (RP_RTCP) | L4+ | TI | FD |  |
| 6.11 | Network-level transport capacity usage (FP_RTCP) | L4+ | TD | FD |  |
| 6.12 | Network-level GoS (Grade of Service) (FP_RTCP) | L4+ | TD | FD |  |
| 6.13 | Application-level transport capacity usage (FP_RTCP) | L4+ | TD | FD |  |
| 6.14 | Application-level GoS (Grade of Service) (FP_RTCP) | L4+ | TD | FD |  |
| 6.15 | Network-level loopback point (LP) | L3 | TD | FD |  |
| 6.16 | Application-level loopback point (LP) | L4+ | TD | FD |  |
| 6.17 | Support of sponsered data services | L3 | TI | FD |  |
| 6.18 | Call tracing | L3 | TI | FD | 3GPP H.248 trace package |
| 6.19 | … |  |  |  |  |

Table II.7 – Functional area "Security area: support of privacy, integrity and confidentiality of user data"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 7.1 | Network-level encryption: IPsec | L3 | TD | FD | none yet defined |
| 7.2 | Transport-level encryption: TLS, DTLS | L4 | TD | FD | H.248.90, H.248.93 |
| 7.3 | Application level framing protocol encryption: SRTP | L4+ | TD | FD | H.248.77 |
| 7.4 | Application level encryption: [-] | L7 | TD | FD | none yet defined |
| 7.5 | Trust models: "e2ae", "e2e" | L4 | TD | FD | 3GPP H.248 profiles |
| 7.6 | Digital watermarking | L7 | TD | FD |  |
| 7.7 | … |  |  |  |  |

Table II.8 – Functional area "Security area: support of network protection"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 8.1 | Gate control | L3 | TD | FD | H.248 Context configuration |
| 8.2 | Incoming network transport address filtering | L3 | TD | FD | H.248.43 gm package |
| 8.3 | Incoming L4+ protocol filtering | L3 | TD | FD | H.248.43 ipf package |
| 8.4 | Outgoing network transport address filtering | L3 | TD | FD | H.248.43 dapf package |
| 8.5 | Outgoing L4+ protocol filtering | L3 | TD | FD | H.248.43 opf package |
| 8.6 | Application control protocol (e.g. SIP) filtering | L7 | TD | FI |  |
| 8.7 | … |  |  |  |  |

Table II.9 – Functional area "Load control & overload protection"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 9.1 | MG node overload control | — | TI | FI | H.248.11 (& H.248.10) |
| 9.2 | MG detailed congestion reporting | — | TI | FI | H.248.32 dcr package |
| 9.3 | MG conditional reporting call-specific metrics | — | TD | FD | H.248.47 (TI or TD, … metric type ...) |
| 9.4 | MGC node overload protection by MG | — | TI | FI | H.248.1 Shaping of notifications |
| 9.5 | … |  |  |  |  |

Table II.10 – Functional area "Service reliability"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 10.1 | Virtual Router Redundancy Protocol (VRRP) | — | TI | FI |  |
| 10.2 | Hanging bearer connection endpoints | — | TI | FD | H.248.36 hangterm package |
| 10.3 | Application data inactivity detection | L3 | TI | FD | H.248.40 adid package |
| 10.4 | IP realm availability detection | L3 | TD | FD | H.248.41 ipra package |
| 10.5 | Remote data storage for MGC (wrt changeover procedures) | — | TI | FI | H.248.45 mgcinfo package |
| 10.6 | Geographical redundancy | — | TD | FI |  |
| 10.7 | Stable bearer preservation | — | TI | FD |  |
| 10.8 | Failover between primary & secondary MGC | — | TI | FI | H.248.1 ServiceChange support |
| 10.9 | … |  |  |  |  |

Table II.11 – Functional area "Virtualization support"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 11.1 | Virtual MGC | — | TI | FI |  |
| 11.2 | Virtual MGC | — | TI | FI |  |
| 11.3 | Platform-as-a-Service (PaaS) support | — | TI | FI | H.248.83 mgi (Media Gateway Instance) package |
| 11.4 | Network-as-a-Service (NaaS) support | — | TI | FI | NaaS impact still open |
| 4.5 | … |  |  |  |  |

Table II.12 – Functional area "Operational services"

| No | Network (Sub-)Function | CL | CT | CF | H.248 area |
| --- | --- | --- | --- | --- | --- |
| 12.1 | Time service (for subsequent network domain) | — | TI | FI | H.Sup8 |
| 12.2 | Clock service (for subsequent network domain) | — | TI | FI | Framework: G.8261 (Timing and synchronization aspects in packet networks) |
| 12.3 | MG internal resource optimization (bearer connection view) | — | TI | FD | H.248.46 ccc package |
| 12.4 | MG internal resource optimization (context view) | — | TI | FD | H.248.63 rmr, ... packages |
| 12.5 | MG internal interlinkage of multiple stream endpoints | — | TI | FD | H.248.92 seplink package |
| 12.6 | Traffic tracing of individual calls (diagnosis) | — | TI | FD | 29.232 calltrace package |
| 12.7 | Media recording | — | TI | FD | H.248.SIPREC |
| 12.8 | Billing/Accounting/Charging (call individual) | — | TI | FD | [mainly via H.248 statistics] |
| 12.9 | Quality alert ceasing | — | TD | FD | H.248.13 qac package |
| 12.10 | Coexistence with legacy network services and functions | — | TD | FD |  |
| 12.11 | Data locality | — | TD | FD |  |
| 12.12 | … |  |  |  |  |

[End Proposed Correction]

____________________