| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4706 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.STGROUP: Cascaded aggregation and de-aggregation streams |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution discusses cascading aggregation and de-aggregation streams.

# 2. Discussion

At the February Q3/16 meeting there was a proposal in C.734 to add text to H.248.STGROUP regarding the interaction between aggregation and de-aggregation streams. During discussions it was agreed further work was needed to be done in this area. The discussions were recorded as an Editor’s note to clause 8.6.7 on “Interaction of stream grouping methods "aggregation" and "de-aggregation".

As noted by the editor’s note, the current model presents the following difficulty for such a cascading: components streams of a de-aggregation stream are internal to the context, they don’t’ exist externally whereas components of an aggregation stream are external to the context. A de-aggregation stream represents an external media stream. As such the component streams of the de-aggregation are a representation of part of the external media stream.

In order to progress the topic the Contributors have analysed the possible “cascades” and have determined 4 theoretically possible combinations for a stream:

- De-aggregation to de-aggregation

- De-aggregation to aggregation

- Aggregation to de-aggregation

- Aggregation to aggregation

These are analysed below.

## 2.1 De-aggregation to de-aggregation

In this combination an incoming data stream is essentially de-multiplexed multiple times as it enters the H.248 Context. This is illustrated in figure 1 below.

Figure 1 – De-aggregation component to de-aggregation stream cascade

A possible use case would be where DTLS/SCTP and SRTP are multiplexed over the same transport address/port. This use case is mandated by clause 3.5/[draft-ietf-rtcweb-transports-08] for WebRTC implementations, i.e.

"WebRTC implementations MUST support multiplexing of DTLS and RTP over the same port pair, as described in the DTLS_SRTP specification [RFC5764], section 5.1.2.  All application layer protocol payloads over this DTLS connection are SCTP packets."

The first de-aggregation could de-multiplex the external data stream into DTLS/SCTP and RTP component streams.  In this case the external data stream could be modelled by two H.248 Streams (i.e. one UDP/TLS/RTP/SAVPF" and one with "UDP/DTLS/SCTP” utilising the same transport address/port). A “bundle” semantic would be specified via the groupse property. However a de-aggregation stream would not be needed because of the use of multiple H.248 streams. So whilst this first “de-mux” could be modelled via a de-aggregation, practically it would be defined via another method.

A de-aggregation would be needed to de-multiplex the SCTP component stream representing the SCTP association into component streams representing the SCTP streams.

This is illustrated in figure 2 below.

Figure 2 – Multiple de-multiplexes one de-aggregation

In considering other scenarios where bundle is used it appears that these can also be handled in the above manner. Thus no practical use case is identified for the support of de-aggregation to de-aggregation cascading.

Conclusion: Whilst multiple de-aggregations are per figure 1 are theoretically possible, their use should be avoided via the use of a configuration such as those in figure 2. Therefore for the current version of the STGROUP package this configuration is not supported. A future version could introduce this configuration.

## 2.2 De-aggregation to aggregation

### 2.2.1 No cascade

This scenario builds on the scenario in clause 2.1. An external data stream is de-multiplexed to several component streams. One or more of these component streams are then used as part of a stream aggregation. This could occur where the media from the de-aggregation stream needs to be aggregated with other streams for example as part of a FID semantic. Figure 3 shows this scenario.

Figure 3 – De-aggregation to aggregation no cascade

Figure 3 shows that whilst de-aggregation and aggregation is used, they are not cascaded.

Conclusion: This is a valid configuration.

### 2.2.2 Component cascade

A slightly modified scenario is where one of the applications on an SCTP stream needs to be included in an aggregation before being forwarded to Termination Tb. This is illustrated in figure 4.

Figure 4 – De-aggregation component to aggregation cascade

This is very similar to scenario in that the same strdeagg and stragg properties are used. However the difference is that the component stream from the de-aggregation is used as part of the aggregation. Whilst theoretically possible no practical use case has been identified. The Stream aggregation semantic could be FID meaning that Sid6 would source media from Sid1, Sid4 or Sid5 however this usage is unlikely.

A more likely scenario is that the de-aggregation component is processed according to a subsequent semantic on another Termination. Figure 5 shows a scenario where a de-aggregation component  (Sid4) becomes part of a BUNDLE on Termination Tb.

Figure 5 – De-aggregation component to subsequent semantic

Conclusion: Whilst multiple de-aggregations are per figure 4 are theoretically possible, their use should be avoided via the use of a configurations such as those in figure 6. Therefore for the current version of the STGROUP package this configuration is not supported. A future version could introduce this configuration.

2.2.2 De-aggregation Stream cascade

In this scenario the de-aggregation Stream is used as an input to an aggregation Stream. Figure 6 illustrates the scenario.

Figure 6 – De-aggregation stream to aggregation cascade

Figure 6 illustrates a scenario where the de-aggregation Stream Sid1 is used as a component to the Aggregation Stream Sid5. This is theoretically possible; however none of the currently defined aggregation semantics (i.e. in clause 8.6.3/[H.248.STGROUP]) would result in such a configuration.

Conclusion: A valid configuration however no semantics are currently defined that require this configuration. Therefore for the current version of the STGROUP package this configuration is not supported. A future version could introduce this configuration.

## 2.3 Aggregation to de-aggregation

In this scenario an aggregation stream acts also acts as a de-aggregation Stream. This is illustrated in figure 7.

Ta

Tb

groupse  ((SEMANTIC1?,1,2),

(SEMANTIC2?,4,5))

Figure 7 – Aggregation to De-aggregation cascade

Given the existing semantics the Contributors have been unable to identify a scenario could result in an aggregation stream being used as a de-aggregation stream. The only currently defined de-aggregation semantic is “SCTP”. Therefore any de-aggregation stream must be SCTP related. The currently defined aggregation semantics (i.e. in clause 8.6.3/[H.248.STGROUP]) do not appear to relate to SCTP streams.

Conclusion: If an MGC tries to set such a configuration the MG shall respond with an appropriate error code.

## 2.4 Aggregation to aggregation

In this scenario the FID semantic is used to identify that one of two RTP streams (RTP1 or RTP2) will be sent at a particular time. Then either resultant stream or RTP3 is sent into the Context due to the second FID semantic.  Figure 8 shows the possible use of two aggregations to achieve this behaviour.

Figure 8 – Aggregation stream as aggregation component

The resultant aggregation of H.248 Stream Sid3 is used as a component stream to the aggregation for Sid5. However this scenario could be more simply modelled via a single aggregation as shown in figure 9.

Figure 9 – Single aggregation

Figure 5 shows the simpler implementation using a single stragg propery. There doesn’t appear to be a valid use case using existing semantics for aggregation to aggregation cascading.

Conclusion: An MGC shall not use an aggregation to aggregation component configuration when the aggregations have the same semantic. An MGC may use such a configuration where the aggregations have different semantics however none of the existing semantics warrant such a configuration.

# 3 Proposal

It is proposed to incorporate the above configurations and conclusions based on discussions at the meeting into H.248.STGROUP. No specific text changes have been proposed. Any changes as a result of the discussion would be to clause 8.6.7/H.248.STGROUP.

__________________________

____________________