| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4710 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-AV: Simulcast |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution discusses the editor’s note on simulcast.

2.	Discussion

H.TPS-SIG clause 7.1.1.13.2 “Simulcast” currently contains the following Editor’s note:

{Editor’s Note: The CLUE WG has decided to utilise the mechanism defined in <draft-ietf-mmusic-sdp-simulcast> “Using Simulcast in SDP and RTP Session”. Contributions to H.TPS-AV and/or H.TPS-SIG are solicited on mechanisms to support simulcast in H.323}

The editor’s note indicates that the CLUE WG has decided to use the simulcast method in <draft-ietf-mmusic-sdp-simulcast>. However this was with respect to the decision to remove the maximum captures attribute. The CLUE WG has taken the approach that the definition of CLUE should be independent from the underlying transport issues. For example CLUE does not define procedures for RTP multiplexing. Likewise CLUE has not described any procedures for simulcast. The simulcast draft draft-ietf-mmusic-sdp-simulcast is not mentioned in the CLUE framework, protocol or signalling drafts.

Whilst H.323 systems support the simultaneous capabilities mechanism there is little discussion regarding simulcast from a single media source other than H.323 Annex B on the support of layered encodings. Further development would have to be done in Q2/16 to document a general purpose simulcast mechanism into H.323. This may be non-trivial given the IETFs decision to utilise a single m-line to represent multiple simulcast streams (unified plan).

Therefore like the CLUE drafts the Contributors believe that H.TPS-Arch should not provide any detailed information or procedures on how simulcast is achieved in H.323 systems other than to note the state of simulcast support in H.323 today. The deletion of the editor’s note and additional text for the supplementary information is proposed.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 247R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

Draft new ITU-T H.??? (ex H.TPS-AV)

Audio/video parameters for telepresence systems

…

2 References

…

[IETF CLUE SIG]	IETF draft-ietf-clue-signaling-05, CLUE Signaling.

[IETF CLUE DM]	IETF draft-ietf-clue-data-model-schema-08, An XML schema for the CLUE Data Model.

[IETF CLUE FW]	IETF draft-ietf-clue-framework-22, Framework for Telepresence Multi-Streams.

…

7.1.1.13.2 Simulcast

| Identity: | simulcast |
| --- | --- |
| Description: | Telepresence systems may provide multiple encodings for the one capture through a technique known as simulcast. For example, this may be achieved by sending multiple video coding streams with different characteristics to allow a receiving endpoint to choose the stream that meets its needs. |
| Format: | - |
| Possible Values: | - |
| Default: | - |
| Reference: | - |
| Signalling Mechanism: | Bearer / media control signalling |
| Supplementary Information: | [ITU-T H.323] defines the simultaneous capabilities mechanism that allows endpoints to indicate which capabilities they can simultaneously decode. However it does not describe how this mechanism is used to represent a single video source with multiple RTP streams other than in[ITU-T H.323 Annex B] which discusses procedures for the use of layered video codecs. |

__________________________

____________________