| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4711 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-SIG: Extended audio, video and data capabilities |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution discusses the support of extended audio, video and data capabilities in H.TPS-SIG.

2.	Discussion

There is an Editor’s note in clause 8.5 regarding extended capabilities, i.e.

{Editor’s Note:  extendedAudioCapability needs to be added to H.245. Real Time text also needs to  be carried, so we should ensure that extended capability is fully generalized.}

ExtendedAudioCapability, ExtendedVideoCapability and ExtendedDataCapability have already been added to the latest draft H.245 (TD 223r1/WP1 from the February Q5/16 Rapporteurs meeting). Thus the extended capability is fully generalised. Indication of the use of real time text is carried via the data application capability. This is not currently mentioned in H.TPS-SIG therefore it is propose to add some text regarding this. If this text is accepted the Editor’s note can be deleted.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 226R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

8.5 Open Logical Channel

After a CLUE Configure is received, the provider opens logical channels for media transport. Each logical channel’s dataType shall carry the encodingID to identify the particular Capture, using the extendedVideoCapability, extendedAudioCapability and extendedDataCapability (for real time text) structures in H.245.

The video media encoding shall be carried in the videoCapability field of the extendedVideoCapability. The audio media encoding shall be carried in the audioCapability field of the extendedAudioCapability. The text media encoding shall be carried in the dataCapability field of the extendedDataCapabilty.

The video encodingID shall be carried in the videoCapabilityExtension, the audio encodingID shall be carried in the audioCapabilityExtension and the text encodingID shall be carried in the dataCapabilityExtension.

The videoCapabilityExtension, audioCapabilityExtension and dataCapabilityExtension (for real time text) shall carry the CLUE Correlation Capability defined in Table 4 with the appropriate encodingID parameter.

…

__________________________

____________________