| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4714 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-SIG: Multipoint Control |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution proposes text for clause 9/H.TPS-SIG on multipoint control.

2.	Discussion

Currently clause 9 on multipoint control contains the following editor’s note as a place holder for normative text on security:

{Editor’s note: The intention of this section is to describe any behaviour specific to MCUs with regards to telepresence support. CLUE itself being a point to point protocol simplifies protocol handing. The support of multipoint is still an open area of discussion in CLUE, i.e. the method of support of switched and composed captures is still to be agreed.}

In order to support telepresence enabled multipoint conferences a multipoint control unit (MCU) needs to support the signalling defined in H.TPS-SIG to allow endpoints to signal their capabilities/characteristics via CLUE. The MCU may then utilise the CLUE Advertisement messages received via endpoints to create its own CLUE Advertisements. In doing so an MCU may decide only to Advertise a sub-set of capabilities or decide to transform the received characteristics into new ones. As such it is proposed to provide some further information regarding these procedures in clause 9.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 226R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

# 9 Telepresence enabled multipoint control

## 9.1 General

In addition to the characteristics defined in clause 6.7/[ITU-T H.323], telepresence enabled multipoint control units (MCU) shall support the signalling defined by this Recommendation to support the CLUE protocol for H.245 based systems. MCUs should support the parameters defined by [ITU-T H.TPS-AV]. Support of the parameter may not need signalling or may need to be signalled according to [ITU-T H.TPS-AV].

## 9.2 CLUE Support

CLUE is an endpoint to endpoint protocol. An MCU in a telepresence enabled multipoint conference may receive CLUE Advertisements from multiple endpoints. The MCU shall use these Advertisements to formulate its own Advertisements to send to endpoints. In doing so the MCU may choose to support a subset of captures from the endpoints to send in the Advertisement or it may choose to construct its own captures based on the source captures. It may add additional captures. These constructed captures may be advertised as a multipoint content capture. The procedures require that the MCU performs a mapping between Capture and Encoding identities received in CLUE Advertisements and Configures to those sent in Advertisements/Configures.

When sending Advertisements the MCU shall ensure that the appropriate capabilities/encodings are sent to the endpoints via TerminalCapabilitySet messages.

Clause 12.1.3/[IETF CLUE FW] provides an example of an MCU Advertisement.

The MCU is also required to map received CLUE Configure messages appropriately. A Configure message received from one endpoint may result in Configure messages sent to multiple endpoints.

As a result of receiving CLUE Configure messages the MCU may need to instantiate a new capture encoding via H.245 OpenLogicalChannel messaging or may need to remove a capture encoding according to clause 8.6.

__________________________

____________________