| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4719 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.235.DTLS: Terminology update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal related to terminology of DTLS.

1	Discussion

Q.3/16 discusses DTLS (and TLS) terminology since the start of work items H.248.90 (TLS) and H.248.93 (DTLS). A LS process with IETF WG TLS was initiated in order to move forward on terminology clarifications.

Progress and consolidated terms are documented in a living list, TD-266 (WP1/16) "H.248.90 (TLS), H.248.91 (Profile "TLS gateways") & H.248.93 (DTLS): Release 2 - Living list items", from Q.3/16 side.

The latest LS with the IETF may be found in COM 16 – LS 142 (http://ifa.itu.int/t/2013/ls/sg16/sp15-sg16-oLS-00142.docx ).

In order to accelerate that clarification process, a proposal was directly submitted to IETF WG TLS:

IETF draft-guballa-tls-terminology (2015), Terminology related to TLS and DTLS.
URL: https://tools.ietf.org/html/draft-guballa-tls-terminology-01

The IETF draft addresses all open issues for clarification related to terminology and proposes solutions, which are all fully consistent with the TLS and DTLS protocol specifications.

Draft H.235.DTLS might be enhanced in updating some DTLS terms.

For instance:

It was in the meanwhile confirmed from IETF side that the "DTLS association" is synonym to a "DTLS connection" (and not "DTLS session"), hence could be replaced in the text.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4672 as input draft to this meeting.

Change proposal #1:

[Begin Proposed Correction]

Draft new ITU-T Recommendation ITU-T H.235.DTLS

H.323 security: Support of DTLS for media streams

Summary

The purpose of this Recommendation is to describe the security procedures for the establishment of media streams utilising Datagram Transport Layer Security (DTLS).

# Scope

The scope of this Recommendation is to provide procedures for the establishment of DTLS [IETF RFC 6347] connections for media streams. DTLS is an evolution of the widely implemented Transport Layer Security (TLS) security protocol that allows the use of a security protocol over a datagram environment.

An important aspect of the establishment of a DTLS connection is that a fingerprint and hash is communicated via an out-of-band means and that the certificate exchange occurs within the established DTLS connection. The fingerprint is used to ensure the integrity of the certificates. DTLS also follows the TLS client-server model for establishment of the DTLS connection where one of the endpoints is responsible for the establishment of the connection. The roles (client or server) need to be negotiated between the endpoints.

In order for endpoints to communicate the fingerprint/hash and roles information, the information needs to be signalled to the peer endpoint. This Recommendation utilises [ITU-T H.245] to signal this information.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2011), Control protocol for multimedia communication

[ITU-T H.323]	Recommendation ITU-T H.323 (12/2009), Packet-based multimedia communications systems

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[IETF RFC 4301]	IETF RFC 4301 (2005), Security Architecture for the Internet Protocol

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)

[IETF RFC 5246]	IETF RFC 2246 (2005), The Transport Layer Security (TLS) Protocol Version 1.2

[IETF RFC 6083]	IETF RFC 6083 (2011), Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)

[IETF RFC 6347]	IETF RFC 6347 (2012), Datagram Transport Layer Security Version 1.2

[IETF DTLS/SCTP]	draft-ietf-tsvwg-sctp-dtls-encaps-08 (xxxx), DTLS Encapsulation of SCTP Packets (Work in progress)

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

- All DTLS related terms are based on [b-IETF tls-terms]

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DTLS | Datagram Transport Layer Security |
| --- | --- |
| IPsec | Internet Protocol Security |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| TLS | Transport Layer Security |

# Conventions

None.

# Overview

This Recommendation defines procedures based on those defined by [IETF RFC 4572], which describes “Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).”

Endpoints involved in a DTLS connection establishment (NOTE 1) indicate their identity by presenting authentication certificates as part of the DTLS handshake procedure. In order to associate the media streams with the connection and to prevent attacks, the endpoints provide a certificate fingerprint. If the presented certificate matches the received fingerprint then the endpoint who sent the fingerprint is the initiator of the connection. [IETF RFC 4572] specifies a fingerprint attribute for this purpose.

NOTE 1: Whether (or not) also a DTLS session would be established is dependent on the "tagging of resumable". There would be then a resumable DTLS session (or semi-permanent DTLS session) existing in parallel to the established DTLS connection.

[IETF RFC 4572] also defines the use of the “setup” and “connection” attributes defined by [IETF RFC 4145] to determine which of the endpoints initiate the DTLS handshake.

The support of DTLS in H.323 follows the same basic flow as described by [IETF RFC 4145]. That is a call signaling association is established between peer endpoints. The endpoints then exchange information regarding the media capabilities, transport and specific DTLS capabilities. If the endpoints agree to use DTLS then they perform a DTLS handshake (NOTE 2) to initiate and authenticate a data stream. The endpoints use the fingerprint information to verify the authentication certificates provided in the DTLS handshake. If the certificates are verified then the data flows. This is illustrated in Figure 1 below:

Figure 1 – Basic DTLS establishment flow

NOTE 2: In more detail, a "DTLS full handshake" (because there doesn't exist any previous DTLS session, which might allow a DTLS abbreviated handshake).

# Usage of DTLS

{Editor’s note: The use of DTLS to exchange SRTP keys is FFS.}

## DTLS considerations

Depending on the protocol stack that DTLS occurs in there are different considerations for its implementation. Different DTLS (?) procedures are applied based where it occurs in a stack. For example: If a DTLS connection occurs within an SCTP association (SCTP/DTLS) [IETF RFC 6083] the implementation considerations are different to when an SCTP association occurs within a DTLS connection (DTLS/SCTP) [IETF DTLS/SCTP].

Therefore the exact DTLS procedures that should be applied to the media stream are determined by the indicated protocol in the H.245 MediaTransportType or DataProtocolCapability element. Any definition of DTLS protocol in these elements shall specify a profile document that describes the operation of DTLS within that protocol stack. For example clause B.2.2.7/[ITU-T H.245] indicates that sctp-dtls is specified by [IETF RFC 6083].

These profile documents may also specify the use of the connection and setup elements and the operation of other protocols in the stack.

## DTLS Parameter Description

Information regarding the DTLS session is exchanged via the following parameters:

- hashFunction within the DTLSSecurityCapability contains the supported hash functions The value is encoded as per “hash-func” in the ABNF syntax as defined by clause 5/[IETF RFC 4572].

- setupInformation within the DTLSSecurityCapability contains the setup information used to determine which endpoint establishes to the DTLS connection as defined by clause 4/[IETF RFC 4572]. The value is encoded as per “role” in the ABNF syntax as defined by clause 4/[IETF RFC 4145].

- connectionInformation within the DTLSSecurityCapability contains the connection information used to determine whether an existing or new connection is used for the DTLS connection as defined by clause 4/[IETF RFC 4572]. The value is encoded as per “conn-value” in the ABNF syntax as defined by clause 5/[IETF RFC 4145].

{Contributor’s note: The proposed ASN.1 above contains a connectionInformation field that is meant to mirror the SDP a=connection field. The a=connection field is used to disambiguate SDP updates as described by clause 5/[IETF RFC 4145]. It is not clear that this same issue applies to the use of H.245 to establish DTLS connections. connectionInformation has been retained in the contribution to stimulate discussion.}

- fingerprint within the DTLSSecurityCapability contains the fingerprint associated with the authentication certificates.  The value is encoded as per “fingerprint” in the ABNF syntax as defined by clause 5/[IETF RFC 4572].

The use of the DTLS security capability parameters in this Recommendation is limited to two-party bidirectional media streams where each source has a unique cryptographic key; support for multicast media streams or multipoint unicast streams is for further study.

## DTLS Parameter Transport

### DTLS Capabilities

The supported hash functions, setup and connection information are carried in the dtlsSecurityCapability field of encryptionAuthenticationAndIntegrity in H235SecurityCapability of the TerminalCapabilitySet capabilityTable. The mediaCapability field of the H235SecurityCapability is associated with the relevant receive or receiveAndTransmit capabilities. The fingerprint is not sent in the TerminalCapabilitySet exchange.

### Fingerprint

The fingerprint associated with the authentication certificate of the OLC initiating endpoint is carried in the dtlsSecurityCapability field of encryptionAuthenticationAndIntegrity in h235Media of the OpenLogicalChannel forwardLogicalChannelParameters dataType field. A single hash function used for the fingerprint, which is supported by both endpoints and chosen from the hash functions  must also be present in the dtlsSecurityCapability field.

The fingerprint associated with the authentication certificate of the peer endpoint is carried in the dtlsSecurityCapability field of the OpenLogicalChannelAck. A single hash function used for the fingerprint must also be present in the dtlsSecurityCapability field.

# Procedures

The DTLS procedures described below shall only be used to negotiate security for two-party bidirectional media streams in situations where the H.245 signalling channel is used. The H.245 signalling channel may be protected by an encapsulating data-security protocol, e.g., IPsec (IETF RFC 4301), TLS (IETF RFC 5246). The exchange of DTLS crypto parameters using H.245 messages shall provide the following functions:

1)	Exchange and negotiation of DTLS capabilities: supported hash types, setup role and whether an existing connection is to be used.

2)	The exchange of fingerprints used to verify the authentication certificates used for the DTLS handshake for the channel.

## Security capability exchange

The initiating endpoint indicates its DTLS capabilities as per clause 7.3.1. The initiating endpoint provides a list of hash functions (via the hashFunction field in DTLSSecurityCapability) that it supports to encode and decode authentication certificates. The peer endpoint replies with the sets of hash functions it supports. The initiating endpoint also sends the DTLS establishment role that it supports (via the setupInformation field in DTLSSecurityCapability) as per clause 7.3.1. As per clause 4.1/[IETF RFC 4145) the initiator provides the roles that it is willing to support. The receiver then chooses based on the received roles, the role that the TerminalCapabilitySet initiator shall use for the DTLS connection.

Note: There is a fundamental difference between H.323 and SIP with regards to the establishment of the media stream. H.323 systems use the H.245 Master Slave determination procedures (clause C.2/[ITU-T H.245]) to determine who is the master. Typically the master is responsible for initiating connections. Whereas in SIP/SDP for connections that require a handshake between a master/slave the SDP "a=setup" attribute is used to determine the initiating party. A problem occurs when interworking between H.323 and SIP systems. It should be possible to open an end to end DTLS connection between the H.323 and SIP endpoints without the need to terminate the data stream (i.e. a gateway operates in a pass through mode). The procedures of this Recommendation allow the initiator of the DTLS handshake to be independent of the H.245 Master.

## Logical Channel initiation and fingerprint exchange

DTLS connections are fundamentally bidirectional in nature. As such the master endpoint initiates a birectional OpenLogicalChannel request containing the DTLS fingerprint and hash function as per clause 7.3.2.  As per clause 6.2.8.2/[ITU-T H.323] the bidirectional logical channel procedures (clause C.5/[ITU-T H.245) are used to signal the initiation of a DTLS connection.

In response to the OpenLogicalChannel the peer endpoint returns an OpenLogicalChannelAck with its DTLS fingerprint and hash function.

The authentication certificates and the associated fingerprint used for the DTLS connection are generated according to the procedures specified in clauses 5 and 6/[IETF RFC 4572]. The endpoint will use one of the hash functions negotiated through the security capability exchange outlined in clause 8.1. If the H.245 connection is secured as per clause 8 then as per clause 6.1/[IETF RFC 4572] any certificate asserting syntactically valid identity may be used. If the H.245 signalling is not secured then an appropriate identity must be provided. This is FFS.

{Contributor’s note: Guidelines need to be formed on the endpoint identification. Clause 6.1/ITU-T H.235.8 based on a public key certificate.}

## DTLS connection initiation

On reception of an OpenLogicalChannel or OpenLogicalChannelAck the endpoint that has been identified as the active endpoint for the DTLS connection initiation will initiate an outgoing DTLS connection to the peer to perform the DTLS handshake. During the handshake phase the certificates are exchanged as per [IETF RFC 6347] and clause 6.2/[IETF RFC 4572].

As discussed in clause 7.1 different uses of DTLS may impose certain requirements for the DTLS handshake phase. The requirements are document as part of profile document and are dependent on the transport, for example as per clause 4.7/[IETF RFC 6083].

The endpoints then verify that the fingerprints of the certificates received during the DTLS handshake match the fingerprints received in the OpenLogicalChannel orOpenLogicalChannelAck messages. In this manner, the endpoints verify each other’s credentials.

If the fingerprints do not match, the endpoint shall close the logical channel immediately.

{Contributor’s note: Silently close or a protocol conformant closure procedure, which would imply the sending of a DTLS closure alert?}

Once the DTLS handshake is successfully completed and the credentials have been verified, data packets are transported in DTLS record layer "application_data" packets.

NOTE – Some usages of DTLS i.e. DTLS-SRTP use alternate methods to transport data packets.

Annex A
Title placeholder

Appendix I
Title placeholder

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

[b-IETF tls-terms]	IETF draft-guballa-tls-terminology (2015), Terminology related to TLS and DTLS.
URL: https://tools.ietf.org/html/draft-guballa-tls-terminology-01

[End Proposed Correction]

____________________