| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4720 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16, 5/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.245 Annex U: editorial update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes some editorial update proposals related to

- WebRTC data channel terminology,

- DTLS terminology,

- SCTP terminology and

- motivation of DTLS-over-L4 transport.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 223 R1 (WP 1/16) as latest draft.

Change proposal:

[Begin Proposed Correction]

Annex U

Support of WebRTC datachannel

(This annex forms an integral part of this Recommendation.)

Editor’s Note: Since the SCTP transport is likely to be used for a variety of applications in H.323 systems, it would be useful to have a table similar to Appendix VIII of H.245, which would list the ITU-T recommendations that use this transport.

## U.1	Scope

This Annex describes the establishment, use and closure of a WebRTC Data Channel via H.245 procedures.

## U.2	Capability Negotiation

Endpoints requiring the use of a WebRTC DataChannel for the transmission of application protocols, such as CLUE (draft-ietf-clue-datachannel [116]), shall use the TerminalCapabilitySet message to indicate the use of the data channel establishment protocol by indicating a bi-directional DataApplicationCapability with the application indicating “dataChannel”. The endpoint should indicate which protocols are used in the dataChannel through the dataChannelProfile protocol element.  In addition, the message shall indicate the use of an SCTP association over a DTLS connection on UDP through the use of the udp-dtls-sctp parameter in DataProtocolCapability or a DTLS connection on TCP through the use of the tcp-dtls-sctp parameter (NOTE 1). The endpoint shall indicate the use of appPPIDs indicating WebRTC usage, i.e. one of 51 “WebRTC String”, 53 “WebRTC Binary”, 56 “WebRTC String Empty” and 57 “WebRTC Binary Empty”.  A PPID of 50 “WebRTC DCEP” is only included if the Data Channel Establishment Protocol (DCEP) is used for the establishment of data channels on the SCTP association.

NOTE 1: Protocol layering of "DTLS-over-TCP" might lead to the impression that DTLS could be replaced by TLS (due to the native layering of "TLS-over-TCP"). However, DTLS will be the common transport-level security protocol due to the assumption of "dynamic L4 protocol" switchover processes (e.g., if UDP is blocked by FWs, then TCP is tried, but without any associated switchover from DTLS to TLS).

The establishmentType in dataChannelProfile indicates whether the data channel is established via DCEP or via an out-of-band OpenLogicalChannel based method. For out-of-band establishment the sctpStreamID indicates the SCTP stream identity to be used for the data channel. Only one method may be supported on a data channel. The use of DTLS is indicated as per clause 7.3.1/ITU-T H.235.DTLS [121].

NOTE 2: PPIDs of the individual data channels associated with WebRTC control are not sent in the DataProtocolCapability.

It must also indicate the use of the SCTP extensions as specified in clause 6.1 / draft-ietf-rtcweb-data-channel [110].  For example, support of IETF RFC3758 [117] is signalled through SCTPChunkType with value 192.

## U.3	SCTP Association over DTLS Establishment

Once the endpoints have negotiated the use of the dataChannel, the bidirectional logical channel procedures (clause C.5/ITU-T H.245) are used to establish the SCTP association over DTLS connection on UDP or TCP.  There is one bidirectional logical channel per SCTP over DTLS on UDP transport. The OpenLogicalChannel message shall contain the parameters negotiated in the terminal capability exchange.

The DTLS connection is first established according to the procedures in clauses 8.2 and 8.3/ITU-T H.235.DTLS [121]. Once the DTLS handshake has been completed (i.e., the DTLS connection is in "data transfer ready" state) the SCTP association is then established via data packets transported as DTLS record layer "application_data" packets.

## U.4	Data Channel Establishment

### U.4.1	Channel Establishment using the Data Channel Establishment Protocol

If the establishmentType for the data channel indicates “DCEP” and once the SCTP association has been established, the Master endpoint (determined through H.245 master/slave determination procedures) is responsible for initiating the Data Channel Establishment Protocol DATA_CHANNEL_OPEN messages (clause 5.1 / draft-ietf-rtcweb-data-protocol [114]). The endpoint should send a DATA_CHANNEL_OPEN for each instance of the DataChannelProfile. If included in the DataChannelProfile, the protocol and label parameters are used in the DATA_CHANNEL_OPEN message. The procedures of clause 6.5 / draft-ietf-rtcweb-data-channel [110] apply.

If the endpoint indicates support of DCEP it may establish additional data channels through the use of the DCEP without additional OpenLogicalChannel signalling.

### U.4.2	Out of band channel establishment

If the establishmentType for data channel includes a sctpStreamID, once an OpenLogicalChannel has been sent containing a mandatory sctpStreamID and protocol and the subsequent OpenLogicalChannelAck is received, the endpoint may start to send data on the relevant data channel.

The endpoint may establish additional data channels using the out-of-band method via updated OpenLogicalChannel messaging for the logical channel.

## U.5	Data Transfer

# Data is sent on the SCTP streams (as used as data channels) as per clause 6.6 / draft-ietf-rtcweb-data-channel [110].

## U.6	Data Channel Closure

# Data channels may be closed using the procedures of clause 6.7 / draft-ietf-rtcweb-data-channel [110], which might result effectively in a reset of the associated SCTP stream.

For data channels established via the out-of-band method, a OpenLogicalChannel removing the dataChannelProfile information for the relevant data channel may also be sent indicating the closure of the data channel.

The SCTP association may be shut down as a result of a CloseLogicalChannel message.  The individual SCTP streams should be reset as per clause 6.7 / draft-ietf-rtcweb-data-channel [110] before the SCTP association is removed (i.e., shut down triggered).

[End Proposed Correction]

____________________