| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4725 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.50 (Rev): Item #1 "Trickle ICE" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides a proposal how we could address trickle ICE under the condition that the IETF drafts will be not ready in due time (from perspective of scheduled finalization date of draft H.248.50 (Rev)).

1	Discussion

Current status and evaluation is documented in clause L.5.2:

### L.5.2	Item #5.2: IETF document "Trickle ICE"

| Reference: | draft-ietf-mmusic-trickle-ice & draft-ietf-mmusic-trickle-ice-sip "Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol" |
| --- | --- |
| Motivation: | - Issue: process of candidate gathering, prioritization and final selection may significantly delay communication establishment ("relatively lengthy session establishment times and degraded user experience"). Such delays do negatively impact primarily realtime conversation services. - Trickle ICE: alternative mode by incremental exchange of candidates already early during session establishment process - trickle ICE = proven technology, included in specifications such as XMPP Jingle - ICE agent: additional capability "candidate harvester" |
| H.248.50 impact? | - Yes: trickle ICE is not backward compatible to standard ICE, hence both ICE agents needs firstly to check "trickle ICE" support or not - the "trickle ICE" capability declaration and negotiation, as well as fallback process is delegated to the IP signalling plane (such as SDP Offer/Answer), thus subject of the H.248 interface in case of ICE involved MGs - concrete H.248.50 changes: to be done |
| update by AVD-4617: | The capability of trickle ICE is part of the overall WEBRTC solution, hence relevant for H.248 WEBRTC gateways (see H.248.WEBRTC). Trickle ICE significantly impacts the existing H.248.50 specification because - the information about vanilla ICE is distributed across multiple places in H.248.50 and - trickle ICE is inherently not backward compatible to vanilla ICE. Thus, an urgent clarification and update of H.248.50 would be beneficial. Such work implies at least following three tasks: T1: Study of signalling scenarios (for trickle ICE only and combined trickle/vanilla ICE) in order to separate "trickle ICE" information at call control signalling level from the remaining part which finally will be visible at the H.248 interface at all. T2: Impact of trickle ICE on all existing H.248.50 packages ("still unclear if at all, but obvious that some package procedure would benefit from more detailled vanilla ICE related information")? T3: Impact of trickle ICE on package-less procedures (caluse 10)? This contribution does not address any of these tasks, the questions are still open. With regards to H.248.WEBRTC gateways it might be that the first H.248 WEBRTC gateway profile versions survive with the model according Appendix I/H.248.50. Taking the high-level assumptions that Assumption 1: trickle ICE will affect primarily the address gathering phase (= step I in Fig. I.1) (to be confirmed) and Assumption 2: subsequent STUN connectivity check phase(s) (= step IV in Fig. I.1) might be independent of the parallel and still ongoing call level trickle ICE signalling (to be confirmed), then the impact might be limited on H.248 packages mgastuns and ostuncc ("which are both subject of first H.248 WEBRTC gateway profile versions"). And there seems to be now impact on the mgastuns package, but impact on the ostuncc package is not obvious due to the "candidate information" used. |

It could be concluded that a comprehensive inclusion of trickle ICE in H.248.50 implies a careful analysis of the entire main body of H.248.50. Trickle ICE does unfortunately not impact a single place only in H.248.50, is rather distributed across multiple clauses.

On the other side, the underlying two IETF documents are still in draft state, but seems to provide already a sufficient technically stable baseline, - however, the overall standardization process is rather slow.

There are mainly two options how to proceed with H.248.50:

O1)	Delay the H.248.50 work item till RFCs are available and provide a complete and comprehensive trickle ICE update.

O2)	Limit the "trickle ICE" scope on the ICE/STUN use case as outlined in Appendix I.1, a scenario between ICE clients residing in a UE and an H.248 IP access gateway. Because there are actually already "successful candidates" required at the "very beginning", when the H.248 context is created, an H.248 stream endpoint added and STUN connectivity checks were successfully executed. Thus, there's a rather stable and static configuration from perspective of the H.248 gateway. The likelihood of "trickling further candidates" is not zero, but seems to be fairly low in such a UE-NAT-MG configuration. Any trickling action would result in SIP INFO message exchanges between the UE and MGC here.

We tend for option O2 because the referred use case is the one relevant for H.248 WebRTC gateways. See also the highlighted text in gree in the living list item at page 2.

2	Conclusions

Moving forward with option O2 would result in following updates:

- terminology (clause 3) could be already completed;

- trickle ICE introduction in clause 6.4.2.1: indicate high level information flow at call control signalling level;

- Appendix IV:
a) provide a summary
b) provide a high level check of existing H.248.50-defined packages vs trickle ICE

- Bibliography: fix the date of the IETF drafts.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4662 as input draft to this meeting.

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	NAT traversal [ITU-T Y.2111]: The operation of adapting the IP addresses so that the packets in the media flow can pass through a far-end (remote) NAT.

3.1.2	network address translation [ITU-T Y.2111]: The operation by which IP addresses are translated (mapped) from one address domain to another address domain.

3.1.3	pinhole [ITU-T H.248.37]: A configuration of two associated H.248 IP terminations within the same context, which allows/prohibits unidirectional forwarding of IP packets under specified conditions (e.g., address tuple).

3.1.4	symmetric NAT [IETF RFC 3489]: A symmetric NAT is one where all requests from the same internal IP address and port, to a specific destination IP address and port, are mapped to the same external IP address and port.

3.1.5	STUN agent [IETF RFC 5389]: an entity that implements the STUN protocol. The entity can be either a STUN client or a STUN server.

3.1.6	STUN client [IETF RFC 5389]: an entity that sends STUN requests and receives STUN responses. A STUN client can also send indications.

3.1.7	STUN server [IETF RFC 5389]: an entity that receives STUN requests and sends STUN responses. A STUN server can also send indications.

3.1.8	Vanilla ICE [b-IETF trickle-ice]: The Interactive Connectivity Establishment protocol as defined in [IETF RFC 5245].

3.1.9	Half trickle ICE mode [b-IETF trickle-ice]: A trickle ICE mode of operation where the offerer gathers its first generation of candidates strictly before creating and sending the offer. Once sent, that offer can be processed by vanilla ICE agents and does not require support for [b-IETF trickle-ice]. It also allows trickle ICE capable answerers to still gather candidates and perform connectivity checks in a non-blocking way, thus roughly offering "half" the advantages of trickle ICE. The mechanism is mostly meant for use in cases where support for trickle ICE cannot be confirmed prior to sending a first offer.

3.1.10	Full trickle ICE mode [b-IETF trickle-ice]: Regular mode of operation for trickle ICE agents, used in opposition to the half trickle mode of operation.

3.1.11	STUN consent freshness [IETF STUNCONFRES]: Maintaining and renewing consent over time. Consent relates to the mechanism of obtaining permission to send to a remote transport address.

NOTE – Initial consent is obtained using ICE or a TCP handshake.

## 3.2	Terms defined in this Recommendation

3.2.1	ICE-full (mode, implementation): The ICE protocol capability set that performs the complete set of ICE requirements according to clause 4.1/[IETF RFC 5245].

NOTE – This definition is based on clause 3/[IETF RFC 5245]: "Full: An ICE implementation that performs the complete set of functionality defined by this specification." The RFC uses the term "full ICE" to reference this functionality.

3.2.2	ICE-lite (mode, implementation): The ICE protocol capability set that performs the subset of ICE requirements according to clause 4.2/[IETF RFC 5245].

NOTE – This definition is based on clause 3/[IETF RFC 5245]: "Lite: An ICE implementation that omits certain functions, implementing only as much as is necessary for a peer implementation that is full to gain the benefits of ICE.  Lite implementations do not maintain any of the state machines and do not generate connectivity checks.".

3.2.3	Trickle ICE: The extended ICE protocol capabilities, beyond vanilla ICE, as defined in [b-IETF trickle-ice]. Trickle ICE includes half trickle and full trickle ICE modes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

### 6.4.2	SDP for extended ICE

#### 6.4.2.1	Trickle ICE

Appendix IV provides some background information of trickle ICE, from perspective of H.248 gateways.

Where trickle ICE is used, the SIP-based call control signalling (according to b-IETF trickle-ice] and b-IETF trickle-sip]) is using following signalling extensions in brief:

- start of call establishment phase: indication of trickle ICE support via SDP attribute "a=ice-options:trickle";

- trickling phase: signalling of candidate updates via SIP INFO messages; and

- stop of trickling phase: announcing end of candidates via SDP attribute " a=end-of-candidates".

The three trickle ICE phases does affect the H.248 MGCs' SIP-based call control signalling interface only. There are not necessarily correspondent impacts on the H.248 gateway control signalling. This seems to be rather conditional, dependent on the network configuration and IP bearer path routes. A detailed analysis of trickle ICE is for further studies.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix IV

ICE extensions: "Trickle ICE"

(This appendix does not form an integral part of this Recommendation.)

## II.1	Introduction

Table IV.1 provides a brief summary of trickle ICE. That information is still preliminary due to the status of the underlying ICE specifications in IETF.

Table IV.1 – Summary "trickle ICE from perspective of H.248 gateways"

| References: | See - [b-IETF trickle-ice]: basic ICE protocol procedures and SDP extensions; - [b-IETF trickle-sip]: trickle ICE support via SIP. |
| --- | --- |
| Motivation: | - Basic issue with vanilla ICE: process of candidate gathering, prioritization and final selection may significantly delay communication establishment ("relatively lengthy session establishment times and degraded user experience"). Such delays do negatively impact primarily realtime conversation services. - Trickle ICE: provides alternative mode by incremental exchange of candidates already early during call establishment process. - ICE agent: additional function "candidate harvester". |
| Principal impact on main body of this Recommendation: | - Yes: trickle ICE is not backward compatible to vanilla ICE, hence both ICE agents (and one is residing in the H.248 MG) needs firstly to check trickle ICE support or not. - There are again the two variants of half trickle and full trickle ICE modes. Such asymmetrical options leading to a number of real life combinations between the H.248 MG and remote ICE clients (modes of ICE client is either "vanilla", "half trickle" or "full trickle"). - The "trickle ICE" capability declaration and negotiation, as well as fallback process is delegated to the IP signalling plane, i.e., a) the SIP-based MGC call control interface (see clause 6.4.2.1), and b) the H.248 gateway control interface (for further studies). Concrete changes are not yet implemented in the H.248 capabilities as defined by this Recommendation. |

Purpose of this Appendix is to illustrate signalling flows with trickle ICE in order to highlight principle impact on H.248 gateway control.  The considered use cases should cover following aspects:

- Comparison of signalling scenarios with trickle ICE only and combined trickle/vanilla ICE modes in order to separate trickle ICE specific information at call control signalling level from the remaining signaling part, which is finally visible at the H.248 interface at all.

- Check impact of trickle ICE on all existing H.248 packages as defined by this Recommendation.

- Verify possible impact of trickle ICE on the package-less procedures according to clause 10.

## II.2	Impact of trickle ICE on H.248.50-defined packages

For further studies.

Table IV.2 – Summary "impact of trickle ICE on H.248.50-defined packages"

| Package | Comments |  |  |
| --- | --- | --- | --- |
| STUN base package (stun) (clause 7.1) |  | No impact. The address mapping principles as introduced for vanilla ICE should be identical to trickle ICE (to be confirmed). |  |  |
| STUN information package (stuni) (clause 7.5) |  | No impact because the NAT type related information is orthogonal to ICE procedures (to be confirmed). |  |  |
| MG STUN client package (mgstunc) (clause 7.2) |  | For further studies. |  |  |
| MGC STUN client package (mgcstunc) (clause 7.4) |  | No impact due to MGC-embedded STUN client function, which is not exposed at the H.248 interface. |  |  |
| STUN consent freshness package (stun) (clause 8.3) |  | No impact due to the assumption that STUN consent freshness procedures will be requested at a "later point in time" of the call phase, i.e., some time after the "end of address gathering" was already announced (to be confirmed). |  |  |
| MG TURN client package (mgstunc) (clause 7.3) |  | No impact (to be confirmed). |  |  |
| MG act-as STUN server package (mgastuns) (clause 8.1) | No impact because the H.248-related STUN server assignment is orthogonal to ICE procedures (to be confirmed). |  |  |
| Originate STUN continuity check package (ostuncc) (clause 8.2) |  |  | No impact because STUN continuity checks are a follow-up activity, after the ICE related address gathering phase. There might be additional STUN continuity checks requested (by the MGC from the MG) after an H.248 stream is already used for application data transfer (to be confirmed). |  |  |
| MGC-originated STUN request package (mgcostunr) (clause 9.1) |  |  | No impact because keep alive mechanism is orthogonal to ICE procedures. Dynamic view: any keep alive activities occur after the ICE based address gathering phase. |  |  |
| Keep alive request package (mgastuns) (clause 9.2) |  |  | No impact because keep alive mechanism is orthogonal to ICE procedures. Dynamic view: any keep alive activities occur after the ICE based address gathering phase. |  |  |

## II.3	Example signalling flows for trickle ICE

For further studies.

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[b-ITU-T H.248.89]	Recommendation ITU-T H.248.89 (10/2014), Gateway control protocol: TCP support packages.

[b-ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[b-ITU-T H.248.93]	Recommendation ITU-T H.248.93 (10/2014), Gateway control protocol: H.248 packages for control of transport security using DTLS.

[b-IETF RFC 4571]	IETF RFC 4571 (07/2006), Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport.

[b-IETF RFC 5626]	IETF RFC 5626 (10/2009), Managing Client-Initiated Connections in the Session Initiation Protocol (SIP).

NOTE 1 – Two keep-alive techniques: CRLF keep-alive technique for TCP and SCTP, and STUN keep-alive technique for UDP.

[b-IETF RFC 6223]	IETF RFC 6223 (04/2011), Indication of Support for Keep-Alive.

{Editor’s note (2013-01 meeting): possible further "keep alive" reference:
"Indication of support for reverse keep-alive" 
(draft-holmberg-sipcore-rkeep). Contributions solicited.}

[b-IETF RFC 6724]	IETF RFC 6724 (09/2012), Default Address Selection for Internet Protocol Version 6 (IPv6).

[b-IETF ice-lite]	IETF draft-rescorla-mmusic-ice-lite (02/2007), Implementing Interactive Connectivity Establishment (ICE) in Lite Mode.
URL: http://tools.ietf.org/html/draft-rescorla-mmusic-ice-lite-00

[b-IETF trickle-ice]	IETF draft-ietf-mmusic-trickle-ice (01/2015), Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol.

[b-IETF trickle-sip]	IETF draft-ietf-mmusic-trickle-ice-sip (01/2015##/201#), A Session Initiation Protocol (SIP) usage for Trickle ICE.

[b-IETF ice-dualstack]	IETF draft-martinsen-mmusic-ice-dualstack-fairness (##/201#), ICE IPv4/IPv6 Dual Stack Fairness.

[End Proposed Correction]

____________________