| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4728 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.50 (Rev): Item #4 "Keep alive support" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal in solving the remaining pending items related to keep alive mechanisms.

1	Discussion and conclusions

The editor's notes are essentially related to IETF SIP documents concerning keep-alive support. Table 1 provides an evaluation. Such kind of call control signalling related information is basically subject of clause 9.2.6.1 "MGC stimuli for IP bearer path keep-alive requests".

Table 1 – "SIP-related keep alive RFCs"

| Reference | Title | Imapct on H.248.50? |
| --- | --- | --- |
| RFC 5626 | Managing Client-Initiated Connections in the Session Initiation Protocol (SIP) | Abstract The SIP allows proxy servers to initiate TCP connections or to send asynchronous UDP datagrams to User Agents in order to deliver requests. However, in a large number of real deployments, many practical considerations, such as the existence of firewalls and NATs or the use of TLS with server-provided certificates, prevent servers from connecting to User Agents in this way. This specification defines behaviors for User Agents, registrars, and proxy servers that allow requests to be delivered on existing connections established by the User Agent. It also defines keep-alive behaviors needed to keep NAT bindings open and specifies the usage of multiple connections from the User Agent to its registrar. Conclusions: - relevant and already covered in clause 9.2.6.1; - no further actions required. |
| RFC 5923 | Connection Reuse in the Session Initiation Protocol (SIP) | Abstract This document enables a pair of communicating proxies to reuse a congestion-controlled connection between themselves for sending requests in the forwards and backwards direction. Because the connection is essentially aliased for requests going in the backwards direction, reuse is predicated upon both the communicating endpoints authenticating themselves using X.509 certificates through TLS. For this reason, we only consider connection reuse for TLS over TCP and TLS over SCTP. This document also provides guidelines on connection reuse and virtual SIP servers and the interaction of connection reuse and DNS SRV lookups in SIP. Conclusions: - the described method of connection reuse is driven by NAT traversal aspects; - however, the SIP specific connection reuse (actually the IP transport connection for SIP traffic itself) seems to be beyond the scope of H.248.50; - thus, RFC is out of scope of H.248.50 |
| RFC 6223 | Indication of Support for Keep-Alive | Abstract This specification defines a new SIP Via header field parameter, "keep", which allows adjacent SIP entities to explicitly negotiate usage of the NAT keep-alive mechanisms defined in SIP Outbound, in cases where SIP Outbound is not supported, cannot be applied, or where usage of keep-alives is not implicitly negotiated as part of the SIP Outbound negotiation. Conclusions: - relevant and already covered in clause 9.2.6.1; - no further actions required. |
| draft-holmberg-sipcore-rkeep-05.txt | Indication of support for reverse keep-alive | Abstract This specification defines a new SIP Via header field parameter, "rkeep". The "rkeep" parameter allows a SIP entity to indicate willingness to receive keep-alives from its adjacent downstream SIP entity, the adjacent downstream SIP entity to indicate willingness to send keep-alives, and for SIP entities willing to send keep-alives to inform the receiver about the keep-alive interval. Conclusions: - relevant because directly related to RFC 6223, which itself was already incorporated in H.248.50; - the quality of "reverse" relates essentially to a more network wide view of keep alive connection segments, however, such a view is beyond the perspective of a single H.248 gateway, i.e., a specific H.248 MGC might be in the role of "downstream SIP entity" which is requested for (reverse) keep alive support … - thus, could be referred as well in clause 9.2.6.1 |

The contribution proposes also some editorial enhancements for clause 9.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4662 as input draft to this meeting.

Change proposal #1:

[Begin Proposed Correction]

# 9	Keep-alive and pinhole support

The packages in this clause provide a means of opening a pinhole through a NAT and to maintain a binding with the NAT. It does not rely on ICE techniques.

## 9.1	MGC-originated STUN request package

|  | # Package name: | MGC Originated STUN Request |
| --- | --- | --- |
|  | # Package ID: | mgcostunr (0x00c4) NOTE 1 – Clause 10 of [IETF RFC 5245] makes use of this mechanism. |
|  | # Description: | The MGC may also periodically request the MG to send a STUN request in order to keep the binding with the NAT "alive". This should be done before the expiry of the keep-alive period. NOTE 2 – This package is an exception to the rule in the scope that the MG determines the address to which a Binding Request is sent. NOTE 3 – The package deals with IP address information, which is direction specific (source and destination) from IP host perspective. The package description qualifies in source and destination specific parts, however, there is basically the assumption of address symmetry (i.e., the local source address is equal to the local destination address, and the same for remote addresses). |
|  | # Version: | 1 |
|  | # Extends: | None |

### 9.1.1	Properties

None

### 9.1.2	Events

#### 9.1.2.1	STUN binding request failure

|  | # Event name: | STUN Binding Request Failure |
| --- | --- | --- |
|  | # Event ID: | fail (0x0001) |
|  | # Description: | This event is triggered if a STUN binding request has failed. |

##### 9.1.2.1.1	EventsDescriptor parameters

###### 9.1.2.1.1.1	From address

|  | # Parameter name: | From Address |
| --- | --- | --- |
|  | # Parameter ID: | fa (0x0001) |
|  | # Type: | List of String |
|  | # Optional: | No |
|  | # Possible values: | "Y" (Yes): Failure reporting is required for this list position. "N" (No): Failure reporting is not required for this list position. |

##### 9.1.2.1.2	ObservedEventsDescriptor parameters

###### 9.1.2.1.2.1	Failure

|  | # Parameter name: | Failure |
| --- | --- | --- |
|  | # Parameter ID: | fail (0x0001) |
|  | # Description: | This parameter contains the reason for the failure. |
|  | # Type: | String |
|  | # Optional: | No |
|  | # Possible values: | If the response of STUN binding request is "time out", error code 1000 follows the <list position>. If the mapping address in the response of STUN binding request changes, error code 1001 and the new mapping address follows <list position>. The format is "<list position>:IP address ":" port. e.g., "1:202.1.2.3:1000" |
|  | # Default: | None |

### 9.1.3	Signals

#### 9.1.3.1	Send STUN request

|  | # Signal name: | Send STUN Request |
| --- | --- | --- |
|  | # Signal ID: | sstunr (0x0001) |
|  | # Description: | This signal instructs a MG to send a binding request from the local source address to the remote destination address as contained in the remote descriptor. |
|  | # Signal type: | Brief |
|  | # Duration: | NA |

##### 9.1.3.1.1	From address

|  | # Parameter name: | From Address |
| --- | --- | --- |
|  | # Parameter ID: | fa (0x0001) |
|  | # Description: | This parameter contains the local source addresses where the binding request should be sent from, to the corresponding remote destination address. |
|  | # Type: | List of String |
|  | # Optional: | No |
|  | # Possible values: | N ("No Request"): do not perform a STUN binding request from the local source address. |
|  | # Default: | None |

##### 9.1.3.1.2	Retransmission time interval

|  | # Parameter name: | Retransmission Time Interval |
| --- | --- | --- |
|  | # Parameter ID: | rti (0x0002) |
|  | # Description: | This parameter contains the initial retransmission time interval for the STUN request, as RTO defined in clause 7.2.1 of [IETF RFC 5389]. |
|  | # Type: | Integer |
|  | # Optional: | Yes |
|  | # Possible values: | 1 ms to 600000 ms (10 minutes) |
|  | # Default: | 100 ms |

### 9.1.4	Statistics

None.

### 9.1.5	Error codes

None.

### 9.1.6	Procedures

The MGC should trigger the MG to initiate a STUN Binding Request to the remote address when it suspects that the local address is behind a restricted cone NAT. This will have the effect of opening a pinhole, allowing the remote end to send packets to the local end. If the MGC initiates a STUN Binding Request to the remote address, it can keep NAT bindings active. If there are no packets sent between the local and remote address pairs being used for media for Tr seconds (where packets include media and previous keep-alives; variable Tr refers to signal parameter value mgcostunr/sstunr/rti), the MG MUST generate a keep-alive on that pair. To detect STUN Binding Request failures the fail event should be set.

This procedure is distinct from the ICE-related STUN continuity check procedures.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## 9.2	Keep alive request package

|  | Package name: | Keep Alive Request |
| --- | --- | --- |
|  | Package ID: | kar (0x00c5) |
|  | Description: | This package enables the MGC to request the MG to send a packet in order to open a pinhole through a server or to maintain a NAT binding. This packet goes through the same way as the media flows. For example it allows the techniques as defined in clause 10 of [IETF RFC 5245]. NOTE – This package is an exception to the rule in the scope that the MG determines the address to which a Binding Request is sent. |
|  | Version: | 1 |
|  | Extends: | None |

### 9.2.1	Properties

None.

### 9.2.2	Events

None

### 9.2.3	Signals

#### 9.2.3.1	Send keepalive packet

|  | Signal name: | Send Keepalive Packet |
| --- | --- | --- |
|  | Signal ID: | skap (0x0001) |
|  | Description: | This signal instructs a MG to send a keepalive packet from the local source address to the remote destination address contained in the remote descriptor. |
|  | Signal type: | Brief |
|  | Duration: | NA |

##### 9.2.3.1.1	From address

|  | Parameter name: | From Address |
| --- | --- | --- |
|  | Parameter ID: | fa (0x0001) |
|  | Description: | This parameter contains the local source addresses where the keepalive packet should be sent from, to the corresponding remote destination address. The MGC shall include a value in each position of list of string for each element as described in the "stunb/ac" property (see clasue 7.1.1.1). |
|  | Type: | List of String |
|  | Optional: | No |
|  | Possible values: | "Send" (S): send a keepalive packet from the local address. "Not Send" (N): do not send a keepalive packet from the local address. |
|  | Default: | None |

##### 9.2.3.1.2	Keep alive transmission interval

|  | Parameter name: | Keep Alive Transmission Interval |
| --- | --- | --- |
|  | Parameter ID: | ti (0x0002) |
|  | Description: | This parameter contains transmission time interval for sending the keepalive packet. The value of this parameter corresponds to the Tr timer from [IETF RFC 5245]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 15000 ms or more |
|  | Default: | 15000 ms (15 seconds), unless provisioned otherwise. |

##### 9.2.3.1.3	Keep alive packet type

|  | Parameter name: | Keep Alive Packet Type |
| --- | --- | --- |
|  | Parameter ID: | kapt (0x0003) |
|  | Description: | This parameter indicates the type of keep alive packet type. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | et (0x0000): Transport (i.e., UDP, DCCP) packet of 0-byte; rm (0x0001): RTCP packets multiplexed with RTP packets; sbi (0x0002): STUN binding indication; cn (0x0003): RTP packet with comfort noise payload; no (0x0004): Reserved for RTP packet with No-Op payload (see [b-IETF no-op]); iv (0x0005): RTP packet with incorrect version number; up (0x0006): RTP packet with unknown payload type. NOTE – Value no (0x0004) is reserved for a future payload type. |
|  | Default: | up (0x0006): RTP packet with unknown payload type |

### 9.2.4	Statistics

None.

### 9.2.5	Error codes

None.

### 9.2.6	Procedures

#### 9.2.6.1	MGC stimuli for IP bearer path keep-alive requests

The IP bearer path related keep-alive functions are part of overall NAT traversal solutions. The MGC may trigger keep-alive procedures due to

- local policy (provisioned policy, e.g. in case that all calls of an IP network domain require keep-alive support) or/and

- call-individual requests (and parameterization) by application control protocol signalling (see e.g. [b-IETF RFC 6223], [b-IETF RFC 5626], [b-IETF rkeep])

Example: A SIP network with usage of [b-IETF RFC 6223]: the MGC could (inter alia) derive the signal parameters "keep alive transmission interval" (kar/skap/ti) and "keep alive packet type" (kar/skap/kapt) from SIP signalling.

#### 9.2.6.2	Triggering keep-alive at MG by MGC

In order to open or maintain a NAT binding, the MGC should send the "Send Keep Alive" (skap) signal to the MG to initiate sending a keep-alive packet to the remote address. This will have the effect of opening a pinhole (or keeping a NAT binding open) allowing the remote end to send packets to the local end. The "Keep Alive transmission interval" (ti) parameter is used in order for the MG to autonomously send a keep-alive packet when no packets have been detected for the address for time interval Tr.

Where a MGC utilizes a "Send Keepalive Packet" request signal with the "RTP packet with unknown payload type", it may be required to communicate this to a peer MGC via the call contrrol signalling embedded SDP. An MGC on reception of this SDP attribute should then set this SDP attribute in the remote descriptor of the applicable MG Termination/Stream. It indicates to the MG that the remote end will utilize a keep-alive using an RTP packet with an unknown payload type.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[b-ITU-T H.248.89]	Recommendation ITU-T H.248.89 (10/2014), Gateway control protocol: TCP support packages.

[b-ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[b-ITU-T H.248.93]	Recommendation ITU-T H.248.93 (10/2014), Gateway control protocol: H.248 packages for control of transport security using DTLS.

[b-IETF RFC 4571]	IETF RFC 4571 (07/2006), Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport.

[b-IETF RFC 5626]	IETF RFC 5626 (10/2009), Managing Client-Initiated Connections in the Session Initiation Protocol (SIP).

NOTE 1 – Two keep-alive techniques: CRLF keep-alive technique for TCP and SCTP, and STUN keep-alive technique for UDP.

[b-IETF RFC 6223]	IETF RFC 6223 (04/2011), Indication of Support for Keep-Alive.

[b-IETF RFC 6724]	IETF RFC 6724 (09/2012), Default Address Selection for Internet Protocol Version 6 (IPv6).

[b-IETF rkeep]	IETF draft-holmberg-sipcore-rkeep (12/2014), Indication of support for reverse keep-alive.
URL: https://tools.ietf.org/html/draft-holmberg-sipcore-rkeep-05

[b-IETF no-op]	IETF draft-ietf-avt-rtp-no-op (05/2007), A No-Op Payload Format for RTP.
URL: https://tools.ietf.org/html/draft-ietf-avt-rtp-no-op-04

[b-IETF ice-lite]	IETF draft-rescorla-mmusic-ice-lite (02/2007), Implementing Interactive Connectivity Establishment (ICE) in Lite Mode.
URL: http://tools.ietf.org/html/draft-rescorla-mmusic-ice-lite-00

[b-IETF trickle-ice]	IETF draft-ietf-mmusic-trickle-ice (##/201#), Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol.

[b-IETF trickle-sip]	IETF draft-ietf-mmusic-trickle-ice-sip (##/201#), A Session Initiation Protocol (SIP) usage for Trickle ICE.

[b-IETF ice-dualstack]	IETF draft-martinsen-mmusic-ice-dualstack-fairness (##/201#), ICE IPv4/IPv6 Dual Stack Fairness.

[End Proposed Correction]

____________________