| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4743 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.50 (Rev): Support of "ICE restarts"? |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a new living list item.

1	Discussion and conclusions

Usage of ICE with WebRTC includes ICE restart procedures. It is still unclear whether and how "ICE restart procedures" would be supported by H.248.50 or impact H.248.50 if not yet supported?

We are proposing to add a living list item due to the lack of time in analysing H.248.50 concerning that ICE capability.

H.248.50 is so far completely silent about ICE restarts.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4662 as input draft to this meeting.

Change proposal:

[Begin Proposed Correction]

Appendix L
Living List related to enhancements and clarifications

(This appendix will be again removed at the stage of consent/approval)

## L.3	Item #3: Package-independent NAT-T procedures

| Motivation: | It turns out that network use cases may be identified with H.248 MGC/MG involvement in ICE/STUN/TURN related NAT-T, but which do not necessarily required H.248.50 defined protocol elements. Just H.248 LD/RD-embedded SDP information element might be sufficient for controlling the NAT-T service. This aspect is similar to H.248.84, which provides NAT-T service support with and without H.248.84 defined packages. #3.1: Correspondent use cases are conditional on particular behaviour, role assignment and/or network configurations, and should be described. |  |
| --- | --- | --- |
| Impacted H.248.50 text: | - Clause 6.1? - New clause on package-independent NAT-T procedures? |  |

## L.4	Item #4: H.248.50 References to IETF ICE documents

The core RFC on ICE [IETF RFC 5245] was augmented and updated by some extensions, which were not referenced by H.248.50 (note: work on H.248.50 was already technically stable in 2007). This clause provides a reference check.

### """"""""L.4.2	Item #4.2: IETF RFC 6724 "IPv6 address selection"

Missing H.248.50 reference:

| [IETF RFC 6724] IETF RFC 6724 (09/2012), Default Address Selection for Internet Protocol Version 6 (IPv6). |  |
| --- | --- |
| Motivation: | - ICE recommends to use the rules defined in [RFC 3484] (= obsoleted by [RFC 6724]) as part of the prioritization formula for IPv6 candidates |
| H.248.50 impact? | - No impact |

## L.5	Item #5: Recent ICE updates by IETF

{Editor’s note (2012-09 meeting): there are a number of IETF activities for improving ICE in different ways (see also email "[MMUSIC] Poll for path on ICE extensions and updates/corrections", http://www.ietf.org/mail-archive/web/mmusic/current/msg09558.html ):
- draft-keranen-mmusic-ice-address-selection-01
- draft-petithuguenin-mmusic-ice-attributes-level-03
- draft-wing-mmusic-ice-mobility-01
- draft-elwell-mmusic-ice-updated-offer-02
There is, in addition, a long discussion in the RTCWEB mailing list about "trickle ICE", and a couple of other ICE-related drafts and discussions in RTCWEB.

Most likely, we can categorize these documents in two big classes:
a) Documents that identify underspecified areas or errors in the spec, which are often driven by implementation experience
b) Documents that update or extend ICE with new functionality, such as "tricke ICE" (see e.g. the e-mail thread at http://www.ietf.org/mail-archive/web/rtcweb/current/msg05121.html) .
Such documents may involve normative updates to the ICE specification or they may simply be ICE extensions.".

Conclusions: there are basically impacts on H.248.50 expected. 
Contributions are solicited.}

### L.5.0	Overview (Status 11/2014)

IETF WG MMUSIC (= technology owner of ICE):

- draft-ietf-mmusic-trickle-ice & draft-ietf-mmusic-trickle-ice-sip
"Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol"

- replaced by: draft-martinsen-mmusic-ice-dualstack-fairness
"ICE IPv4/IPv6 Dual Stack Fairness"""

- draft-petithuguenin-mmusic-ice-attributes-level-04 (2012-10-08) 
"Media level ice-options SDP attribute"

- draft-keranen-mmusic-ice-address-selection-01 (2012-07-16) 
"Update on Candidate Address Selection for Interactive Connectivity Establishment (ICE)"

- draft-wing-mmusic-ice-mobility-07 (2014-06) 
"Mobility with ICE (MICE)"

- others?

### L.5.1	Item #5.1: IETF document "Update on Candidate Address Selection"

| Reference: | draft-keranen-mmusic-ice-address-selection-01 (2012-07-16) "Update on Candidate Address Selection for Interactive Connectivity Establishment (ICE)" URL: http://tools.ietf.org/id/draft-keranen-mmusic-ice-address-selection-01.txt |
| --- | --- |
| Motivation: | - update of rules for candidate address selection - primarily driven by IPv6 aspects related to ICE HOST CANDIDATES (differences between IPv4 and IPv6 based local interfaces) - # |
| H.248.50 impact? | - No impact (because rule selection algorithm is "ICE internal" functionality without any impact on "H.248 MG reporting of successful candidate/transport pairs") TO BE CHECKED |

### L.5.2	Item #5.2: IETF document "Trickle ICE"

| Reference: | draft-ietf-mmusic-trickle-ice & draft-ietf-mmusic-trickle-ice-sip "Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol" |
| --- | --- |
| Motivation: | - Issue: process of candidate gathering, prioritization and final selection may significantly delay communication establishment ("relatively lengthy session establishment times and degraded user experience"). Such delays do negatively impact primarily realtime conversation services. - Trickle ICE: alternative mode by incremental exchange of candidates already early during session establishment process - trickle ICE = proven technology, included in specifications such as XMPP Jingle - ICE agent: additional capability "candidate harvester" |
| H.248.50 impact? | - Yes: trickle ICE is not backward compatible to standard ICE, hence both ICE agents needs firstly to check "trickle ICE" support or not - the "trickle ICE" capability declaration and negotiation, as well as fallback process is delegated to the IP signalling plane (such as SDP Offer/Answer), thus subject of the H.248 interface in case of ICE involved MGs - concrete H.248.50 changes: to be done |
| update by AVD-4617: | The capability of trickle ICE is part of the overall WEBRTC solution, hence relevant for H.248 WEBRTC gateways (see H.248.WEBRTC). Trickle ICE significantly impacts the existing H.248.50 specification because - the information about vanilla ICE is distributed across multiple places in H.248.50 and - trickle ICE is inherently not backward compatible to vanilla ICE. Thus, an urgent clarification and update of H.248.50 would be beneficial. Such work implies at least following three tasks: T1: Study of signalling scenarios (for trickle ICE only and combined trickle/vanilla ICE) in order to separate "trickle ICE" information at call control signalling level from the remaining part which finally will be visible at the H.248 interface at all. T2: Impact of trickle ICE on all existing H.248.50 packages ("still unclear if at all, but obvious that some package procedure would benefit from more detailled vanilla ICE related information")? T3: Impact of trickle ICE on package-less procedures (caluse 10)? This contribution does not address any of these tasks, the questions are still open. With regards to H.248.WEBRTC gateways it might be that the first H.248 WEBRTC gateway profile versions survive with the model according Appendix I/H.248.50. Taking the high-level assumptions that Assumption 1: trickle ICE will affect primarily the address gathering phase (= step I in Fig. I.1) (to be confirmed) and Assumption 2: subsequent STUN connectivity check phase(s) (= step IV in Fig. I.1) might be independent of the parallel and still ongoing call level trickle ICE signalling (to be confirmed), then the impact might be limited on H.248 packages mgastuns and ostuncc ("which are both subject of first H.248 WEBRTC gateway profile versions"). And there seems to be now impact on the mgastuns package, but impact on the ostuncc package is not obvious due to the "candidate information" used. |

### L.5.4	Item #5.4: IETF document "Media level ice-options SDP attribute"

| Reference: | draft-petithuguenin-mmusic-ice-attributes-level-04 (2012-10-08) "Media level ice-options SDP attribute" URL: http://tools.ietf.org/id/draft-petithuguenin-mmusic-ice-attributes-level-04.txt |
| --- | --- |
| Motivation: | - Disaggregated media: refers to the ability for a user to create a multimedia session combining different media streams, coming from different devices under his or her control, so that they are treated by the far end of the session as a single media session - Redefinition of ICE SDP attributes: - ice-options (as a session-level and media-level attribute) special attention to value: "a=ice-options:rtp+ecn" - ice-lite: not redefined - ice-mismatch - ## |
| H.248.50 impact? | - # TO BE CHECKED |

### L.5.5	Item #5.5: IETF document "Mobility with ICE"

| Reference: | draft-wing-mmusic-ice-mobility-02 (2012-10-06) "Mobility with ICE (MICE)" URL: http://tools.ietf.org/id/draft-wing-mmusic-ice-mobility-02.txt |
| --- | --- |
| Motivation: | - Issue: mobility may lead to location changes which lead to address changes - ICE does not expect either the local host or the remote host to change their IP addresses. If … then "ICE restart" … (but takes time, service interruption) - Two mobility mechanisms: - Both endpoints ICE capable - Not …, with TURN server - STUN extensions: - new STUN Attribute MOBILITY-TICKET - new STUN Error Response Code - # |
| H.248.50 impact? | - # TO BE CHECKED |

### L.5.6	Item #5.6: "Support of ICE restarts?"

| Reference: | AVD-4743 URL: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/AVD-4731.zip |
| --- | --- |
| Motivation: | It is still unclear whether and how "ICE restart procedures" would be supported by H.248.50 or impact H.248.50 if not yet supported? Regarding support of ICE restarts, the main point would be to ensure that media packet flows continue to use the previously selected candidate pairs, while the ICE restart related processes (like address gathering and connectivity checking) are ongoing. For instance, the MGC may have to send an updated Remote descriptor to the involved H.248 SEP containing the new set of peer candidates. This new Remote descriptor would also contain a remote transport address (in the SDP "c="- and "m="-lines), which the MG should typically not use as new remote destination address. Rather, the MG should continue using the previously selected remote transport address as as remote destination address long as the new ICE process is not completed. |
| H.248.50 impact? | That aspect might need to be described in H.248.50, - but where? E.g., would section 8 (the text before section 8.1) be a potential place for that? The actual local address gathering, STUN connectivity check and candidate discovery and reporting procedures are based on H.248 signals, events and properties, which seems to be independent of actual parallel user plane packet transmission and reception processes and could possibly be equally applied to ICE restarts as they are applied to the initial ICE procedures. |

[End Proposed Correction]

____________________

“Addresses obtained from local network interfaces, called host candidates, are recommended as high-priority ones to be tested first since if they work, they provide usually the best path between the two hosts.“

Topic = http://tools.ietf.org/html/draft-loreto-splices-disaggregated-media-02 (see clause 3.2.1. Megaco issues)