| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4744 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.77 (Rev): Discussion – H.248 package(s) design versus SRTP key management schemes |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution discusses some general aspects related to H.248.77 package(s) versus support of multiple SRTP key management schemes.

1	Introduction

Starting situation is the fact that there are multiple SRTP key management options, as described in detail by following two RFCs:

IETF RFC 5479 (04/2009), Requirements and Analysis of Media Security Management Protocols.

IETF RFC 7202 (04/2014), Securing the RTP Framework: Why RTP Does Not Mandate a Single Media Security Solution.

RFC 5479 did already identify 15 key management schemes in 2009 (see Figure 1). There are even more in the meanwhile (due to some proprietary ones).

The actual number is out of discussion here, we are rather concerned with the various categories of schemes from H.248 protocol design perspetive. There are three categories with a different distribution of information flows over the network signaling and/or media plane. Figure 2 outlines a H.248 gateway perspective.

Figure 1 – RFC 5479: Overview of existing keying mechanisms (Status 2009)

Figure 2 – Example scenario: H.248 Media Gateway peering an IP Terminal
Key exchange management between SRTP endpoints

The three categories are termed as signaling path keying techniques (1), media path keying techniques (2) and signaling and media path keying techniques (3).

2	Discussion

2.1	Topic #1: Key management category dependent H.248 support?

It is obvious from Figure 2 that H.248 signalling is directly involved in categories 1 and 3, where all or partial SRTP key management information is subject of the (call and gateway control) signaling plane. H.248.77 (09/2010) does support only the SDES based key management scheme, which belongs to category one. The current revision of H.248.77 adds DTLS-SRTP as a second supported key management scheme, which belongs to category 2.

Well, the information flow of category 2 is solely along the (IP) media path (aka H.248 bearer plane), - why is any H.248 support required at all? H.248.93 on DTLS could also address DTLS-SRTP as well, right?

The rationale behind again is the fact that a single H.248 MG must support multiple SRTP key management schemes in parallel. There is consequently a H.248 specific signalling requirement for the indication of a particular SRTP key management scheme, - independent of the three categories.

2.2	Topic #2: Generic versus key management specific H.248 signalling requirements

Figure 3 illustrates the srtp package design as well as key management specific usage profiles:

Figure 3 – H.248 srtp package – Structure and key management specific usage profiles

There are essentially two network subfunctions behind each srtp package element:

- the "secure RTP function" in the packet path and

- the SRTP key management function in the control path.

The original package design was done from SDES specific view only. Figure 3 makes an attempt in qualifying the srtp v1 package elements as "generic" (when SRTP key management scheme independent) and "specific" (when tightly coupled to SDES functionality).

Further, Figure 3 provides key management scheme specific columns "SDES" and "DTLS-SRTP". All elements in the SDES column are of course tagged as "Used". The basic questions for "DTLS-SRTP" are then the followings:

- Is the package element required at all for the next SRTP key management scheme?

- If yes, is the package element completely reusable without any modification?

- If no, could the package element used as baseline for an extension?

- If no, is a new package element required?

Figure 3 provides a provisional classification ("which is used as baseline for the AVD-4692 change proposals").

2.3	Topic #3: H.248.77 packages design considerations

There are basically three options how new SRTP key management schemes could be integrated in H.248.77, see Figure 4:

Figure 4 – H.248.77 packages design considerations

The basic result of Figure 3, - which implies that multiple srtp v1 packages elements needs to be revised from DTLS-SRTP perspective -, could lead to the conclusion that a new DTLS-SRTP specific extension package (option 2) or a new self-contained DTLS-SRTP package (option 3) might be the cleanest approach from perspective of protocol semantics and structured specification.

On contrary, the "generic" attribution of the majority of srtp package elements (see Figure 3) underlines the design approach in just upversioning the srtp package (option 1) when new SRTP key management schemes are required to be supported.

We are still undecided at this stage, more analysis seems to be beneficial. Present draft H.248.77 follows option 1. Option 1 might be also in the end sufficient when H.248.77 could survive with two SRTP key management schemes only in future. But what happens in case of a 3rd scheme …?

("Well, all answers to these questions demand "just" the evaluation of all RFC 5479 SRTP key management schemes, or alternatively the consideration of the three categories only, from perspective of H.248.77 design …")

3	Summary

This contribution provides a general discussion on the subject of multiple SRTP key management schemes versus H.248 support by srtp package(s).

The contribution does not request any action.

____________________