| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4759 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ITU-T SG 12 |  |  |  |
| Title: | LS/i/r concerning Recommendation ITU-T G.1091 (COM16-LS-134) [from ITU T SG 12] |  |  |  |
| Purpose: | Liaison |  |  |  |

| INTERNATIONAL TELECOMMUNICATION UNION |  |  | STUDY GROUP 16 |  |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | TD 316 (GEN/16) |  |
|  |  |  | English only Original: English |  |
| Question(s): | 5/16 |  | Lucca, Italy, 12 - 23 October 2015 |  |
| TD (Ref.: COM 12 - LS 81) |  |  |  |  |
| Source: | ITU-T SG 12 |  |  |  |
| Title: | LS/i/r concerning Recommendation ITU-T G.1091 (COM16-LS-134) [from ITU- T SG 12] |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |
| For action to: |  | - |  |  |  |
| For comment to: |  | ITU-T Q5/16 |  |  |  |
| For information to: |  | - |  |  |  |
| Approval: |  | ITU-T SG12 meeting (Geneva, 14 May 2015) |  |  |  |
| Deadline: |  | 2 November 2015 |  |  |  |
| Contact: | Akira Takahashi NTT Japan |  |  | Tel: +81-422-59-7188 Fax: +81-422-59-5671 Email: takahashi.akira@lab.ntt.co.jp |  |
| Contact: | Marie-Neige Garcia Deutsche Telekom AG Germany |  |  | Tel: +49 30 8353 58591 Fax: Email: marie-neige.garcia@telekom.de |  |

ITU-T Q13/12 thanks ITU-T SG16 for their liaison (your reference COM16-LS-134) concerning the scope of Recommendation ITU-T G.1091 “Quality of Experience requirements for telepresence services”, and potential mis-alignment with draft Rec. H.TPS-AV “Audio/video parameters for telepresence systems”. Unfortunately, ITU-T Q13/12 was unable to provide a reply prior to your deadline of 1 February 2015, but we hope that this reply is not too late.

With respect to the question about the scope of G.1091, specifically whether it is intended to specify normative requirements for telepresence systems, or simply to provide guidance on QoE, the answer is the latter, where the scope reads:

This Recommendation provides guidelines for achieving high quality of experience (QoE) in telepresence services.

This is typically the approach followed in other performance-related Recommendation produced by ITU-T SG12. Of course, these guidelines may be adopted (in whole, or part) as normative specifications for specific equipment, but this is outside of the scope of G.1091.

With respect to specific mis-alignments between G.1091 and H.TPS-AV, the table below provides a comparison between corresponding parameters in each document.

| H.TPS-AV | G.1091 |
| --- | --- |
| 7.1.3.5 Sending Loudness Rating (+ 13–Fs) dB | 7.1 SLR (+ 13 – Fs) dB, |
| 7.1.3.6 Weighted Microphone Loudspeaker Coupling Loss ≥ 46 dB nominal volume, ≥ 40 dB maximum volume | 7.6 Acoustic echo/double talk Single talk: The weighted terminal coupling loss (TCLw) should be greater than or equal to 45 dB |
| 7.1.4.1 End to End Video Delay less than 320 milliseconds. | 10.1 Absolute delay of audio and video streams For telepresence systems: the end-to-end video delay shall be less than or equal to 320 ms. |
| 7.1.4.2 End to End Audio Delay less than 280 milliseconds. | 10.1 Absolute delay of audio and video streams End-to-end audio delay shall be less than or equal to 280 ms. |
| 7.1.4.3 Audio / Video Synchronization Within 40 and -60 milliseconds (i.e. synchronization error is less than 40 ms if the audio stream is ahead of the video stream and less than 60 ms if the video stream is ahead of the audio stream). | 10.2 Lip sync Audio streams should be less than 40 ms ahead of video streams, or less than 60 ms behind video streams. |
| 7.2.2.1 Receiving Loudness Rating (+ 5–Fr) dB | 7.1 (+ 5 – Fr) dB |
| 7.3.2.1 Reverberation time Reverberation time should be typically between 0.3 and 0.4 seconds, but at a minimum of 0.2 seconds, when measured in the 125 Hz-4 kHz octave bands. Such values would apply to medium size meeting rooms with a volume of approximately 100 m3 or less. For larger meeting rooms, a slightly higher reverberation time would be acceptable | 11.2 Reverberation Early reflecting sound should be between 1 kHz and 8 kHz and 15 ms after direct sound should attenuate 10 dB against direct sound. |
| 7.3.2.2 Ambient noise <= 35 dBA | 11.1 Background noise Telepresence systems shall enjoy a comfortable environment with background noise less than 40 dBA. |

As can be seen, apart from the difference in ambient noise (<= 35dBA compared to < 40 dBA), the different method of specifying reverberation, and a 1 dB difference in TCLw, there is exact agreement between the two documents. In fact, for ambient noise, it is noted that H.HPS-AV mentions:

This can be a difficult goal to meet in some buildings, and most smaller telepresence meeting rooms (holding 20 people or less)  will work well with a higher ambient noise limit like 40 dBA

Nevertheless, this is good feedback for ITU-T Q13/12, and as the scope of Recommendation ITU-T G.1091 mentions:

The list of parameters and requirements is not to be considered as an exhaustive list, and it is expected that this Recommendation will be updated as more experience is gained with telepresence systems.

ITU-T Q13/12 welcomes further input from ITU-T SG16, for future upgrades of G.1091.

_______________________