| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4772 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | H.TPS-AV: Signalling of sendingLoudnessRating |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

Clause 7.1.3.5 defines the sendLoudnessRating parameter, which indicates the acoustic to electric transfer function of the Telepresence audio stream.  Signalling of this parameter is presently TBD.  Polycom proposes changing this to “Not Signalled”.

# Introduction

The three purposes for specifying sendLoudnessRating are all related to the need to set the microphone preamplifiers used in a telepresence system properly.

The resulting digital audio signal is then converted by the telepresence system to the appropriate nominalAudioLevel for the receiving device.  In the case of other telepresence systems, that level is defined as -26 dBov in clause 7.1.3.3.  However, non-telepresence devices may require that audio be sent using a different nominalAudioLevel.

The current text of H.TPS-AV also specifies that the nominalAudioLevel sent to another telepresence system shall be -26 dBov, and simultaneously specifies that the sendLoudnessRating shall be (+ 13–Fs) dB.  These specifications in combination create needless confusion for implementers.

# Discussion

The sendLoudnessRating parameter is useful when setting up a telepresence room, especially in cases where the room equipment is not all supplied by the same vendor. It allows the microphone preamplifier gains to be set to produce the electrical levels the telepresence system expects.  Since it is useful, Polycom sees value in standardizing the value and its associated measurement procedure in H.TPS-AV.

However, the audio level sent to other systems is defined in clause 7.1.1.3 (nominalAudioLevel.  That nominal level can be changed on a call-by-call basis, depending on the nominal audio level expected by the receiving system.   If that level is unknown, the telepresence system should use a nominalAudioLevel of -26 dBov.

Therefore, when rendering or mixing audio, there is no need for the receiving system to know the sending system’s sendLoudnessRating.  It is sufficient to know the nominalAudioLevel.

We also believe that the text in 7.1.3.5 should make it clear that a sendLoudnessRating of (+ 13–Fs) dB in fact is consistent with a nominalAudioLevel of -26 dBov.

# Conclusion

We propose that the Signalling Mechanism be changed to “Not Signalled” per the corrections below.

We also propose that a small modification to the text in 7.1.3.5 to make it clear to implementers that there is no conflict between clause 7.1.3.3 and clause 7.1.3.5.

[Begin Correction]

#### 7.1.3.5 	Sending Loudness Rating

| Identity: | sendLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the acoustic to electric transfer function of the Telepresence audio stream. |
| Format: | Numeric, in dB |
| Possible Values: | +4 to +16 |
| Default: | (+ 13–Fs) |
| Reference: | - |
| Signalling Mechanism: | Not Signalled |

[End Correction]

[Begin Correction]

When the nominalAudioLevel is -26 dBov, Telepresence systems shall send audiowith a SendLoudnessRating of (+ 13–Fs) dB, measured in accordance with [ITU-T P.300].

[End Correction]

[Begin Correction]

### I.1.3	Audio parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| dynamicAudioLevel | N | Y | See [IETF RFC 6464] |
| sendLoudnessRating | N | N | Not signalled. |
| weighted Microphone-Loudspeaker Coupling Loss | N | N | Not signalled. |

[End Correction]

________