| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4774 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16, 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | Support for RTP Multiplexing in H.245 |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

Polycom suggests that the better support for RTP multiplexing in H.245 systems is needed.

# Introduction

There are several mechanisms for multiplexing media streams onto a single UDP port.  These include the multiplexing mechanism in H.460.19, the Multiplexed stream capabilities defined in H.245 B.2.2.12, as well as IETF BUNDLE, SSRC and payload type multiplexing.  In addition RFC 5761 describes the multiplexing of RTP and RTCP traffic on a single port.

# Discussion

The increased use of media relays and the increased support for multiple media streams in videoconferencing systems has resulted in more adoption of RTP multiplexing.  Reducing the number of UDP ports used in a call simplifies NAT traversal and improves scalability of the traversal servers.

We also believe that there might be some need to allow direct reception of media streams using the IETF BUNDLE protocol when using non-transcoding gateways, and perhaps need to support ICE.

While more analysis and contributions are clearly needed, Polycom believes many of these needs can be addressed with minimal changes to existing signalling.  For instance, the addition of an optional SSRC field in H2250LogicalChannelParameters, RedundancyEncodingElement, MultiplePayloadStreamElement, H2250LogicalChannelAckParameters would facilitate SSRC signalling.

# Conclusion

Polycom believes that improved support for RTP multiplexing is needed in H.323 systems, and encourages future contributions on this topic from Q2 experts.  We also suggest that care needs to be taken in ongoing work (particularly H.245-DTLS) to ensure that H.323 systems are not locked into a single multiplexing method.

__________________