| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4775 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | All/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteurs for Questions 2, 3, 5 and 21/16 |  |  |  |
| Title: | Report for Joint Rapporteur’s Meeting |  |  |  |
| Purpose: | Report |  |  |  |

AVD- and TD-numbered documents referred to are available at:

http://ftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/1506_Che.html

1	General	1

2	Question 3/16 – Multimedia Gateway Control Architectures and Protocols	4

3	Question 2/16 – “Packet-based conversational multimedia systems and functions” and Question 5/16 – “Telepresence Systems”	19

4	Question 21/16 – Multimedia framework, applications and services	26

___________________

# General

This joint Rapporteur meeting of Questions 2, 3, 5 and 21/16 was held from 8 - 12 June 2015 in Chengdu China, hosted by the University of Electronic Science and Technology of China (UESTC). There were a total of 18 participants present at the meeting. The meeting organizer for the joint Rapporteur meeting of Questions 2, 3, 5 and 21/16 was Mr Christian Groves and the document manager was Mr Paul Jones.

The Rapporteurs want to extend their thanks to the UESTC for hosting the meeting and making the arrangements necessary for us to have a productive meeting.

The meeting participants reviewed a total of 105 input documents. This included 14 incoming liaison statements. There were 30 TDs. The objective of this meeting was to progress the ongoing work in each Question. The specific objectives for each Question are listed in the sub-sections designated for each Question.

There are 3 Annexes at the end of this report. Annex A contains a description of a new work item in Q21/16. Annex B contains the list of participants. Annex C lists the documentation for this meeting.

All AVD- and TD-numbered documents referred to in this report are available at the WP 1/16 Questions repository at http://ftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/.

## Questions and Rapporteurs for this meeting

| 2/16 | H.323 real-time multimedia system (Mr Paul Jones, Cisco/USA) Note: Mr Stephen Botzko acted as Rapporteur for the meeting. Q2 and Q5 meet jointly for the entire meeting. |
| --- | --- |
| 3/16 | Multimedia gateway control architectures and protocols (Mr Christian Groves, Huawei/China) |
| 5/16 | Telepresence systems (Mr Stephen Botzko, Polycom/USA) |
| 21/16 | Multimedia framework, applications and services (Mr Noah Luo, Huawei, China & Kai Wei, MIT, China) |

## Time Schedule

Meetings began each morning at 09:00 and normally ended at 17:30.

The table below shows the meeting schedule.

| Date | Day | 1q (9:00-10:30) | 2q (11:00-12:30) | 3q (14:00-15:30) | 4q (16:00-17:30) | 5q ? |
| --- | --- | --- | --- | --- | --- | --- |
| 8-Jun | Mon | All* | Q5 | Q3 | Q3 | - |
| 9-Jun | Tue | Q2&Q5 | Q2&Q5 | Q21 | Q3 | - |
| 10-Jun | Wed | Q5¦Q21 | Q5¦Q21 | Q3 | Q3 | - |
| 11-Jun | Thu | Q2&Q5 | Q3 | Q21 | - | - |
| 12-Jun | Fri | All** | All** | All** |  |  |

*	Document allocation and logistics.  Note: Starting time of 9.00am.

**	Review of meeting reports and output documents

/	Serial session

|	Parallel session

&	Joint Session

Note 1: Liaisons were discussed in the opening session.

## Documentation for this meeting

This was an electronic meeting, with Internet access provided by the host.  All documents for the meeting were available here:

http://ftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/

## Liaison statement ownership

| AVD | Q (owner) | Title | Source | For |
| --- | --- | --- | --- | --- |
| AVD-4746 | ALL | LS/i on activities of the Focus Group on Smart Sustainable Cities [from ITU-T FG-SSC] [288 (GEN/16)] | FG SSC - LS 34R1 | C |
| AVD-4747 | ALL | LS on ITU-T study group responsibilities in WTSA Resolution 67 (Rev. Dubai, 2012) – Terms and Definitions harmonization [296 (GEN/16)] | SG2 - LS 98 | A |
| AVD-4748 | ALL | LS on Telecommunication Management and OAM Project Plan [297 (GEN/16)] | SG2 - LS 86 | A |
| AVD-4749 | ALL | LS on material on ITU inter-Sector coordination of mutual interest [to ITU-T SGs] [298 (GEN/16)] | TSAG - LS 10 | A |
| AVD-4750 | ALL | LS on Candidate topics on working methods for ITU inter-Sector coordination [to RAG, TDAG, ITU-T SGs] [299 (GEN/16)] | TSAG - LS 11 | A |
| AVD-4751 | 21 | LS/r on SNLP (reply to ASTAP-24/OUT-18) [303 (GEN/16)] | SG2 - LS 100 | I |
| AVD-4752 | 21 | LS/r on comments and requests on proposed new work item on user interface for face-to-face speech translation human factor (reply to SG16-LS099) [304 (GEN/16)] | SG2 - LS 101 | I |
| AVD-4753 | 21 | LS on on-going work item of ITU-T Q4/2 on user interface for face-to-face speech translation considering human factors [305 (GEN/16)] | SG2 - LS 102 | I |
| AVD-4754 | ALL | LS/i/r on collaboration between ITU-T and testing laboratories for ITU C&I programme (COM-16-LS94) [308 (GEN/16)] | ITU-T SG11 | A |
| AVD-4755 | ALL | LS/i on approval of SG11 Guidelines on "Testing Laboratories recognition procedure" (ex. Q.TL-rec-pro) and first meeting of ITU-T CASC [from ITU-T SG11] [309 (GEN/16)] | ITU-T SG11 | A |
| AVD-4756 | ALL | LS/i on invitation to update the information in the cloud computing standards roadmap [from ITU-T SG13] [312 (GEN/16)] | ITU-T SG13 | A |
| AVD-4757 | ALL | LS/i on establishment of new ITU-T Focus Group "IMT-2020" [from ITU-T SG13] translation considering human factors [from ITU-T SG 2] [313 (GEN/16)] | ITU-T SG13 | I |
| AVD-4758 | ALL | LS/i on Final Deliverables of the Focus Group on Smart Sustainable Cities and Proposal of a New Study Group [from ITU-T FG-SSC] [314 (GEN/16)] | ITU-T FG-SSC | I |
| AVD-4759 | 5 | LS/i/r concerning Recommendation ITU-T G.1091 (COM16-LS-134) [from ITU T SG 12] [316 (GEN/16)] | ITU-T SG 12 | C |

Note: A – Action, C – Comment, I - Information

## Topics of General Nature

The liaison statements addressed to “all” were discussed in the opening session. The results of the discussion can be found in TD-22. No liaison responses were generated from the general discussions or from the individual questions.

## Future meetings

Questions 2, 3, 5 and 21/16 are planning to meet at the next Study Group 16 meeting 12-23 October 2015.

# Question 3/16 – Multimedia Gateway Control Architectures and Protocols

Q3/16 met for 7 quarters to review some 77 documents. Q3/16 continued work on our existing work items as well as adding a new work item for H.Sup 14 (rev). Q.3 did not produce any liaison statements.

The Rapporteur would like to thank the University of Electronic Science and Technology of China (UESTC) for hosting the meeting and for providing the facilities, the contributors to the Q.3 work and especially the editors for their work in preparing the work items.

Question 3/16 was addressed in 7 sessions during the meeting under the chairmanship of Christian Groves (Huawei). The group adopted the agenda in TD-03.

## Objectives

The objectives for this meeting were:

- Coordinate with other SDOs, Questions, or Study Groups

- Progress work on:

  - H.248.41 (Rev.)

  - H.248.50 (Rev.)

  - H.248.66 (ex H.248.RTSP)

  - H.248.74 (ex H.248.MRCP)

  - H.248.WEBRTC

  - H.248.RTPMUX

  - H.248.SCTP

  - H.248.CLOUD

  - H.248.SHAPER

  - H.248.PAURES

  - H.248.SIPREC

  - H.248.CodecSDPprofile

  - H.248 Sub Series IG

  - H Series Sup. 2

  - H Series Sup. 13 Release 2

  - H Series Sup.Openflow

  - H Series Sup.ALTC

- Consider new material.

## Documentation

The following documents were examined:

- AVDs:  4662, 4663, 4664, 4665, 4666, 4667, 4668, 4669, 4670, 4672, 4673, 4674, 4675, 4776, 4677, 4678, 4679, 4680, 4681, 4682, 4683, 4684, 4685, 4686, 4687, 4688, 4689, 4690, 4691, 4692, 4693, 4694, 4695, 4696, 4697, 4699, 4700, 4701, 4702, 4703, 4706, 4707, 4708, 4717, 4718*, 4721, 4722, 4723*, 4724, 4725, 4726, 4727, 4728, 4729, 4730, 4731, 4733, 4740, 4742, 4743, 4744, 4760 (62 contributions)

- TDs: 3, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 23, 24, 29 (15 documents)

*Indicates late document.

## Report of interim activities

There were no interim meetings.

H.248 related work also progresses by on the IETF MEGACO e-mail reflector (megaco@ietf.org). Documentation is found in external FTP area for Q3/16 at: http://ftp3.itu.ch/av-arch/avc -site.

## Discussions

### Incoming liaison statements

None.

### Co-ordination

None.

### General

| AVD-4717 | Draft Q3 question text for the 2017-2020 Study Period | Q3 Rapporteur |
| --- | --- | --- |

This contribution was discussed and was accepted with the following comments:

- ITU-T SG11 should be modified to indicate work on SDN and Cloud.

- ITU-T SG13 should be updated to NGN-e and Future Networks.

- ETSI NFV should be added.

- AVTEXT, TLS and XRBLOCK should be added to the IETF WGs.

The updated question text is contained in TD-23. It was agreed to provide this text to the TSB.

### H.248 Sub-Series Implementors Guide

| AVD-4677 | H.248.x-IG "H.248 Sub-series Implementors’ Guide" (Rev.): Input draft | Editor H.248.x IG |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting.

### Revised H.248.41 IP domain connection package

| AVD-4673 | H.248.41 “Gateway control protocol: IP domain connection package” (Rev.): Output draft | Editor H.248.41 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting. The delegates believed that it is ready to be Consented.

### H.248.50 NAT traversal toolkit packages

| AVD-4662 | H.248.50 "Gateway control protocol: NAT traversal toolkit packages" (Rev.): Input draft Ed. 0.13 | Editor H.248.50 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4724 | H.248.50 (Rev): Ready for completion? Status of open items. | Alcatel-Lucent International |
| --- | --- | --- |

The contribution was discussed and the proposals were accepted with the following comments:

- Clause L.4 should be removed.

| AVD-4725 | H.248.50 (Rev): Item #1 "Trickle ICE" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

| AVD-4726 | H.248.50 (Rev): Item #2 "Dual stack ICE" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

| AVD-4727 | H.248.50 (Rev): Item #3 "ICE for TCP" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- The contributor’s note in clause 8.2.6.0 should be turned into an editor’s note

- There was some discussion where the delegates thought that NAT should only be applied to the de-aggregation stream. Further work is needed to look at aggregation stream semantics to see how NAT would apply to the components.

| AVD-4728 | H.248.50 (Rev): Item #4 "Keep alive support" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Clause 9.1.6 it was confirmed variable Tr related to the rti parameter.

| AVD-4743 | H.248.50 (Rev): Support of "ICE restarts"? | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

There was a discussion regarding whether H.248.50 will be ready for Consent in October. The main issue raised was the support of Trickle ICE and TCP and that these were required for WebRTC. There is also a dependency from 3GPP on this work.

The output draft revised ITU-T H.248.50 can be found in TD-10.

### H.248.66 (ex H.248.RTSP) Packages for RTSP and H.248 interworking

| AVD-4674 | H.248.66 (ex H.248.RTSP) "Packages for RTSP and H.248 interworking" (New): Input draft. | Editor H.248.66 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting. This work item is ready for Consent pending an RFC number for RTSP2.0.

### H.248.74 (ex H.248.MRCP) Media Resource Control Enhancement Packages

| AVD-4675 | H.248.74 (ex H.248.MRCP) "Media resource control enhancement packages" (New): Input draft | Editor H.248.74 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting. This work item is ready for Consent pending an RFC number for RTSP2.0.

### H.248.77 Secure real-time transport protocol (SRTP) package and procedures

| AVD-4676 | H.248.77 "Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures" (Rev.): Input draft. | Editor H.248.77 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4692 | H.248.77 (Rev): proposal for srtp package version 2 | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Interworking between SDES and DTLS/SRTP needs to be considered. An editor’s note should be added to this effect.

| AVD-4744 | H.248.77 (Rev): Discussion - H.248 package(s) design versus SRTP key management schemes | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed. It was agreed that there could be multiple key management schemes but it was felt that it was best to concentrate on SDES and DTLS/SRTP in the short term. The Privacy Enhanced Routing (PERC) work in the IETF may be a source of future work.

The output draft revised ITU-T H.248.77 can be found in TD-16.

### H.248.88 RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations

| AVD-4721 | H.248.88: Living list item "Lip synchronization issue in audio/video RTP topologies".. | Alcatel-Lucent |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Another possible solution may be to use the grouping framework with the LS (Lip sync) semantic.

The output draft for the living list can be found in TD-24.

### H.248.92 Stream endpoint interlinkage package

| AVD-4682 | H.248.92 Release 2: additional support of "intra-Termination interlinkage" (seplink package version 2). | Alcatel-Lucent |
| --- | --- | --- |

This contribution was discussed and it was not agreed. It was agreed that SCTP was a use case but there was some concern that as this layering was not connection oriented the use of SEPlink may not be appropriate. The intra-termination interlinkage is probably only applicable to release procedures. Should limit the interlinkage to connection oriented layers and de-aggregation streams.

Further contribution invited.

### H.248.WEBRTC Guidelines on H.248 capabilities for Web-based real-time communication services

| AVD-4663 | H.248.WEBRTC "Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles" (New): Input draft Ed. 0.17 | Editor H.248.WEBRTC |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4693 | H.248.WEBRTC: clause 8 - update of requirements | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Figure 1: A note should be added that the DTLS and SRTP key exchange use the same UDP port.

- R-8.1.2/5: Replace the note with the following text “NOTE – This is an optional capability for the basic WebRTC call establishment. Its use relates to the network operator policy concerning IP version preferences.”

- Clause 6.3.3: Replace second bullet with “SRTP key exchange using DTLS with SRTP-over RFC 4571 based framing.”

| AVD-4694 | H.248.WEBRTC: update of profile guideline sections | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Clauses 11.7.3 and 11.7.5: Termination type “ALL except ROOT” should be changed to “IP”.

| AVD-4731 | H.248.WEBRTC: Distributed ToIP endpoints in WebRTC | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- There was a discussion on why interworking between RTP and SCTP for T.140 was a problem. It was clarified that this was because of the tight coupling between the behaviour of T.140 (in the case of idle periods) to RTP behaviour. It was thought it could be solved by a back-to-back T.140 agent however further analysis is needed.

- It was thought that the IETF RTCWeb gateway draft (draft-ietf-rtcweb-gateways-00) would be the applicable place to discuss this issue. Delegates should contribute to the work in the IETF.

| AVD-4733 | H.248.WebRTC/H.248.SCTP: Autonomous SCTP stream reset | Huawei |
| --- | --- | --- |

This contribution was discussed and the proposal with option B was accepted.

The output draft revised ITU-T H.248.WEBRTC can be found in TD-11.

### H.248.RTPMUX H.248 support for RTP multiplexing

| AVD-4679 | H.248.RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing” (New): Input draft | Editor H.248.RTPMUX |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4684 | H.248.RTPMUX: terminology updates due to alignment with IETF "RTP taxonomy" draft | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Clause 1: Should be updated to read “Communication services with the use of the Real Time Protocol (RTP) as application level framing protocol use normally a native IP-based transport mechanism [IETF RFC 3550], which results in using two separate UDP ports for the RTP and RTCP traffic components.”

| AVD-4685 | H.248.RTPMUX: update proposals to clauses 6 to 9 | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Clause 6.5: Should be updated to read “UDP payload multiplexing represents the case:”

- Clause 6.5: A new bullet 1 should be added “A single UDP connection is used.”

- Clause 7.1: Remove first bullet.

- Clause 7.1: Change “scenario” to “reason”.

- Clause 8.2: Change to “In order to multiplex several RTP streams the MGC shall set the…”

The output draft revised ITU-T H.248.RTPMUX can be found in TD-17.

### H.248.STGROUP H.248 Stream grouping and aggregation

| AVD-4680 | H.248.STGROUP “Gateway Control Protocol: H.248 Stream grouping and aggregation” (New): input draft | Editor H.248.STGROUP |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4686 | H.248.STGROUP: consistency check of terminology (clauses 3 and 6) | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were agreed.

| AVD-4687 | H.248.STGROUP: clause L.1 – discussion and update proposal for "use case #1: alternate speech-text telephony" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Figure L.1 – Need to explain that the grey arrows indicate not used H.248 streams and the first box on the bi-directional case should be removed.

- It was agreed to include this use case in an Appendix.

| AVD-4688 | H.248.STGROUP: clause L.2 – discussion and update proposal for "use case #2: WebRTC" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- The motivation for the use case was discussed. The motivation was to minimise the size of the stream descriptor to minimise the effort in processing the descriptor when changes were made (e.g. for ICE or DTLS updates). It was agreed to add further motivation to the use case.

| AVD-4689 | H.248.STGROUP: clause L.3 – discussion and update proposal for "use case #3: Telepresence media server" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed. It was noted that there is already an IETF defined semantic for “CLUE” that doesn’t relate to multiplexing. It was thought that the use case in clause L.2 already covered the use of CLUE via WebRTC datachannel. As such it was agreed to remove use case L.3.

| AVD-4690 | H.248.STGROUP: clause L.4 – discussion and update proposal for "use case #4: ALTC" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- The ALTC semantic should be replaced by the ANAT semantic as ALTC provides the same functionality ANAT over the H.248 interface.

- The proposed editor’s note under figure 5 is to be removed. It was thought that the MG would have to handle the case where the two FID components use the same or different SSRCs. What would be the impact to the MG?

| AVD-4691 | H.248.STGROUP: clause L.5 – discussion and update proposal for "use case #5: SCTP transport modes" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- There was some discussion regarding how component streams fit in. There were concerns with allowing multiple levels of de-aggregation cascading in this version of the Recommendation.

- Figure L.11: The lines representing stream S2 should be modified to indicate that it’s a component stream.

| AVD-4706 | H.248.STGROUP: Cascaded aggregation and de-aggregation streams | Huawei |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- There was general agreement with the conclusions.

- Clause 2.2.1: remove “One or more of these component streams are then used as part of a stream aggregation”.

- Figure 3: groupse (FID,2,6) should be (FID,1,5)

- Add to clause 2.2.2 Conclusion “...and would need to indicate that an aggregation component may be an internal stream.”

| AVD-4707 | H.248.STGROUP: Stream level interactions | Huawei |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Policy rule enforcement, multiple levels could apply to aggregation and de-aggregation.

- Need to capture that there shouldn’t be conflict between component and aggregation and de-aggregation stream properties and properties values.

- There was a concern with having properties set on aggregation streams.

- An approach may be to only specify behaviour for mandatory parameters.

- Based on the discussion of AVD-4760 it was decided to incorporate the proposal.

| AVD-4760 | H.248.STGROUP Miscellaneous updates | Ericsson |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- The relationship of the proposals to AVD-4707 was discussed. It was agreed that the procedures defined in AVD-4760 should be the primary goal in minimising property conflict. As such the text provided in clauses 8.6.5 and 8.6.6 should be the base line. However it was agreed to allow properties on both aggregation and de-aggregation and their component streams. In such case the procedures from AVD-4707 would apply.

- It was agreed to remove text from the draft Reommendation indicating that aggregation streams have a bothway default.

- It was agreed for the editor to harmonise the proposals from AVD-4760 and AVD-4707.

The output draft revised ITU-T H.248.STGROUP can be found in TD-18.

### H.248.SCTP H.248 support for control of SCTP bearer connections

| AVD-4740 | H.248.SCTP "Gateway control protocol: H.248 support for control of SCTP bearer connections" (New): Input draft Ed. 0.11 | Editor H.248.SCTP |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4730 | H.248.SCTP: clause 11.3 – SCTP over a virtual DTLS connection | Alcatel-Lucent International |
| --- | --- | --- |

There were some concerns that introducing the figure y.1 would be misleading as while the use of multiple SCTP associations over a single DTLS session is theoretically possible no mechanism exists to signal its use. As such the proposals were not agreed. Instead it was agreed to incorporate the following text:

“11.2 Isolation of DTLS in case of SCTP over DTLS

11.2.1 Single SCTP association over a DTLS connection

{Editor’s note: See living list H.248.STGROUP}

11.2.2	Grouping of multiple SCTP associations over the same DTLS connection

The DTLS encapsulation of SCTP traffic supports multiplexing multiple SCTP associations over a single DTLS connection" (see clause 6.1/[draft-ietf-tsvwg-sctp-dtls-encaps]). The SCTP port element is used for multiplexing and demultiplexing the SCTP associations. However this is not currently supported by this Recommendation as there is no mechanism (e.g. via SDP) to indicate its use.”

The text proposal regarding 11.3 was accepted.

| AVD-4742 | H.248.SCTP Miscellaneous updates | Ericsson |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Editor’s note under fig.12 remove “…It should be noted that there is a use case where a single DTLS connection carries multiple SCTP associations according to draft-ietf-tsvwg-sctp-dtls-encaps.”

- Clause 5.3: The term “SCTP bearer” should be changed to “SCTP stream”.

- Clause 5.5: Delete the last paragraph starting “The Stream Endpoint (SEP) subject…”

The output draft revised ITU-T H.248.SCTP can be found in TD-29.

### H.248.CLOUD Cloudification of packet gateways

| AVD-4664 | H.248.CLOUD "Gateway control protocol: Cloudification of packet gateways" (New): Input draft Ed. 0.6 | Editor H.248.CLOUD |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4700 | H.248.CLOUD: update proposals for clauses 6 and 7 incl. a new Appendix I | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- There were some concerns with clause I.3 mentioning service chains with relation to NFV. It was agreed to add an editor’s note to remove the notion of service chaining and replace it with another concept.

- Update text under figure 1: “Figure 1 uses the metaphor of a multi-dimensional space where each specific evolution path reflects a "directional" change, thus it does not represent a divergence…”

- In general change “reactionless” to “no impact” or similar.

- It was questioned in whether Appendix I needs to use mathematic functions to state the methodology. An editor’s should be added to consider if this can be simplified.

| AVD-4701 | H.248.CLOUD: new Appendix II related to an example classification of low-level gateway functions | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

| AVD-4718 | H.248.CLOUD: mapping an H.248 packet gateway (MG / MGC) to ETSI NFV / VNF model | Alcatel-Lucent International |
| --- | --- | --- |

Withdrawn.

The output draft revised ITU-T H.248.CLOUD can be found in TD-12.

### H.248.SHAPER H.248 support for traffic shaping

| AVD-4665 | H.248.SHAPER "Gateway control protocol: H.248 support for traffic shaping" (New): Input draft Ed. 0.6 | Editor H.248.SHAPER |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting.

### H.248.PAURES Support of remote media pause and resume

| AVD-4678 | H.248.PAURES “Gateway control protocol: Support of remote media pause and resume” (New): Input draft | Editor H.248.PAURES |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4695 | H.248.PAURES: H.248 use cases and design considerations (clauses 1 to 7) | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Clause 7.3 remove editor’s note.

- Clause 8.2.1 Description last sentence change to “It is applicable to an H.248 stream endpoint that realizes the RTP sender function.”

- Clause 8.6 bullet 2 – should say RTP receiver

- Clause 8.6 bullet 3 - … towards a remote RTP sender

- Clause 8.6 bullet 4: Update “MGC initiated local pause/resume where the MGC receives the pause and resume request results from MG.”

- It was agreed that the package should be used primarily for conversational services therefore the MG should silently discard any packets received in a pause state, i.e. no buffering. This should be made clear in the scope and appendix.

| AVD-4696 | H.248.PAURES: Part II – Package review | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted with the following comments:

- Clause 8: Reword text regarding P/R signalling “P/R-signalling procedures in both directions are disjoint.”

- Clause 8.1.1: The proposed note should be removed.

| AVD-4697 | H.248.PAURES: Part III – Possible interactions with RTP topologies | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

The output draft revised ITU-T H.248.PAURES can be found in TD-19.

### H.248.SIPREC Profile guidelines for H.248 supported SIP-based Media Recording

| AVD-4667 | H.248.SIPREC: "Gateway control protocol: Profile guidelines for H.248 supported SIP-based Media Recording" (New): Input draft Ed. 0.5 | Editor H.248.SIPREC |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4729 | H.248.SIPREC: update of RTP terminology | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

The output draft revised ITU-T H.248.SIPREC can be found in TD-14.

### H.248.CodecSDPprofile Profiling codec configurations via SDP in H.248 profiles

| AVD-4668 | H.248.CodecSDPprofile: "Gateway control protocol: Profiling codec configurations via SDP in H.248 profiles – Guidelines and examples" (New): Input draft Ed. 0.5 | Editor H.248. CodecSDPprofile |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting.

### H Series Supplement 2 Release 16 Packages Guide

| AVD-4681 | H-Sup.2 “H.248.x sub-series packages guide – Release 16” (Rev.): Input draft | Editor H. Sup.2 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4708 | H.Supp.2: Updates | Huawei |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

The output draft revised ITU-T H.Sup 2 can be found in TD-20.

### H.Sup.13 (Rel.2) Common ITU-T H.248 terminology - Release 2

| AVD-4669 | H.Sup.13: "Gateway control protocol: Common ITU-T H.248 terminology – Release 2" (Rev.): Input draft Ed. 0.2 | Editor H. Sup.13 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4699 | H.Sup13: Gateway term | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and the proposals were accepted.

The output draft revised ITU-T H.Sup 13 can be found in TD-15.

### H.Sup.14 SDP codepoints for gateway control - Release 2

| AVD-4683 | H.Sup14: update proposal for next version of "draft-schwarz-mmusic-sdp-for-gw" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and was noted. The IETF draft has to be progressed in the IETF. Delegates were encouraged to participate in the work there.

| AVD-4722 | H.Sup14: IG item on "IANA registration process" | Alcatel-Lucent International |
| --- | --- | --- |

This contribution was discussed and was agreed to create a work item for H.Sup.14 release 2. This proposed text is to form a baseline for this work.

Mr.Schwarz (Alcatel-Lucent) volunteered to be the editor.

The output draft revised ITU-T H.Sup 14 can be found in TD-13.

### H.Sup.OpenFlow: Protocol evaluation – OpenFlow versus H.248

| AVD-4666 | H.Sup.OpenFlow: "Gateway Control Protocol: Protocol evaluation of OpenFlow versus ITU-T H.248" (New): Input draft Ed. 0.10 | Editor H.Sup.OpenFlow |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting.

| AVD-4723 | H.Sup.OpenFlow: update proposal | Alcatel-Lucent International |
| --- | --- | --- |

Withdrawn

### H.Sup.ALTC: Alternate IP connectivity support by H.248 gateway control protocol

| AVD-4670 | H.Sup.ALTC "Gateway Control Protocol: Alternate IP connectivity support by H.248 gateway control protocol" (New): Input draft Ed. 0.3 | Editor H.Sup.ALTC |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes proposed there was no output draft from the meeting.

### New Material

None.

## Intellectual property statements

No IPR statements were received at this meeting.

## Outgoing liaison statements

No outgoing liaisons were prepared.

## Work programme

### Future work

E-mail correspondences pertaining to the activities of this group are routinely conducted using the e-mail reflector currently hosted by Packetizer. Those wishing to subscribe or unsubscribe to this email reflector should visit the Packetizer mailing list web page at:

http://lists.packetizer.com/mailman/listinfo/itu-sg16

E-mail correspondences pertaining to H.248 are routinely conducted using the IETF MEGACO e-mail reflector. Those wishing to subscribe or unsubscribe to this email reflector should visit the IETF Megaco Mailing List web page at:

https://www.ietf.org/mailman/listinfo/megaco

E-mails regarding H.248 should be sent to megaco@ietf.org.

Rapporteur group documentation can be found in the external FTP server for Q3/16 located at http://ftp3.itu.int/av-arch/avc-site.

The up-to-date work programme for Q3/16 can be checked online at: http://itu.int/ITU-T/workprog/wp_search.aspx?q=3/16.

The currently open work items for Q3/16 are as follows:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| H.248 IG | H.248 Sub-series Implementors' Guide | C.Groves | 10/2015 | AVD-4677 |
| H.248.41 (Rev.) | IP domain connection package | C.Groves | 10/2015 | AVD-4673 |
| H.248.50 (Rev.) | NAT traversal toolkit packages (Revised) | A.Schwarz | 10/2015 | TD-10 |
| H.248.66 (ex H.248.RTSP) | Packages for RTSP and H.248 interworking (New) | Y.Lin | 10/2015 | AVD-4674 |
| H.248.74 (ex H.248.MRCP) | Media Resource Control Enhancement Packages (New) | T.Young | 10/2015 | AVD-4675 |
| H.248.77 (Rev.) | Secure real-time transport protocol (SRTP) package and procedures (Rev) | C.Groves | 10/2015 | TD-16 |
| H.248.WEBRTC | Guidelines on the use of H.248 capabilities for Web-based Real-Time Communication Services in H.248 Profiles (New) | A.Schwarz | 10/2015 | TD-11 |
| H.248.RTPMUX | Gateway control protocol: H.248 Support for RTP multiplexing | T.Young | 10/2015 | TD-17 |
| H.248.STGROUP | Gateway control protocol: H.248 Stream grouping and aggregation | C.Groves | 10/2015 | TD-18 |
| H.248.SCTP | Gateway control protocol: H.248 support for control of SCTP bearer connections (New) | A. Martin De Nicolas | 10/2015 | TD-29 |
| H.248.CLOUD | Gateway control protocol: Cloudification of packet gateways | A.Schwarz | 2016 | TD-12 |
| H.248. SHAPER | Gateway control protocol: H.248 support for traffic shaping | A.Schwarz | 2016 | AVD-4665 |
| H.248.PAURES | Gateway control protocol: Support of remote media pause and resume | C.Groves | 10/2015 | TD-19 |
| H.248.SIPREC | Gateway control protocol: Profile guidelines for H.248 supported SIP-based Media Recording | A.Schwarz | 2016 | TD-14 |
| H.248.CodecSDPprofile | Gateway control protocol: Profiling codec configurations via SDP in H.248 profiles | A.Schwarz | 2016 | AVD-4668 |
| H.Sup.2 (Rev.) | H.248.x sub-series packages guide-Release 16 (Revised) | C.Groves | 10/2015 | TD-20 |
| H.Sup.13 (Rev.) | Gateway control protocol: Common ITU-T H.248 terminology – Release 2 | A.Schwarz | 2016 | TD-15 |
| H.Sup.14 (Rev.) | Gateway control protocol: SDP codepoints for gateway control - Release 2 | A.Schwarz | 2016 | TD-13 |
| H.Sup.Openflow | Technical report: Protocol evaluation-OpenFlow versus H.248 (New) | A.Schwarz | 2016 | AVD-4666 |
| H.Sup.ALTC | Alternate IP connectivity support by H.248 gateway control protocol (New) | A.Schwarz | 10/2015 | AVD-4670 |

### Future meetings

Geneva, Switzerland 12-23 October 2015

# Question 2/16 – “Packet-based conversational multimedia systems and functions” and Question 5/16 – “Telepresence Systems”

The joint experts meeting of Question 2/16 (Packet-based conversational multimedia systems and functions) and 5/16 (Telepresence Systems) was held in Chengdu, China during the week of 8 - 12 June 2015.  The meeting was chaired by the Q5/16 Rapporteur, Mr. Stephen Botzko. Q2, 3, 5, and 21/16 met concurrently and joint sessions were held among the Questions.

Q2 and Q5 thank the University of Electronic Science and Technology of China (UESTC), for providing the excellent meeting facilities and for their generous hospitality.

## Objectives

The objectives for this meeting were to:

- Coordinate with other Questions

- Progress Topics related to

  - H.TPS-AV

  - H.TPS-SIG

  - H.235.DTLS

  - H.225.0

  - H.245

- Consider new material

## Documentation

Documents considered by Q2 and Q5/16 at this meeting can be found in the AVC documents area at http://ftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che.

- AVD:  4671, 4672, 4702, 4703, 4704, 4705, 4709, 4710, 4711, 4712, 4713, 4714, 4715, 4716, 4719, 4720, 4738, 4739, 4741, 4745, 4759, 4770,  4771, 4772, 4774* (25 Contributions);

- TDs: 04, 07, 21, 25a, 27, 28

Note that AVD-4774 was registered on 1 June 2015 (after the registration deadline).  Experts agreed to review this document despite the late registration.

## Review of the Meeting Agenda

The draft agenda TD-04 was accepted.

## Intellectual property statements

Members were reminded to make disclosures of known patents to the ITU Secretariat as defined in the IPR policy in a timely manner. To the question “does anyone have knowledge of patents or pending patent applications, the use of which may be required to implement Recommendations being considered?”, no answers were received.

## Incoming liaison statements

| AVD-4759 | LS/i/r concerning Recommendation ITU-T G.1091 (COM16-LS-134) [from ITU T SG 12] | SG12 |
| --- | --- | --- |

This was reviewed.  Experts felt that no response was needed at this time, but that an output liaison to SG12 should be prepared when H.TPS-AV is Consented.

## Discussions

### H.TPS-AV

| AVD-4771 | H.TPS-AV "Audio/Video Parameters for Telepresence systems" (New): Input Draft | Editor H.TPS-AV |
| --- | --- | --- |

This was accepted as baseline text.

| AVD-4709 | H.TPS-AV: Addressing open editor’s notes | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

This was accepted with editorial refinements.  In addition experts agreed to include a definition of endpoint based on the definition found in ITU-T Rec. H.323.

| AVD-4710 | H.TPS-AV: Simulcast | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

This was accepted with editorial refinements.  The text referring to ITU-T Rec. H.323 Annex B was replaced with similar text referring to ITU-T Rec. H.241.

| AVD-4772 | Signaling of sendingLoudnessRating | Polycom |
| --- | --- | --- |

## This was accepted.

| TD-25a | H.TPS-AV "Audio/Video Parameters for Telepresence systems" (New): Output Draft | Editor H.TPS-AV |
| --- | --- | --- |

This was accepted as output text, and incorporates agreed upon changes from AVD-4709, AVD-4710, AVD-4770, and AVD-4772.

### H.TPS-SIG

| AVD-4741 | H.TPS-SIG "Signalling for telepresence-enabled conferencing" (New): Input Draft | Editor H.TPS-SIG |
| --- | --- | --- |

This was accepted as baseline text.

| AVD-4711 | H.TPS-SIG: Extended audio, video and data capabilities | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

This was accepted.

| AVD-4712 | H.TPS-SIG: Editor’s notes – H.245 CLUE control capability | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

Experts agreed that only one mechanism for transport of CLUE messages was needed, and that providing two mechanisms would create interworking barriers.  After some discussion, experts concluded that the CLUE data transport would be kept, and the H.245 generic messages approach would be removed.  Text revisions to H.TPS-SIG effecting this change was developed in the meeting and incorporated into the output document.

| AVD-4713 | H.TPS-SIG: Superfluous Editor’s notes | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

This was accepted.

| AVD-4739 | H.TPS-SIG: delete two editor’s notes | ZTE |
| --- | --- | --- |

This was accepted.

| AVD-4714 | H.TPS-SIG: Multipoint Control | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

The proposed text for clause 9.1 was accepted and integrated with the text from AVD-4738.  The text proposed for clause 9.2 was accepted with editorial refinements

| AVD-4738 | H.TPS-SIG: 9 Multipoint control | ZTE |
| --- | --- | --- |

The proposed text was accepted and integrated with the proposed text for clause 9.1 from AVD-4714.

| AVD-4715 | H.TPS-SIG: H.TPS-AV parameter signalling | Huawei Technologies Co. Ltd |
| --- | --- | --- |

The text was accepted with some editorial refinement.  The text describing future use of the CLUE extension mechanism was turned into a note.  Experts agreed to add a similar note to H.TPS-AV.

| AVD-4716 | H.TPS-SIG: Security | Huawei Technologies Co. Ltd |
| --- | --- | --- |

This text was accepted with some editorial refinement.

| TD-27 | H.TPS-SIG "Signalling for telepresence-enabled conferencing" (New): Output Draft | Editor H.TPS-SIG |
| --- | --- | --- |

This was accepted as output text, and incorporates agreed upon changes from AVD-4711, AVD-4712, AVD-4713, AVD-4714, AVD-4739, AVD-4738, AVD-4715, AVD-4716, and AVD-4705.

Section 11 and Appendix 1 are currently empty.  Experts agreed to delete them at the next meeting if contributions related to these sections are not received.

### H.235.DTLS

| AVD-4672 | H.235.DTLS “H.323 security: Support of DTLS for media streams” (New): Input draft | Editor H.235.DTLS |
| --- | --- | --- |

This was accepted as baseline text.

| AVD-4702 | H.235.DTLS: Support of SRTP | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

Experts did not agree on the use of H.460.19 for multiplexing.  H.460.19 is a client-server protocol, and would require adaptation to use in the client-client case.  The remaining text was accepted with some editorial refinement.

| AVD-4719 | H.235.DTLS: Terminology update proposals | Alcatel-Lucent |
| --- | --- | --- |

This was accepted.  Text on proper termination was also prepared in the meeting (instead of an editor’s note).

| TD-21 | H.235.DTLS “H.323 security: Support of DTLS for media streams” (New): output draft | Editor H.235.DTLS |
| --- | --- | --- |

This was accepted as output text, and incorporates agreed upon changes from AVD-4702 and AVD-4719.

### H.225.0

| AVD-4745 | H.225.0 Time Synchronization Mechanism | Spranto |
| --- | --- | --- |

Experts felt that procedures were needed on the proper use of this mechanism.  supportsTimeSync appears to be a request for Time Synchronization, and not an indication of support. timeSyncronization (sic) appears to be a response to that request.  Perhaps an indication of the clock accuracy might also be useful, as it would provide a hint to the endpoint as to whether the endpoint should reset its clock to the provided time.

### H.245

| AVD-4671 | Proposed revisions to H.245 | Cisco Systems |
| --- | --- | --- |

This proposal appears to be technically equivalent to the current ASN.1, but would require revisions to track future IETF work (for instance new hash functions).  It was unclear to experts whether the proposal would actually reduce the size of the ASN.1 structure.  Interworking seems simpler in gateways with the current structure.

Experts at the meeting expressed a preference to maintain the existing structures.

| AVD-4703 | H.245 SCTP transport editor’s note | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

Experts agreed to drop the editor’s note.

Experts observed that if the use of SCTP transport is only for carriage of IETF defined protocols, then tables have little value.  If the transport is also used for ITU-T defined protocols, then it would have merit. Experts agreed that the tables can be added later on then the value is more apparent.

| AVD-4704 | H.245: DC parameter format update | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

This was accepted.

| AVD-4705 | H.245 Logical Channel Update | Huawei Technologies Co. Ltd. |
| --- | --- | --- |

Experts agreed to use Approach 2.3.  Text was developed in the meeting for both H.245 Annex U and H.TPS-SIG.

| AVD-4720 | H.245 Annex U: editorial update proposals | Alcatel-Lucent |
| --- | --- | --- |

This was accepted.

| AVD-4770 | Signaling of nominalAudioLevel | Polycom |
| --- | --- | --- |

This was accepted, with the addition of a comment to the ASN.1 specifying the units.

| AVD-4774 | Support for RTP Multiplexing in H.245 | Polycom |
| --- | --- | --- |

Experts agree there is a need for increased support of RTP multiplexing.  Some also cautioned that the approach should target specific applications, since otherwise the work could become very complex.

| TD-28 | H.245 "Control protocol for multimedia communication" (Rev.): Output text | Editor H.245 |
| --- | --- | --- |

This was accepted as output text, and incorporates agreed upon changes from AVD-4703, AVD-4704, AVD-4705, AVD-4720, and AVD-4770.

## New Material

None.

## Work Schedule

Experts discussed the current Q5 work schedule (and H.235.DTLS and H.245).  All are scheduled for Consent in October 2015.  The consensus is that RTP multiplexing is the biggest technical issue, and that is broader than CLUE/telepresence.  Since during SIP/H.323 Interworking the CLUE channel needs to be terminated by a gateway, that same gateway could also demultiplex the media.

Therefore, if experts defer work on RTP multiplexing the documents will be technically stable by October.  More contributions are solicited in order to improve the text and address any remaining editor notes.

Experts observed that there are dependencies on RFC publication in the IETF, and that those dependencies might require Consent to be postponed.

## Presentation of Reports

The draft report for Q2/16 and Q5/16 was reviewed and the experts accepted its content. The report can be found in TD-07.

## Outgoing liaison statements

No output liaison statements were prepared at this meeting.

## Work programme

### Future work

E-mail correspondences pertaining to the activities of Q2/16 and Q5/16 are routinely conducted using the e-mail reflector currently hosted by the ITU, t13sg16q5@lists.itu.int, as well as the mailing list hosted by Packetizer, sg16-avd@lists.packetizer.com.

Those wishing to subscribe or unsubscribe to e-mail reflectors hosted by ITU should follow the instructions at http://itu.int/ITU-T/edh/faqs-email.html.  Those wishing to subscribe or unsubscribe to the Packetizer reflector should follow the instructions at: http://lists.packetizer.com/mailman/listinfo/sg16-avd.

Rapporteur group documentation can be found in the external FTP server located at http://ftp3.itu.int/av-arch/avc-site.

The currently open work items for Q2/16 are as follows:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| AMS Catalogue | AMS Catalogue | Paul Jones (paulej@packetizer.com) | 2016 | TD 211/WP2 (2009-10) |
| AMS Project Description | AMS Project Description | Paul Jones (paulej@packetizer.com) | 2016 | TD 562/WP2 (2008-04) |
| AMS Working Assumptions | AMS Working Assumptions | Paul Jones (paulej@packetizer.com) | 2016 | TD 27 (2013-06) |
| H.235.DTLS | H.323 security: Support of DTLS for media streams | Jing Xiao (lennard.xiao@huawei.com) | 2015-10 | TD-21 |
| H.245 | Revised Rec. H.245 "Control protocol for multimedia communication" (Rev.) | Liang Wang (wang.liang12@zte.com.cn) | 2015-10 | TD-28 |
| H.325 (H.ams) | AMS: Advanced multimedia systems for next generation and other packet-switched networks | Paul E. Jones (paulej@packetizer.com) | 2016 | TD 671R1/WP2 (2011-11) |
| H.460.27 (ex H.460.SessionID) | End-to-End Session Identifier for H.323 Systems (New) | Paul Jones (paulej@packetizer.com | 2015-02 | TD 345/Plen (2015-02) |
| H.AMS.CIS | Common Interface Syntax | Paul Jones (paulej@packetizer.com) | 2016 | AMS-0001 (2009-12) |
| H.EMQ | A cooperative architecture for enhanced multimedia QoS/QoE (New) | Ki-Jong Koo (kjkoo@etri.re.kr) Seong-Ho Jeong (shjeong@hufs.ac.kr) | 2015-10 | TD 259/WP1 (2015-02) |
| H.supp-web-apps | Supplement describe use cases to enhancements to H.32x-series recommendations to support web applications | Stephen Botzko (Stephen.Botzko@polycom.com) | 2016 | AVD-4444 (2013-06) |
| H.323v8 | Packet-based multimedia communications system | Stephen Botzko (Stephen.Botzko@polycom.com) | 2015-10 | TD 174/WP1 (2014-06) |
| H.225.0v8 | Call signalling protocols and media stream packetization for packet-based multimedia communication systems | Paul E. Jones (paulej@cisco.com) | 2015-10 | TD 170/WP1 (2014-06) |
| H.323 Annex for ASSET | H.323 Associative Simple Endpoint Type (ASSET) | Simon Horne (s.horne@spranto.com) | 2015-10 | COM16-C.268 (2013-10) |
| H.ASSET | H.323 Associative Set (ASSET) Feature | Simon Horne (s.horne@spranto.com) | 2015-10 | AVD-4527 (2014-03) + COM16-C. C.703R1 (2015-02) |
| H.460.22 | Negotiation of security protocols to protect H.225.0 call signalling messages | Simon Horne (s.horne@spranto.com) | 2015-02 | TD 319/Plen (2015-02) |
| H.natbind | Common NAT/FW Procedures for H.300-series packet-based multimedia systems | Simon Horne (s.horne@spranto.com) | 2015-10 | AVD-4524 (2014-03) |

The currently open work items for Q5/16 are as follows:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| H.TPS-AV | Audio/Video Parameters for Telepresence systems | S. Botzko (Stephen.Botzko@polycom.com) | 2015-10 | TD-25a |
| H.TPS-SIG | Signalling for telepresence-enabled conferencing | A. Martin de Nicolas (arturo.martin-de-nicolas@ericsson.com) | 2015-10 | TD-27 |

### Future meetings

Questions 2/16 and 5/16 are planning to meet at the next Study Group 16 meeting 12 – 23 October 2015 in Geneva, Switzerland.

# Question 21/16 – Multimedia framework, applications and services

Under the chairmanship of Mr. Wei Kai, Question 21 met for four quarters during this meeting. There are 15 contributions and 4 liaison statements were received. Question 21 continued the work on existing work items with 14 contributions which were reviewed and accepted. One contribution was received but not reviewed because the author’s absence. Work items H.IQAS, F.CSVSReqs,F.VSSIReqs, H.IVSArch, H.VSMprot, H.VCDNArch, F.VCDNReqs were updated.

Question 21/16 was addressed in four sessions during the interim meeting under the chairmanship of Mr. Kai Wei (CATR of MIIT, China). The group adopted the agenda in TD-05 (2015-06).

## Objectives

The objectives for this meeting were:

- General topic and new work items

- Progress work on: H.IQAS, F.CSVSReqs,F.VSSIReqs, H.IVSArch, H.VSMprot, H.VCDNArch, F.VCDNReqs

- Plan for future work

## Documentation

The following documents were examined:

- Contributions:AVD-4773, AVD-4732, AVD-4734, AVD-4735, AVD-4736, AVD-4737, AVD-4761, AVD-4762, AVD-4763, AVD-4764, AVD-4765, AVD-4766, AVD-4767, AVD-4768, AVD-4769

- LSs: AVD-4751, AVD-4752, AVD-4753

- Agenda & Report: TD-05, TD-08b

- Outputs: TD-26, TD-31, TD-32, TD-33, TD-34,  TD-35, TD-36

## Report of interim activities

Question 21/16 did not hold an interim Rapporteur meeting.

## Discussions results

### Incoming Liaison Statements

| AVD | Q (owner) | Title | Source | For |
| --- | --- | --- | --- | --- |
| AVD-4751 | 21 | LS/r on SNLP (reply to ASTAP-24/OUT-18) [303 (GEN/16)] | SG2 - LS 100 | Information |
| AVD-4752 | 21 | LS/r on comments and requests on proposed new work item on user interface for face-to-face speech translation human factor (reply to SG16-LS099) [304 (GEN/16)] | SG2 - LS 101 | Information |
| AVD-4753 | 21 | LS on on-going work item of ITU-T Q4/2 on user interface for face-to-face speech translation considering human factors [305 (GEN/16)] | SG2 - LS 102 | Information |

Q21 noted the new work item and interested on it. Q21 will solicit contribution in the future and keep a close cooperation with SG2 on this topic. No outgoing LSs were produced at this meeting.

### Contributions

There 15 contributions were received, and 14 contributions were discussed and were accepted by Q21/16 this meeting.

|  | AVD-4773a | Proposal for new work item on Content Delivery Network- The architecture of Integration Multimedia Content Delivery Network | China Telecom, MITT, ZTE |
| --- | --- | --- | --- |

This contribution proposed to start a new work item, H.CDNFI: “Framework and interface for multimedia content delivery network (New)”. It was accepted, Chen Ge (China Telecom) was appointed as the editor, with Nie Xiuying (CATR of MIIT) and Miao Chuanyang (ZTE) as co-editors. Experts suggested that the scope of this new work item need to avoid the overlap with F.CCNMMS. The initial text of H.CDNFI can be found in TD-34.

|  | AVD-4732 | H.IQAS: Update of functional description of Intelligent Question Answering Service Framework | ETRI |
| --- | --- | --- | --- |

It was discussed and accepted with some modifications. The output document of H.IQAS as TD-26 was produced accordingly. The editor is planning to subject the draft Recommendation H.IQAS to Consent at the next SG16 meeting.

|  | AVD-4734 | H.IVSArch: proposal for updated outlines | ZTE |
| --- | --- | --- | --- |
|  | AVD-4735 | H.IVSArch: proposed content for components description | ZTE |

Those contributions were discussed and accepted. The output document of H.IVSArch was produced as TD-32.

|  | AVD-4736 | F.VCDNReqs: proposed new text for virtual storage requirement in 9.2.2 | ZTE |
| --- | --- | --- | --- |

Those contributions were discussed and accepted with some modifications. The output document of F.VCDNReqs was produced as TD-35.

|  | AVD-4737 | H.VCDNArch: propose to add new functional block description in section 7 | ZTE |
| --- | --- | --- | --- |

Those contributions were discussed and accepted with some modifications. The output document of H.VCDNArch was produced as TD-36.

|  | AVD-4761 | F.CSVSReqs: Proposal to add scenarios in clause 6 | BUPT, China Telecom |
| --- | --- | --- | --- |

It was discussed and accepted with some modifications.

|  | AVD-4762 | F.CSVSReqs: Proposal to add requirements in clause 7 | BUPT, China Telecom |
| --- | --- | --- | --- |

It was discussed and accepted with some modifications. The output document of F.CSVSReqs was produced as TD-30.

|  | AVD-4763 | F.VSSIReqs: Proposal to add scenarios in clause 6 | BUPT, China Telecom |
| --- | --- | --- | --- |

It was discussed and accepted with some modifications. Need to explain what is a single “VSS” in the future study.

|  | AVD-4764 | F.VSSIReqs: Proposal to add requirements in clause 7 | BUPT, China Telecom |
| --- | --- | --- | --- |

It was discussed and accepted with some modifications. The output document of F.VSSIReqs was produced as TD-31.

|  | AVD-4765 | Proposal to add clause 6.1 in H.IVSArch | China Telecom |
| --- | --- | --- | --- |
|  | AVD-4766 | Proposal to add reference point in clause 6.2 in H.IVSArch | China Telecom |

Those contributions were discussed and accepted with some modifications. The output document of H.IVSArch was produced as TD-32.

|  | AVD-4767 | Proposal to update clause 6.1 and 6.2 in H.VSMprot | China Telecom |
| --- | --- | --- | --- |
|  | AVD-4768 | Proposal to update clause 6.3 and add 6.4 in H.VSMprot | China Telecom |

Those contributions were discussed and accepted with some modifications. The output document of H.VSMprot was produced as TD-33.

|  | AVD-4769 | F.IEReqs: Proposed Definitions | BUPT |
| --- | --- | --- | --- |

This contribution was not discussed because the contributor didn’t attend the meeting. The author is encouraged to resubmit the contribution in the next SG16 meeting.

## Intellectual property statements

No IPR statements were received at this meeting.

## Outgoing liaison statements

No ongoing LSs were produced at this meeting.

## Work programme

### Future work

E-mail correspondences pertaining to the activities of this group are routinely conducted using the sg16-avd@lists.packetizer.com e-mail reflector hosted by Packetizer. Those wishing to subscribe or unsubscribe to this email reflector should follow the instructions at http://lists.packetizer.com/mailman/listinfo/sg16-avd.

Rapporteur group documentation can be found in the external FTP server for Q21/16 located at http://ftp3.itu.int/av-arch/avc-site.

The meeting agreed to start work on the following new work item:

- ITU-T H.CDNFI “Framework and interface for multimedia content delivery network(New)”

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| ITU-T H.CDNFI | Framework and interface for multimedia content delivery network(New) | Editor: Chen Ge(China Telecom) cheng@gsta.com Co-Editors: Nie Xiuying (CAICT) niexiuying@ritt.cn Miao Chuanyang(ZTE) miao.chuanyang@zte.com.cn | 2017 | AVD-4773a(2015-06) |

The currently open work items for Q21/16 are as follows:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| F.745 | Functional requirements for network-based speech-to-speech translation services | Chiori Hori, Shoichi Senda | 2016 | TD 263-WP1 (2015-02) |
| F.CCNMMS | Requirements and architecture for CCN-based mobile multimedia services | Seong-Ho Jeong | 2015-10 | TD 192-WP1 (2014-06) |
| F.CSVSReqs | Requirements for cloud storage of visual surveillance | Huadong Ma, Haitao Zhang | 2016 | TD-30(2015-06) |
| F.EMSarch | Energy management service requirements and architecture | Kaiyu Zhou | 2016 | TD 267-WP1 (2015-02) |
| F.MAFFReqs | Requirements for mobile animation file formats | Qing Xiong | 2016 | TD 268-WP1 (2015-02) |
| F.NBLICreqs | Requirements for Network based location information conversion for location based applications and services | Kaiyu Zhou | 2016 | TD 185-WP1 (2014-06) |
| F.PDRDReqs | Personal devices registration and discovery requirements | Aijun Wang, Kaiyu Zhou | 2015-10 | TD 191-WP1 (2014-06) |
| F.VCDNReqs | Functional requirements and architecture model for virtual content delivery networks | Zhi Qiao, Xiaoyu Wang | 2016 | TD-35(2015-06) |
| F.VSSIReqs | Requirements for visual surveillance system interworking | Huadong Ma, Haitao Zhang | 2016 | TD-31(2015-06) |
| F.VSTPIW | Scenarios and requirements of interworking between video surveillance and telepresence | Dan Peng | 2015 | - |
| H.625 | Architecture for network-based speech-to-speech translation services | Chiori Hori, Shoichi Senda | 2016 | TD 264-WP1 (2015-02) |
| H.IQAS | Intelligent question answering service framework | Miran Choi | 2015-10 | TD-26(2015-06) |
| H.IVSArch | Architecture of intelligent visual surveillance systems | Doudou Hu, Yuan Zhang | 2016 | TD-32(2015-06) |
| H.P2PVSArch | Architecture for point-to-point visual surveillance | Ning Cao, Xiaoyang Ye, Yanxia Zhang | 2015-10 | TD 261-WP1 (2015-02) |
| H.VCDNArch | Architecture of virtual CDN | Zhi Qiao, Xiaoyu Wang | 2016 | TD-36(2015-06) |
| H.VHN | Service capabilities and framework for virtual home network | Ilyoung Chong | 2015-10 | TD 260-WP1 (2015-02) |
| H.VSMprot | Signalling and protocols for mobile visual surveillance | Yanxia Zhang, Ming Feng, Duguo Liang, Xiaoyang Ye | 2015-10 | TD-33(2015-06) |
| H.CDNFI | Framework and interface for multimedia content delivery network | Chen Ge, Nie Xiuying , Miao Chuanyang | 2017 | TD-34(2015-06) |

### Future meetings

Q21/16 is not planning to hold any Rapporteur meeting before the next SG16 meeting in Geneva, Switzerland, 12-23 October 2015.

Annex A 
Q21/16 A.1 justification for proposed draft new Recommendation H.CDNFI

| Question: | 21/ | 16 | Proposed new ITU-T Recommendation | Chengdu, 8 - 12 June 2015 |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Recommendation ITU-T H.CDNFI"Framework and interface for multimedia content delivery network" |  |  |  |  |
| Base text: | TD-34 (2015-06) |  |  | Timing: | 2017 |
| Editor(s): | Editor: Chen Ge(China Telecom) cheng@gsta.com Co-editor: Nie Xiuying (CAICT)niexiuying@ritt.cn Co-editor: Miao Chuanyang(ZTE)miao.chuanyang@zte.com.cn |  |  | Approval process: | AAP |
| Scope: |  |  |  |  |  |
| This recommendation identifies the framework and interface for hierarchical multimedia content delivery network. In order to support various multimedia services, the detailed interface attributes and procedures are identified in this recommendation by focusing on the content ingestion, content distribution and content delivery aspects. |  |  |  |  |  |
| Summary: |  |  |  |  |  |
| A content delivery network (CDN) is an infrastructure of network elements operating at layer 4 through layer 7, arranged for the efficient distribution and delivery of digital content. Such content includes, but is not limited to, web pages and images delivered via HTTP, and streaming of continuous media delivered via HTTP, RTSP, RTMP, etc. CDNs typically provide services to multiple content service providers (CSPs). This Recommendation will define the main function requirements, logical architecture, content access, request redirection, media services, etc. |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| CDN related standards were studied within IETF CDI Working group around 2003. IETF CDI working group was closed in 2003 after RFC 3466 - A Model for Content Internetworking (CDI) had been released. There is an active working group CDNI within IETF, in which standards related with interworking of CDNs are studied, and several RFCs have be approved. Two work items about VCDN were set up within Q.21 of ITU-T SG 16 last year. But there is not a standard of the architecture of integration multimedia content delivery network. So we propose to set up a new work item on content delivery network- The architecture of multimedia content delivery network within Q.21/16 to standardize the architecture of multimedia content delivery (distribution) network. The current Recommendation Y.2019 has defined the architecture of content delivery functions. It is a general architecture but it is only focused on the IPTV service. Currently, many other services such as OTT may also apply this architecture but some functional blocks may need to be enhanced to support the various services. In order to support those services, some interface and protocols may also need updated correspondently. So the current context in Y.2019 may not sufficient to satisfy those new requirements. |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Non |  |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |  |
| China Telecom, CATR of MIIT, ZTE |  |  |  |  |  |

Annex B – Registered Participants

The participants for the meeting can be found in TD-9a from the meeting.

http://ftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/TD-09a.zip

Annex C – Documentation

The document list for the meeting can be found in TD-02a from the meeting.

http://ftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/TD-02a.zip

________________