| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | TD-19 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.PAURES |  |  |  |
| Title: | H.248.PAURES “Gateway control protocol: Support of remote media pause and resume” (New): output draft |  |  |  |
| Purpose: | Information |  |  |  |

1.	Introduction

This document contains the Editor's output draft for H.248.PAURES “Gateway control protocol: Support of remote media pause and resume” (New).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.PAURES Ed. 0.1 | Initial baseline text for H.248.PAURES. Based on AVD-4641 from the Seoul, November 2014 ITU-T SG16 meeting. |
| H.248.PAURES Ed. 0.2 | Input draft to the Geneva, 9-20 February 2015 meeting. TD 231/WP1: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0231/en |
| H.248.PAURES Ed. 0.3 | Output draft from the Geneva, 9-20 February 2015 meeting. Changes due to: C.731 |
| H.248.PAURES Ed. 0.4 | Input draft for the Chengdu, 8 - 12 June 2015 meeting. |
| H.248.PAURES Ed. 0.5 | Output draft for the Chengdu, 8 - 12 June 2015 meeting. Changes due to: AVD-4695, AVD-4696 and AVD-4697. |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 6 5 Conventions 7 6 Relation to other Recommendations in the ITU-T H.248 sub-series 7 7 Design considerations 8 7.1 Real-time nature 8 7.2 Message direction 8 7.3 Apply to individual sources 9 7.4 Consensus 9 7.5 Message acknowledgements 10 7.6 Retransmitting Requests 10 7.7 Sequence Numbering 10 8 RTP topology dependent RTCP handling 10 8.1 Introduction 10 8.2 Interactions with H.248.88 11 8.3 Interactions with compound and reduced-size RTCP packets 11 8.4 Gathering SSRC values, relevant for P/R-signaling, from the IP media path 11 8.4.1 Active call interactions with H.248.71 11 8.4.2 Pre-call usage of H.248.71 12 8.4.3 RTP topology "RTP source translator" (H.248.88) 12 9 Remote pause and resume package 12 9.1 Properties 13 9.1.1 Autonomous Response 13 9.2 Events 13 9.2.1 RTP Pause State 13 9.2.2 Detect Pause / Resume Request 15 9.2.3 Detect Pause / Resume Result 16 9.3 Signals 17 9.3.1 Local Pause 17 9.3.2 Local Resume 18 9.3.3 Refuse 19 9.3.4 Remote Pause 20 9.3.5 Remote Resume 20 9.4 Statistics 21 9.5 Error Codes 21 9.6 Procedures 21 9.6.1 General - SDP parameter setting 22 9.6.2 General – Event and Signal Scope 22 9.6.3 MG autonomous handling of remote requests 22 9.6.4 MGC reception of pause and resume requests 23 9.6.5 MGC initiation of remote pause and resume requests 24 9.6.6 MGC initiation of local pause and resume 24 9.6.7 MG initiation of remote pause and resume 25 9.6.8 Error handling 25 9 Package-less alternative: partial emulation service using RTCP FB messages related to Temporary Maximum Media Stream Bit Rate signalling 25 10 Examples 25 Appendix I Network use cases with H.248 media gateways in the end-to-end RTP path with Pause/Resume signaling 26 I.1 Introduction 26 I.2 Single RTP Termination 26 I.3 Dual RTP Terminations 26 I.4 Multiple RTP terminations 29 Appendix II State models for Pause/Resume signaling endpoints 31 II.1 Introduction 31 II.2 RTP pause state model at RTP sender side 31 II.2 RTP pause state model at RTP receiver side 32 Appendix III SSRC identifiers for Pause/Resume signalling endpoints 33 III.1 SSRC information in RTCP FB "Pause/Resume" messages 33 III.2 Example use case 34 Bibliography 35

List of Tables

Page

Table 1 – Allowed setting of “rempr” package signals and events 25

List of Figures

Page

Figure 1 – RTP stream Pause and Resume as feedback-based control loop 9 Figure II.1 – RTP sender (RTP source): RTP Pause State model – extended model with additional indication of H.248 signalling elements 32 Figure II.2 – RTP receiver (RTP sink): RTP Pause State model – extended model with additional indication of H.248 signalling elements 33 Figure III.1 – RTCP FB packets for Pause/Resume-signaling – SSRC related fields 34 Figure III.2 – Example use case #3b (from Appendix I) – Extended for bidirectional P/R-signalling and indication of SSRCs. 35

Draft new Recommendation H.248.xxx (ex ITU-T H.248.PAURES)

Gateway control protocol: Support of remote media pause and resume

Summary

This Recommendation allows the RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] to be supported on H.248 controlled gateways.

# 1	Scope

This Recommendation describes the support of RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] on H.248 controlled gateways. It describes the mapping of call control SDP related to pause/resume to H.248 SDP and vice versa. It also contains a package that allows several different implementation types to be supported. This allows the Media Gateway (MG) to operate in an autonomous manner by receiving, making pause/resume decisions and sending RTCP level pause/resume messages with minimal MGC involvement. A Media Gateway Controller (MGC) controlled configuration is also supported whereby the reception of pause/request requests/responses is indicated to the MGC for it to make decisions and determine a response. It also allows the MGC to initiate pause/resume requests.

It also contains considerations related to the support of RTP stream Pause/Resume in H.248 controlled gateways.

The Recommendation also details the relationship to other existing H.248.x sub-series Recommendations where pause/resume functions are supported.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.1 (12/2009), Gateway control protocol: Advanced media server packages.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.66]	Draft Recommendation ITU-T H.248.66 (xxxx), Gateway control protocol: Packages for RTSP and H.248 interworking.

[ITU-T H.248.STGROUP]	Recommendation ITU-T H.248.STGROUP (xxxx), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 3264]	IETF RFC 3264 (2002), An Offer/Answer Model with Session Description Protocol (SDP).

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 5104]	IETF RFC 5104 (2008), Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF).

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF BUNDLE]	draft-ietf-mmusic-sdp-bundle-negotiation-16 (xxxx), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[IETF STREAM PAUSE]	IETF draft-ietf-avtext-rtp-stream-pause-07 (xxxx), RTP Stream Pause and Resume.

[IETF draft taxonomy]	IETF draft-ietf-avtext-rtp-grouping-taxonomy (XX/2015), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	RTP stream (clause 2.1.10/[IETF draft taxonomy]): A stream of RTP packets containing media data, source or redundant. The RTP stream is identified by an SSRC belonging to a particular RTP session.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	P/R-signalling: the "RTP pause and resume" signalling as defined by [IETF STREAM PAUSE].

NOTE – The signalling protocol syntax is based on dedicated RTCP "feedback" packet types, the signalling protocol procedures use the associated bidirectional RTCP control flow and the signalling protocol semantics are defined in clauses 6 to 9 in [IETF STREAM PAUSE].

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AVPF | [IETF RFC 5104]: "The extended RTP profile for RTCP-based feedback" |
| --- | --- |
| CCM | Codec Control Message |
| DTMF | Dual Tone Multi Frequency |
| FB | (RTCP) Feedback (message or packet type) |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| FB | (RTCP) Feedback (message or packet type) |
| RTCP | Real-Time Control Protocol |
| RTCP FB | RTCP Feedback (message / packet type) |
| RTP | Real-Time Protocol |
| SDP | Session Description Protocol |
| SSRC | Synchronisation Source Identifier |
| TMMBN | Temporary Maximum Media Stream Bit Rate Notification [IETF RFC 5104] |
| TMMBR | Temporary Maximum Media Stream Bit Rate Request [IETF RFC 5104] |

# 5	Conventions

Elements of the ITU-T H.248 protocol model, e.g., Context, Termination, Stream, Event are represented using the first letter capitalized. Property, Event, Signal and Parameter identities are given in italics.

The suffix ".req" added to an ITU-T H.248 command name stands for a command request, while the suffix ".rep" stands for a command reply. For example "Notify.req" represents a Notify Request.

Capitalised names PAUSE, RESUME, REFUSED, PAUSED refer to Codec Control Messages (CCM) defined in [IETF STREAM PAUSE].

NOTE – The CCMs TMMBR and TMMBN [IETF RFC 5104] could alternatively be used for emulating specific pause/resume behaviour (besides the "native CCMs" [IETF STREAM PAUSE] for RTP stream pause and resume). See also clause 9.

# 6	Relation to other Recommendations in the ITU-T H.248 sub-series

Gateways utilising H.248 currently have several methods to pause the transmission of media.

[ITU-T H.248.1]: Contains the Local Control Descriptor ‘StreamMode’ property that allows the MGC to effectively stop sending a RTP media flow (i.e. change from send/receive to receive only) whilst still maintaining an RTCP control flow. To resume the flow of media the MGC can change the StreamMode back to send/receive. However this is a local MG action rather than a request to a remote endpoint. The action is also triggered from a call control level stimulus.

[ITU-T H.248.1]: The H.248 connection model (including the TopologyDescriptor) allows Terminations/Streams to be isolated from other Terminations/Streams in the Context. This allows a MGC to isolate an H.248 media stream from other input streams effectively stopping the flow of RTP media. The MGC may then move the Termination/Stream back into a Context to resume RTP media flow.

[ITU-T H.248.9]: Contains the “Play Collect” package that enables a user to request (via a DTMF digit) that the playout of media associated with a signal be paused. It also enables the user to resume the RTP media flow via a DTMF digit.

[ITU-T H.248.66]: Contains the “Signal pause package” which is a generic package that allows the MGC to request a pause in the playout of media and later resume RTP media flow. It is suited to scenarios where media playout has been initiated through the use of a H.248 Signals rather than real-time media flows. It again is a local MG action rather than a request to a remote endpoint and the action is also triggered from a call control level stimulus.

[ITU-T H.248.71]: Contains the "RTCP source description package" which allows the MGC to request the MG to monitor, extract and report SSRC values from RTP streams. Clause 8 describes how that capability may be used together with RTP stream pause and resume services.

[ITU-T H.248.88]: Describes RTCP handling by an H.248 MG in association with a particular enforced local RTP topology. Clause 8 provides more detailed information on the subject.

None of the above methods match the functionality supported by [IETF STREAM PAUSE]. [ITU-T H.248.1] and [ITU-T H.248.66] do not support user originated requests being terminated at the MG and [ITU-T H.248.9] and [ITU-T H.248.66] do not allow the pause of H.248 media streams not related to the playout of signals.

Pause/resume relates to real time media flows (as subject of conversational communication services). Therefore the resumption of an RTP media flow is based on the media at the time of the resume request rather than at the pause point. [ITU-T H.248.66] (typically applied in the context of streaming communication services) assumes that the playout occurs from the pause point unless another point is indicated in the resumption request.

# 7	Design considerations

Clause 3/[IETF STREAM PAUSE] describes the main use cases for RTP stream pause and resume. It lists a number of RTP topologies. A H.248 media gateway may act as a media sender, media receiver, mixer and/or other RTP topologies with involvement in P/R-signalling. Appendix I provides a list of example use cases from the perspective of H.248 gateways.  As such the design criteria listed in clause 4/[IETF STREAM PAUSE] should be considered when implementing RTP stream pause/resume on gateways.

## 7.1	Real-time nature

Given the real time aspects described in clause 4.1/[IETF STREAM PAUSE] a gateway should ensure prompt handling of resume requests, i.e. a MG and optionally MGC should process and act upon any requests to resume a RTP stream in an expedited manner.

Pause requests are time-dependent; however handling is not as stringent as resume requests.

## 7.2	Message direction

A gateway may initiate a local pause or resume of an RTP stream, request a remote RTP sender to pause or resume an RTP stream or respond to a request from a remote RTP receiver to pause or resume and RTP stream (see also Figure 1).

Figure 1 – RTP stream Pause and Resume as feedback-based control loop

(NOTE: the "P/R-signalling"-path is highlighted in red)

Clause 9 provides functionality for an MG to act autonomously or for it to co-ordinate with the MGC to implement RTP stream pause/resume. Furthermore it allows the specification of which pause CCM messages are used. (NOTE - there are configuration options in order to support partial implementations, see clause 9.6.1.1).

## 7.3	Apply to individual sources

Clause 4.3/[IETF STREAM PAUSE] indicates that the pause CCM messages relate to a single RTP stream, i.e. relates to a single synchronization source identifier (SSRC). This usually relates to a single H.248 stream. An RTP session may have multiple RTP streams A single SDP m=line typically describes an RTP session. However where media multiplexing is used (e.g. as described by [IETF BUNDLE]) multiple SDP m=lines can describe an RTP session. Thus a H.248 Stream using a SDP “m=” line may describe one or more RTP streams. [IETF RFC 5576] provides attributes for source specific parameters.

This has implications for Signals and Events related to pause and resume on H.248 streams. In order for any Signals and Events to be correctly scoped where a single SDP m=line describes multiple sources the Signal or Event must be able to reference the source. Clause 9 allows the SSRC to be associated with pause / resume related Signals and Events. The MGC shall include the ssrc parameter in rempr Package Signals when there are multiple SSRCs in a H.248 Stream to uniquely identify the RTP stream instance.

Alternatively the MGC may consider alternate means (such as through H.248 stream grouping [ITU-T H.248.STGROUP]) to ensure that there is a single RTP stream per H.248 stream.

## 7.4	Consensus

Clause 4.4/[IETF STREAM PAUSE] indicates that there must be consensus between RTP receivers before an RTP sender can pause a RTP stream. This requires that all RTP receivers be aware of the current RTP stream state (i.e. paused or playing, see clause 6/[IETF STREAM PAUSE]) and that all receivers be aware of the pause request. Relevant example network use cases with H.248 media gateways are indicated in Appendix I.4. As described in clause 5.2/[IETF STREAM PAUSE] the RTP sender should delay the pausing of a stream for a wait period. As the gateway acts as an RTP sender it should ensure that pausing meets these requirements. If MG autonomous response is indicated, the MG shall be responsible for maintaining the wait period. If MGC controlled mode is indicated, the MGC shall be responsible for maintaining the wait period.

If a new RTP receiver that supports pause messages is added whilst an RTP stream is in the paused state, the gateway shall send it a PAUSED indication as per clause 5.3/[IETF STREAM PAUSE]. The MG is responsible for sending the PAUSED indication. In the case of MGC controlled mode the MG shall only send a PAUSED indication if the MGC had previously sent a “Local Pause” (rempr/lpause) Signal for the particular RTP stream.

## 7.5	Message acknowledgements

As described in clause 4.5/[IETF STREAM PAUSE] the RTP and RTCP do not guarantee reliable data transmission, meaning that the PAUSE, RESUME, PAUSED and REFUSED messages may be lost. H.248 MGCs and MGs should ensure that they follow the procedures in [IETF STREAM PAUSE] with respect to sending PAUSED and REFUSED and detecting paused and resumed states. Clause 9 provides Events for detecting when a RTP media stream has been paused or resumed to allow co-ordination between MGC and MG.

## 7.6	Retransmitting Requests

As per clause 4.6/[IETF STREAM PAUSE], PAUSE and RESUME requests sent from a H.248 gateway (as RTP receiver) to an RTP stream sender may result in no response (either the receipt of a PAUSED or REFUSED, or media action (pause/resume)) being detected from the sender. The gateway may assume that the PAUSE or RESUME was lost and thus may need to retransmit the message. Depending on the configuration the MGC or MG is responsible for the retransmission once the timeout period is met. If MG autonomous response is indicated, the MG shall be responsible for retransmitting the messages. If MGC controlled mode is indicated, the MGC shall be responsible for retransmitting the message (i.e. by sending a new signal to the MG indicating pause or resume).

## 7.7	Sequence Numbering

Clause 4.7/[IETF STREAM PAUSE] indicates that PAUSE request messages have a sequence number associated with them. This number is then referenced by the other pause CCM messages. As the pause identity is updated for each PAUSE request the gateway needs to manage the Pause identities (i.e., parameter PauseID in the Feedback Control Information field). If MG autonomous mode is indicated, then the MG shall be responsible for assigning the identities. If MGC controlled mode is indicated, then the MGC shall be responsible for assigning the identities.

# 8	RTP topology dependent RTCP handling

## 8.1	Introduction

The H.248 stream endpoint(s) or/and Context involved in RTP stream pause and resume services may be requested to support additional RTP / RTCP related services, leading to potential interactions which should be considered by H.248 profile designers. A brief summary of the problem statement may be found in clause 6/[ITU-T H.248.88]. There are basically three aspects:

- correct handling of RTCP FB "Pause/Resume" messages, dependent on the end-to-end use case of RTP stream pause and resume (see Appendix I) and dependent on the MG-internally enforced RTP topology (see clause 8.2);

- correct handling of compound RTCP packets (see clause 8.3); and

- correct handling of SSRC values due to their usage as P/R-signalling endpoint identifiers as well as their occurrence at "multiple places" in SDP and H.248 signalling elements (see clause 8.4).

## 8.2	Interactions with H.248.88

[IETF STREAM PAUSE] defines a service, with RTCP-based signalling along the IP media path, which belongs to the following [ITU-T H.248.88] category:

- RTP profile dependent RTCP service (clause 8.3.2/[ITU-T H.248.88]) because it is required to use RTP profiles RTP/AVPF or RTP/SAVPF;

- handling of RTCP FB "Pause/Resume" messages is dependent on the network use case:

  - local P/R-signalling endpoint: requires the local origination and termination of such RTCP FB messages (i.e., support of function F-6 in clause 7.6/[ITU-T H.248.88]);

  - transparent forwarding of P/R-signalling (see e.g. use case #4 in Appendix I): requires the unmodified forwarding of such RTCP FB messages (i.e., support of function F-4 in clause 7.6/[ITU-T H.248.88]); and

- such required RTCP handling behaviour should be consistent when RTP topologies "point-to-point" (aka B2BRE), "transport translator" or "mixer" is be enabled (i.e., a property setting of rtpt/rtptopo equal to 'PP', 'TR' or 'MX').

The question remains how the MG knows how RTCP FB "Pause/Resume" messages needs to be processed, i.e., transparently forwarding or origination/termination mode? The solution would be typically:

- the H.248 rempr package would be part of the operated H.248 profile;

- the usage of rempr package elements (in a H.248 command request) implies the origination/termination mode; and

- the omission of rempr package elements implies the transparent forwarding mode (i.e., the MG is not allowed to block such RTCP FB messages).

## 8.3	Interactions with compound and reduced-size RTCP packets

P/R-signalling could use reduced-size RTCP packets, which facilitates their processing by MGs because there are not any interactions.

However, compound RTCP packets may require enhanced processing because the compound RTCP packet might contain multiple RTCP packet types with different destination RTP endpoints. For instance, an RTCP XR packet or RTCP FB ECN packet might be destined for a next RTP node in the forwarding path whereas the RTCP FB "Pause/Resume" packet needs to be terminated locally. Hence, the MG might be required to rewrite a (compound) RTCP packet (i.e., support of function F-5 in clause 7.6/[ITU-T H.248.88]).

## 8.4	Gathering SSRC values, relevant for P/R-signaling, from the IP media path

## 8.4.1	Active call interactions with H.248.71

The RTCP source description package according to clause 6/[ITU-T H.248.71] allows the observation and extraction of SSRC values from remote and local RTP endpoints. The H.248.71 rtcpsdes package is typically applied for network wide performance monitoring services for RTP traffic, which use RTCP XR packets for transporting measurement data (see [b-ITU-T H.248.48] and [b-ITU-T H.248.87]). There are basically two H.248 profile types:

- H.248 profile with RTP stream Pause and Resume service support, but without any additional RTCP XR based services; or

- H.248 profile with support of both RTCP based services.

The H.248.71 rtcpsdes package could be applied together with the rempr package in case 1, but is not required for the P/R-signalling service. SSRC information would be extracted directly via H.248 Events (of the rempr package) from RTCP FB Pause/Resume messages.

With respect to case 2, where both packages are in use: same usage as in case 1 during active call phase.

### 8.4.2	Pre-call usage of H.248.71

The MGC might need to know the local SSRC value (e.g., for call control signalling purposes) of the RTP endpoint in the MG, participating in RTP stream pause and resume service, already before any RTP and RTCP packets are transmitted. Hence, the H.248 Events (of the rempr package) would not work. The solution to such a requirement could be addressed by the H.248.71 rtcpsdes package (see clause 6.6.1/[ITU-T H.248.71]).

### 8.4.3	RTP topology "RTP source translator" (H.248.88)

# An H.248 Context enabled both for a) transparent forwarding of P/R-signalling and b) a RTP source translator topology (see clause 11.1/[ITU-T H.248.88] needs careful consideration because the RTP source translator may modify the SSRC field which is used for RTP sender / RTP receiver identification in P/R-signalling. Such an interaction is conditionally, typically dependent on the used RTCP packet structure (see clause 8.3).

# 9	Remote pause and resume package

|  | Package name: | Remote Pause and Resume |
| --- | --- | --- |
|  | Package ID: | rempr (0x0123) |
|  | Description: | This package allows a MGC to request that the MG issue a request to a remote endpoint to pause (and subsequently resume) the transmission of an RTP media stream. These pause and resume requests typically apply to conversional media services. The H.248 media stream and RTP media flow is supposed to be used in bidirectional manner. Hence there are two RTP streams, one in incoming and one in outgoing direction. The package makes the assumption that the MG may provide P/R-signaling for both unidirectional RTP streams. P/R-signalling procedures in both directions are disjoint. The package description makes the assumption that the MG could provide the RTP receiver or/and the RTP sender function in parallel on a particular H.248 SEP enabled for support of this package. There are two slightly different RTP stream pause state models at RTP sender and RTP receiver side, see also Appendix II. |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

### 9.1.1	Autonomous Response

|  | Property name: | Autonomous Response |
| --- | --- | --- |
|  | Property ID: | ar (0x0001) |
|  | Description: | The property indicates whether the MG shall provide an autonomous response to PAUSE and RESUME requests or whether it should notify the MGC and await confirmation from the MGC on what action to take. |
|  | Type: | Boolean |
|  | Possible values: | On MG autonomous: The MG responds autonomously Off MGC controlled: The MG notifies the MGC and awaits a response |
|  | Default: | On (unless provisioned otherwise) |
|  | Defined in: | LocalControl or TerminationState |
|  | Characteristics: | Read/Write |

## 9.2	Events

### 9.2.1	RTP Pause State

|  | Event name: | RTP Pause State |
| --- | --- | --- |
|  | Event ID: | rtpps (0x0001) |
|  | Description: | This event allows the MG to indicate when the state associated with an RTP stream Pause / Resume changes, i.e., the H.248 stream endpoint realizes the RTP sender function. |

#### 9.2.1.1	EventsDescriptor parameters

##### 9.2.1.1.1	State

|  | Parameter name: | State |
| --- | --- | --- |
|  | Parameter ID: | state (0x0001) |
|  | Description: | This parameter indicates which pause states the MGC wishes to be notified about. Based on the RTP pause state model according to clause 6/[IETF STREAM PAUSE] (see also Appendix II.2). |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | paused The MG has paused RTP media flow as a result of the receipt PAUSE request from a remote RTP endpoint. resumed The MG has resumed RTP media flow as a result of the receipt of a RESUME request from a remote RTP endpoint localPause The MG has paused RTP media flow based on a local decision. localResume The MG has resumed RTP media flow based on a local decision. |
|  | Default: | All (paused, resumed, localPause, localResume) |

##### 9.2.1.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. {Editor's note (2015-02): how does the MGC know this information. Possible solution see E.N. in clause 9.3.1.1.2.} |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 9.2.1.2	ObservedEventsDescriptor parameters

##### 9.2.1.2.1	Observed State

|  | Parameter name: | Observed State |
| --- | --- | --- |
|  | Parameter ID: | obstate (0x0001) |
|  | Description: | This parameter indicates the detected pause state. It is reported whenever there is a transition between the states. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | As per “states” clause 9.2.1.1.1 |
|  | Default: | None |

##### 9.2.1.2.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 9.2.2	Detect Pause / Resume Request

|  | Event name: | Detect Pause Resume Request |
| --- | --- | --- |
|  | Event ID: | dprreq (0x0002) |
|  | Description: | This event allows the MG to indicate when a PAUSE or RESUME request has been detected. Thus, the H.248 stream endpoint realizes the RTP sender function. |

#### 9.2.2.1	EventsDescriptor parameters

##### 9.2.2.1.1	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0001) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. {Editor's note (2015-02): how does the MGC know this information. Possible solution see E.N. in clause 9.3.1.1.2.} |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 9.2.2.2	ObservedEventsDescriptor parameters

##### 9.2.2.2.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained the detected request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.2.2.2.2	Request Type

|  | Parameter name: | Request Type |
| --- | --- | --- |
|  | Parameter ID: | reqt (0x0002) |
|  | Description: | This parameter indicates whether a PAUSE or a RESUME is requested for the RTP Stream. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “PAUSE” A PAUSE has been requested. “RESUME” A RESUME has been requested. |
|  | Default: | None |

##### 9.2.2.2.3	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 9.2.3	Detect Pause / Resume Result

|  | Event name: | Detect Pause Resume Result |
| --- | --- | --- |
|  | Event ID: | dprres (0x0003) |
|  | Description: | This event allows the MG to detect and indicate whether the remote RTP sender has: sent a PAUSED indication, a REFUSE indication or resumed RTP transmission in response to a request initiated by the MGC. It also detects a failure of the RTP sender to take action on a PAUSE or RESUME request. Thus, the H.248 stream endpoint realizes the RTP receiver function. |

#### 9.2.3.1	EventsDescriptor parameters

##### 9.2.3.1.1	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0001) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 9.2.3.2	ObservedEventsDescriptor parameters

##### 9.2.3.2.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the detected PAUSED or REFUSED indication. It may also contain the pauseID related to a failed PAUSE or RESUME request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.2.3.2.2	Response Type

|  | Parameter name: | Response Type |
| --- | --- | --- |
|  | Parameter ID: | rest (0x0002) |
|  | Description: | This parameter indicates whether a PAUSED or a REFUSED indication has been received. It also indicates whether a RESUME request as resulted in the flow of RTP packets from the RTP sender. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “paused” A PAUSED indication has been received. “refused” A REFUSED indication has been received. “resumed” The MG has detected a resumed RTP flow. “failed” No response from the RTP sender has been detected |
|  | Default: | None |

##### 9.2.2.2.3	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

## 9.3	Signals

### 9.3.1	Local Pause

|  | Signal name: | Local Pause |
| --- | --- | --- |
|  | Signal ID: | lpause (0x0001) |
|  | Description: | This signal indicates that the MG shall pause the RTP stream and send a PAUSED indication to the remote RTP receiver(s). Thus, the H.248 stream endpoint realizes the RTP sender function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 9.3.1.1	Additional parameters

##### 9.3.1.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID of the PAUSE to which the PAUSED indication relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.3.1.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the PAUSE relates. {Editor's note (2015-02): there are basically three sources of information at MGC level with regards to SSRC values: a) as detect and reported by the MG via events rtpps/ssrc or dpreq/ssrc; b) via SDP attributes in SIP signalling (tbc); or/and c) via H.248.71 "statistic" rtcpdes/rssrc. It is supposed that method (a) is used in context of this package.} |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 9.3.2	Local Resume

|  | Signal name: | Local Resume |
| --- | --- | --- |
|  | Signal ID: | lresume (0x0002) |
|  | Description: | This signal indicates that the MG shall resume transmission of the RTP stream. Thus, the H.248 stream endpoint realizes the RTP sender function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 9.3.2.1	Additional parameters

##### 9.3.2.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID indicating the PAUSE to which the RESUME relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.3.2.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the RESUME relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 9.3.3	Refuse

|  | Signal name: | Refuse |
| --- | --- | --- |
|  | Signal ID: | refuse (0x0003) |
|  | Description: | This signal indicates that the MG shall not change the state of the RTP stream. If the MG had received a remote request to change the state, it should send a REFUSE indication to the remote end. Thus, the H.248 stream endpoint realizes the RTP sender function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 9.3.3.1	Additional parameters

##### 9.3.3.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID indicating the PAUSE to which the REFUSE relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.3.3.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the REFUSE relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 9.3.4	Remote Pause

|  | Signal name: | Remote Pause |
| --- | --- | --- |
|  | Signal ID: | rpause (0x0004) |
|  | Description: | This signal indicates that the MG shall send a PAUSE request to the remote RTP sender. Thus, the H.248 stream endpoint realizes the RTP receiver function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 9.3.4.1	Additional parameters

##### 9.3.4.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the PAUSE request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.3.4.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the PAUSE relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 9.3.5	Remote Resume

|  | Signal name: | Remote Resume |
| --- | --- | --- |
|  | Signal ID: | rresume (0x0005) |
|  | Description: | This signal indicates that the MG shall send a RESUME request to the remote RTP sender. Thus, the H.248 stream endpoint realizes the RTP receiver function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 9.3.5.1	Additional parameters

##### 9.3.5.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the RESUME request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.3.5.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the RESUME relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

## 9.4	Statistics

{Editor’s note: There may be a need for a statistic to indicate the ratio of inactivity (due to a pause) to activity. Contributions are solicited.}

None

## 9.5	Error Codes

None

## 9.6	Procedures

This Package allows the gateway to act in four main modes:

- MG autonomous handling of pause and resume requests from a remote RTP receiver where the MGC is notified of RTP stream state change. The MG acts in the RTP sender role.

- MGC controlled handling of pause and resume requests from a remote RTP receiver where the MGC is notified of pause and resume messages from a remote RTP receiver and provides a response to the MG regarding the action to take. The MG acts in the RTP sender role.

- MGC initiated pause/resume requests towards a remote RTP sender where the MGC receives the pause and resume request results from MG. The MG acts in the RTP receiver role.

- MGC initiated local pause/resume requests towards a remote RTP receiver where the MGC receives the local pause and local resume request results from MG. The MG acts in the RTP sender role.

The procedures for these modes are described in clauses 9.6.3 to 9.6.7. Clauses 9.6.1 and 9.6.2 provide general procedures these modes.

### 9.6.1	General - SDP parameter setting

On H.248 Streams where RTP stream Pause / Resume is supported, for all three modes of operation the MGC shall set SDP media descriptions in the Local and/or Remote Descriptors indicating the support of “pause” codec control message (CCM) [IETF RFC 5104] as per clause 9/[IETF STREAM PAUSE]. This allows the MGC to indicate to the MG the support of MG initiated local pause and resume (as RTP sender and RTP receiver) without the need for H.248 Signals and Events.

{Editor's note (2015-06): the RTP stream pause and resume behaviour could be (partially) emulated via RTCP FB TMMBR & TMMBN message types, too. This option is in scope of /[IETF STREAM PAUSE], but not (yet) in H.248.PAURES. It should be explicitly mentioned in this Draft. => new clause 10 proposed … }

#### 9.6.1.1	Pause

The SDP for “pause” allows a “config” parameter to be set against an m-line which indicates what Pause/Resume CCM messages it supports. This description is based on the m-line describing a bi-directional bearer however the Local and Remote Descriptor describes a unidirectional bearer. Rather than mapping the “config” value to a separate value for the Local and Remote Descriptor the same value shall be set on both these Descriptors.

### 9.6.2	General – Event and Signal Scope

Given that pause and resume relate to a single RTP stream, Signals and Events should be set in such a way that the affected RTP streams are unambiguously identified. Where a H.248 Stream contains a single RTP stream (i.e. a single SSRC) the MGC may omit the ssrc parameter when setting any Signals and Events in this package. Where a H.248 Stream contains multiple RTP streams the MGC shall include the ssrc parameter indicating the SSRC of the applicable RTP stream.

The H.248 ssrc parameter shall be mapped by the MG to the correspondent SSRC field in a RTCP Pause/Resume CCM message, dependent on the applied in RTP sender or RTP receiver role (see also Appendix III).

Where there are multiple H.248 Streams on a Termination the MGC shall set the StreamID related to the RTP Stream in the Events and Signals in this package.

### 9.6.3	MG autonomous handling of remote requests

The MGC controller may confer handling of RTP Pause/Resume requests to the MG by setting the “Autonomous Response” (rempr/ar) property to “on” indicating that the MG shall autonomously process and respond to any RTP pause/resume messages as defined by [IETF STREAM PAUSE]. If the MGC requires to be notified of any changes in RTP transmission state the MGC shall set the “RTP Pause State” (rempr/rtppps) event with the states that it requires to be informed about. The MGC shall also set the ‘pause’ parameter in the Local and Remote Descriptor as appropriate according to clause 9.6.1.

On recept of a PAUSE request message the MG shall as RTP sender act according to clauses 5.2 and 5.3/[IETF STREAM PAUSE]. This may result in the MG pausing the RTP stream and the sending of a PAUSED indication or it sending a REFUSE message and continuing the transmission of the RTP stream. If requested when entering the PAUSED state the MG will generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has entered the “paused” state (obstate=“paused”).

On receipt of a RESUME request message the MG shall as RTP sender act according to clause 5.4 /[IETF STREAM PAUSE]. This may result in the MG resuming the RTP stream or the sending of a REFUSE message and the continuation of the paused state. If requested when resuming media flow the MG will generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been resumed (obstate=“resumed”).

According to clause 5.3/[IETF STREAM PAUSE] an MG may pause an RTP Stream at any time. If the MG initiates a local pause and it has been requested to report this state, it shall generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been paused (obstate=“localPause”) due to a local decision.

According to clause 5.4/[IETF STREAM PAUSE] an MG may resume an RTP Stream at any time. If the MG initiates a local resume and it has been requested to report this state, it shall generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been resumed (obstate=“localResume”) due to a local decision.

### 9.6.4	MGC reception of pause and resume requests

The MGC may require that it be responsible for the decision of whether or not to pause or resume an RTP Stream. In this case it shall set the “Autonomous Response” (rempr/ar) property to “off” indicating to the MG that it shall defer the decision to pause/resume an RTP Stream to the MGC. The MGC shall also set the “Detect Pause/Resume Request” (rempr/dprreq) to request notification of pause or resume requests.

On receipt of a PAUSE request the MG shall send a Notify.req with the “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent indicating a request type indicating a pause (reqt=”PAUSE”). The MGC shall take into account any topology (end-to-end RTP session topology, see clause 7 and MG-local internal RTP topology, see Appendix I ), service or other local considerations to determine whether to grant the pause request. In order to grant the request the MGC shall send the “pause” (rempr/lpause) signal to the MG. On receipt of the signal the MG shall pause the RTP Stream and send a PAUSED indication as per clause 5.3/[IETF STREAM PAUSE]. To deny the request, the MGC shall send the “refuse” (rempr/refuse) signal to the MG. On receipt of the signal the MG shall send a REFUSED indication as per clauses 5.2 and 5.3/[IETF STREAM PAUSE].

On receipt of a RESUME request the MG shall send a Notify.req with the “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent indicating a request type indicating a resume (reqt=”RESUME”). The MGC shall take into account any topology (see clause 7), service or other local considerations to determine whether to grant the resume request. In order to grant the request the MGC shall send the “resume” (rempr/lresume) signal to the MG. On receipt of the signal the MG shall resume the RTP Stream as per clause 5.4/[IETF STREAM PAUSE]. Media is resumed at a time shifted point rather than from the pause point. To deny the request, the MGC shall send the “refuse” (rempr/refuse) signal to the MG. On receipt of the signal the MG shall send a REFUSED indication as per clauses 5.4/[IETF STREAM PAUSE].

The MG shall not send a Notify.req with “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent with a request type for a pause state that the RTP stream is already in. I.e. the MG shall not send a request type equal to pause if the RTP stream is already paused.

### 9.6.5	MGC initiation of remote pause and resume requests

The MGC may require that for a particular RTP stream that the remote RTP sender pauses RTP transmission. In order to request the MG to send a PAUSE request to the remote RTP sender, the MGC shall send the “Remote Pause” (rempr/rpause) Signal to the MG. On reception of the signal the MG shall initiated a PAUSE request on the applicable RTP stream. The MGC may provide or wildcard CHOOSE a Pause Identity (pauseid). If the pauseid is wildcarded, the MG shall return the chosen value in a command reply. The MG will send the pause identity in the PAUSE request. The use of the pause identity allows the MGC and MG to correlate pause/resume requests and replies.

The MGC may require that for a particular RTP stream that the remote RTP sender resumes RTP transmission. In order to request the MG to send a RESUME request to the remote RTP sender, the MGC shall send the “Remote Resume” (rempr/rresume) Signal to the MG. On reception of the signal the MG shall initiate a RESUME request on the applicable RTP stream. The MGC shall provide a fully specified a Pause Identity (pauseid).

In order that the MGC may be notified of the response from the remote RTP sender regarding MGC initiated PAUSE and RESUME requests it shall set the “Detect Pause Resume Result” (rempr/dprres) Event. On reception of a rempr/rpause signal with a pause identity, the MG arms the Event and sends a Notify.req to the MGC when it detects a PAUSED or REFUSED indication for the applicable pause identity. On reception of a rempr/rresume signal with a pause identity, the MG arms the Events and sends a Notify.req to the MGC when it detects a REFUSED indication or when it detects a resumed RTP flow for the applicable RTP stream. The Notify.req contains the dprres ObservedEvent containing the Response Type (rest). If after a suitable timeout period (see clause 4.6/[IETF STREAM PAUSE] the MG does not detect any response from the RTP sender (either by signaling pause CCM messages or via a modification in the sending of the RTP stream) the MG shall send a Notify.req containing the dprres ObservedEvent containing the Response Type indicating a failure.

### 9.6.6	MGC initiation of local pause and resume

The MGC may require that for a particular RTP stream that the MG as RTP sender pauses RTP transmission. In order to request the MG to pause RTP sending, the MGC shall send the “Local Pause” (rempr/lpause) Signal to the MG. On reception of the signal the MG shall initiate a PAUSE on the applicable RTP Stream. The MG shall send a PAUSED indication to the RTP receivers. The MGC may provide or wildcard CHOOSE a Pause Identity (pauseid). If the pauseid is wildcarded, the MG shall return the chosen value in a command reply. The MG will send the pause identity in the PAUSED indication.

The MGC may require that for a particular RTP stream that the MG as RTP sender resumes RTP transmission. In order to request the MG to resume RTP sending, the MGC shall send the “Local Resume” (rempr/lresume) Signal to the MG. On reception of the signal the MG shall initiate a RESUME request on the applicable RTP stream. The MGC shall provide a fully specified a Pause Identity (pauseid).

In order that the MGC may be notified of the response of the MG to MGC initiated local PAUSE and RESUME requests it shall set the “Detect Pause Resume Result” (rempr/dprres) Event. On reception of a rempr/lpause signal with a pause identity, the MG sends a Notify.req to the MGC when it enters the local paused state. On reception of a rempr/lresume signal with a pause identity, the MG sends a Notify.req to the MGC when it resumes RTP flow for the applicable RTP stream. The Notify.req contains the dprres ObservedEvent containing the Response Type (rest).

### 9.6.7	MG initiation of remote pause and resume

{Editor’s note: MG autonomous initiation of remote pause and resume should be considered.}

### 9.6.8	Error handling

The MGC shall not send signals or set events that are incompatible with the SDP “config” CCM parameters. For example, the MGC shall not request the “rpause” signal on a Stream if it has not set the “config” parameter equal to 1,2 or 4. The table below indicates the allowed settings:

Table 1 – Allowed setting of “rempr” package signals and events

| Package “rempr” Signals | Valid “configs” | Comments |
| --- | --- | --- |
| Local Pause (lpause) | 1,2,3,5,6,8 |  |
| Local Resume (lresume) | 1,2,4,5 |  |
| Refuse (refuse) | 1,2,5 |  |
| Remote Pause (rpause) | 1,2,4 |  |
| Remote Resume (rresume) | 1,2,4 |  |
| Package “rempr” Events | Valid “configs” | Comments |
| RTP Pause State (rtpps) | 1,2,3,4,5,6,7,8 | As the event may detect in state it may be set against any configuration. |
| Detect Pause Resume Request (dprres) | 1,2,3,4,6,7 |  |
| Detect Pause Resume Result (dprres) | 1,3,5 |  |

If the MGC sends a Signal or Event without having also set the having set the ‘pause’ or ‘paused’ SDP parameter in a Local or Remote Descriptor then the MG shall respond with error 472 “Required Information Missing”. If the MGC sends a Signal or Event on a Stream where the ‘pause’ or ‘paused’ SDP parameter is set contravening table 1, then the MG shall respond with error 473 “Conflicting Property Values.

# 10	Package-less alternative: partial emulation service using RTCP FB messages related to Temporary Maximum Media Stream Bit Rate signalling

{Editor’s note (2015-06): this option is indicated in [IETF STREAM PAUSE]. It would lead to an package-less approach because the current rempr package does not address the RTCP FB TMMBR / TMMBN message types. It's also a kind of emulation service because a) the original semantic of these RTCP FB message types was slightly different, and b) the solution does not address all requirements and design considerations. Contributions are solicited. }

# 11	Examples

{Editor’s note: TBD}

Appendix I

Network use cases with H.248 media gateways in the end-to-end RTP path with Pause/Resume signaling

(This appendix does not form an integral part of this Recommendation.)

## I.1	Introduction

Clause 3/[IETF STREAM PAUSE] describes the main use cases for RTP sessions with usage of RTP stream pause and resume. The example use cases consider the unidirectional RTP traffic flow between two or more RTP endpoints, without or with additional RTP nodes in between. Such interim RTP nodes realize a specific RTP topology function, which basically impact the variation of RTP pause/resume scenarios. This Appendix provides a mapping of the clause 3/[IETF STREAM PAUSE] use cases on H.248 gateways, i.e., when an H.248 media gateway provides an RTP endpoint and an associated RTP topology function. The result of that analysis is a list of example reference network configurations with RTP stream pause/resume usage for a particular H.248 gateway (profile).

The H.248 use cases might be classified in H.248 Context models with a single (clause I.2), two (clause I.3) or more than two (clause I.4) H.248 RTP-enabled Terminations.

All use cases illustrate only one traffic direction (of a uni- or bidirectional RTP session) due to the asymmetric nature of RTP stream pause / resume between RTP sender(s) and RTP receiver(s).

## I.2	Single RTP Termination

This use case relates to H.248 connection models with a single RTP-enabled Termination per H.248 Context. For instance this use case includes all variants of H.248 physical-to-RTP media gateway types within PSTN emulation subsystems. The H.248 RTP Termination represents an RTP endpoint, i.e., all RTCP packet types are terminated (inclusive of RTCP FB) and there are not any Context-internal P/R-signalling aspects as opposed to subsequent use case discussions of the following clauses I.3 and I4.

## I.3	Dual RTP Terminations

The notion of "dual RTP Terminations" relates to an H.248 Context with two H.248 RTP-enabled Terminations (which are labelled as T1 and T2) in the diagrams below. Clause 3.1/[IETF STREAM PAUSE] provides use cases for a "Point-to-Point" RTP topology, from an end-to-end communication perspective. Such an end-to-end (RTP)-connection might be routed via an interim H.248 IP-to-IP media gateway, which could have multiple different "RTP topology" modes, such as an RTP media translator, RTP transport translator or Back-to-Back RTP endsystem topology (see [b-ITU-T H.248.88]). This clause lists some use cases for such an H.248 connection model, but without discussion of the applied RTP topology by the (T1, T2) Context configuration.

Figure I.1 illustrates two use cases, termed "Point-to-Point Downstream" and "Point-to-Point Upstream":

NOTE – The legend applies to all Figures in this Appendix.

Figure I.1 – Use-cases #1 "Point-to-Point Downstream" & #2 "Point-to-Point Upstream"

Description of the remote RTP nodes (from H.248 MG perspective): Node Y is a pure RTP sink. Node X represents an optional, virtual RTP source (from MG perspective) because the real RTP source X' might not be visible from MG perspective. There are RTCP-based services where the real RTP source X' might become visible, such as in the case of RTCP XR based performance monitoring (see [b-ITU-T H.248.48] and [b-ITU-T  H.248.87].

The H.248 MG provides support of RTP stream pause/resume only in an outgoing (#1) or in an incoming (#2) RTP traffic direction. The P/R-signalling related RTCP packet types needs to be terminated by the correspondent H.248 Terminations (and not Context-internally forwarded).

Use cases #1 and #2 may be supported in parallel, leading to the configurations as outlined by Figure I.2:

Figure I.2 – Use-cases #3 "Point-to-Point Down- and Up-stream"

There are multiple options: firstly, the P/R-signalling loops might be fully decoupled (#3a) or coupled (#3b). It has to be emphasized that the P/R-signalling loops were intentionally established with a local scope (i.e., X-to-T1 and T2-to-Y) and not as a signal loop with a global scope (i.e., end-to-end loop between X and Y). Use case #3a looks similar as use case #1, with the difference that the MG is aware of P/R-signalling towards X. The decoupled mode (#3a) could lead to situations of a paused RTP source T2 but a still playing RTP source X, which leads to the question whether the MG should buffer or discard RTP packets received at T1? Due to that P/R-signalling defined by [IETF STREAM PAUSE] is typically for conversational services, the RTP packets are silently discarded.

Use case #3b reflects the "coupled RTP stream pause and resume segments" case where the "RTP sink" requests to stop the RTP send process, which means that the actual RTP source should be switched to pause mode (i.e., X or even X' and not T2). Hence, the two P/R-signalling loops needs to be coupled. There are two coupling options: either indirectly via the MGC or directly MG-internally.

{Editor's note (2015-06): the MGC method could be already supported by applying the rempr package at both terminations T1 and T2. The MG method is not yet supported …}

Finally, there might be a single, end-to-end P/R-signalling loop across the H.248 MG, see Figure I.3:

Figure I.3 – Use-cases #4 "Point-to-Point transparent"

The "end-to-end" RTP stream pause and resume scope implies that neither termination T1 nor T2 provides an "RTP sender" or "RTP receiver" role. P/R-signalling information needs to be transparently forwarded by the MG. Such a forwarding requirement for dedicated RTCP packet types needs to be taking into account by a suitable RTP toplogy (see [b-ITU-T H.248.88]).

## I.4	Multiple RTP terminations

Such a use case would be the result when an H.248 media gateway realizes the "RTP mixer" function in all examples of clauses 3.2 to 3.5 in [IETF STREAM PAUSE].

NOTE – Another multipoint configuration is the (RTP) relay topology in [IETF STREAM PAUSE], which represents a "RTP translator with only unicast paths" according to [b-IETF rtp-topologies-update].

Figure I.4 illustrates a general multipoint configuration with enabled P/R-signalling between all RTP endpoints:

Figure I.4 – Use-case #5 "Multipoint-to-Multipoint" (with six RTP participants)

The MG behaviour (in use case #5) with respect to P/R-signalling handling is similar to  use case #3, apart from the additional complexity that the MG-internal P/R-signalling interworking effort correlates with the applied mixer function for a multiparty, multimedia communication service. Three specific multipoint use case examples are discussed in more detail below.

Figure I.5 illustrates a configuration for three RTP participants where the downstream RTP sink Y1 gets the RTP traffic forwarded from one-out-of-two upstream RTP sources X1 or X2:

Figure I.5 – Use-case #6 "Dual point-to-Point switching" (with three RTP participants)

Such a use case is for example discussed in clause 10.3/[IETF STREAM PAUSE] ("voice activated mixer"). The two self-contained P/R-signalling loops X1-to-T1,1 and X2-to-T1,2 need to be coupled by the H.248 gateway due to the applied service configuration.

Figure I.6 illustrates a configuration between a sender X1 and two receivers Y1 and Y2 and an interim H.248 MG in RTP transport relay topology (see also Figure 18/[IETF STREAM PAUSE]):

Figure I.6 – Use-case #7 "Point-to-Dual point copying" (with three RTP participants) 
and a "P/R-signalling active MG"

Both receivers Y1 and Y2 may request the single sender X1 to pause the RTP playout process, hence there are inherent interactions between the concurrent P/R-signalling loops. Use case #7 represents a network configuration where the H.248 gateway resolves such interactions. Use case #8 (Figure I.7) shows the same end-to-end scenario, but with a "dumb H.248 gateway" because the MG transparently forwards all RTCP FB "P/R signalling messages".

Figure I.7 – Use-case #8 "Point-to-Dual point copying" (with three RTP participants) 
and a "P/R-signalling passive MG"

In summary, the H.248 MG provides the same RTP topology in use cases #7 and #8 (to be confirmed), but once an H.248 profile with and without H.248 rempr package usage.

Appendix II

State models for Pause/Resume signaling endpoints

(This appendix does not form an integral part of this Recommendation.)

## II.1	Introduction

Clause 6/[IETF STREAM PAUSE] defines the RTP pause state model at RTP sender side. This model is the normative framework for the H.248 Events and Signals as defined by the rempr package in clause 9.

The purpose of this Appendix is to illustrate and indicate the state transitions which are correlated with the H.248 signalling elements. In case of any discrepancies between this Appendix and clause 8, the package specification in clause 9 takes precedence.

## II.2	RTP pause state model at RTP sender side

Figure II.1 illustrates the RTP sender side state model:

Figure II.1 – RTP sender (RTP source): RTP Pause State model – extended model with additional indication of H.248 signalling elements

## II.2	RTP pause state model at RTP receiver side

Figure II.2 illustrates the RTP receiver side state model:

{Editor's note (2015-06): [IETF STREAM PAUSE] does not provide any information about a potential state model at RTP receiver side. Below figure reflects a proposal by Q.3/16. The state names are provisional. There might be also a chance to reduce the number of state from three to two. Contributions are solicited.}

Figure II.2 – RTP receiver (RTP sink): RTP Pause State model – extended model with additional indication of H.248 signalling elements

Appendix III

SSRC identifiers for Pause/Resume signalling endpoints

(This appendix does not form an integral part of this Recommendation.)

## III.1	SSRC information in RTCP FB "Pause/Resume" messages

Figure III.1 outlines the structure of such a RTCP FB message type:

Figure III.1 – RTCP FB packets for Pause/Resume-signaling – SSRC related fields

There are (at least) three SSRC identifiers in such a RTCP FB packet:

- SSRC packet sender: the SSRC of the RTP endpoint issuing a P/R-signalling message in a RTP sender or RTP receiver role;

- SSRC media source: original semantic see [IETF RFC 5104], not used in P/R-signalling messages, set to value '0';  and

- SSRC target: the SSRC of the remote RTP endpoint acting as the target of P/R-signalling (i.e., the associated RTP sender or RTP receiver).

It could be concluded that the H.248 signalled ssrc parameter (as part of Event configuration, Event notification and Signal configuration) is either mapped on SSRC #1 (packet sender) or SSRC #3 (target), dependent whether the H.248 unidirectional media stream endpoint is acting in RTP sender or RTP receiver role.

## III.2	Example use case

Figure III.2 provides an example, based on use case #3b from Appendix I, extended to bidirectional support of P/R-signalling. The H.248 rempr package supports this case, which facilitates the understanding the scope of the ssrc information in H.248 Signals and Events of this package. It has to be noted that the MG provides an [ITU-T H.248.88] B2BRE RTP topology. Other RTP topologies might lead to different SSRC assignments. Figure III.2 outlines the SSRC values which appear in the SSRC fields #1 and #3 of the RTCP FB packets.

Figure III.2 – Example use case #3b (from Appendix I) – Extended for bidirectional P/R-signalling and indication of SSRCs.

Bibliography

[b-ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[b-ITU-T H.248.82]	Recommendation ITU-T H.248.82 (03/2013), Gateway control protocol: Explicit congestion notification support

[b-ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[b-IETF rtp-topologies-update]	IETF RFC ### (###), RTP Topologies.
URL: http://tools.ietf.org/wg/avtcore/draft-ietf-avtcore-rtp-topologies-update/

____________________