| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | TD-32 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors H.IVSArch |  |  |  |
| Title: | H.IVSArch "Architectural requirements for intelligent visual surveillance" (New): Output draft |  |  |  |
| Purpose: | Information |  |  |  |

Introduction

This document contains the editors’ output draft for H.IVSArch "Architectural requirements for intelligent visual surveillance" (New))from the SG16,Chengdu, 8 - 12 June 2015.

Updates were based on the following contributions:

| AVD-4734 | ZTE | H.IVSArch: proposal for updated outlines | Q21/16 |
| --- | --- | --- | --- |
| AVD-4735 | ZTE | H.IVSArch: proposed content for components description | Q21/16 |
| AVD-4765 | China Telecom, Beijing Univ. of Posts and Telecom | Proposal to add clause 6.1in H.IVSArch | Q21/16 |
| AVD-4766 | China Telecom, Beijing Univ. of Posts and Telecom | Proposal to add reference point in clause 6.2in H.IVSArch | Q21/16 |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations 6 5 Conventions 6 6 Functional Requirements 7 6.1 Service Function 7 6.1.1 IVU order and change 7 6.1.2 IVU registration and configuration 7 6.1.3 Task schedule 7 6.1.4 Image and video acquisition 7 6.1.5 Intelligent event report 7 6.1.6 Intelligent analysis result show 7 6.1.7 OSD parameters Issuing to IPU 7 6.2 Service management 7 6.2.1 IVM Management 7 6.2.2 IVU Management 7 6.2.3 Network Management 8 6.2.4 Remote upgrade 8 7 Functional Architecture 8 7.1 Functional architecture framework 9 7.2 Functional entities 9 7.2.1 IPU 9 7.2.2 ICU 9 7.2.3 IVM 9 7.2.4 IVU 9 7.3 Reference Points 9 7.3.1 Reference Point: IPU - CMU 10 7.3.2 Reference Point: IPU - SCU 10 7.3.3 Reference Point: IPU - MDU 10 7.3.4 Reference Point: IPU - ICU 10 7.3.5 Reference Point: ICU - CMU 10 7.3.6 Reference Point: ICU - SCU 10 7.3.7 Reference Point: ICU - MDU 10 7.3.8 Reference Point: ICU - MSU 10 7.3.9 Reference Point: IVU - SCU 10 7.3.10 Reference Point: IVU - MDU 10 7.3.11 Reference Point: IVU – IVM 10 7.3.12 Reference Point: IVM – CMU 10 7.3.13 Reference Point: IVM – SCU 10 8 Service control flow and signalling 10 8.1 Registration 10 8.1.1 IPU registration 10 8.1.2 IVU registration 10 8.2 Configuration 10 8.2.1 IPU configuration 10 8.2.2 IVU configuration 10 8.3 IVU calling 11 8.3.1 Start IVU manually 11 8.3.2 Stop IVU manually 11 8.3.3 Start IVU by event trigger 11 8.3.4 Stop IVU by event trigger 11 8.4 Intelligent event report 11 8.4.1 IVU report intelligent event 11 8.4.2 IPU report intelligent event 11 8.5 OSD parameters Issuing to IPU 11 8.5.1 Issuing procedure 11 8.5.2 OSD parameters profile 11 8.6 Service subscription 11

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 7 –System composition of intelligent Visual Surveillance 5

Draft new ITU-T Rec. H.IVSArch

Architectural requirements for intelligent visual surveillance

AAP Summary

[To be provided before Consent]

Summary

TBD.

Keywords

Visual surveillance, architecture,intelligent

# Scope

The Intelligent Visual Surveillance systemis focusing on value-added intelligent applications based on visual (audio included also) application technology and intelligent algorithm,which providesservices of target recognition, event recognition and intelligent alarm, data analysis, intelligent video diagnosis, intelligent video retrieval, intelligent video synopsis to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability.

Intelligent visual surveillance could automatically identify specific objects, behaviours or attributes in video. It processes the video stream and transforminto structured data to be transmitted or archived so that the visual surveillance system can act accordingly.

The Recommendation H.IVSArch focuses on the functional requirements of anintelligent visual surveillance system, including the functional requirements, model, architecture, entities, signalling and reference points.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	RecommendationITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626(2011), Architectural requirements for visual surveillance.

[ITU-T H.627]	Recommendation ITU-T H.627(2009), Signalling and Protocols for visual surveillance.

[ITU-T H.626.1]	Recommendation ITU-T H.626.1(2011), "Architectural requirements for mobile visual surveillance".

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP)

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003),RTP: A Transport Protocol for Real-Time Applications.

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]:<optional quoted definition>

3.1.2	<Term 2> [Reference]:<optional quoted definition>

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>:<definition>

# Abbreviations

| APP | Application |
| --- | --- |
| CIV | Client Intelligent Video |
| CMU | Centre management unit |
| CU | Client Unit |
| ICU | Intelligent Customer Unit |
| IPU | Intelligent Premises Unit |
| IVM | Intelligent Video Management |
| IVU | Intelligent Video Unit |
| IVS | Intelligent Visual Surveillance |
| MDU | Media Distribution Unit |
| MSU | Media Storage Unit |
| OSD | On Screen Display |
| PIV | Premises Intelligent Video |
| PTZ | Pan/Tilt/Zoom |
| SCU | Service Control Unit |

# Conventions

TBD.

# Functional Requirements

## {Editor’s note: this section provides the main functions description of intelligent visual surveillance system. And in the section8, it will provide the corresponding procedures and signalling.

## 6.1 Service Function

### IVU order and change

Users are recommended to order and change thecategory, supplier and capacity of IVU through IVM portal.

### IVUregistration and configuration

IVMis required to support strategiesconfiguration ofintelligent applicationby users and video sources schedule dynamically according to the strategies.IVM can accept registration, deletion, capability report from IVU and schedule IVU dynamically

### Task schedule

Users are recommendedto operate IVM to manage the intelligent analysis task such as query, start, pause and stop

### Image and video acquisition

Users are recommended toacquire the device list and query the RTSP address of streaming media. Then IVUcan load the local video or get streaming media through theRTSP address from the video surveillance platform

### Intelligent event report

IVU is required to identify specific objects automatically, and outputs recognition results to IVM. The recognize information includes event trigger information and data acquisition information. One or more intelligent analysis algorithms can be loaded into or unloaded from IVU according to different requirements.

### Intelligent analysis result show

IVU is recommended to send the intelligent analysis results to APP.APP show those results using statistical charts, event screenshot, event videosegment and so on.

## OSD parameters issuing to IPU

## 6.2 Service management

### 6.2.1 IVM Management

IVM is required to store, manage, and schedule those capabilities dynamically. IVM is required to be invoked by an intelligent application from CMU and manage IVU validity check.

### 6.2.2 IVU Management

IVU is required tosend capability set and status to IVM. IVS system is required to have the ability to develop new algorithms（IVU）, such as items left behind detection (to detect dangerous and suspicious item in certain area), item preservation (to guard certain item from being moved), license plate recognition, etc.

IVU is required to provide at least two types of controls of algorithm parameter configuration for administrator and ordinary user, and require the administrator to finish the initial scene studying, advanced parameter setting and other professional operations, while the business is opened via controls, in order to improve the alarm accuracy.

### 6.2.3 Network Management

IVS is required to manage the intelligent analysis process and CPU resources. IVM is required to have the ability to provide the NTP clock synchronization service to IVU managed by it.

IVS system is recommended to classify users into several levels. Users of different levels have different operational rights; high-level users have more operation privileges than low-level ones. Low-level users can do basic operations which meet the following basic requirements, and high-level users can do advanced operations which meet the following advanced requirements.

### 6.2.4 Remote upgrade

IVU is recommended to get the remote software upgrade at regular intervals

# Functional Architecture

# Figure 7 –System composition of intelligent Visual Surveillance

IVU: identifies specific objectsautomatically, and outputs recognition results to IVM. The recognize information includes event trigger information and data acquisition information. One or more intelligent analysis algorithms can be loaded into or unloaded from IVU according to different requirements.

IVM: supports strategiesconfiguration of intelligent application by users and video sources schedule dynamically according to the strategies.IVM can accept registration, deletion, capability report from IVU and schedule IVU dynamically. It can also store, manage, and schedule those capabilities dynamically. IVM can be invoked by an intelligent application from the CMU.

APP:itis the third-party data analysis system, which can be used to configure the data analysis algorithms and strategies.APP can receiveand analyse data from the visual surveillance platform, whereas it can send alarm information to the visual surveillance platformif needed. With the consideration that alarm processing is mostly a heavy load function, so it can be stripped from CMU to the third-party APP so as to make the function of platform unit clear and extendible.

ICU: In order to achieve comprehensive intelligent video recognition, we can add CIV (Client Intelligent Video) module to CU.CIV recognizes required information from input videos, and output recognition results. In addition, CIV can search designated records from video files. The identifiable information includes event trigger information and data acquisition information

IPU:  it just adds PIV (Premises Intelligent Video) module to PU, PIV is intelligent recognition module of PU, which can be achieved in PU devices and be PU integration. PIV can recognize required information from input videos, and output recognition results. The recognize information includes event trigger information and data acquisition information, etc.

PIV:  it is an intelligent identification module in PU. It provides functions such as identifying required information from input video and outputting the identified result.

CIV: it is an intelligent identification module in CU. It provides functions such as identifying required information from input video and outputting the identified result, retrieve the video with specified information from recorded video data.

# The result identified by PIV or CIV can be used for video indexing, image indexing, intelligent PTZ control, subtitle, etc. PIV and CIV can submit the result to CMU.

## Functional architecture framework

{Editor’s note: this section provides the figure of system composition of intelligentvisual surveillance system, and brief description for the components.}

## Functional entities

{Editor’s note: this section defines the functional entities which are introduced for intelligentfunctions based on normal visual surveillance system.}

### IPU

### ICU

### IVM

### IVU

## Reference Points

{Editor’s note: this section describes the reference points between the components based on the framework shown in the figure of system composition.}

### Reference Point: IPU - CMU

### Reference Point: IPU - SCU

### Reference Point: IPU - MDU

### Reference Point: IPU - ICU

### Reference Point: ICU - CMU

### Reference Point: ICU - SCU

### Reference Point: ICU - MDU

### Reference Point: ICU - MSU

### Reference Point: IVU - SCU

### Reference Point: IVU - MDU

### Reference Point: IVU – IVM

### Reference Point: IVM – CMU

# Reference Point: IVM – SCU

# Service control flow and signalling

{Editor’s note: this section provides service flow and signalling which will be used to implement the functions defined above. A proposed category for the service flow is provided as below, procedures, signalling, protocols and messages definition are need to be added accordingly in the future. }

## Registration

### IPU registration

### IVU registration

## Configuration

### IPU configuration

### IVUconfiguration

## IVU calling

### Start IVU manually

### Stop IVU manually

### Start IVU by event trigger

### Stop IVU by event trigger

## Intelligent event report

### IVU report intelligent event

### IPU report intelligent event

## OSD parameters issuing to IPU

### Issuing procedure

### OSD parameters profile

## Service subscription

_______________