| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | TD-33 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.VSMprot |  |  |  |
| Title: | H.VSMprot “Signalling and protocols for mobile visual surveillance” (New): Output draft |  |  |  |
| Purpose: | Information |  |  |  |

# Introduction

This document contains the editors’ output draft for H.VSMprot “Signalling and protocols for mobile visual surveillance” (New) from the SG16, Chengdu, 8-12 June 2015 meeting.

Updates were based on the following contributions:

| AVD-4767 | China Telecom, Beijing Univ. of Posts and Telecom | H.VSMprot: Proposal to update clause 6.1 and 6.2 in H.VSMprot | Q21/16 |
| --- | --- | --- | --- |
| AVD-4768 | China Telecom, Beijing Univ. of Posts and Telecom | H.VSMprot: Proposal to update clause 6.3 and add 6.4 in H.VSMprot | Q21/16 |

NOTE– Editorial adjustments introduced by TSB.

# CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations 6 5 Conventions 6 6 Protocol reference points 6 6.1 Reference point Cms: CMU-MSP 7 6.1.1 Interface function 7 6.1.2 Interface protocol 8 6.1.3 Message format 8 6.2 Reference point Msm: MSP-M_CU 15 6.2.1 Interface function 15 6.2.2 Interface Protocol 15 6.2.3 Message Format 15 6.3 Reference point Wm:WAP GW-MSP 18 6.3.1 Interface function 18 6.3.2 Interface protocol 18 6.3.3 Message format 18 6.4 Reference point Mv:M_CU-VAU/PSS 19 6.4.1 Interface function 19 6.4.2 Interface protocol 19 6.4.3 Message format 19 6.5 Reference point Mv:M_CU-VAU （PTZ control） 20 6.5.1Interface function 20 6.5.2Interface protocol 20 6.5.3Message format 20 6.6 VAU and PSS management interface 23 6.6.1Interface function 23 6.6.2Interface protocol 23 6.6.3Message format 23 7 Signalling and protocols 27 7.1 Real-time video/audio acquisition 27 7.2 Recording and playback 29 7.2.1 Playback in platform 29 7.2.2 Playback in PU 31 7.3 PTZcontrol 32 7.4 Service management 33

# List of Tables

Page

Table 6-1-1: Interface: MSP->CMU 8 Table 6-1-2: Interface: CMU->MSP 8 Table 6-1-3: Interface: MSP->CMU 9 Table 6-1-4: Interface: CMU->MSP 9 Table 6-1-5: Interface: MSP->CMU 10 Table 6-1-6: Interface: CMU->MSP 10 Table6-1-7: Interface: MSP->CMU 11 Table 6-1-8: Interface: CMU->MSP 11 Table 6-1-9: Interface: MSP->CMU 11 Table 6-1-10: Interface: CMU->MSP 12 Table 6-1-11: reqVauUrlReques Interface 12 Table 6-1-12: reqVauAddResponse Interface 13 Table 6-1-13: contain fields 14 Table 6-2-1:M_CU Authentication certification and Packet access 15 Table 6-2-2:MSP-MCU 15 Table 6-2-3:Surveillance points list query 16 Table 6-2-4: returns list of surveillance point 16 Table6-2-5: Video surveillance point play entrance query 17 Table 6-2-6:Returns the link address 17 Table 6-2-7: The statistics of mobile phone surveillance video data flows, duration 18 Table 6-3-1: Interface: WAP GW->MSP 18 Table 6-6-1:VAU interface 23 Table 6-6-2:VAU multi –platform data model 24 Table 6-6-3:VAU state management information in system model 24 Table 6-6-4:PSS configuration management information in system model 26 Table 6-6-5:PSSstate management information in system model 26

# List of Figures

Page

Figure 5-1 Functional Architecture Framework of mobile Visual Surveillance 7 Figure 6-2 Interface protocol 19 Figure 6-3 Signalling 19 Figure 7-1 Procedural flows for real-time media acquisition 28 Figure 7-2 Procedural flows for playback in platform 29 Figure 7-3 Procedural flows for playback in PU 31 Figure 7-4 Signalling of PTZ control 32 Figure 7-5 Signalling of VAU Registration 33 Figure 7-6 Signalling of MCU Authorization 33 Figure 7-7 Signalling of PU related parameters query 34

# Draft new Recommendation ITU-T H.VSMprot

# Signalling and protocols for mobile visual surveillance

# AAP Summary

[To be provided before Consent]

# Summary

TBD.

# Keywords

# Mobile Visual Surveillance, Architecture, Signalling and Protocols

# 1	Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The Recommendation H.VSMprot focuses on the Signalling and Protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

This Recommendation defines the reference points, signalling and protocols of the mobile visual surveillance system, based on the architecture defined in the Recommendation ITU-T H.VSMarch.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	RecommendationITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626(2011), Architectural requirements for visual surveillance.

[ITU-T H.627]	Recommendation ITU-T H.627(2012), Signalling and Protocols for visual surveillance.

[ITU-T H.VSMarch]	Recommendation ITU-T H.VSMarch (2011), Architectural requirements for mobile visual surveillance.

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP).

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1.

[IETF RFC 3261]	IETF RFC 3261 (2002),SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003),RTP: A Transport Protocol for Real-Time Applications.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]:<optional quoted definition>

3.1.2	<Term 2> [Reference]:<optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>:<definition>

# 4	Abbreviations

# TBD.

# 5 	Conventions

# In this Recommendation, the following conventions apply.

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended.

# 6	Protocol reference points

Recommendation [ITU-T H.VSMarch] describes overall architecture framework of mobile Visual Surveillance as the following figure, and it also introduces reference points of architecture. This document gives more detailed description of those reference points as well as some newly defined reference points.

# Figure 6-1  Functional Architecture Framework of mobile Visual Surveillance

# TBD.

The interface protocols which involved in mobile video surveillance include following protocols:

- Reference point Cms: CMU-MSP, uses the WebServiceinterfaceprotocol (SOAP/HTTP);

- Reference point Msm: MSP-M_CU, uses the HTTP protocol;

- Reference point Mv: M_CU-VAU, uses the UDPcommunication protocol;

- Reference point Vm: VAU-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Psm: PSS-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Vs: VAU-SCU, uses the HTTP protocol;

- Reference point Mdv: MDU-VAU, uses the RTP/RTCP protocol;

- Reference point Pv: PU-VAU, uses the RTP/RTCP protocol;

- Reference point Vp: VAU-PSS, uses the RTSP protocol;

- Reference point Wm:WAP GW-MSP, WAP GW puts the MDN number in the head domain of HTTP

## 6.1	Reference point Cms: CMU-MSP

### 6.1.1	Interface function

The interface is used forquerying the attributions of the surveillance point when the users login, and the information of all surveillance points available for mobile surveillance service.

- User authentication and surveillance point query: When user logs in MSP by M_CU, MSP could get the UserID and password of the current user (If user choose the universal sign-in way, MSP needs to translatethe MDN number into the corresponding UserID and password locally), then MSP authenticates the user to the CMU of video surveillance platformvia the UserID and password. If authentication failures, the failed message would return. If success, CMU would inquiry the surveillance point information according to current UserID, CMU returns the corresponding surveillance point and parameter list under the UserID in the MSP.

- Inquiry the enterprise surveillance pointsattribution: When enterprise users login Enterprise Self-SeviceSystem through the MSP, MSP could get the inquired information of the current surveillance point list that enterprise has activatedon CMU video surveillance platform through the enterprise account (such as CustID).

- RTSP media stream entrance inquiry:When user inquiry some kind of media streaming of one surveillance point, CMU would returns RTSP access addressthat user get through internal inquiry scheduling on the platform, which need the accesstoken.

When universal sign-in user requests resource, according to the original interface specification, the method called "reqUserChannelInfo".

### 6.1.2	Interface protocol

The interface is hosted on the IP protocol, using the Web Service interface protocol(SOAP/HTTP).

### 6.1.3	Message format

1)	The enterprise attribute information query

MSP query the enterprise attribute information (used when handling business opening worksheet by Telecom administrator)according to the corporate identify (customer ID)

# Table 6-1-1: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerId | Enterprise ID | string(64) | Telecom administrator manually input when the MSP service opened (18 bit) |

Explanation: the first six bitsof customerID refer to the domain ID (Platform domain)

# Table 6-1-2: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerName | Customer names | String(60) |  |
| CustomerType | Customer type | int(4) | Only : Enterprise:1 |
| Address | Customer address | String(100) |  |
| PhoneNumber | Customer phone number | String(30) |  |
| Fax | Customer fax number | String(30) |  |
| ZipCode | Customer zip code | String(6) |  |
| Email | Customer E-mail | String(60) |  |
| TransactPerson | The operator | String(30) |  |
| AdminAccount | video surveillance admin account | String(64) |  |
| ResultCode | Return code | Int(1) | 0 for enterprise ID not exist 1 for accessionsuccess |

Explanation: User use "adminAccount prefix" tologin MSP Enterprise Self Service page, the user ID and user account arethe same in format.

2)	Search enterprise information list

MSP query the enterprise attribute information (when handling business opening worksheet by Telecom administrator)according to the corporate identify (customer ID) or customer name.

# Table 6-1-3: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| searchCID | Enterprise ID | string(64) | Telecom administrator manually input when the MSP service opened (18) |
| searchCN | Enterprise name | string(255) | Company name |

Explanation: the first six bits of customerIDrefer to the domain ID (Platform domain).

# Table 6-1-4: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerName | Customer names | String(60) |  |
| CustomerType | Customer type | int(4) | Only : Enterprise:1 |
| Address | Customer address | String(100) |  |
| PhoneNumber | Customer phone number | String(30) |  |
| Fax | Customer fax number | String(30) |  |
| ZipCode | Customer zip code | String(6) |  |
| Email | Customer E-mail | String(60) |  |
| TransactPerson | The operator | String(30) |  |
| AdminAccount | video surveillance admin account | String(64) |  |

Explanation: User use "adminAccount prefix" to login MSP Enterprise Self Service page, the user ID and user account are thesame in format.

3)	Accept MSP user authentication

Accept MSP user authentication, including user’s login through the mobile phone and the enterprise administrator.

# Table 6-1-5: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| AccountType | User account type | Int(1) | 0 for user 1 for enterprise administrator |

Explanation: The account is MSP user account or the enterprise administrator account which has prefix processed.

# Table 6-1-6: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Authorization | Certification results | Int(1) | 0 for failed, 1 for success |
| The following list of fields |  |  |  |
| CategoryID | Catalogue number | String(128) |  |
| CategoryName | Catalogue name | string(128) |  |
| PID | The parent ID | string(128) | The parent directory CategoryID When the root directory PID element value defined for null values |

4)	User account query under the customer

The enterprise administrator could query for the list of all user accounts opened by enterprise.

# Table6-1-7: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerId | EnterpriseID | string(64) |  |

Explanation:the first six bits of customerID refer to the domain ID.

# Table 6-1-8: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| ResultCode | Return code | Int(1) | 0 for enterprise ID not exist 1 for accessionsuccess |
| Account | User account | String(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |

5)	Surveillance point query where user attribution

Query the authorized surveillance list under the specific user account.

# Table 6-1-9: Interface: MSP->CMU

| MSP-CMUinterface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| CategoryID | Catalogue number | string(64) | Catalogue numbers is a null value representing the query root directory |

Explanation: The account is MSP user account which has prefix processed.

# Table 6-1-10: Interface: CMU->MSP

| MSP-CMUinterface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| PuId-ChannelNo | PU ChannelNumber | string(128) |  |
| PuName | PUname | string(128) |  |
| PuProperty | PUProperty | structure type | IncludeptzFlag, playbackFlag, online parameters at present, |
| PtzFlag | Control authority flag | int(1) | 0 for no authorization; 1 for authorization |
| PlaybackFlag | Playback authority flag | int(1) | 0 for no authorization; 1 for authorization |
| Online | Online flag | Int(1) | 0 for offline; 1 for online |
| ResultCode | Return code | Int(1) | 0 for no user account; 1 for success |

6)	Query the VAU entrance

Query the RTSP address of VAU entrance、PTZ entrance.

# Table 6-1-11: reqVauUrlReques Interface

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| M_cuIp (Optional) | M_CU’s IP address | String(32) | Because VAU can span multiple network deployment. This parameter is provided to CMUand used to determine the return VAU which segment address. By default M_CU fill in 0.0.0.0, CMU returns VAU foreign service public address. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | NetType | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | Surveillance point unique identifier | String(128) |  |
| StreamingType | Streaming mediatype | Int(4) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |
| PlayMethod | Play type | Int(1) | 0:real-time video |

Response message (video surveillance system ->MSP)

# Table 6-1-12: reqVauAddResponse Interface

| CMU-MSPinterface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CMUPtzUrl | CMU PTZ URL address | String(128) | Return CMU PTZ URL address |
| VideoPlayUrl | Direct access entrance | String(128) | Return Direct access entrance URL address (PSS or VAU address) |

Among these, in addition to the VideoPlayUrl VAU/PSS address, it should contain the following fields:

# Table 6-1-13:  contain fields

| Parameter | Parameter name | Data type | Notes |
| --- | --- | --- | --- |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| NetType | NetType | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | Surveillance point unique identifier | String(128) |  |
| StreamingType | Streaming mediatype | Int(4) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |
| PlayMethod | Play type | Int(1) | 0:real-time video (fixed for 0 at present) |
| ExpireTime | Requesting the link failure time | String(16) | 20120406155530 Year-month-day-hour-min-sec |
| HashToken | MD5abstract | String(64) | Sixteen hexadecimal calculated by the MD5 (Account=xx&Password =MD5 (password) &PuId-ChanelId=xx&StreamingType=xx&ExpireTime=xx) |

HashToken requirements:

- If the HashToken parameter values in VideoPlayUrl was artificially modified, VAU should denied accessment;

- If VideoPlayUrl video link requests time exceeds the ExpireTime failure time, the link is invalid;

Video streaming could play sustainably in the established RTSP media channels, not subject to ExpireTime time limit;

## 6.2	Reference point Msm: MSP-M_CU

### 6.2.1	Interface function

M_CU and MSP interface use Web Service interface protocol (SOAP/HTTP), mainly to complete the following functions:

- Send the service request; realize video surveillance business docking and authentication through MSP;

- Get control point play entrance;

- Check version;

### 6.2.2	Interface Protocol

The interface is hosted on the IP protocol, using the Web Service interface protocol(SOAP/HTTP).

### 6.2.3	Message Format

# Table 6-2-1:M_CU Authentication certification and Packet access

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| LoginType | Login type | Int(1) | 0: Account-password login 1: Local login |
| Version | M_CU version | String(16) | For example: V1.05.004, larger version needs to be consistent with MSP. If the version of MSP>M_CU that M_CU L / class; if M_CU>MSP version, then prompts the current version of M_CU is too high, please use the low version. |

Table 6-2-2:MSP-MCU

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter nam | Data type(unit: bit) | Notes |
| Result | Certification results | Int(2) | 0:Success 1: user name not exist 2:wrong password 3: M_CU version too new 4:M_CU version too old |
| The following list |  |  |  |
| CategoryID | CategoryID | String(128) |  |
| CategoryName | CategoryName | string(128) |  |
| PID | The parent ID | string(128) | The parent directory CategoryID When the root directory PID element value defined for null values |

Table 6-2-3:Surveillance points list query

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| M_cuIp | M_CU’s IP address | String(32) | Because VAU can span multiple network deployment. This parameter is provided to CMU and used to determine the return VAU which segment address. By default M_CU fill in 0.0.0.0, CMU returns VAU foreign service public address. |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| CategoryID | Catalogue number | string(64) | Catalogue numbers is a null value representing the query root directory |

Table 6-2-4: returns list of surveillance point

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| PuId-ChannelNo | PU number and Channel Number | String(64) |  |
| PuProperty | PU ability of PTZ control | Int(4) | If no control ability, default is 0 |
| PuName | Surveillance point name | String(64) | For prompting the user to PU the specific significance in M_CU |
| Online | Online flag | Int(1) | 0 for offline; 1 for online |

PuId-ChannelNo: Corresponding to the PU number and channel number；

PuProperty: PU ability of PTZ control

# Table6-2-5: Video surveillance point play entrance query

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | PU number and Channel Number | String(64) |  |
| StreamingType | Streaming media type | Int(2) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |

Table 6-2-6:Returns the link address

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CMUPtzUrl | CMU PTZ URL address | String(128) | Return CMU PTZ URL address |
| VideoPlayUrl | Direct access entrance | String(128) | Return Direct access entrance URL address (PSS or VAU address) |

Table 6-2-7: The statistics of mobile phone surveillance video data flows, duration

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| BRun | Start / end flag | Int(2) | 0: Start 1: Process (heartbeat) 2:Quit |
| NStreamBit | Video data flow | Int(2) | The unit is KB |
| NStreamTime | Video data time | Int(2) | The unit is Second |

Note: the statistical MCU access to the MSP, and the total length of the video data flow from MCU to MSP, report, in theory every normal exit report once. But the need to consider abnormal exit MCU, general users in more than 10 seconds, you need every 10 seconds to be reported to MSP a, MSP judge, statistics according to the account, to "exit" identification report shall prevail, or report the last to process shall prevail.

## 6.3	Reference point Wm:WAP GW-MSP

### 6.3.1	Interface function

Reference point Mw is between MSP and WAP GW.

It is used to return the MDN number of user’s cell phone to MSP.

### 6.3.2	Interface protocol

TheWAP GW put the MDN number in the head domain of HTTP.

### 6.3.3	Message format

MSP query the enterprise attribute information (used when handling business opening worksheet by Telecom administrator)according to the corporate identify (customer ID)

# Table 6-3-1: Interface: WAP GW->MSP

| WAP GW->MSP interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| user-agent | Terminal Type | string(256) | Identify the type of browser used in user’s terminal (255 bit),Apply for WAP1.X and WAP2.0. For terminals that do not support UAProfile, this parameter represents the type of terminal. |
| x-up-calling-line-id | MDN number(cell phone number) | String(32) | User ID, the MDN number (cell phone number) of user. |

## 6.4	Reference point Mv:M_CU-VAU/PSS

### 6.4.1	Interface function

Reference point Mv is between M_CU and VAU/PSS.

It is used to transmit the real-time video through RTSP by establishing connection and  transmitting the real-time stream of media and real-time video.

### 6.4.2	Interface protocol

The Interface is actually between the terminal and the streaming server, it follows the TCP / IP, RTP / RTSP / RTCP protocol, as shown in the Figure 5-2.

| RTP | RTSP |  |
| --- | --- | --- |
| UDP | TCP | UDP |
| IP |  |  |

Figure 6-2  Interface protocol

And the signalling is shown in Figure 6-3.

M_CU

VAU/ PSS

RTSP Request

Media stream

# Figure 6-3 Signalling

### 6.4.3	Message format

M_CU send RTSP requests through VideoPlayUrl CMS querying by MSP.

URL is like this:

rtsp://VideoPlayUrl/service?Account=xx&PuId-ChanelId=xx&StreamingType=xx&ExpireTime=xx&HashToken=xx

The RTSP interface meets following needs:

- RTSP interface requirements

- Meet the minimum requirements of client and server basic playback in the Appendix D  ofRFC2326 "Real Time Streaming Protocol (RTSP)".

- In accordance with the requirements of 3GPP TS 26.234, the client and server should support the RTSP DESCRIBE method.

- In accordance with the requirements of 3GPP TS 26.234, the client and server should support the RTSP Range header field;

- In accordance with the requirements of 3GPP TS 26.234, the server need to include the Range fieldin theresponding message to PLAY.

- RTP packet format

VAU packet format adoptthe TS26.234 (based on RFC 3984) standard, requiring VAU to support Single NAL Unit Packet format or FU-A packet format.

## 6.5	 Reference point Mv:M_CU-VAU （PTZ control）

### 6.5.1	Interface function

It is used to control PTZ for end-users when browsing real-time video.

### 6.5.2	Interface protocol

The communication  between M_CU and  VAU is according to UDP protocol ，The default communication port:5060. The pack for request and response reserves 200 bytes for via fields in head of SIP

M_CU sends the PTZ control messages to VAU without 200OK response or resend. But if within the definition time M_CU doesn’t send the stop message to VAU, VAU will send PTZstop message to the platform automatically.

Considering several M_CUs visit one PUID-ChannelNoat the same time, M_CU will send the PUID-ChannelNo carried by RTSP request to VAU.VAU distinguish PTZ messagetarget deviceaccording to the PTZ message body

<IE_HEADER MessageType="MSG_PTZ_SET_REQ"

UserID=”User name”

DestID = "PUID-ChannelNo"

/>

- M_CU

M_CU sends request and receives response .When it receives response, M_CU must check XML

Message_Type and Sequence_Number.  If the parameter is wrong, the response must be discarded

- VAU

VAU should be able to receive request from any IP address and port, and reply the port from  M_CU request

### 6.5.3	Message format

generic-message = start-line

*message-header

CRLF

[ message-body ]

1） start-line   start-line = Request-Line / Status-Line

SIP  request ：

Request-Line = Method SP Request-URI SP SIP-Version CRLF

SIP response ：

Status-Line = SIP-Version SP Status-Code SP Reason-Phrase CRLF

2） message-header

Request  for message-header：

INFO sip:xxx

Content-Type: application/global_eye_v10+xml

Content-Length: xxx

To: xxx

# From: xxx

CSeq: xxx INFO

Call-ID: xxx

Max-Forwards: xxx

Via: xxx

Contact: xxx

[CrLF]

Response  to message-header：

200 OK sip:xxx

Content-Type: application/global_eye_v10+xml

Content-Length: xxx

To: xxx

# From: xxx

CSeq: xxx INFO

Call-ID: xxx

Max-Forwards: xxx

Via: xxx

Contact: xxx

[CrLF]

3）  Message-body

PTZ control

a）Function:Theuser platform sends PTZ control message to another platform .

b）  Message ：

<!—PTZ control  -->

<?xml version="1.0" encoding="UTF-8" ?>

<Message Verison="1.0">

<IE_HEADER MessageType="MSG_PTZ_SET_REQ"

UserID=”User name”

DestID=”PUID-ChannelNo”

/>

<IE_PTZ OpId="code"

Param1="parameter1"

Param2="parameter2"

/>

</Message>

This message need no response

4）  Code and parameter

1：turn up（1--10）

2：turn down（1--10）

3：turn left（1--10）

4：turn right（1--10）

5：open iris（1--10）

6：close iris  （1--10）

7：zoom in（1--10）

8：zoom out（1--10）

9：focus near（1--10）

10：focus far（1--10）

11：RESERVED

12：RESERVED

13：RESERVED

14：RESERVED

15：stop

20：scan off

21：scan on（1--10）

24：RESERVED

25：RESERVED

26：RESERVED

## 6.6	VAU and PSS management interface

### 6.6.1	Interface function

This interface transfers performance data,alarm data and topological data.

### 6.6.2	Interface protocol

Interface protocol is HTPP＋XML、SOAP or CORBA，for  different platforms and systems.

### 6.6.3	Message format

6.6.3.1VAU interface

It is to control and manage VAU according to the definition of MIB.

- Configuration management

VAUinformation model as following table:

# Table 6-6-1:VAU interface

| Name | Data type | Read/Write |
| --- | --- | --- |
| VAU device ID | String | R/W |
| Version number | String | R/W |
| Equipment manufacturer code | String | R/W |
| Equipment manufacturer name | String | R/W |
| IP network parameter | String | R/W |
| Maximum number of video input | Integer | R/W |
| Maximum number of video output | Integer | R/W |
| Transcoding type | String |  |
| Platform number | Integer |  |

Table 6-6-2:VAU multi –platform data model

| Name | Data type | Read/Write |
| --- | --- | --- |
| CMS device ID | String | R/W |
| IP address | String | R/W |
| Port | String | R/W |

- state management

# Table 6-6-3:VAU state management information in system model

| Name | Date type | Read/Write |
| --- | --- | --- |
| VAU device ID | String | R |
| CPUusage | String | R |
| Memory usage | String | R |
| Current processes number | Integer | R |
| Current thread number | Integer | R |
| Registration status | String | R |
| Current video inputs | Integer | R |
| Current video outputs | Integer | R |
| Current transcoding number | Integer | R |
| VAU heartbeat statue | String（1） | R |

6.6.3.2 PSS interface

It is just the MIB definition of PSS management, which can be used to control and manage PSS

- Configuration management

# Table 6-6-4:PSS configuration management information in system model

| Name | Data type | Read/Write |
| --- | --- | --- |
| PSS device ID | String | R/W |
| Version Number | String | R/W |
| Equipment manufacturer code | String | R/W |
| Equipment manufacturer name | String | R/W |
| Maximum number of video input | Integer | R/W |
| Maximum number of video output | Integer | R/W |

- Statue management

# Table 6-6-5:PSSstate management information in system model

| Name | Data type | Read/Write |
| --- | --- | --- |
| PSS device ID | String | R |
| CPU usage | String | R |
| Memory usage | String | R |
| Current processes number | Integer | R |
| Current thread number | Integer | R |
| Registration status | String | R |
| Current video inputs | Integer | R |
| Current transcoding number | Integer | R |
| PSS heartbeat statue | String（1） | R |

# 7	Signalling and protocols

[NOTE – This clause will provide the identification of signalling or protocols.]

The following is some illustrative items:

- The SIP message format is required to  comply with the definition in IETF RFC3261;

- The RTSP message format is required to comply with the definition in IETF RFC2326;

- The HTTP message format is required to comply with HTTP1.1 defined in IETF RFC2616;

- RTP/RTCP signaling and message is required to  comply with IETF RFC3550;

- TBD.

## 7.1	Real-time video/audio acquisition

The Real-time Video/Audio Acquisitionis used to view the real-time surveillance video/audiodata by mobile terminals.After MSP performs authentication and returnsthe VAU information,the M_CU choose and request to the VAU for real-time video/audio data.

When a user wants to view the real-time surveillance video/audio by mobile terminal, the M_CU initiates a request to the VAU for real-time video/audio streams after MSP performing authentication and returning VAU information to M_CU.

# Figure 7-1 Procedural flows for real-time media acquisition

# This figure shows the procedural flows for real-time Media Acquisition for one M_CU:

1)	First, the M_CU initiates a request and creates a connection with the MSP.

2)	The MSP initiates a request and creates a connection with the CMU.

3)	The CMU returns the VAU information.

4)	The MSP gets the request and returns VAU information to M_CU.

5)	When the M_CU gets VAU information, it requests the VAU for the real-time media address with the information about the network type in which the M_CU is located.

6)	VAU receives this request and requests to SCU for the media address. At the same time, VAU queries the corresponding transcoding parameters from SCU.

7)	The SCU transfers this request to the MDU.

8)	After receiving messages, the MDU returns the real-time media address to the SCU.

9)	The SCU returns the real-time media address to the VAU.

10)	After receiving the real-time media address, the VAU requests the MDU to start transmitting media stream.

11)	The MDU sends a request to the PU for initiating a media flow.

12)	The PU creates media channel with the MDU and sends media flow to the MDU.

13)	The MDU transfers the media flow to the VAU. VAU performs transcoding according to the transcoding parameters.

14)	The transcoded media flow is transferred from the VAU to the M_CU. The M_CU decodes and plays the received media flow.

15)	When the M_CU stops the real-time media acquisition it sends a stop-media request to the VAU.

16)	VAU stops transcoding after receives the request. The VAU stops sending media stream to the MDU.

17)	The MDU sends response to the VAU for stopping media flow.

18)	The VAU requests the SCU to disconnect which then returns a response to the VAU.

19)	Then the VAU ends this real-time media acquisition and sends response to the SCU.

## 7.2	Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

### 7.2.1	Playback in platform

When users want to playback the surveillance point using mobile phones, they must log into the MSP by the M_CU. The MSP performs authentication and returned the playback surveillance list.

After the users choose one surveillance point name, the M_CU sends a request for this surveillance video to the VAU, and then VAU returns the media stream to the M_CU to play.

# Figure 7-2 Procedural flows for playback in platform

# This figure shows the procedural flows for playback in platform:

1)	The M_CU sends a request to the MSP for querying recording information.

2)	The MSP sends a request to the CMU for querying recording information.

3)	The CMU sends a request to the MSU for querying recording information.

4)	The MSU returns the querying result to the CMU.

5)	When the CMU gets the result, it transfers the result to the MSP.

6)	When the MSP gets the result, it transfers the result to the M_CU.

7)	The M_CU selects one playback point and starts a request to playback.

8)	The MSP sends a request for the playback address to the CMU.

9)	The request is transferred from the CMU to the SCU.

10)	The request is transferred from the SCU to the MDU.

11)	The MDU responses to the request and sends the playback address back to the SCU.

12)	The playback address is sent from the SCU to the CMU.

13)	The CMU sends the playback address to the MSP.

14)	The playback address is sent from the MSP to the M_CU.

15)	The M_CU starts playback request for the VAU with the playback address and the network type.

16)	The VAU starts playback request for the MSU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

17)	The MSU returns a response to the VAU and sends the media stream to the VAU.

18)	Once the VAU gets the media stream, it performs transcoding according to the transcoding parameters and then transfers the transformed stream to the M_CU.

19)	The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receiving the request.

20)	The VAU sends a stop playback request to the MSU.

21)	In response, the MSU stops sending the stream.

22)	At the same time, the MSU notifies the MDU to stop playing.

23)	The MDU returns a response to the MSU.

### 7.2.2	Playback in PU

# Figure 7-3 Procedural flows for playback in PU

# This figure shows the procedural flows for playback in PU:

1)	The M_CU sends a request to the MSP for querying recording information.

2)	The MSP sends a request to the CMU for querying recording information.

3)	The CMU sends a request to the PU for querying recording information.

4)	The PU returns the querying result to the CMU.

5)	When the CMU gets the result, it transfers the result to the MSP.

6)	When the MSP gets the result, it transfers the result to the M_CU.

7)	The M_CU selects one playback point and starts a request to playback.

8)	The MSP sends a request for the playback address to the CMU.

9)	The request is transferred from the CMU to the SCU.

10)	The request is transferred from the SCU to the MDU.

11)	The MDU responses to the request and sends the playback address back to the SCU.

12)	The playback address is sent from the SCU to the CMU.

13)	The CMU sends the playback address to the MSP.

14)	The playback address is sent from the MSP to the M_CU.

15)	The M_CU starts playback request for the VAU with the playback address and network type.

16)	The VAU starts playback request for the PU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

17)	The PU returns a response to the VAU and sends the media stream to the VAU.

18)	Once the VAU gets the media stream, it performs transcoding based on the transcoding parameters and then transfers the transformed stream to the M_CU.

19)	The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receiving the request.

20)	The VAU sends a stop playback request to the PU.

21)	In response, the PU stops sending stream.

22)	At the same time, the PU notifies the MDU to stop playing.

23)	The MDU returns a response to the PU.

## 7.3	PTZcontrol

The PTZ Control function is used to pan, tilt and zoom cameras, switchauxiliary peripherals (such as flashlight, fan and rain brush), and to preset the position of relatedphysical devices.

VAU receives a PTZ control request from the M_CU, parses and transfers this request to SCU. After authentication, SCU transfers this request to PU to perform PTZ operation.

M

_

CU

PU

7

.

Upload logs

8

.

Response

VAU

CMU

1

.

PTZ control Request

2

.

PTZ control Request

SCU

3

.

PTZ control Request

4

.

PTZ control Response

5

.

PTZ control Response

6

.

PTZ  control Response

# Figure 7-4 Signalling of PTZ control

# This figure shows the procedural flows for PTZ Control:

1)	M_CU sends a PTZ control request to VAU.

2)	VAU transfers this request to SCU.

3)	After authentication, SCU transfers this request to PU.

4)	PU parses this request and transfers PTZ control commands to related physical devices. Then the devices perform PTZ operations. And PU returns a response to SCU

5)	SCU transfers this response to VAU.

6)	VAU returns this response to M_CU.

7)	SCU writes logs about this PTZ control event, such as operation time and operation type, then uploads the logs to CMU.

8)	CMU returns a response to SCU to complete log uploading.

## 7.4	 Service management

#### 7.4.1 VAU registration

VAU registers to CMS with its ID number and the capability information (including the transcoding and distribution capability), and re-registers automatically in periods according to the configuration of the registration information.

VAU

CMS

VAU gets the registration information

Registration Request

Registration Response

# Figure 7-5 Signalling of VAU Registration

#### 7.4.2	MCU authorization

MCU sends request to MSP. MSP gets the UserID and the password and send authorization request to CMS. The CMS returns the ok information or the error code.

M

_

CU

WAPGW

CMS

Authorization Request

Verify the User Info

MSP

Return OK or Fail Info

Returnok or error code

# Figure 7-6 Signalling of MCU Authorization

#### 7.4.3 PU related parameters query

# This figure shows the procedural flows of PU related parameters query:

1)	MSP queries the source list of the current UserID.

2)	The mobile VS platform transfers the PU list and related information to MSP. The information includes the PUID-ChannelNo, PUName and PuProperty.

3)	MSP processes the information and returns it to MCU for display.

M

_

CU

MSP

Authorization

Query the related source

CMS

Return the related information

Return the processed Info

# Figure 7-7 Signalling of PU related parameters query

_____________________