| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4787 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE, China Telecom, MIIT |  |  |  |
| Title: | H.IVSArch: reference points between IPU and other units |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution tries to describe the reference points between IPU and other units including SCU, MDU and ICU. It also makes some editorial changes about the reference point between IPU and CMU.

# Discussion

Section 7.3 defines the reference points which are introduced for intelligent functions between the components based on the framework shown in the figure of system composition. It is necessary to provide some descriptions of those reference points. So this contribution proposes to add contents for reference points.

# Conclusion

It is proposed to make these changes in H.IVSArch. Change is marked up against the version from the last meeting.

[Begin Proposal]

## 7.3	Reference Points

{Editor’s note: this section describes the reference points between the components based on the framework shown in the figure of system composition.}

### Reference Point: IPU – CMU

The reference point Pm is located between the IPU and the CMU. It is used for the following functions:

- For the IPU to make the registration to the CMU.

- For the IPU to report the identified result such as event trigger information or data acquisition information to the CMU.

- For the CMU to manage the IPU.

- For the CMU to query the running status of IPU.

- For the CMU to configure parameters and to upgrade the software version.

### Reference Point: IPU – SCU

The reference point Ps is located between IPU and SCU. It is used for the following functions:

- For the IPU to report the identified result such as event triggers information or data acquisition information to the SCU.

- For the SCU to control the service of the SCU, to control the alarm and linkage action, media acquisition, instant image snapshot, recording and playback and voice (audio) communication

- For the SCU to manage the IPU device, to query running status of IPU devices, to configure parameters, to upgrade software version, and to control PTZ by forwarding PTZ control operation commands and return result.

### Reference Point: IPU – MDU

The reference point Pd lies between IPU and MDU. It is used for the following functions:

- For the MDU and IPU to support the video/audio and image acquisition.

- For the MDU and IPU to support the recording in the service platform.

- For the MDU and IPU to support voice (audio) communication.

### Reference Point: IPU – ICU

The reference point Pc/Pc’ lies between IPU and ICU. It is used to:

- Deliver interaction signals directly between the IPU and the ICU (such as an RTSP request or response).

- Directly deliver media streams from IPU to ICU.

[End Proposal]

__________________