| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4789 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.VCDN.1-NV: proposed updated Figure 6-1and the related content |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update the Figure 6-1 with more related Functions.

# Introduction

In the last meeting, the group created a new draft Recommendation to describe the network virtualization in the vCDN service.  The Figure 6-1 shows the overview of VCDN architecture with the scope of network virtualization functions. The VCDN has 2 core functions: Resource Control Functions and Resource Function. Those 2 functions combination provides the capability to virtualize the CDN main functions into a virtualized resource. But in fact, the virtualized CDN functions should be followed the orchestrator and management system from a network virtualization system. Furthermore, the orchestrator and management system can be integrated with vCDN resource control functions or be separated. The requirement of CDN function virtualization is coming from CDN management system then vCDN will implement it.

In this contribution, it is proposed to modify the Figure 6-1 and the related content. The detailed functional blocks will be expected in the future contributions.

# Proposal

- Proposes to update Figure 6-1 and the related content

# Start proposal

# 6   Introduction

This section is going to describe overview of functional requirements, components and the possible approach of network virtualization in VCDN.

The target of legacy CDN is to push the content to the server node which is nearest to the end-user. It also provides the capability of locating the content, content backtracking, content delivery route, etc. But in real implementation, the efficiency of CDN will be affected by many factors, such as loading balance, bandwidth, network traffic and the underlay network topology. The current network is fixed, non-service aware and hard to replace. In some CDN solution, CDN node may have the capability to detect the network status periodically. The problem is the network information detected is unreliable, which reduce the CDN service efficiency. In the other hand, it is difficult for Service Provider to create and distribute a new service based on the fixed network hardware and configuration.

Therefore, as the fundamental element of CDN, network innovation will bring a great benefit to Service Provider or CDN operator to optimize their CDN implementation.  A flexible, programmable and service-aware network would give an opportunity to solve the problem just mentioned above and could reduce the cost of CDN equipments investment and maintenance.

Figure 6-1 shows the functional component that related to the network virtualization in VCDN.

Figure 6-1: Functions related to network virtualization in VCDN architecture

The Figure 6-1 provides the basic idea of network virtualization. The physical network resource, such as router and switches, in Network Functions can be abstracted in the Resource Function and be orchestrated and managed in Resource Control Functions (RCF). The orchestrator and management can be implemented in a VN MANO (virtual network management and orchestrator) Functions which could be integrated with RCF. Alternatively, the VN MANO Functions can be separated from RCF as a 3rd party Functions. The RCF can communicate with 3rd party MANO with a standard interface.

The network resource controlling may be based on the request from the Content Delivery Management Functional Block or dynamically modified according to the network information from under layer network.

The Resource Functions (RF) will provide abstract infrastructure resource based on the physical resource from Content Delivery Functions (CDF) and created the required virtual resource by the instruction from RCF.

# End Proposal

__________________