| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4790 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.VCDN.1-NV: proposed functions and functional blocks in Resource Control Functions |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to give the detailed functional blocks related to Network Virtualization in Resource Control Functions.

# Introduction

In the last meeting, the group created a new draft Recommendation to describe the network virtualization in the vCDN service.  In this contribution, it is proposed to present a figure with the architecture of functions and functional block in Resource control functions.

In general, if CDN functions are intended to be virtualized, the couples of requirements should be firstly received from the Content delivery management system. It determines the requirement of expected functions to implement the service logic corresponding with the Operator’s purpose. Then the requirements are transported to a MANO system to instantiate the detailed VN applications and allocated the required network resource. The lifecycle of APP and resource can be managed by RCF according to the existing period of vCDN service. The MANO system can be implemented in the RCF but also could be a 3rd party entity. The Reference Point should be defined based on the relationship between functions and functional blocks in the future contributions.

# Proposal

- Proposes to add the new Figure 7-1 and descriptions.

# Start proposal

## 7.1 Functions and Functional blocks in Recourse Control Functions

This clause is going to describe the detailed functional blocks which related to the virtual network creation, configuration and management in the RCF .

The Resource Control Functions handles the orchestration and management of the virtual network resource from the Resource Functions. It also creates the application instance to march the vCDN operator’s requirements. With the combination of those components, the RCF is able to control the virtual network resource creation, allocation and release.

The Figure 7-1 shows the Functions and Functional Blocks that contains in the RCF.

Figure 7- 1: Functional blocks in Recourse Control Functions related to Virtual Network instance

## (Contributor note: VN=Virtual Network)

## [Notes:] The architecture of Virtual Network Resource Control Functions in this Recommendation covers the instantiation of Virtual Network. But it could also cover the instantiation of Virtual Storage and Virtual Computing as well.

## 7.1.1 Virtual Network Application Functions

The Virtual Network Application (VNA) Functions enables the RCF to implements the CDN service logical operations as instances with virtualized network resource, e.g. content distribution with virtualized CDN nodes.

### 7.1.2 Virtual Network MANO Functions

The Virtual Network MANO (VN-MANO) Functions handles the lifecycle of vCDN service, VNAF and the virtualized network resource control corresponding with the vCDN service request. It is composed of Orchestrator, VN application management and VN resource control functional blocks.

It is noted that the VNMANOF is alternatively existed out of the RCF as a 3rd party Functions.

#### 7.1.2.1 Virtual Network Orchestrator Functional Block

The Orchestrator is responsible for the lifecycle management of VN service, the coordination of VN applications management in vCDN and the orchestration of VN resource.

#### 7.1.2.2 Virtual Network Application Management Functional Block

The Virtual Network Application Management (VNAM) functional block is responsible for the instantiation and lifecycle management of the VN application related to vCDN service. It is noted that the VNAM functional block is possibly composed of many independent instances.

#### 7.1.2.3 Virtual Network Resource Control Functional Block

The Virtual Network Resource Control (VNRC) functional block is responsible for the controlling and managing the Virtual Network Resource from the Resource Functions.

# End Proposal

__________________