| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4792 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.VCDN.1-NV proposed use case of vCDN nodes deployment |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to provide a use case of vCDN node deployment in a CDN network overview.

# Introduction

In the last meeting, the group created a new draft Recommendation to describe the network virtualization in the vCDN service.  The central principle of vCDN is to virtualized CDN functions and migrate them in a general hardware/software resource layer. CDN has many functions such as content distribution, delivery, loading balance and user request redirect. For the different purposes, vCDN may have different approaches. This contribution will discuss one of the use cases: vCDN node deployment dynamically.

# Proposal

- Proposes to add the new Appendix I.1 virtualized CDN node deploy dynamically

# Start proposal

Appendix I

Use cases of network virtualization supporting vCDN services

(This Appendix does not form an integral part of this Recommendation.)

The legacy telecom service, such as IPTV, is usually deployed on the Telecom Operator’s dedicate network. CDN is also a distribute network overlay with the legacy transport network. But in current market, CDN is expected to be designed for the integrated service in an open network environment. The integrated services may be run on the different platforms or networks with their own service logic. According to the features of the integrated service, network virtualization is a prospective approach to improve the CDN capability to use the network resource with higher efficiency and flexibility by ignoring the difference of hardware/software.

So the network virtualization for vCDN is considered to have the following use cases.

I.1 virtualized CDN node deploy dynamically and flexibly

In general, CDN nodes deployment depends on many factors such as geography location, target user density, etc. Usually the deployment purpose and node device should be determined by CDN operators. Once the CDN nodes are deployed, the configuration and maintenance for CDN nodes are usually keep in stable. But in that situation, if one CDN node capability is exceeded its limitation by the rapid increasing of user request in a peak time, it is difficult to add a new CDN node in time. Also a deployed CDN node resource may not be used efficiently in the normal time. vCDN approach is able to create a new logical CDN node is network device supporting network virtualization by Resource control request and to release the resource when the peak time is passed.

Figure A-1: vCDN nodes deploy dynamically

As the Figure A-1shows, a vCDN node can be created and deployed by the service request. The functionality of CDN node can be virtualized and programmed on the virtual machine in a server located in a data centre or any device which supports NFV/SDN functions.

# End Proposal

__________________