| RAPPORTEUR MEETING OF QUESTION 21/16 |  | TD-07 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | STUDY GROUP 16 |
|  |  | Original: English |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |
| RAPPORTEUR MEETING DOCUMENT |  |  |
| Source: | H.IVSArch Editor |  |
| Title: | H.IVSArch "Architectural requirements for intelligent visual surveillance" (New):Output draft (Changzhou, 26-29 September 2016) |  |
| Purpose: | Proposal |  |

This document contains the editors’ output draft for H.IVSArch "Architectural requirements for intelligent visual surveillance" (New) from the SG16, Changzhou, 26-29 September 2016.

Updates were based on the following contributions:

| AVD-4787 | ZTE, China Telecom, MIIT | H.IVSArch: reference points between IPU and other units | Q21/16 |
| --- | --- | --- | --- |
| AVD-4788 | ZTE, China Telecom, MIIT | H.IVSArch: registration and configuration service control flow for ICU | Q21/16 |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations 6 5 Conventions 6 6 Functional Requirements 7 6.1 Service Function 7 6.1.1 IVU order and change 7 6.1.2 IVU registration and configuration 7 6.1.3 Task schedule 7 6.1.4 Image and video acquisition 7 6.1.5 Intelligent event report 7 6.1.6 Intelligent analysis result show 7 6.1.7 OSD parameters issuing to IPU 7 6.2 Service management 7 6.2.1 IVM Management 7 6.2.2 IVU Management 7 6.2.3 Network Management 8 6.2.4 Remote upgrade 8 7 Functional Architecture 8 7.1 Functional architecture framework 9 7.2 Functional entities 9 7.2.1 Intelligent Premises Unit 9 7.2.2 Intelligent customer unit 10 7.2.3 Intelligent video management 11 7.2.4 Intelligent video unit 11 7.3 Reference Points 12 7.3.1 Reference Point: IPU – CMU 12 7.3.2 Reference Point: IPU – SCU 12 7.3.3 Reference Point: IPU – MDU 12 7.3.4 Reference Point: IPU – ICU 12 7.3.5 Reference Point: ICU - CMU 13 7.3.6 Reference Point: ICU - SCU 13 7.3.7 Reference Point: ICU - MDU 13 7.3.8 Reference Point: ICU - MSU 13 7.3.9 Reference Point: IVU - SCU 13 7.3.10 Reference Point: IVU - MDU 13 7.3.11 Reference Point: IVU – IVM 13 7.3.12 Reference Point: IVM – CMU 13 7.3.13 Reference Point: IVM – SCU 13 8 Service control flow and signaling 13 8.1 Registration 13 8.1.1 IPU registration 13 8.1.2 IVU registration 14 8.1.3 ICU registration 14 8.2 Configuration 14 8.2.1 IPU configuration 15 8.2.2 IVU configuration 15 8.2.3 ICU configuration 16 8.3 IVU calling 16 8.3.1 Start IVU manually 16 8.3.2 Stop IVU manually 17 8.3.3 Start IVU by event trigger 17 8.3.4 Stop IVU by event trigger 17 8.4 Intelligent event report 17 8.4.1 IVU report intelligent event 17 8.4.2 IPU report intelligent event 17 8.5 OSD parameters issuing to IPU 17 8.5.1 Issuing procedure 17 8.5.2 OSD parameters profile 17 8.6 Service subscription 17

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 7 – System composition of intelligent Visual Surveillance 8 Figure 7-2-1 – Function modules of IPU 10 Figure 7-2-2 – Function modules of ICU 11 Figure 8-2-1 – Flow for IPU configuration 15 Figure 8-2-2 – Flow for IVU configuration 15 Figure 8-2-3 – Flow for ICU configuration 16 Figure 8-3-1– Flow for start IVU manually 16 Figure 8-3-2 – Flow for stop IVU manually 17

Draft new ITU-T Rec. H.IVSArch

Architectural requirements for intelligent visual surveillance

AAP Summary

[To be provided before Consent]

Summary

TBD.

Keywords

Visual surveillance, architecture, intelligent

Scope

The Intelligent Visual Surveillance system is focusing on value-added intelligent applications based on visual (audio included also) application technology and intelligent algorithm, which provides services of target recognition, event recognition and intelligent alarm, data analysis, intelligent video diagnosis, intelligent video retrieval, intelligent video synopsis to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability.

Intelligent visual surveillance could automatically identify specific objects, behaviours or attributes in video. It processes the video stream and transform into structured data to be transmitted or archived so that the visual surveillance system can act accordingly.

The Recommendation H.IVSArch focuses on the functional requirements of an intelligent visual surveillance system, including the functional requirements, model, architecture, entities, signalling and reference points.

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance.

[ITU-T H.627]	Recommendation ITU-T H.627 (2009), Signalling and Protocols for visual surveillance.

[ITU-T H.626.1]	Recommendation ITU-T H.626.1 (2011), "Architectural requirements for mobile visual surveillance".

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP)

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol – HTTP/1.1

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

Definitions

Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

Abbreviations

| APP | Application |
| --- | --- |
| CIV | Client Intelligent Video |
| CMU | Centre management unit |
| CU | Client Unit |
| ICU | Intelligent Customer Unit |
| IPU | Intelligent Premises Unit |
| IVM | Intelligent Video Management |
| IVU | Intelligent Video Unit |
| IVS | Intelligent Visual Surveillance |
| MDU | Media Distribution Unit |
| MSU | Media Storage Unit |
| OSD | On Screen Display |
| PIV | Premises Intelligent Video |
| PTZ | Pan/Tilt/Zoom |
| SCU | Service Control Unit |

Conventions

TBD.

Functional Requirements

{Editor’s note: this clause provides the main functions description of intelligent visual surveillance system. And in the section8, it will provide the corresponding procedures and signalling.

- 6.1 Service Function

IVU order and change

Users are recommended to order and change the category, supplier and capacity of IVU through IVM portal.

IVU registration and configuration

IVM is required to support strategies configuration of intelligent application by users and video sources schedule dynamically according to the strategies. IVM can accept registration, deletion, capability report from IVU and schedule IVU dynamically

Task schedule

Users are recommended to operate IVM to manage the intelligent analysis task such as query, start, pause and stop

Image and video acquisition

Users are recommended to acquire the device list and query the RTSP address of streaming media. Then IVU can load the local video or get streaming media through the RTSP address from the video surveillance platform

Intelligent event report

IVU is required to identify specific objects automatically, and outputs recognition results to IVM. The recognize information includes event trigger information and data acquisition information. One or more intelligent analysis algorithms can be loaded into or unloaded from IVU according to different requirements.

Intelligent analysis result show

IVU is recommended to send the intelligent analysis results to APP.APP show those results using statistical charts, event screenshot, event video segment and so on.

OSD parameters issuing to IPU

- 6.2	Service management

- 6.2.1	IVM Management

IVM is required to store, manage, and schedule those capabilities dynamically. IVM is required to be invoked by an intelligent application from CMU and manage IVU validity check.

- 6.2.2	IVU Management

IVU is required to send capability set and status to IVM. IVS system is required to have the ability to develop new algorithms（IVU）, such as items left behind detection (to detect dangerous and suspicious item in certain area), item preservation (to guard certain item from being moved), license plate recognition, etc.

IVU is required to provide at least two types of controls of algorithm parameter configuration for administrator and ordinary user, and require the administrator to finish the initial scene studying, advanced parameter setting and other professional operations, while the business is opened via controls, in order to improve the alarm accuracy.

- 6.2.3	Network Management

IVS is required to manage the intelligent analysis process and CPU resources. IVM is required to have the ability to provide the NTP clock synchronization service to IVU managed by it.

IVS system is recommended to classify users into several levels. Users of different levels have different operational rights; high-level users have more operation privileges than low-level ones. Low-level users can do basic operations which meet the following basic requirements, and high-level users can do advanced operations which meet the following advanced requirements.

- 6.2.4	Remote upgrade

IVU is recommended to get the remote software upgrade at regular intervals

Functional Architecture

Figure 7 – System composition of intelligent Visual Surveillance

IVU: identifies specific objects automatically, and outputs recognition results to IVM. The recognize information includes event trigger information and data acquisition information. One or more intelligent analysis algorithms can be loaded into or unloaded from IVU according to different requirements.

IVM: supports strategies configuration of intelligent application by users and video sources schedule dynamically according to the strategies. IVM can accept registration, deletion, capability report from IVU and schedule IVU dynamically. It can also store, manage, and schedule those capabilities dynamically. IVM can be invoked by an intelligent application from the CMU.

APP: it is the third-party data analysis system, which can be used to configure the data analysis algorithms and strategies. APP can receive and analyse data from the visual surveillance platform, whereas it can send alarm information to the visual surveillance platform if needed. With the consideration that alarm processing is mostly a heavy load function, so it can be stripped from CMU to the third-party APP so as to make the function of platform unit clear and extendible.

ICU: In order to achieve comprehensive intelligent video recognition, we can add CIV (Client Intelligent Video) module to CU.CIV recognizes required information from input videos, and output recognition results. In addition, CIV can search designated records from video files. The identifiable information includes event trigger information and data acquisition information

IPU:  it just adds PIV (Premises Intelligent Video) module to PU, PIV is intelligent recognition module of PU, which can be achieved in PU devices and be PU integration. PIV can recognize required information from input videos, and output recognition results. The recognize information includes event trigger information and data acquisition information, etc.

PIV:  it is an intelligent identification module in PU. It provides functions such as identifying required information from input video and outputting the identified result.

CIV: it is an intelligent identification module in CU. It provides functions such as identifying required information from input video and outputting the identified result, retrieve the video with specified information from recorded video data.

The result identified by PIV or CIV can be used for video indexing, image indexing, intelligent PTZ control, subtitle, etc. PIV and CIV can submit the result to CMU.

Functional architecture framework

{Editor’s note: this section provides the figure of system composition of intelligent visual surveillance system, and brief description for the components.}

Functional entities

{Editor’s note: this section defines the functional entities which are introduced for intelligent functions based on normal visual surveillance system.}

Intelligent Premises Unit

Intelligent Premises Unit (IPU) is the premise subsystem within the intelligent visual surveillance system.

It implements the following functions like normal PU defined in [ITU-T H.626]:

- Captures multimedia information (such as audio, video, image, alarm signals, etc.) from the surveilled object.

- Encodes multimedia (audio, video, and image).

- Outputs alarm signal to external linkage device.

- Parses PTZ command codes and transmit them to the devices in order to control pan and Camera.

- Provides network transport, and transmits bidirectional media stream to other entities and alarm signals.

It also implements some additional functions for intelligent application:

- Identify required information from input videos, and output identified results.

- Report the identified result such as event trigger information or data acquisition information to the service platform.

Figure 8 below shows the function modules of IPU based on PU defined in [ITU-T H.626]. Only PIV is the new module to implement intelligent identification functions or corresponding operations.

Figure 7-2-1 – Function modules of IPU

Intelligent customer unit

Intelligent Customer Unit (ICU) is the client subsystem within the intelligent visual surveillance system. It is used to present multimedia information (such as audio, video, image, alarm signal, etc.) to the end user, pick the useful information and trigger the related actions.

It implements the following functions like normal CU defined in [ITU-T H.626]:

- Multimedia decoding function.

- Audio play and video/image display function.

- Console interface for end user to operate the VS system.

It also implement some additional functions for intelligent application:

- Identifying required information from input video and outputting the identified result.

- Retrieve the video with specified information from recorded video data.

- Report and present the identified result to end users.

The figure below shows the function modules of ICU based on CU defined in [ITU-T H.626]. Only CIV is the new module to implement intelligent identification related functions.

Figure 7-2-2 – Function modules of ICU

Intelligent video management

Intelligent Video Management (IVM) is the premise subsystem within the intelligent visual surveillance system.

It implements some normal functions for intelligent application:

- Support strategies configuration of intelligent application by users and video sources schedule dynamically according to the strategies

- Accept registration, deletion, capability report from IVU and schedule IVU dynamically.

- Store, manage, and schedule IVU capabilities dynamically

- Accept intelligent capabilities query from CMU ,then feedback

- Accept intelligent strategies query from CMU ,then feedback

- IVM can be invoked by an intelligent application from CMU and transfer IVU address to CMU

- Support the standard video stream

It also implements some additional functions for intelligent application:

- Manage the IVU license in some methods, such as USB key and software key.

Intelligent video unit

Intelligent Video Unit (IVU) is the premise subsystem within the intelligent visual surveillance system.

It  implements some normal functions for intelligent application:

- Identify specific objects automatically, and output recognition results to IVM.

- The recognize information includes event trigger information and data acquisition information.

- One or more intelligent analysis algorithms can be loaded into or unloaded from IVU according to different requirements.

- Support the standard video stream: RTSP,SIP

- Support acquiring video stream from the video surveillance platform using API

- Support images analysis ,using HTTP protocol

It also implements some additional functions for intelligent application:

Support expanding IVU capability, using IVU virtualization

Reference Points

{Editor’s note: this section describes the reference points between the components based on the framework shown in the figure of system composition.}

Reference Point: IPU – CMU

The reference point Pm is located between the IPU and the CMU. It is used for the following functions:

- For the IPU to make the registration to the CMU.

- For the IPU to report the identified result such as event trigger information or data acquisition information to the CMU.

- For the CMU to manage the IPU.

- For the CMU to query the running status of IPU.

- For the CMU to configure parameters and to upgrade the software version.

Reference Point: IPU – SCU

The reference point Ps is located between IPU and SCU. It is used for the following functions:

- For the IPU to report the identified result such as event triggers information or data acquisition information to the SCU.

- For the SCU to control the service of the IPU, to control the alarm and linkage action, media acquisition, instant image snapshot, recording and playback and voice (audio) communication

- For the SCU to manage the IPU device, to query running status of IPU devices, to configure parameters, to upgrade software version, and to control PTZ by forwarding PTZ control operation commands and return result.

Reference Point: IPU – MDU

The reference point Pd/Mp lies between IPU and MDU. It is used for the following functions:

- For the MDU and IPU to support the video/audio and image acquisition.

- For the MDU and IPU to support the recording in the service platform.

- For the MDU and IPU to support voice (audio) communication.

Reference Point: IPU – ICU

The reference point Pc/Pc’ lies between IPU and ICU. It is used to:

- Deliver interaction signals directly between the IPU and the ICU (such as an RTSP request or response).

- Directly deliver media streams from IPU to ICU.

Reference Point: ICU - CMU

Reference Point: ICU - SCU

Reference Point: ICU - MDU

Reference Point: ICU - MSU

Reference Point: IVU - SCU

Reference Point: IVU - MDU

Reference Point: IVU – IVM

Reference Point: IVM – CMU

Reference Point: IVM – SCU

Service control flow and signaling

{Editor’s note: this section provides service flow and signalling which will be used to implement the functions defined above. A proposed category for the service flow is provided as below, procedures, signalling, protocols and messages definition are need to be added accordingly in the future. }

Registration

IPU registration

Figure 8-1-1- Flow for IPU registration

Step 1: IPU sends registration request to CMU.

Step 2: CMU sends registration response to IPU including result with status code.

IVU registration

Figure 8-1-2- Flow for IVU registration

Step 1: IVU sends registration request to IVM.

Step 2: IVM sends registration response to IVU including result with status code.

ICU registration

Figure 8-1-3- Flow for ICU registration

Step 1: ICU sends registration request to CMU.

Step 2: CMU sends registration response to ICU including result with status code.

Configuration

IVU / IPU is required to provide at least two types of controls of algorithm parameter configuration for administrator and ordinary user, and require the administrator to finish the initial scene studying, advanced parameter setting and other professional operations, while the business is opened via controls, in order to improve the alarm accuracy.

ICU is required to recognize the intelligent video. The algorithm parameter configuration for CIV should be provided by the configuration.

IPU configuration

Figure 8-2-1 – Flow for IPU configuration

Step 1: IPU sends configuration request to CMU.

Step 2: CMU sends configuration response to IPU including result with status code and controls of algorithm parameter configuration

IVU configuration

Figure 8-2-2 – Flow for IVU configuration

Step 1: IVU sends configuration request to IVM.

Step 2: IVM sends configuration response to IVU including result with status code and controls of algorithm parameter configuration

ICU configuration

Figure 8-2-3 – Flow for ICU configuration

Step 1: ICU sends configuration request to CMU.

Step 2: CMU sends configuration response to ICU including result with status code and controls of algorithm parameter configuration

IVU calling

Start IVU manually

Figure 8-3-1– Flow for start IVU manually

Step 1: IVM  sends start request to IVU.

Step 2: IVU starts intelligent analysis tasks and sends response to IVM including result with status code. IVU identifies specific objects automatically, and outputs recognition results to IVM .One or more intelligent analysis algorithms can be loaded into or unloaded from IVU according to different requirements.

Stop IVU manually

Figure 8-3-2 – Flow for stop IVU manually

Step 1: After IVU finishes intelligent analysis tasks, IVM sends stop request to IVU.

Step 2: IVU stops tasks and sends response to IVM including result with status code

Start IVU by event trigger

Stop IVU by event trigger

Intelligent event report

IVU report intelligent event

IPU report intelligent event

OSD parameters issuing to IPU

Issuing procedure

OSD parameters profile

Service subscription

____________________