| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A1104 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): RD-Optimization |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Sehoon Yea | Tel: Email: | +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document defines Core Experiment 4 on RD-optimization to be performed for the 2nd JCT-3V meeting.

# Introduction

The goal of this Core Experiment (CE) is to investigate the methods for RD-optimization proposed at the 1st JCT-3V meeting.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| LG | Sehoon Yea (Coordinator) Jiwook Jung Hongbin Liu | sehoon.yea@lge.com jiwook.jung @lge.com hongbin.liu@lge.com | P,C |
| Zhejiang | Lu Yu Lily Wang Yichen Zhang | yul@zju.edu.cn wangli@zju.edu.cn felixzyc@gmail.com | P,C |
| HHI | Gerhard Tech | gerhard.tech@hhi.fraunhofer.de | C |
| Samsung | Byung Tae Oh Jaejoon Lee Kwan-Jung Oh | byung.oh@samsung.com jaejoon1.lee@samsung.com kwanjung.oh@samsung.com | C |
| Nokia | Miska Hannuksela | miska.hannuksela@nokia.com | C |

(P=proponent, C=crosss checker)

# Tools under Test

## Depth distortion metric with a weighted depth fidelity term [JCT2- A0119]

This contribution proposes to add a depth-fidelity term in the distortion metric currently used for RD-optimization at the encoder for depth coding. Comparisons between the cases with and without the proposed depth-fidelity term are provided to highlight how the proposed change can mitigate the abovementioned potential problems. The average change of luma BD-Rate (BDR) in synthesized result is 0.4% on CTC. And, on 3DVC Cfp conditions using VSRS3.5, the average change of luma BDR is      -2.4%. Especially, the luma BDR of Undo_Dancer and GT_Fly is -9.5% and -5.1%, respectively. The encoding time is increased by 1.8%.

## 3D-CE8.h results on JRDO [JCT2-A0057]

This document presents the RD performance of JRDO implemented on HTM3.1, evaluated with two view synthesis tools. A harmonization scheme of JRDO and VSO is also proposed. The harmonized scheme achieves better RD performance than JRDO and VSO on both VSRS and VSRS-1D-Fast.

# Mandates

Mandates for the CE are as follows:

- To find out optimal values for the weighting factors in the proposal JCT2-A0019, the SW implementation of which will be provided in the 3DV-HTM.

- To investigate ways to further improve the anchor RD-optimization scheme by combining the scheme proposed in JCT2-A0057.

# Software, Configuration and Evaluation

## Software

Experiments in CE4 will use the 3D-HTM-4.0 software that is recommended in JCT2-A1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT2-A1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT2-A1100.

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

Coding Performance Measurements: Objective rate-distortion measurements using 4-point BD-Rate for random access configuration.

Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.