| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0032 |
| --- | --- |

| Title: | CE6.H: Results on CABAC Context Reduction for Simplified Depth Coding (SDC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fabian Jäger Institut für Nachrichtentechnik RWTH Aachen University Jin Heo, Sehoon Yea LG Electronics | Tel: Email: | +49 (0) 241 80 27678 jaeger@ient.rwth-aachen.de jin78.heo@lge.com, sehoon.yea@lge.com |
| Source: | RWTH Aachen University, LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents a reduction of required CABAC contexts for the Simplified Depth Coding (SDC) tool, which was proposed to the 2nd JCT-3V meeting in JCT3V-B0036 [1].

In JCT3V-B0036 separate CABAC contexts were used for each segment of an SDC-coded block. Moreover, another set of CABAC contexts was used for coding the sign flag of the two DC offsets of a SDC block. In this proposal, the number of CABAC contexts is reduced by only using one set of contexts together for both segments of an SDC block and by removing the contexts for the sign flag completely. Finally, the initialization of all remaining CABAC contexts is fixed in the software by using UChar data types instead of Short.

For the CTC configuration [2] the proposed context reduction yields 0.1% coding gain. For the CE6 All-Intra coding configuration the proposed context reductions result in a very minor coding loss of 0.1% compared to the HTM 6.0 anchor.

# Modifications to Required CABAC Contexts

The CABAC contexts that are used for SDC as described in JCT3V-B0036 are partially redundant, unused or independently defined for each segment of an SDC coded block. In this contribution it is proposed to remove this differentiation of contexts for each segment in a block, which affects the symbols sdc_residual_flag and sdc_residual_abs_minus1. Moreover, the contexts for the sdc_residual_sign_flag are removed completely.

| Symbol(s) | Number of Contexts ( HTM 6.0 / draft text ) | Number of Contexts ( Proposal ) | Reduction |
| --- | --- | --- | --- |
| sdc_flag | 3 | 3 | 0 |
| sdc_residual_flag | 2 | 1 | 1 |
| sdc_residual_sign_flag | 2 | 0 | 2 |
| sdc_residual_abs_minus1 | 20 | 8 | 12 |
| sdc_pred_mode | 3 | 3 | 0 |
| OVERALL | 30 | 15 | 15 |

# Modifications to Specification Text

Table G-6 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | sdc_flag | Table G-15 | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Table G-16 | 0..1 0 | 2..3 1 | 4..5 2 |
|  | sdc_residual_sign_flag | Table G-17 | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Table G-18 | 0..19 0..7 | 20..39 8..15 | 40..59 16..23 |
|  | sdc_pred_mode | Table G-19 | 0..2 | 3..5 | 6..8 |

Initialization process for context variables

Table G-15 – Values of variable initValue for sdc_flag ctxIdx

| Initialization variable | sdc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 0 154 | 0 154 | 64 154 | 0 154 | 0 154 | 0 154 | 64 154 | 0 154 | 0 154 |

Table G-16 – Values of variable initValue for sdc_residual_flag ctxIdx

| Initialization variable | sdc_residual_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 251 154 | 255 154 | 0 154 | 0 154 | 56 154 | 0 154 |

Table G-17 – Values of variable initValue for sdc_residual_sign_flag ctxIdx

| Initialization variable | sdc_residual_sign_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 255 | 255 | 0 | 252 | 55 | 0 |

Table G-18 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 255 | 255 | 64 | 0 | 2 | 0 | 64 | 0 | 6 | 0 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 67 | 0 | 8 | 0 | 61 | 0 | 7 | 0 | 47 | 0 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 243 | 255 | 12 | 0 | 14 | 0 | 33 | 0 | 243 | 255 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 2 | 0 | 66 | 0 | 0 | 0 | 63 | 0 | 1 | 0 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| initValue | 12 | 0 | 50 | 0 | 14 | 0 | 27 | 0 | 0 | 0 |
|  | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 239 | 255 | 14 | 9 | 27 | 0 | 0 | 0 | 239 | 255 |

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table G-19 – Values of variable initValue for sdc_pred_mode ctxIdx

| Initialization variable | sdc_pred_mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 9 154 | 255 154 | 70 154 | 0 154 | 9 154 | 255 154 | 70 154 | 0 154 | 9 154 |

Table G-20 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| sdc_flag | 0 | FL, cMax = 1 | 0 | Table G-15 | 0 |
|  | 1 |  | 0 | Table G-15 | 3 |
|  | 2 |  | 0 | Table G-15 | 6 |
| sdc_residual_flag | 0 | FL, cMax = 1 | 0 | Table G-16 | 0 |
|  | 1 |  | 0 | Table G-16 | 2 1 |
|  | 2 |  | 0 | Table G-16 | 4 2 |
| sdc_residual_sign_flag | 0 | FL, cMax = 1 | 0 | Table G-17 | 0 |
|  | 1 |  | 0 | Table G-17 | 2 |
|  | 2 |  | 0 | Table G-17 | 4 |
| sdc_residual_abs_minus1 | 0 | FL, cMax = dltFlag ? Floor( Log2( num_depth_values_in_dlt ) : BitDepthY | 7 | Table G-18 | 0 |
|  | 1 |  | 7 | Table G-18 | 20 8 |
|  | 2 |  | 7 | Table G-18 | 40 16 |
| sdc_pred_mode | 0 | U | 0 | Table G-19 | 0 |
|  | 1 |  | 0 | Table G-19 | 3 |
|  | 2 |  | 0 | Table G-19 | 6 |

Table G-21 – Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| sdc_flag | 0,1,2 (subclause G.9.3.3.1.1) | na | na | na | na | na |
| sdc_residual_flag | 0,1 0 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_sign_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_abs_minus1 | 0,10 0 (subclause G.9.3.3.1.7) |  |  |  |  |  |
| sdc_pred_mode | 0 | 1 | 2 | na | na | na |

Derivation process of ctxIdxInc using segment index

Input to this process is the luma location ( xC, yC ) specifying the top-left luma sample of the current luma coding block relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

If IntraPredMode[ xC ][ yC ] is not equal to 0 and IntraPredMode[ xC ][ yC ] is not equal to 1, ctxIdxInc depends on the SdcSegmentIndex = 0..1, as specified in Table G-23.

Otherwise, the ctxIdxInc is equal to 0.

Table G-23 – Specification of ctxIdxInc using segment index

| Syntax element | IntraPredMode | SdcSegmentIndex | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_residual_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_sign_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_abs_minus1 | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 10 |

# Simulation Results

The simulations were performed according the common test conditions [2]. For the All-Intra coding scenario, configuration files from Core Experiment 6 on Depth Map Intra Coding Tools were used.

The anchor for the simulations was HTM 6.0.

## Random Access Coding Configuration (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -0,1% | 0,1% | 0,0% | -0,1% | -0,1% | 99,2% | 101,2% |
| Kendo | 0,0% | -0,1% | 0,0% | 0,0% | -0,2% | -0,2% | 101,4% | 100,7% |
| Newspaper_CC | 0,0% | -0,1% | 0,1% | 0,0% | -0,1% | -0,1% | 101,9% | 100,4% |
| GT_Fly | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% | -0,1% | 103,0% | 99,2% |
| Poznan_Hall2 | 0,0% | -0,6% | 0,2% | -0,1% | -0,1% | -0,1% | 98,2% | 99,9% |
| Poznan_Street | 0,0% | 0,0% | 0,2% | 0,0% | 0,0% | 0,1% | 98,6% | 100,6% |
| Undo_Dancer | 0,0% | -0,2% | -0,2% | 0,0% | -0,1% | -0,1% | 99,4% | 100,0% |
| 1024x768 | 0,0% | -0,1% | 0,1% | 0,0% | -0,1% | -0,1% | 100,8% | 100,8% |
| 1920x1088 | 0,0% | -0,2% | 0,0% | 0,0% | 0,0% | -0,1% | 99,8% | 99,9% |
| average | 0,0% | -0,1% | 0,1% | 0,0% | -0,1% | -0,1% | 100,2% | 100,3% |

## All-Intra Coding Configuration (AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 99,3% | 95,6% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | 0,0% | 99,4% | 94,3% |
| Newspaper_CC | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 99,4% | 99,0% |
| GT_Fly | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% | 99,8% | 99,1% |
| Poznan_Hall2 | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | 0,2% | 98,8% | 97,1% |
| Poznan_Street | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% | 99,5% | 99,5% |
| Undo_Dancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% | 99,5% | 97,1% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 99,4% | 96,3% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% | 99,4% | 98,2% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% | 99,4% | 97,4% |

# Cross Check

The cross check of the proposed modifications to the SDC tool was done by Ghent University. They investigated the source code modifications and ran the simulations for verification of the presented results.

In their investigation they did not find any problems with the source code. Their simulation results perfectly match with those presented in this document.

# Conclusion

In this contribution CABAC context reductions for the Simplified Depth Coding method are proposed. These modifications result in a simplification of the decoder due to reduced number of contexts to be adapted. Moreover, the coding gain is only slightly reduced for the All-Intra coding configuration and for the configuration following the CTC [1], coding gain is even slightly increased. Furthermore, the draft text for the 3D-HEVC Test Model is reduced and simplified significantly by the proposed modifications.

These results show the benefits of the proposed modifications to the SDC scheme and therefore should be adopted to the 3D-HEVC Test Model [3].

# Patent rights declaration

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).