# Description of 3D-ATM (release candidate r12rc) run-time parameters

Dong Tian, Shinya Shimizu

May 12, 2014

Contents

1.	Encoder run-time configuration parameters	2

1.1.	Texture coding parameters	2

1.1.1.	Additional texture coding parameters	4

1.2.	Depth coding parameters	7

1.2.1.	Additional depth coding parameters	9

2.	Decoder run-time configuration parameters	12

Apendix 1: Example of camera parameter file	13

Apendix 2: Example of translational parameter file	13

Apendix 3: Example of ZNear parameter file	14

Apendix 4: Example of ZFar parameter file	14

Acknowledgements	14

The 3D-ATM codec supports the multi-view plus depth (MVD) data coding and takes input parameters from configurations files. This document can be used to replace the relative part of the software manual after removing the relative changes to the public 3D-ATM version.

In the provided software package, example configuration files are provided under ./configs folder, which has the following contents.

Table 1 Main Texture Configurations

./configs:

mvc+d/  3davc/  cam_param/  decoder.cfg

./configs/mvc+d:

encoder_texture.cfg  texture_additional.cfg  depth_additional.cfg  encoder_depth.cfg

./configs/3davc:

encoder_texture.cfg  texture_additional.cfg  depth_additional.cfg  encoder_depth.cfg

./configs/cam_param:

cam_param_kendo.txt            cam_param_poznanhall2.txt          cam_param_balloons.txt            cam_param_gt_fly.txt           cam_param_lovebird1_4_33views.txt  cam_param_newspaper.txt

… …

File ./configs/decoder.cfg is an example decoder configuration file for both MVC+D and 3D-AVC.

Under ./configs/mvc+d/ are example encoder configuration files for MVC+D (4 files).

Under ./configs/3davc/ are example encoder configuration files for 3D-AVC (4 files).

# Encoder run-time configuration parameters

In this section, the configuration parameters for a 3D-ATM encoder are described.

Encoding configurations for texture and depth component are specified separately in two major input files, one for texture coding (see Section 1) and another for depth coding (see Section 2).

Note that each of the component (texture or depth) has an additional configuration file targeted for a view-specific parameters, such as view coding order, camera parameters and so on.

# Texture coding parameters

Compared to JM encoding parameters, the major parameters for texture coding are listed in the following table. For the parameters with strikethrough indicates that those parameters appeared in ATM public version, but removed in this improved version.

Table 2 Main Texture Configurations

# Highest level controls

3DVCoding               = 1   # 1: Enables MVD coding (texture and depth coded into 1 bitstream).

# 0: Disable

CompatibilityCategory   = 1   # 0: MVC-compatible; 1: AVC-compatible

3DVCoding               = 1   # 0: MVC-compatible; 1: AVC-compatible

NumberOfViews           = 3   # Number of texture views to be coded

3DVCodingOrder          = "T0D0D1D2T1T2" # Specifies coding order for MVD components views

3DVConfigFile           = 3dv_texture_balloons.cfg  # Sequence-specific configuration for current

# texture component of MVD data

# Normative switches for 3D-AVC only coding tool

#

DepthBasedMVP           = 1   # Enable depth-based motion vector prediction for texture

SliceHeaderPred         = 1   # Enable slice header prediction from texture to depth or

# depth to texture, depending on coding order.

PredSliceHeaderSrc      = 2   # Parameter 1 for slice header prediction

PredRefListsSrc         = 0   # Parameter 2 for slice header prediction

PredWeightTableSrc      = 0   # Parameter 3 for slice header prediction

PredDecRefPicMarkingSrc = 2   # Parameter 4 for slice header prediction

VSP_Enable              = 1   # Enable VSP for dependent texture views

AaptiveLuminanceCompensation = 1   # Enable adaptive luminance compensation

AlcSearchRangeX         = 20       # Parameter for ALC

AlcSearchRangeY         = 6        # Parameter for ALC

RLCSkip                 = 1   # Enable run-length coding for skip flag

RLESkip                 = 1   # Enable run-length coding for skip flag

# Encoder only switches: MVC+D and 3D-AVC

#

InterPredictionAtAnchor    = 0   # Disable inter prediction at anchor picture(Default:1)

InterPredictionAtAnchorOff = 0   # Disable inter prediction at anchor picture(Default:1)

GradualViewRefresh         = 0   # Enable Gradual View Refresh (0: disable/default, 1: enable)

# 3DV SEIs

#

SEI_MultiviewAcquisitionInfoFlag = 0 # Enable to send Multiview acquisition SEI

SEI_3DReferenceDisplaysInfo      = 0 # Enable to send 3D reference displays information SEI

# Multiview acquisition SEI must be enabled to send this SEI

SEI_3DReferenceDisplaysFile      = 3dv_reference_display.cfg

3DVCoding specifies the compatibility category of the resulting bitstream. Value 0 specifies that resulting bitstream complies to MVC+D profile (a.k.a. HP), and value 1 specifies that resulting bitstream complies to 3D-AVC profile (a.k.a. EHP). Default value is 1 if not present.

NumberOfViews specifies number of viewpoints to be coded for texture views. Note that the number of coded texture views can be different from the number of coded depth views. Typically, this parameter can be set equal to 2, or 3. Default value is 3 if not present.

3DVCodingOrder is a string, specifying the coding order of MVD  views and components (texture and depth). Letter 'T' indicates a texture view component, letter 'D' indicates a depth view component. A number following a letter 'T' or 'D' indicates the viewpoint index in coding order starting from 0.

For example, "T0D0D1D2T1T2" specifies the following coding order: 1) texture from viewpoint 0; 2) depth from viewpoint 0; 3) depth from viewpoint 1; 4) depth from viewpoint 2; 5) texture from viewpoint 1; and 6) texture from viewpoint 2. This example was used in the CTC test condition, and is often referenced as depth first coding.

"T0D0T1D1T2D2" specifies the following coding order: 1) texture from viewpoint 0; 2) depth from viewpoint 0; 3) texture from viewpoint 1; 4) depth from viewpoint 1; 5) texture from viewpoint 2; and 6) depth from viewpoint 2.  This example is often referenced as texture first coding.

3DVConfigFile specifies a file name for view-specific configuration which provides parameter settings for coding the texture views. See section 1.1.1 for more details.

DepthBasedMVP specifies whether to enable Depth based MVP (DMVP) for dependent texture view. Parameter set to 0 disables DMVP, parameter set to 1 enables DMVP. Parameter cannot be set equal to 1 if 3DVCoding is equal to 0. Default value is 0 if not present.

SliceHeaderPred specifies whether to enable slice header prediction. Value 0 indicates the tool is turned off. Value 1 indicates the tool is turned on. Parameter cannot be set equal to 1 if 3DVCoding is equal to 0. Default value is 0 if not present.

PredSliceHeaderSrc, PredRefListsSrc, PredWeightTableSrc, and PredDecRefPicMarkingSrc specifies slice header prediction related parameters. See the semantics of the syntax elements in slice header in the specification for details.

VSP_Enable specifies whether to enable VSP coding tool for dependent texture view. Parameter set to 0 disables VSP, parameter set to 1 enables VSP. Parameter cannot be set equal to 1 if 3DVCoding is equal to 0.  Default value is 0 if not present.

AaptiveLuminanceCompensation specifies whether to enable ALC coding tool for dependent texture view. Parameter set to 0 disables ALC, parameter set to 1 enables ALC. Parameter cannot be set equal to 1 if 3DVCoding is equal to 0. Default value is 0 if not present.

AlcSearchRangeX and AlcSearchRangeY specifies ALC coding tool's parameters.

RLESkip specifies specifies whether to enable RLESkip coding toot for depend texture view of MVD data. Parameter set to 0 disables RLESkip, parameter set to 1 enables RLESkip. Parameter cannot be set equal to 1 if 3DVCoding is equal to 0. Default value is 0 if not present.

# Additional texture coding parameters

View-specific parameters for texture is specified in a file named by parameter 3DVConfigFile, such as view coding order, input sequence file name, etc. An example of such parameters for texture is shown below.

Table 3 Additional Texture Configurations – PIP interview prediction

ViewCodingOrder    3-1-5

ViewId             3

InputFile          balloons_3.yuv

ViewId             1

InputFile          balloons_1.yuv

QPOffset           4

ReorderAtAnchor    01

ReorderAtNonAnchor 01

FwdAnchorRefs      1 3

BwdAnchorRefs      0

FwdNonAnchorRefs   1 3

BwdNonAnchorRefs   0

ViewId             5

InputFile          balloons_5.yuv

QPOffset           4

ReorderAtAnchor    01

ReorderAtNonAnchor 01

FwdAnchorRefs      1 3

BwdAnchorRefs      0

FwdNonAnchorRefs   1 3

BwdNonAnchorRefs   0

ViewCodingOrder specifies the coding order of different views. It is composed of a string of view numbers. The view appearing first in the string is coded first.

Following the ViewCodingOrder, there is one section for each texture view describing view-specific configurations, including inter-view dependencies. The first section is recommended for base view section.

ViewId specifies the view number for current view section, which should appear at the first place of the current view section.

InputFile specifies the input file name of the current view of this view section.

QPOffset specifies the QP offset of the current view relative to the base view.

ReorderAtAnchor specifies ordering of temporal (set by 0) and inter-view (set by 1) references in picture list at the Anchor picture. Example of parameter is “01”, which specifies that temporal reference pictures are placed in beginning of the reference picture list, with inter-view references following them.

ReorderAtNonAnchor specifies ordering of temporal (set by 0), inter-view (set by 1) references in picture list at the Non-Anchor picture. Example  of parameter is “01”, which specifies that temporal reference pictures are placed in beginning of the reference picture list, with inter-view references following them.

FwdAnchorRefs specifies the reference views in LIST0. The first number indicates the number of LIST0 references and followed by the view numbers used as reference views. e.g. "0" indicates there is no LIST0 references. "1 3" indicates there is one LIST0 reference and it is from view 3.

BwdAnchorRefs specifies the reference views in LIST1. The first number indicates the number of LIST1 references and followed by the view numbers used as reference views. e.g. "0" indicates there is no LIST1 references. "1 3" indicates there is one LIST1 reference and it is from view 3.

Note that the interview prediction pattern is implemented by modifying the configuration in this file (and the one corresponding to depth). In the example provided above, the interview prediction is called "PIP", where the center view is coded as Intra view, the left and right views are coded as "P" views, predicted from the center view.

In the next example, the interview prediction is known as "IBP", where the left view is coded as Intra view, the right view is coded as "P" view and the central view is coded as "B" view.

Table 4 Additional Texture Configurations – IBP interview prediction

ViewCodingOrder    1-5-3

ViewId             1

InputFile          balloons_1.yuv

ViewId             5

InputFile          balloons_5.yuv

QPOffset           4

ReorderAtAnchor    01

ReorderAtNonAnchor 01

FwdAnchorRefs      1 1

BwdAnchorRefs      0

FwdNonAnchorRefs   1 1

BwdNonAnchorRefs   0

ViewId             3

InputFile          balloons_3.yuv

QPOffset           4

ReorderAtAnchor    01

ReorderAtNonAnchor 01

FwdAnchorRefs      1 1

BwdAnchorRefs      1 5

FwdNonAnchorRefs   1 1

BwdNonAnchorRefs   1 5

In another example, the interview prediction "IPP", where the left view is coded as Intra view; the central view is coded as "P" view which is predicted from the left view; the right view is coded as "P" view which is predicted from the central view and left view with both being putting in LIST0.

Table 5 Additional Texture Configurations – IPP interview prediction

ViewCodingOrder    1-3-5

ViewId             1

InputFile          balloons_1.yuv

ViewId             3

InputFile          balloons_3.yuv

QPOffset           4

ReorderAtAnchor    01

ReorderAtNonAnchor 01

FwdAnchorRefs      1 1

BwdAnchorRefs      0

FwdNonAnchorRefs   1 1

BwdNonAnchorRefs   0

ViewId             5

InputFile          balloons_5.yuv

QPOffset           4

ReorderAtAnchor    01

ReorderAtNonAnchor 01

FwdAnchorRefs      2 3 1

BwdAnchorRefs      0

FwdNonAnchorRefs   2 3 1

BwdNonAnchorRefs   0

# Depth coding parameters

Similar to texture views, depth views also have some major new parameters relative to JM encoder configurations, which are listed in the following table.

Table 6 Main Depth Configurations

# Highest level controls

#

NumberOfViews           = 3  # Number of views to encode

ForceYUV400             = 1  # 1: Code depth with 400 format. 0: Code depth with 420 format

3DVConfigFile           = 3dv_depth_balloons.cfg # Depth config file

# Normative parameters for 3D-AVC only

#

# Depth resolution related parameters in seq_parameter_set_3davc_extension()

depth_hor_mult_minus1   = 0

depth_ver_mult_minus1   = 0

depth_hor_rsh           = 1

depth_ver_rsh           = 1

# Depth cropping related parameters in seq_parameter_set_3davc_extension()

depth_frame_cropping_flag       =   0

depth_frame_crop_left_offset    =   0

depth_frame_crop_right_offset   =   0

depth_frame_crop_top_offset     =   0

depth_frame_crop_bottom_offset  =   0

# Normative parameters for 3D-AVC, but non-normative for MVC+D

#

# Depth representations

NonlinearDepth          = 1

NonlinearDepthModel     = "2;4;7;8;10;12;14;16;17;19;20;21;22;23;24;25;26;26;27;27;27;27;27;27;26;26;25;24;23;22;20;19;17;15;13;11;9;6;3"

NonlinearDepthThreshold = 50

# Encoder controls

#

ViewSynRDO              = 1

ViewSynCfg              = "[ C L R ; 0.25 0.5 0.75 ]"  # 3-view case, default : [ C C C ; 0.5 ]

#ViewSynCfg             = "[ L R ; 0.25 0.5 0.75 ]"    # 2-view case, default : [ C C ; 0.5 ]

VSDWeight               = 4

SSEWeight               = 1

NormalizeDepth          = 1 # 0: Output depth resolution is kept the same as its input resolution

# 1: Output depth resolution is aligned with texture resolution

PostDilation            = 1 # 0: No dilation for depth

# 1: Dilation for depth when writing the output YUV files

# SEIs

# Multiview Acquisition

PrecFocalLength        =    18 # Specifies the exponent of the maximum allowable truncation

# error for focal_lenth_x and focal_length_y as given by  2^(-

# PrecFocalLength).(Default:18)

PrecPrincipalPoint     =    18 # Specifies the exponent of the maximum allowable truncation error

# for principal_point_x and principal_point_y as given by 2^(-

# PrecPrincipalPoint).(Default:18)

PrecTranslation        =    18 # Specifies the exponent of the maximum allowable truncation error

# for translation as given by 2^(-PrecTranslation).(Default:18)

MantissaLenDepthRange  =    25 # Specifies the mantissa length of depth range(Default:25)

NumberOfViews specifies number of viewpoints to be coded for depth views. Not that the number of coded texture views can be different from the number of depth views. This parameters is typically set equal to 2, or 3. Default value is 3 if not present.

ForceYUV400 specifies the coding format for the depth component. Value 0 means YUV420 format is used for depth coding. Value 1 indicates YUV400 format is used for depth coding. Default is 1 if not present.

3DVConfigFile specifies a file name for view-specific configuration which provides parameter settings for coding of depth views. See section 1.2.1 for more details.

NonlinearDepth specifies the way to represent depth values. Value 1 indicates to enable nonlinear depth representation. Value 0 indicates to disable nonlinear depth representation. Default value is 0 if not present.

NonlinearDepthModel specifies the nonlinear depth model when NonlinearDepth is equal to 1. For example, "10;19;24;27;26;22;13" as deviations from diagonal line 0..255.

NonlinearDepthThreshold specifies the nonlinear depth threshold when NonlinearDepth is equal to 1. The parameter equal to non-zero would automatically turn on/off the Nonlinear Depth Representation (NDR) tool based on average depth value in first depth frame of the base view. NDR is turned off if the average depth value is less than NonlinearDepthThreshold ; NDR is turned on if the average depth value is not less than NonlinearDepthThreshold . NonlinearDepthThreshold  equal to 0 specifies that  NDR is always turned on.

depth_hor_mult_minus1, depth_ver_mult_minus1, depth_hor_rsh, and depth_ver_rsh specify parameters to depth-based disparity value derivation process (specified in subclause J.8.2.1.1 of the specification). When not present, depth_hor_mult_minus1 and depth_ver_mult_minus1 are inferred to be equal to 1, and depth_hor_rsh and depth_ver_rsh are inferred to be equal to 0. depth_hor_mult_minus1 and depth_ver_mult_minus1 shall be in the range of 0 to 1023, inclusive. depth_hor_rsh and depth_ver_rsh shall be in the range of 0 to 31, inclusive.

depth_frame_cropping_flag equal to 1 specifies that the frame cropping offset parameters for depth view components follow next in the sequence parameter set. Value 0 indicates such parameters not present. Default value is 0 if not present.

depth_frame_crop_left_offset, depth_frame_crop_right_offset, depth_frame_crop_top_offset, and depth_frame_crop_bottom_offset specify the samples of the decoded depth view components in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in frame coordinates for output.

ViewSynRDO equal to 1 specifies VSO is turned on for rate distortion optimization. Value 0 indicates VSO is turned off. Default value is 0 if not present.

ViewSynCfg specifies the parameters on how the VSO is configured. It must be present if ViewSynRDO is equal to 1. In particular, the parameter, a string composed of letter and numbers, specifies the rendering direction and positions for view synthesis. Letters, "C"," "L, and "R", indicate the rendering directions of texture/depth views. Letter "L" indicates the current view would be used as reference for view synthesis for views on its left, letter "R" indicates the current view would be used as reference for              view synthesis for views on its right, and letter "C" indicates the current view would be  used as reference for view synthesis for views on its left and right. The first letter corresponds to the view with view index 0, the second letter corresponds to the view with view index 1, etc. In addition, numbers, e.g. 0.25, 0.5, 0.75, specify the viewpoints of the virtually rendered views.

For example, [ C L R ; 0.25 0.5, 0.75 ] specifies that texture/depth views from viewpoint 0 are used as reference to render a virtual view on its left  and right (C), texture/depth views from viewpoint 1 are used as reference to render a virtual view on its left (L), and texture/depth views from viewpoint 2 are used as reference to render a virtual view on its right. The target virtual viewpoints are positioned at 0.25, 0.5 and 0.75 between two neighboring texture/depth views that are coded.

VSDWeight further specifies the weighting factor of view synthesis distortion in the RDO optimization. Default value is 4 if not present.

SSEWeight further specifies the weighting factor of SSE distortion in the RDO optimization. Default value is 1 if not present.

NormalizeDepth equal to 0 specifies the depth is outputted at the same resolution as its input resolution. Value 1 specifies that the depth is output at the same  resolution as the accompany texture view. Default value is 0 if not present.

PostDilation equal to 1 specifies that a dilation post processing is applied to depth before it is outputted. Value 0 specifies that no such dilation is applied. Default value is 0 if not present.

# Additional depth coding parameters

Further view-specific parameters for depth is specified in a file named by parameter 3DVConfigFile, such as view coding order, input sequence file name, etc. An example of such parameters for texture is shown below.

Table 7 Additional Depth Configurations – PIP interview prediction

# Overview

ViewCodingOrder     3-1-5

CameraOrder         1-3-5

CameraParameterFile cam_param_balloons.txt

AcquisitionIdc      1

# View section 3

ViewId              3

InputFile           depth_balloons_0.5_3.yuv

ZNear               448.251214

ZFar                11206.28035

CameraName          param_balloons3

TanslationFile      NULL

ZNearFile           NULL

ZFarFile            NULL

GridPosX            0

GridPosY            0

# View section 1

ViewId              1

InputFile           depth_balloons_0.5_1.yuv

QPOffset            0

ReorderAtAnchor     01

ReorderAtNonAnchor  01

FwdAnchorRefs       1 3

BwdAnchorRefs       0

FwdNonAnchorRefs    1 3

BwdNonAnchorRefs    0

ZNear               448.251214

ZFar                11206.28035

CameraName          param_balloons1

TanslationFile      NULL

ZNearFile           NULL

ZFarFile            NULL

GridPosX            0

GridPosY            0

# View section 5

ViewId              5

InputFile           /data/3dtv/3dv_cfp_seqs/LowRes/depth_balloons_0.5_5.yuv

QPOffset            0

ReorderAtAnchor     01

ReorderAtNonAnchor  01

FwdAnchorRefs       1 3

BwdAnchorRefs       0

FwdNonAnchorRefs    1 3

BwdNonAnchorRefs    0

ZNear               448.251214

ZFar                11206.28035

CameraName          param_balloons5

TanslationFile      NULL

ZNearFile           NULL

ZFarFile            NULL

GridPosX            0

GridPosY            0

ViewCodingOrder specifies the coding order of different views. It is composed of a string of view numbers. The view appearing first in the string is coded first.

CameraOrder specifies view orders appearing from the left to the right. It is composed of a string of view numbers. The view appearing first in the string is on the leftmost.

CameraParameterFile specifies the camera parameter file. The example of camera parameter file is given in Appendix 1.

AcquisitionIdc specifies a type of depth representation approach. Parameters being set to 0 specifies that depth represents the disparity information. Parameter being set to 1 specifies that depth represents the distance information measured in a 3D coordination system, at the same time, all pictures within one view share the same camera and depth range information. Parameter being set to 2 specifies that the camera and depth range information may vary from picture by picture. Picture-varying camera and dynamical range parameters to be specified in external files through parameters  TanslationFile, ZNearFile and ZFarFile.

Next, there is one view section for each view to be coded. The first section is recommended for the base view.

ViewId specifies the view number for current view section, which should appear at the first place of the current view section.

InputFile specifies the input file name of the current view. The first view section corresponds to the base view.

QPOffset specifies the QP offset of the current view relative to the base view.

ReorderAtAnchor specifies ordering of temporal (set by 0) and inter-view (set by 1) references in picture list at the Anchor picture. Example of parameter is “01”, which specifies that temporal reference pictures are placed in beginning of the reference picture list, with inter-view references following them.

ReorderAtNonAnchor specifies ordering of temporal (set by 0), inter-view (set by 1) references in picture list at the Non-Anchor picture. Example  of parameter is “01”, which specifies that temporal reference pictures are placed in beginning of the reference picture list, with inter-view references following them.

FwdAnchorRefs specifies the reference views in LIST0. The first number indicates the number of LIST0 references and followed by the view numbers used as reference views. e.g. "0" indicates there is no LIST0 references. "1 3" indicates there is one LIST0 reference and it is from view 3.

BwdAnchorRefs specifies the reference views in LIST1. The first number indicates the number of LIST1 references and followed by the view numbers used as reference views. e.g. "0" indicates there is no LIST1 references. "1 3" indicates there is one LIST1 reference and it is from view 3.

ZNear and ZFar specify the nearest and farthest Z values. Together with camera parameters, they are used for conversion between depth value and disparity value.

CameraName specifies the camera name used to indicate the current view in the camera parameter file.

TanslationFile, ZNearFile and ZFarFile specify the file names of translation parameter, ZNear and Zfar parameter if AcquisitionIdc is set equal to 2. Each of these parameters are are provided on a basis of frames. The example of such parameter file is given in Apendix 2, Apendix 3 and Apendix 4.

GridPosX specifies, for the current view, a horizontal offset of a depth sampling grid relative to the luma texture sampling grid in texture luma sample units.

GridPosY specifies, for the current view, a vertical offset of a depth sampling grid relative to the luma texture sampling grid in texture luma sample units.

Note that the example provided above corresponds to "PIP" interview prediction. Similar modifications as in texture (see Table 4 and Table 5) could be made to support "IBP" and "IPP" interview predictions.

# Decoder run-time configuration parameters

All JM decoder configuration parameters are reused. Especially, parameter DecodeAllLayers equal to 0 specifies that only the texture base view is to be decoded. DecodeAllLayers equal to 1 specifies that all coded texture and depth view components are to be decoded.

In addition to JM decoder configuration parameters, there are a few new parameters as listed in the following table.

Table 8 Decoder Configurations

#

DecodeAllLayers        = 1                 # decode all view and all components

OutputRecYUVFile       = 1                 # output YUV reconstructed YUV files

OutputFileForTexture   = "texture_dec.yuv" # Output file, YUV/RGB

WriteUVForTexture      = 1                 # 1: write UV components for texture; 0: not writing

OutputFileForDepth     = "depth_dec.yuv"   # Output file, YUV/RGB

WriteUVForDepth        = 1                 # 1: write UV components for depth; 0: not writing

NormalizeDepth          = 1 # 0: Output depth resolution is kept the same as its input resolution

# 1: Output depth resolution is aligned with texture resolution

PostDilation            = 1 # 0: No dilation for depth

# 1: Dilation for depth when writing the output YUV files

CameraFile             = "camera.txt"      # Output file, camera parameters

TextConfigFile          = "texture_additional.cfg"

DepthConfigFile         = "depth_additional.cfg"

3DReferenceDisplaysOutputFile = "ReferenceDisplaySEIOutput.txt"

OutputRecYUVFile equal to 1 specifies that the decoded YUV are to be outputted to files. Value 0 specifies that no YUV files from the decoding will be outputted. Default value is 0 if not present.

OutputFileForTexture specifies the file name base of texture YUV files. The view index will be appended to the end of the file name.

WriteUVForTexture equal to 1 specifies that the chroma components will be written to the YUV output file. Value 0 specifies that no chroma components will be written to the YUV output file when the texture is coded in YUV400 format. Note that if the texture is coded with non-YUV400, this parameter does not have impacts and the chroma components are always outputted. Default value is 1 if not present.

OutputFileForDepth specifies the base file name of depth YUV files. The view index will be appended to the end of the file name.

WriteUVForDepth equal to 1 specifies that the chroma components will be written to the YUV output file. Value 0 specifies that no chroma components will be written to the YUV output file when the depth is coded in YUV400 format. Default value is 0 if not present.

NormalizeDepth equal to 0 specifies the depth is outputted at the same resolution as its input resolution. Value 1 specifies that the depth is output at the same  resolution as the accompany texture view. Default value is 0 if not present.

PostDilation equal to 1 specifies that a dilation post processing is applied to depth before it is outputted. Value 0 specifies that no such dilation is applied. Default value is 0 if not present.

CameraFile specifies the file name used to store the outputted camera parameters. It may be provided as input for a view synthesis software to render a virtual view.

# Apendix 1: Example of camera parameter file

Following is an example of camera parameter file.

Table 9 Example camera file

param_balloons00
2241.25607 0.0 701.5
0.0 2241.25607 514.5
0.0 0.0 1.0 
0.0 
0.0 
1.0 0.0 0.0 0.0
0.0 1.0 0.0 0.0 
0.0 0.0 1.0 0.0

param_balloons01
2241.25607 0.0 701.5
0.0 2241.25607 514.5
0.0 0.0 1.0 
0.0 
0.0 
1.0 0.0 0.0 5.0
0.0 1.0 0.0 0.0 
0.0 0.0 1.0 0.0

param_balloons02
2241.25607 0.0 701.5
0.0 2241.25607 514.5
0.0 0.0 1.0 
0.0 
0.0 
1.0 0.0 0.0 10.0
0.0 1.0 0.0 0.0 
0.0 0.0 1.0 0.0

# Apendix 2: Example of translational parameter file

Following is an example of translational parameter file, composing of translation parameters for frames 0, 1 and 2.

Table 10 Example translational file

2824.7961840820300
2796.7812475585900
2768.8285180664000

# Apendix 3: Example of ZNear parameter file

Following is an example ofZNear parameter file, composing of ZNear parameters for frames 0, 1 and 2.

Table 11 Example ZNear file

70625
69812.5
68937.5

# Apendix 4: Example of ZFar parameter file

Following is an example ofZfar parameter file, composing of ZFar parameters for frames 0, 1 and 2.

Table 12 Example ZFar file

662000
663000
664500

# Acknowledgements

We would like to thanks the colleagues below for their inputs on our inquiries related to the run-time parameters:

Olgierd Stankiewicz, Jin Young Kim, Li Zhang, Ying Chen, Dmytro Rusanovskyy, Miska Hannuksela.