| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G135 WG11 Number: 21687 |
| --- | --- |

| Title: | Rectangular (2NxN and Nx2N) Intra Prediction |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Shan Liu Ximin Zhang Zhi Zhou Shawmin Lei 2860 Junction Ave. San Jose, CA95134, USA | Emails: | shan.liu@mediatek.com ximin.zhang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek USA Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes to add two new Intra prediction modes, i.e. 2NxN and Nx2N Intra prediction to the existing 2Nx2N and NxN (only in SCU) Intra prediction modes in current HM. Experimental results report average 1.60% BD-rate reduction for All Intra HE and average 2.54% BD-rate reduction for All Intra LC. With the current version implementation, encoder runtime is increased by 32% and 41% for All Intra HE and LC settings, respectively. Decoding runtime is increased 3% for All Intra settings in average. Further coding efficiency improvement and/or implementation complexity reduction are expected with further investigation.

- Introduction

The current HEVC (High Efficiency Video Coding) defines coding unit (CU) and prediction unit (PU), where a CU may consist of one or multiple PUs. An Intra 2Nx2N CU should consist of one 2Nx2N PU if it is greater than smallest coding unit (SCU); or it may consist of either one 2Nx2N PU or four NxN PUs if it is an SCU. An Inter 2Nx2N CU may consist of one 2Nx2N PU, two rectangular shape (e.g. 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, nRx2N) PUs, or four NxN PUs if it is an SCU. Various PU types for a 2Nx2N CU are shown in Figure 1.Partition types and binarization for Intra Prediction is illustrated in Table 1.

# Proposed Methods

In this contribution, we propose to add two rectangular shape partition types to Intra prediction, i.e. 2NxN and Nx2N Intra prediction, to current HEVC/HM. Consequently, in the proposed scheme, an Intra 2Nx2N CU may consist of one 2Nx2N PU, two 2NxN or Nx2N PUs, or four NxN PUs if it is an SCU. The proposed partition types are shown in Figure 2, with binarization shown in Table 2. For each proposed rectangular shape (i.e. 2NxN and Nx2N) partition, 35 prediction modes are allowed, the same as those are used for 8x8, 16x16 and 32x32 Intra predictions in HM4.0.

Figure 1 PU (partition) types for a 2Nx2N CU in HM4.0.

Table 1 Partition types and binarization for Intra prediction in HM4.0.

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | log2CUSize > 3 | log2CUSize ==3 |
| I | 0 | MODE_INTRA | PART_2Nx2N | - | 1 |
|  | 1 | MODE_INTRA | PART_NxN | - | 0 |

Figure 2 Proposed PU (partition) types for a 2Nx2N CU.

Table 2 Proposed partition types and binarization for Intra prediction.

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | log2CUSize > 3 | log2CUSize ==3 |
| I | 0 | MODE_INTRA | PART_2Nx2N | 1 | 1 |
|  | 1 | MODE_INTRA | PART_2NxN | 01 | 001 |
|  | 2 | MODE_INTRA | PART_Nx2N | 00 | 000 |
|  | 3 | MODE_INTRA | PART_NxN | - | 01 |

The transforms used in the current implementation are shown in Figure 3 and Table 3. They are similar to that have been used in the previously proposed SDIP (Short Distance Intra Prediction) method. Another transform structure is currently under investigation, which is similar to NSQT, shown in Figure 4 and Table 4.

Figure 3 Transform for the proposed rectangular Intra PUs.

Table 3 Transform for proposed rectangular Intra prediction.

| PU Size | TU size |
| --- | --- |
| 32x16 | 32x8, 32x2 |
| 16x32 | 8x32, 2x32 |
| 16x8 | 16x4 |
| 8x16 | 4x16 |
| 8x4 | 8x2 |
| 4x8 | 2x8 |

Figure 4 Transform for the proposed rectangular Intra PUs (on-going investigation)

Table 4 Transform for the proposed rectangular Intra PUs (on-going investigation)

| PU Size | TU size |
| --- | --- |
| 32x16 | 32x16, 32x8, 16x4 |
| 16x32 | 16x32, 8x32, 4x16 |
| 16x8 | 16x8, 16x4 |
| 8x16 | 8x16, 4x16 |
| 8x4 | 8x4 |
| 4x8 | 4x8 |

# Experimental Results

Simulations were conducted following common test conditions defined in JCTVC-F900 [1]. Anchor data was generated using HM4.0 software [2]. Results produced by current software implementation are reported in table 5.

Table 5 Results for proposed rectangular Intra prediction.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.63% | -1.59% | -1.94% | -1.40% | -2.27% | -2.64% |
| Class B | -1.51% | -2.39% | -2.53% | -2.64% | -2.87% | -3.28% |
| Class C | -1.82% | -1.53% | -1.69% | -2.56% | -2.78% | -3.10% |
| Class D | -1.83% | -1.23% | -1.21% | -2.39% | -2.36% | -2.47% |
| Class E | -2.45% | -4.97% | -4.51% | -4.07% | -7.03% | -5.65% |
| Overall | -1.60% | -2.21% | -2.28% | -2.54% | -3.26% | -3.31% |
|  | -1.60% | -2.20% | -2.26% | -2.54% | -3.25% | -3.29% |
| Enc Time[%] | 132% |  |  | 141% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |

# Conclusions

This contribution reports the method and results for the proposed rectangular shape (i.e. 2NxN and Nx2N) Intra prediction. Experimental results reportedly show average BD-rate reductions of 1.60% and 2.54% for All Intra HE for All Intra LC settings, respectively. With current version implementation, encoder runtime is increased by 32% and 41% for All Intra HE and LC settings; decoding runtime is increased by 3% in average. Further coding efficiency improvement and/or implementation complexity reduction are expected with further investigation. It is recommended to further study the proposed rectangular Intra prediction in related CE.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, Italy, July 2011.

- HM 4.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-4.0.