INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2012/M23895

February 2012, San Jose, USA

| Source | Samsung Electronics Co., Ltd. |
| --- | --- |
| Status | Input Contribution |
| Title | Proposed Padding Indication Method on D2-LD EE |
| Authors | Sunghee Hwang, Sungoh Hwang, Kyungmo Park, Sungryeul Rhyu, Hyun-Koo Yang |

This document proposes a padding indication method on D2-LD EE for MMT Transport Packet.

Background

When padding bytes are added at the end of RTP Payload or RTCP control information, the last byte of the padding is set to a count of how many padding bytes should be ignored, including itself. This means that the number of padding bytes is restricted to 255.[Ref. 1]

However, when we consider the maximum size of MMT Transport Packet, more padding bytes than 255 are possible.

Therefore, we need a padding indication method to support more padding bytes.

Proposed Padding Indication Method

D.2 Layer Header shall include Padding Flag (1bit) field to indicate whether MMT Transport Packet has padding byte(s) or not. If the padding bit is set, the packet contains one or more additional padding bytes at the end which are not part of the payload. If the number of padding bytes (P) is less than 256, the last byte of the padding shall be set to P. If the number of padding bytes (P) is equal or greater than 256, the last byte of the padding shall be set to “0” to indicate the existence of the secondary padding size field which consists of 256th and 255th bytes from the last byte and which is set to “P-256”.

Figure 1. Proposed Padding Indication Method

Conclusion

It is proposed the padding indication method on D2-LD EE for MMT Transport Packet as follows:

- Padding Flag Field (1bit) is prepared on D.2 Layer Header.

- When one or more padding bytes are added at the end of MMT Payload, Padding Flag Field shall be set to 1.

  - In case of P < 256, the last byte of the associated MMT Transport Packet shall be set to P. Where, P is a count of how many bytes should be ignored.

  - In case of P ≥ 256, the last byte of the associated MMT Transport Packet shall be set to 0 to indicate the existence of Secondary Padding Size Field (2bytes) which consists of 256th and 255th bytes from the last byte, and the Secondary Padding Size Field shall be set to P-256. Where, P is a count of how many bytes should be ignored.

Proposal on Working Draft

Proposed descriptions on Chapter 6.3 MMT Transport Packet of N12334 (Working Draft of MMT Transport Packet) are as follows:

************************************************************************

6.3.x Padding Indication Method

D.2 Layer Header shall have Padding Flag field (1bit) to indicate a padded status on MMT Transport Packet.

When padding is needed for MMT Transport Packet, one or more padding bytes are added at the end of MMT Payload and Padding Flag field shall be set to “1”, otherwise 0.

In case of P < 256, the last byte of the associated MMT Transport Packet shall be set to P. In case of P ≥ 256, the last byte of the associated MMT Transport Packet shall be set to 0 to indicate the existence of Secondary Padding Size Field (2bytes) which consists of 256th and 255th bytes from the last byte, and the Secondary Padding Size Field shall be set to P-256.

Where, P is a count of how many bytes should be ignored.

All padded bytes, except the Last byte and Secondary Padding Size Field(if any) of padded data, shall be set to “00h”.

Example for padding indication method is depicted as shown Figure x.

Figure x. Example of Padding Indication Method

************************************************************************

Reference

[1] IETF RFC3550: “RTP: A Transport Protocol for Real-Time Applications”.