#### Sample adaptive offset parameter syntax

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| sao_num_units_in_width_minus1 | ue(v) |
| sao_num_units_in_height_minus1 | ue(v) |
| if (sao_num_units_in_width_minus1 ¦¦ sao_num_units_in_ height_minus1){ |  |
| sao_one_luma_unit_flag | u(1) |
| if(sao_one_luma_unit_flag) |  |
| sao_offset_vlc( 0, 0, 0 ) |  |
| if( sao_cb_enable_flag ){ |  |
| sao_one_cb_unit_flag | u(1) |
| if( sao_one_cb_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag ){ |  |
| sao_one_cr_unit_flag | u(1) |
| if( sao_one_cr_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 2 ) |  |
| } |  |
| } |  |
| for( ry = 0; ry <= sao_num_units_in_ height_minus1; ry++ ) { |  |
| for( rx = 0; rx <= sao_num_units_in_width_minus1; rx++ ) { |  |
| if( aps_sample_adaptive_offset_flag && ! sao_one_luma_unit_flag ) { |  |
| if( ry > 0 && rx = = 0 ) |  |
| sao_repeat_row_flag[ 0 ] | u(1) |
| sao_unit_vlc(rx, ry, 0 ) |  |
| } |  |
| if( sao_cb_enable_flag && ! sao_one_cb_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 1 ] | u(1) |
| sao_unit_vlc(rx, ry, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag & & !sao_one_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 2 ] | u(1) |
| sao_unit_vlc( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### Sample adaptive offset parameter semantics

sao_cb_enable_flag equal to 1 specifies that the sample adaptive offset process for Cb is applied to the current picture.

sao_cr_enable_flag equal to 1 specifies that the sample adaptive offset process for Cr is applied to the current picture.

sao_num_units_in_width_minus1 plus 1 specifies the number of SAO parameter units in picture width.

sao_num_units_in_height_minus1 plus 1 specifies the number of SAO parameter units in picture height.

sao_one_luma_unit_flag equal to 1 specifies that all of coding treeblocks in luma component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the luma component are signalled on a coding treeblock basis.

sao_one_cb_unit_flag equal to 1 specifies that all of coding treeblocks in Cb component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cb component are signalled on a coding treeblock basis.

sao_one_cr_unit_flag equal to 1 specifies that all of coding treeblocks in Cr component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cr component are signalled on a coding treeblock basis.

sao_repeat_row_flag[ cIdx ] equal to 1 specifies that the SAO parameters of coding treeblocks in the current coding treeblock row are the same as those of above coding treeblocks for the colour component cIdx; equal to 0 specifies that the current coding treeblock row has new SAO parameters. When sao_repeat_row_flag is not present, it is inferred to be equal to 0.

#### 7.4.2.7    Sample adaptive offset unit VLC semantics

The number of times the SAO parameters corresponding to a coding treeblock are repeated for subsequent coding treeblocks in the same row is represented by saoRun[ cIdx ][ rx ][ ry ]. The array index cIdx specifies the colour component; cIdx is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices rx and ry specify the location ( rx, ry ) of the considered coding treeblock relative to the top-left coding treeblock of the picture.

sao_run_diff specifies the saoRun of current coding treeblock if the current row is the first row, otherwise specifies the difference between the run of current coding treeblock and the run of the above coding treeblock. When saoRun is greater than or equal to 0, the syntax elements in sao_offset_vlc() are derived from the corresponding syntax elements of the left coding treeblock. When ry is equal to 0, the length of the sao_run_diff syntax element is Ceil( Log2(sao_num_units_in_width_minus1 − rx + 2) ) bits.

#### 7.4.2.8 Sample adaptive offset VLC semantics

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-6 of current coding treeblock at position Ctbx and Ctby for the colour component cIdx where;

- rx = floor (((Ctbx + 1) * sao_num_units_in_width_minus1) / num_LCU_in_width) – 0.0001)

- ry = floor (((Ctby + 1) * sao_num_units_in_height_minus1) / num_LCU_in_height) – 0.0001)

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry ].

Table 7-6 – Specification of the edge type for SAO

| sao_type_idx[ cIdx ][ rx ][ ry ] | Edge type (informative) |
| --- | --- |
| 0 | Not applied |
| 1 | 1D 0-degree edge |
| 2 | 1D 90-degree edge |
| 3 | 1D 135-degree edge |
| 4 | 1D 45-degree edge |
| 5 | Band |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the pixel range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5.

When sao_band_position[ cIdx ][ rx ][ ry ] is not present it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx − 1 ][ ry ].

sao_offset[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding treeblock at position Ctbx and Ctby for the colour component cIdx.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

It is a requirement of bitstream conformance that when sao_type_idx[ cIdx ][ rx ][ ry ] is not equal to 5, the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of 0 to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive and otherwise, the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of −( 1<< ( Min( bitDepth, 10 ) − 5 ) ) to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive.

When sao_offset[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx − 1 ][ ry ][ i ].

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 5 and i is larger than 1,offsetSign is set to −1.

- Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5 or i is less than 2), offsetSign is set to 1.

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-36)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*sao_offset[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) ) 	(7-37)