| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April-7 May 2012 | Document: JCTVC-I0433 |
| --- | --- |

| Title: | Adaptive rate control for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junjun Si, Siwei Ma, Wen Gao Peking University Yiheyuan Road 5, Haidian District, Beijing, China, 100871 Mingyuan Yang Huawei Technologies Co., Ltd. | Tel: Email: | 86-10-62753424 jjsi@jdl.ac.cn, swma@pku.edu.cn wgao@pku.edu.cn myyang@huawei.com |
| Source: |  |  |  |

_____________________________

- Abstract

This contribution provides a frame level rate control scheme for HEVC, which originates from ABR (Adaptive Bitrate) used in the popular x264 codec with fine tuning for HEVC. The proposed rate control scheme is designed deliberately for Low delay and Random Access coding respectively and implemented into HM5.0. Compared with the original rate control scheme proposed by JCTVC-H0213, the average coding gain can be up to 0.64dB for LB-HE & LB-LC(for LP-HE: 0.33dB; RA-HE: 0.42dB; LP-LC: 0.31dB; RA-LC: 0.44dB ).

Introduction

Rate control is indispensable for the real application of video coding standards. A rate control algorithm has been proposed by JCTVC-H0213 and adopted at the last San Jose meeting. However, the performance of H0213 is not so good or even worse than the HM anchor. The average loss may be up to -0.62dB for LB-LC(LB-HE: -0.61dB, LP-HE: -0.39dB ; RA-HE: -0.42dB; LP-LC: -0.35dB; RA–LC: -0.43dB). For the worst case, the loss is more than 1dB, which is unacceptable. Based on the ABR(Adaptive Bit Rate) rate control scheme used in the popular x264 codec, a new adaptive rate control scheme is proposed for HEVC in this proposal. The rate control algorithm is designed carefully for LD and RA setting respectively.

As ABR for x264, the complexity of previously encoded frames is used for experimental rate model modeling, and the experimental results show that the rate modeling can achieve accurate rate estimation for HEVC. The video buffer verifier (VBV) operation model used in x264 is established to avoid overflow and underflow of client buffer. Moreover, adaptive QP adjusting strategies are set up to ensure the generated bitrate meets the target requirement with comparable quality as HM5.0.

Proposed rate control algorithm

2.1 Rate modeling

Based on the rate distortion modeling study of the rate control algorithm in x264, a linear R-D model is proposed for HEVC, as shown in Equation (1). In the proposed model, SATD (Sum of Absolute Transformed Difference) is used as complexity estimation. Besides, the complexities of previously encoded frames are taken into account in the proposed rate model, providing efficient information to smooth the coding performance. The proposed rate model is shown as,

(1)

where:

n is the current frame number;

is a constant. The recommended value is 0.6;

is the quantization parameters of the n-th frame;

is the target bits of the n-th frame;

is the actual bits of the n-th frame;

is the weighted sum of SATD values of previously encoded (n+1) frames.is defined as:

(2)

Figure.1, The relationship between the generated bits and the estimated bits. QP is set to 32.

Figure.1 shows the performance of the proposed rate model. The test sequence is encoded with a fixed quantization parameter. Obviously, the mismatch of generated bits and estimated bits per frame is relatively small.

2.2 Rate control scheme

Considering the difference between LD and RA settings, the proposed rate control algorithm is designed discriminatingly. A GOP level QP adjustment strategy is designed for the RA setting to maintain the generated bitrate with the target. Frame level adaptive QP adjustment schemes are designed for both LD and RA settings to make the bitrate match the target requirement as soon as possible. Finally, the quantization parameter is adjusted to make sure no buffer overflow or underflow.

The proposed rate control algorithm is detailed as follows:

2.2.1 Rate control algorithm for LD setting

Step 1: Initialize the VBV buffer. Set the initial buffer fullness, where is the buffer size. Letdenote the current buffer fullness and set;

Step 2: Bit allocation for the current frame, shown as,

(3)

where T is the target bits, R is the target bitrate and Fr is the frame rate.is defined as follows:

(4)

where is the difference between the current and the initial VBV buffer fullness, andis set to 0.1 by default.

Step 3: Estimate the frame complexity SATD by doing rough motion estimation over LCUs, which can be merged with motion estimation of CU in the future without increasing complexity. And Experimental results show that this pre-analysis hardly effects the overall encoding time.

Step 4: Calculate the quantization parameter using the rate model proposed in section 2.1. As for the first frame of the sequence, a constant valueis used to substitute , defined as:

(5)

Step 5: Check whether the calculated QP in step3 complies with the VBV buffer constraints. If not, clip QP to avoid buffer overflow or underflow. The detail steps are shown as:

- If the current buffer fullnessis less than the half of the buffer size, it means the former encoded frame has consumed too much bits, so the QP value of current frame is increased as the following equation:

(6)

whereis clipped between 0.5 and 1.

- Predict the bits to be consumed by the current frame withand the SATD value of the current frame, defined as:

(7)

whereis the predicted bits count. are constants, which are updated with the final QP value and bits generated after one frame is encoded.

- If the  is bigger than, then the QP value should be increased as following:

(8)

- If the  is smaller than, then the QP value should be decreased as following:

(9)

Step 6: Implement adaptive frame level QP adjustment: regulate the quantization parameter for the current frame based on the difference between the generated bits and the target bits so far.

(10)

whereare constants, andis between 0~1. is bigger than 1. is the difference between the target bits and the generated bits of the encoded frames.

Step 7: For quality smoothness, QP derived from step6 is clipped around a small range of the QP value of the closest frame of the same type.

(11)

is 2 or 4 used in the experiments according to the bit rate fluctuation .

Step 8: The QP value is further clipped between the minimum and maximum QP value allowed. The range is set to (0, 51) in experiments.

Step 9: Update the VBV buffer fullness after the frame is encoded using the following equation:

(12)

wheredenotes the current buffer fullness.

Step 10: Go to Step 2 to continue coding the next frame until the sequence is finished.

2.2.2 Rate control algorithm for RA setting

Figure.2, The hierarchical B structure in RA setting.

Step 1: For the first frame of the sequence, it is coed with an initial QP. For the other frame, average bit allocation for the current frame is applied:

(14)

Step 2: Same as step3 of LD setting. Estimate the frame complexity SATD by doing rough motion estimation over LCUs, which can be merged with motion estimation of CU in the future without increasing complexity.

Step3: As shown in Figure 2, for the referenced frames (frame in depth 0/1/2), QP determination is the same as step4 of LD setting. And QP of the unreferenced B frames (depth 3) is derived from a linear interpolation method as follows:

(15)

where  and  are the quantization parameters of two nearest reference frames respectively.  is a constant set to 1.4. And , , where POC is the picture order count for display order.

Step4: Perform GOP level adaptive QP adjustment for I frames according to the coding status of the previous GOP.

(16)

where  are the generated and target bits of the previous GOP respectively.  is the average QP of the previous encoded GOP. For the first GOP this step is skipped.

Step5: Implement adaptive frame level QP adjustment: regulate the quantization parameter for the current frame based on the difference between the generated bits and the target bits so far. And quantization parameter of B-frames should not be less than its reference frames.

(17)

whereare constants, andis between 0~1, is bigger than 1. is the difference between target bits and generated bits of encoded frames. depth_i is the depth of the current picture.

Step6: For quality smoothness, QP derived from step6 is clipped with the QP value of the closest frame in the lower depth. As for frames in depth 0, the previous encoded frame in the same depth is used as an alternative.

(18)

is 1 or 2 used in the experiments according to the bit rate fluctuation .

Step7: Same as step8 of the LD setting. The QP value is further clipped between the minimum and maximum QP value allowed. The range is set to (10, 46) in experiments.

Step8: Go to Step 1 to continue coding the next frame, until the sequence is finished.

Experimental results

The proposed rate control scheme is implemented on HM5.0 reference software. The control accuracy and R-D performance are compared with that of H0213 and HM5.0, respectively. R-D curves and PSNR curves for typical test sequences are also provided.

Table 1. Rate control performance with RA, LB, and LP with HE compared with JCTVC-H0213

Table 2. Rate control performance with RA, LB, and LP with LC compared with JCTVC-H0213

Table 1-2 show the control accuracy and R-D performance of the proposed method compared with H0213. Obviously the proposed method outperforms H0213 in terms of BD-PSNR and BD-RATE.

Table 3. Rate control performance with RA, LB, and LP with HE compared with HM5.0

Table 4. Rate control performance with RA, LB, and LP with LC compared with HM5.0

Table 3-4 show the control accuracy and R-D performance of the proposed method compared with HM5.0. The proposed method show comparable R-D performance and the control accuracy is within 1%.

Figure.3, R-D curve of typical sequences with RA.

Figure.4, R-D curve of typical sequences with LB.

Figure.5, R-D curve of typical sequences with LP.

Figure 3-5 show the R-D curves of typical test sequences with RA, LB and LP respectively. The R-D performance of the proposal is better than that of H0213 and comparable with HM5.0.

Figure.6, PSNR curve of “BQTerrace” with LB HE

Figure.7, PSNR curve of “BQTerrace” with LB LC

Figure.8 PSNR curve of “BQSqure” with LP HE

Figure.9, PSNR curve of “BQSqure” with LP LC

Figure.10, PSNR curve of “PeopleOnStreet” with RA HE

Figure.11, PSNR curve of “PeopleOnStreet” with RA LC

Figure 6-11 show the PRNR curves of typical test sequences, obviously, the proposed rate control scheme results in smoothness of quality.

Reference

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Rate control based on unified RQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H0213 (m23088), San José, CA, USA, Feb. 2012

- Frank Bossen, “Common test conditions and software reference configurations” , JCTVC-H1100, 8th JCT-VC Meeting, San Jose, CA, USA, 1-10 February, 2012.

Patent rights declaration(s)

PKU and Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).