#### Coding tree block modification process

Inputs to this process are:

–	picture sample array recPicture for the colour component cIdx,

–	a variable cIdx specifying colour component index,

–	a pair of variables ( rx, ry ) specifying the coding tree block location,

–	a coding tree block size nS.

Output of this process is a modified picture sample array saoPicture for the colour component cIdx.

The variable saoTypeIdx is set equal to sao_type_idx[ cIdx ][ rx ][ ry ].

The variable bitDepth is derived as follows.

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The variables xC and yC are set equal to rx*nS and ry*nS, respectively.

For i = 0..nS−1 and j = 0..nS−1, depending on the value of saoTypeIdx, pcm_loop_filter_disable_flag, pcm_flag, and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xC + i, yC + j ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- saoTypeIdx is equal to 0.

–	Otherwise, if saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k=0..1 are specified in Table 8-14 based on sao_type_idx[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows.

- If one or more of the following conditions for k=0..1 are true, edgeIdx is set equal to 0.

- The sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) is outside picture boundary

- The sample at location ( xC + i + hPos[ k ], yC + j + vPos[ k ] ) belongs to a different slice and slice_loop_filter_across_slices_enabled_flag is equal to 0 in the current slice and one or more of the following conditions are true,

        - The sample at location ( xC + i, yC + j - 1) belongs to a different slice and vPos[ k ] is equal to -1.

        - The sample at location ( xC + i - 1, yC + j ) belongs to a different slice, hPos[ k ] is equal to -1 and vPos [ k ] is not equal to 1 if the sample at location ( xC + i, yC + j + 1) belongs to a different slice.

- The sample at location ( xC + i + hPos[ k ], yC + j + vPos[ k ] ) belongs to a different slice and slice_loop_filter_across_slices_enabled_flag is equal to 0 in the slice to which the sample at location ( xC + i + hPos[ k ], yC + j + vPos[ k ] ) belongs and one or more of the following conditions are true,

        - The sample at location ( xC + i, yC + j + 1) belongs to a different slice and vPos[ k ] is equal to 1.

        - The sample at location ( xC + i + 1, yC + j ) belongs to a different slice, hPos[ k ] is equal to 1 and vPos [ k ] is not equal to -1 if the sample at location ( xC + i, yC + j – 1 ) belongs to a different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to a different tile.

- Otherwise, edgeIdx is derived as follows.

edgeIdx = 2 + ∑k( Sign3( recPicture[ xC + i, yC + j ] – 
	recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-321)

When edgeIdx is equal to 0, 1, or 2, it is modified as follows.

edgeIdx = (edgeIdx = = 2) ? 0 : (edgeIdx + 1)	(8-322)

- The modified picture sample array saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(8-323)

- Otherwise (saoTypeIdx is equal to 5), the following ordered steps apply:

- The variable bandShift is set equal to bitDepth − 5.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The list bandTable is defined with 32 elements and all elements are initially set to 0. Then, four of its elements (indicating the starting position of bands for explicit offsets) are modified as follows.

for( k = 0; k < 4; k++ )
	bandTable[ (k + saoLeftClass) & 31 ] = k + 1	(8-324)

- The variable bandIdx is set equal to bandTable[ recPicture[ xC + i, yC + j ] >> bandShift ].

- The modified picture sample array saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
			SaoOffsetVal[ cIdx ][ rx ][ ry ][ bandIdx ] )	(8-325)

Table 8-14 – Specification of hPos and vPos according to the sample adaptive offset type

| sao_type_idx[ cIdx ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |

##### Granularity block boundary padding process

Inputs of this process are:

–	a reconstructed picture sample array recPicture prior to ALF,

–	a location ( xB, yB ) specifying the top-left sample of the current granularity block relative to the top left sample of the current picture,

–	a location (xD, yD) specifying the top-left sample of the current granularity block relative to the top left sample of the current coding block,

–	a variable gbSize specifying the granularity block size,

–	a (cbSize+8)x(cbSize+8) padded sample array padBlock.

Output of this process is the modified (cbSize+8)x(cbSize+8) padded sample array padBlock.

Depending on slice_loop_filter_across_slices_enabled_flag and loop_filter_across_tiles_enabled_flag, the padded sample array padBlock is derived as follows.

There are 8 neighbouring blocks that may be referenced during the filtering process of the current block. The neighbouring blocks include left, right, above, bottom, above-left, above-right, below-left, and below-right blocks. The availability of each neighbouring block is derived as follows:

- If all of the following conditions are true, the neighbouring block is marked as "available for ALF".

  - the neighbouring block and the current block belong to the same slice.

  - The left boundary of the current block is the left boundary of the current slice or the top boundary of the current block is the top boundary of the current slice, the neighboring block is one of the above-right, above, above-left or left neighboring blocks and it belongs to a different slice, slice_loop_filter_across_slices_enabled_flag is equal to 1 in the current slice.

  - The right boundary of the current block is the right boundary of the current slice or the bottom boundary of the current block is the bottom boundary of the current slice, the neighboring block is one of the right, below-right, below or below-left neighboring blocks and it belongs to a different slice, slice_loop_filter_across_slices_enabled_flag is equal to 1 in the slice to which the neighboring block belongs.

  - the neighbouring block and the current block belong to the same tile or loop_filter_across_tiles_enabled_flag is equal to 1.

  - the neighbouring block is not outside picture boundary.

- Otherwise, the neighbouring block is marked as "not available for ALF"

The padded sample array padBlock[ x , y] is derived by the following ordered steps:

1. padBlock[ xD+4+x, yD+4+y ] is set equal to recPicture[ xB+x, yB+y ] for x = 0..(gbSize−1) and y = 0..(gbSize−1).

2. The left margin is derived as follows:

  - If the left block is marked as “available for ALF",

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB+y ] for x = −1..− 4 and y = 0..(gbSize−1)

  - Otherwise (the left block is marked as “not available for ALF”),

  - If the below-left block is marked as "available for ALF" and one or more of the following conditions are true,

    - the left boundary of the current block does not coincide with an CTB boundary.

    - both the left and bottom boundaries of the current block coincide with CTB boundaries.

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB, yB+y ] for x = −1..−4 and y = 0..(gbSize− 4−1)

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB+gbSize ] for x = −1..− 4 and y= (gbSize−4)..(gbSize−1)

  - Otherwise, the following applies:

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB, yB+y ] for x = −1..− 4 and y = 0..(gbSize−1)

3. The right margin is derived as follows:

  - If the right block is marked as “available for ALF",

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize+x, yB+y ] for x = 0..(4−1) and y = 0..(gbSize−1)

  - Otherwise (the right block is marked as “not available for ALF”),

  - If the above-right block is marked as "available for ALF" and all of the following conditions are true,

    - the right boundary of the current block coincides with an CTB boundary.

    - the top boundary of the current block does not coincides with an CTB boundary.

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize+x, yB−1 ] for x= 0..(4−1) and y= 0..(4−1)

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize−1, yB+y ] for x= 0..(4−1) and y= 4..(gbSize−1)

  - Otherwise, the following applies:

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize−1, yB+y ] for x= 0..(4−1) and y= 0..(gbSize−1)

4. The above margin is derived as follows:

  - If the above block is marked as “available for ALF",

padBlock[xD+4+x, yD+4+y ] = recPicture[ xB+x, yB+y ] x= 0..(gbSize−1) and y= −1..−4

  - Otherwise (the above block is marked as “not available for ALF”),

  - If the above-right block is marked as "available for ALF" and all of the following conditions are true,

    - the right boundary of the current block coincides with an CTB boundary.

    - the top boundary of the current block does not coincide with an CTB boundary.

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB ] for x= 0..(gbSize−4−1) and y= −1..−4

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+gbSize, yB+y ] for x= (gbSize−4)..(gbSize−1) and y= −1..−4

  - Otherwise, the following applies

padBlock[  xD+4+x, yD+4+y ] = recPicture[ xB+x, yB ] for x= 0..(gbSize−1) and y= −1..−4

5.  The below margin is derived as follows:

  - If the below block is marked as “available for ALF",

padBlock[ xD+4+x, yD+4+gbSize+y ] = recPicture[ xB+x, yB+gbSize+y ] for x= 0..(gbSize−1) and y= 0..(4−1)

  - Otherwise (the below block is marked as “not available for ALF”),

  - If the below-left block is marked as "available for ALF" and one or more of the following conditions are true,

    - the left boundary of the current block does not coincide with an CTB boundary.

    - both the left and bottom boundaries of the current block coincide with CTB boundaries.

padBlock[ xD+4+x, yD+4+gbSize+y ] = recPicture[ xB−1, yB+gbSize+y ] for x= 0..( 4−1) and y= 0..(4−1)

padBlock[ xD+4+x, yD+4+gbSize+y ] = recPicture[ xB+x, yB+gbSize−1 ] for x= 4..(gbSize−1) and y= 0..(4−1)

  - Otherwise, the following applies:

padBlock[ xD+4+x, yD+4+gbSize+y ] = recPicture[ xB+x, yB+gbSize−1 ] for x= 0..(gbSize−1) and y= 0..(4−1)

6. The above-left margin is derived as follows:

  - If the above-left block is marked as “available for ALF",

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB+y ] for x= −1..−4 and y= −1..−4

  - Otherwise (the above-left block is marked as “not available for ALF”),

  - If the above block and the left block are both marked as “not available for ALF”,

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB, yB ] for x= −1..−4 and y= −1..−4

  - Otherwise, if the above block is marked as "available for ALF" and one or more of the following conditions are true,

    - the left block is marked as "not available for ALF".

    - the left block is marked as "available for ALF", and the left boundary of the current block coincides with an CTB boundary, and the top boundary of the current block does not coincide with an CTB boundary.

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB, yB+y ] for x= −1..−4 and y= −1..−4

  - Otherwise (none of the conditions above is true), the following applies:

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB ] for x= −1..−4 and y= −1..−4

7. The above-right margin is derived as follows:

  - If the above-right block is marked as “available for ALF",

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize+x, yB+y ] for x= 0..(4−1) and y= −1..−4

  - Otherwise (the above-right block is marked as “not available for ALF”),

  - If the above block and the right block are both marked as “not available for ALF”,

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize−1, yB ] for x= 0..(4−1) and y= −1..−4

  - Otherwise, if the above block is marked as "available for ALF" and the right block is marked as "not available for ALF",

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize−1, yB+y ] for x= 0..(4−1) and y= −1..−4

  - Otherwise (none of the conditions above is true), the following applies:

padBlock[ xD+4+gbSize+x, yD+4+y ] = recPicture[ xB+gbSize+x, yB ] for x= 0..(4−1) and y= −1..−4

8. The below-left margin is derived as follows:

  - If the below-left block is marked as “available for ALF",

padBlock[ xD+4+ x, yD+4+gbSize+y ] = recPicture[ xB+x, yB+gbSize+y ] for x= −1..−4 and y= 0..(4−1)

  - Otherwise (the below-left block is marked as “not available for ALF”),

  - If the below block and the left block are both marked as “not available for ALF”,

padBlock[ xD+4+ x, yD+4+gbSize+y ] = recPicture[ xB, yB+gbSize−1 ] for x= −1..−4 and y= 0..(4−1)

  - Otherwise, if the below block is marked as "available for ALF" and the left block is marked as "not available for ALF",

padBlock[ xD+4+ x, yD+4+gbSize+y ] = recPicture[ xB, yB+gbSize+y ] for x= −1..−4 and y= 0..(4−1)

  - Otherwise (none of the conditions above is true), the following applies:

padBlock[ xD+4+ x, yD+4+gbSize+y ] = recPicture[ xB+x, yB+gbSize−1 ] for x= −1..−4 and y= 0..(4−1)

9. The below-right margin is derived as follows:

  - If the below-right block is marked as “available for ALF",

padBlock[ xD+4+gbSize+x, yD+4+gbSize+y ] = recPicture[ xB+gbSize+x, yB+gbSize+y ] for x= 0..(4−1) and y= 0..(4−1)

  - Otherwise (the below-right block is marked as “not available for ALF”),

  - If the below block and the right block are both marked as “not available for ALF”,

padBlock[ xD+4+gbSize+x, yD+4+gbSize+y ] = recPicture[ xB+gbSize−1, yB+gbSize−1 ] for x= 0..(4−1) and y= 0..(4−1)

  - If the below block is marked as "available for ALF" and one or more of the following conditions are true,

    - the right block is marked as "not available for ALF".

    - the right block is marked as "available for ALF", and the right boundary of the current block does not coincide with an CTB boundary or the bottom boundary of the current block coincides with an CTB boundary.

padBlock[ xD+4+gbSize+x, yD+4+gbSize+y ] = recPicture[ xB+gbSize−1, yB+gbSize+y ] 
for x= 0..(4−1) and y= 0..(4−1)

  - Otherwise (none of the conditions above is true), the following applies:

padBlock[ xD+4+gbSize+x, yD+4+gbSize+y ] = recPicture[ xB+gbSize+x, yB+gbSize−1 ] for x= 0..(4−1) and y= 0..(4−1)