| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 October 2012 | Document: JCTVC-K0217 |
| --- | --- |

| Title: | Proposed Changes on Coding Tree Unit Syntax and Sequence parameter set RBSP syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Fang, Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3618 x.fang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution proposes high level syntax modifications in two sections. The first section proposes modification of Coding Tree Unit Syntax change regarding the InverseRasterScan function. The second section addresses the clean up as well as simplification related to PCM parameters in Sequence parameter set RBSP syntax.

.

# Coding Tree Unit Syntax

## Possible Issues

According to HEVC spec [1], Coding Tree Unit (CTU) contains a luma location ( xCtb, yCtb ) specifying the top-left sample of a current tree unit (CU) relative to the top-left luma sample of a current picture, as shown in Table 1 and highlighted in red.

Table 1 Coding Tree Unit Syntax

| coding_tree_unit( xCtb, yCtb ) { | Descriptor |
| --- | --- |
| NumPCMBlock = 0 |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| CtbAddrInSlice = CtbAddrRS − slice_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> Log2CtbSizeY, yCtb >> Log2CtbSizeY ) |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| } |  |

where InverseRasterScan() is given as

InverseRasterScan( a, b, c, d, e ) = 	(5-7)

When pic_width_in_luma_samples is not dividable by the tree unit size CtbSize which is equal to 1 << Log2CtbSizeY, xCtb and yCtb will not be calculated correctly in some cases.

For example, given a picture size of 416x240 and CtbSize of 64x64, there will be 7x4 CTU in total. Each CTU row has 7 CTUs according to the PicWidthInCtbsY defined in equation (7-20). For CTU of CtbAddrRS = 6, the calculated location of the up right most unit, (xCtb, yCtb), will be (0, 64) while the actual position should be (384, 0).

PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )	(7-20)

PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )	(7-22)

## Proposal Editoral Changes

| coding_tree_unit( xCtb, yCtb ) { | Descriptor |
| --- | --- |
| NumPCMBlock = 0 |  |
| xCtb = (CtbAddrRS %PicWidthInCtbsY)<< Log2CtbSizeY |  |
| yCtb = (CtbAddrRS / PicHeightInCtbsY) << Log2CtbSizeY |  |
| CtbAddrInSlice = CtbAddrRS − slice_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> Log2CtbSizeY, yCtb >> Log2CtbSizeY ) |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| } |  |

Here (CtbAddrRS/ PicHeightInCtbsY) and (CtbAddrRS/PicWidthInCtbsY) are corresponding to the row and column coding tree unit address of a picture. PicHeightInCtbsY is defined in equation (7-22).

# PCM in Sequence parameter set RBSP syntax

## PCM parameter Issues in Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Issue 1: PCM parameters are spread in three locations of the syntax, each with a condition check if pcm_enabled_flag is on.

Issue 2: log2_min_pcm_luma_coding_block_size_minus3 is already limited within the range set by log2_min_luma_coding_block_size_minus3 and log2_diff_max_min_luma_coding_block_size. Coding the delta from log2_min_luma_coding_block_size_minus3 highlited in cyan can save bits and define more clearly without the sematic “The variable Log2MinIpcmCbSizeY shall be in the range of Log2MinCbSizeY to Min( Log2CtbSizeY, 5 ), inclusive”.

## Proposal PCM parater cleanup

### Group all the PCM related parameters together and even have a pcm_parameters().

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| …….. |  |

### Change log2_min_pcm_luma_coding_block_size_minus3 to log2_diff_min_pcm_luma_coding_block_size.

log2_diff_min_pcm_luma_coding_block_size +  log2_min_luma_coding_block_size_minus3 + 3 specifies the minimum size of coding blocks with pcm_enabled_flag equal to 1.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_diff_min_pcm_luma_coding_block_size | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| …….. |  |

Or if proposal 2.2.1 is adopted,

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_diff_min_pcm_luma_coding_block_size | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| …….. |  |

The following is one of the possible syntax examples of proposals above.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_diff_min_pcm_luma_coding_block_size | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” JCTVC-K0030.