| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CH, 14–21 October 2016 | Document: JCTVC-Y0045r1 |
| --- | --- |

| Title: | AHG14: cross-checking results of JCTVC-Y0042 on usage of TMI, CRI and MDCV SEIs for HDR Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check report |  |  |
| Author(s) or Contact(s): | F. Hiron, E. François (Technicolor) | Email: | franck.hiron@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document reports crosscheck results for proposal JCTVC-Y0042 on usage of colour remapping information (CRI) and tone mapping information (TMI) SEI messages for HDR coding with SDR compatibility. Two bitstream SDR compatible implementations are investigated in JCTVC-Y0042, the first one using TMI, the second using CRI. Mapping applies in RGB domain. Cross-checking results reportedly confirmed the objective results reported in JCTVC-Y0042, with minor mismatches in a few cases, due to using different compiler. During visual check of the SDR quality, some colors deviation compared to the HDR rendering are observed. The SDR versions were in general judged by the cross-checker a bit too dark and not enough contrasted, especially for dark content. Globally, the SDR quality was judged as acceptable compared to PQ version rendered on an SDR display.

# Introduction

Contribution JCTVC-Y0042 have been cross-checked by three tests:

- Test1: Usage of Tone Mapping Information (TMI) SEI message to provide SDR compatibility for HDR content. The input HDR content is mapped in a pre-processing step to HDR, using a dynamic function applying in 4:4:4 RGB domain. In pre-processing, a gamma of 1/2.2 is applied to generate the SDR version after linear-light mapping of R, G, and B components. A TMI SEI is used to model and code the inverse mapping function, and TMI is applied in RGB domain in a post-decoding process to convert the decoded video to an HDR version.

- Test2: Usage of Colour Remapping Information (CRI) SEI message to provide SDR compatibility for HDR content. The input HDR content is mapped in a pre-processing step to HDR, using a dynamic function applying in 4:4:4 RGB domain. In pre-processing, a gamma of 1/2.4 is applied to generate the SDR version after linear-light mapping of R, G, and B components. A TMI CRI is used to model and code the inverse mapping function, using the three CRI post-LUTs (the 3 pre-Luts and the 3x3 matrix are not activated). CRI is applied in RGB domain in a post-decoding process to convert the decoded video to an HDR version.

# Simulation results

## Objective results

Simulations are performed on Linux platform. As shown in Table 1 and Table 2, cross-checking confirms the objective metrics of compression results obtained in Test1 and Test2, with however slight mismatches that are most probably due to using a different compiler than the JCTVC-Y0042 proponent.

Comparisons with the HM anchor 3.2 (as recommended in CTCs, cf JCTVC-X1020), are shown in Table 3. It is observed a PSNR_DeltaE BD-rate gain of 15.3% for TMI case, and 21.6% for CRI case, and a PSNR_L BD-rate loss of 7.8% for TMI case, and 6.9% for CRI case. Test2 (CRI) outperform Test1 (TMI) (6% more gain in PSNR_DeltaE BD-rate, 1% less loss in PSNR_L BD-rate).

Table 1. Objective results for Test1 (TMI) – Cross-checker vs Proponent.

Table 2. Objective results for Test2 (CRI) – Cross-checker vs Proponent.

Table 3. Objective results for Test1 (TMI, left table) and Test2 (CRI, right table) versus Anchor 3.2.

Test1 (TMI)	Test2 (CRI)

# SDR quality visual assessment

For Test1 and 2 on SDR compatible solutions, a visual assessment of the SDR versions has been made, with a check of the conformity of the color and texture to the HDR version displayed on a Sim2 monitor. Comparisons with SDR versions internally created by the cross-checker, produced to get a rendering as similar as possible to the HDR, were also made.

SDR videos were displayed both on a consumer TV and on a professional monitor (TVlogic XVM 245W). It was observed that visual quality of resulting SDR version of HDR content is acceptable, but often too dark and with color shift in different cases (especially on saturated colors). CRI solution is judged a bit better, thanks to using a gamma of 1/2.4 (instead of 1/2.2 in case of Test1) in the final mapping stage, leading to a bit brighter pictures. More visual assessment comments and snapshots are provided in section 4.

It can be expected an improved rendering of the SDR by using the full model of CRI, that is, the three Pre-LUTs, the three-by-three matrix and the three Post-LUTs. The usage of the three-by-three matrix may help managing the color shift issues. It is also worth mentioning that the usage of cross-plane scaling, as previously proposed in the ETM (JCTVC-W0031 and JCTVC-W0092), may solve the hue and saturation issues that are observed when applying the reshaping functions independently to each component.

# Conclusion

The cross-checking confirms the objective results reported in JCTVC-Y0042 with some minor mismatches probably due to using a different compiler than the proponent. During visual check of SDR quality, SDR quality is judged as acceptable. However, some colors deviation and loss of contrast, compared to the HDR rendering are observed. Usage of full CRI model, or of cross-plane scaling, may solve these issues.

# SDR pictures snapshots

In the following, for each test picture, three snapshot SDR versions are presented. The first picture corresponds to an SDR version internally created by the cross-checker, produced to get a rendering as similar as possible to the HDR. It is a kind of reference for the SDRs coming from Test1 (2nd picture) and Test2 (3rd picture). For m	any cases, the SDRs from Test1 and Test2 look too dark. This sis especially visible on rather dark content such as the StEM content. In all cases, the CRI version looks better than the TMI version because it is a bit brighter. Slight hue shift can also be observed in different cases.

In more details, the following observations are made:

- balloonFestival: colors in sky a bit desaturated, overall brightness and contrast ok

- bikeSparklers: colors ok, loss of contrast, pictures too dark

- fireEater: overall rendering conform to HDR

- garageExit: colors ok, loss of contrast, pictures too dark

- EBU_Hurdles: hue shift and desaturation observed (red/orange and blue areas), loss of contrast

- StEM_MagicHours: picture too dark, colors ok

- market: hue shift on red banner on the left and purple dress on the right, brightness ok

- showGirl: slight hue shift and desaturation observed in colored areas (flowers), pictures too dark

- EBU_Starting: slight hue shift on deep blue, otherwise globally ok

- sunrise: globally good

- StEM_WarmNight: picture too dark, colors ok

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)	Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

Reference	Test1 (TMI)	Test2 (CRI)

# Usage of NEW Tone Mapping Information (new TMI) SEI message to provide SDR compatibility for HDR content

## Simulation results

Simulations are performed on Linux platform. As shown in Table 4. Objective results for Test3 (new TMI) – Cross-checker vs Proponent, cross-checking confirms the objective metrics of compression results obtained in Test3.

Comparisons with the HM anchor 3.2 (as recommended in CTCs, cf JCTVC-X1020), are shown in Table 5. It is observed a PSNR_DeltaE BD-rate gain of 38.7% for new TMI case, and a PSNR_L BD-rate loss of 11.1% for new TMI case.

Table 4. Objective results for Test3 (new TMI) – Cross-checker vs Proponent

Table 5. Objective results for Test3 (new TMI) versus Anchor 3.2.

## Conclusion

The cross-checking confirms the objective results reported in JCTVC-Y0042 v5. During visual check of SDR quality, for many cases, the SDRs from Test3 look too bright, hue shift and desaturation observed, loss of contrast.

## SDR pictures snapshots

In the following, for each test picture, three snapshot SDR versions are presented. The first picture corresponds to an SDR version internally created by the cross-checker, produced to get a rendering as similar as possible to the HDR. It is a kind of reference for the SDRs coming from Test3 (2nd picture)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)	            Reference   Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)

Reference	Test3 (new TMI)