| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z1020 |
| --- | --- |

| Title: | Common Test Conditions for HDR/WCG video coding experiments |  |  |
| --- | --- | --- | --- |
| Status: | Approved Output Document of JCT-VC |  |  |
| Purpose: | Report of agreed test conditions |  |  |
| Author(s) or Contact(s): | E. François, J. Sole, J. Ström, P. Yin | Tel: Email: | edouard.francois@technicolor.com pyin@dolby.com joels@qti.qualcomm.com jacob.strom@ericsson.com |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in the context of experiments for JCT-VC HDR/WCG video coding experiments.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments. This document defines the CTC for the HDR/WCG Video Coding Experiments. There are two anchors defined in CTC: 1) NCL YCbCr and 2) ICTCP. Input contributions to JCT-VC should provide a set of results as complete as possible that apply to the proposal compared to the corresponding anchor. Results should be reported using the Excel sheets provided by the JCT-VC AHG on HEVC HM and HDRTools software development and software technical evaluation.

The following sections define test sequences, quantization parameter values, encoder configurations, and the pre- and post-processing options to be used.

# Test material

## File exchange formats

The filenames are specified as follows:

Name_Resolution_Fps_Format_ContentPrimaries_ContainerPrimaries_ChromaFormat_xxx.yyy

with

- Name: 	sequence name

- Resolution: 	picture size (e.g. 1920x1080p)

- Fps: 	frame rate in frames per second

- Format: 	format of the samples (e.g. ff for 32-bit floating point, hf for half-float 16-bit floating point, 10 for 10-bit integer)

- Content primaries: 	colour primaries of the colour volume of the content, e.g. ITU-R Recommendations BT.709 and BT.2020, SMPTE ST 428-1:2006 P3 with D65 white point (P3D65)

- Container primaries:	colour primaries of the container (when different from the content primaries), e.g. ITU-R Recommendations BT.709 and BT.2020, SMPTE ST 428-1:2006 P3 with D65 white point (P3D65)

- Chroma format:	e.g. 4:2:0, 4:2:2, or 4:4:4 (when applicable; for instance TIFF format involves 4:4:4 interleaved)

- xxx: 	frame number (when applicable)

- yyy:	exr, tif, tiff or yuv

## Test sequences

All the test sequences have the following characteristics:

- Resolution: 1920x1080 progressive

- Colour format: RGB 4:4:4

- Container: BT.2020 or P3D65 depending on the content

The test sequences considered for the evaluation tests are listed in Table 1.

Table 1. HDR test sequences.

| Class | seq | gamut | TF | Sequence name | fps | Frames |
| --- | --- | --- | --- | --- | --- | --- |
| A | S00 | BT.709 | Linear | FireEater2Clip4000r1_1920x1080p_25_hf_709_ct2020_444_xxx.exr | 25 | 0-199 |
|  | S02 | BT.709 | Linear | Market3Clip4000r2_1920x1080p_50_hf_709_ct2020_444_xxx.exr | 50 | 0-399 |
| B | S04 | P3D65 | PQ12b | BikeSparklersClip4000_1920x1080p_24_12_P3_ct2020_xxx.tif 1 | 24 | 23-128 + 216-340 |
| C | S05 | P3D65 | PQ12b | ShowGirl2TeaserClip4000_1920x1080p_25_12_P3_ct2020_xxx.tif | 25 | 0-338 |
| D | S06 | P3D65 | PQ12b | StEM_MagicHour_1920x1080p_24_12_P3_xxx.tif 2 | 24 | 3527 to 3608 + 3609 to 3803 + 3804 to 3887 |
|  | S07 | P3D65 | PQ12b | StEM_WarmNight_1920x1080p_24_12_P3_xxx.tif 2 | 24 | 6280 to 6481 + 6482 to 6640 |
| G | S08 | BT.709 | Linear | BalloonFestival_1920x1080p_24_hf_709_ct2020_444_xxx.exr | 24 | 0-239 |
| H | S10 | BT.709 | Linear | EBU_04_Hurdles_1920x1080p_25_hf_709_ct2020_444_xxx.exr | 100 | 0-499 |
|  | S11 | BT.709 | Linear | EBU_06_Starting_1920x1080p_25_hf_709_ct2020_444_xxx.exr | 100 | 0-499 |
| A | S12 | BT.709 | Linear | Sunrise_1920x1080p_25_hf_709_ct2020_444_xxx.exr | 25 | 0-199 |
| B | S13 | P3D65 | PQ12b | GarageExit_1920x1080p_24_12_P3_ct2020_xxx.tif | 24 | 0-287 |

1 for BikeSparklers, only 1st and 3rd cuts are kept ([023-128] + [216-340])

2 for StEM sequences, the successive cuts of S06 and S07 are considered

Material originally in a P3D65 container will be evaluated after direct conversion into a BT.2020 container, as specified in section 4.2.2.2 for YCbCr and in section 5.2.2.2 for ICTCP.

Notes:

- StEM (S06, S07) and ShowGirl2Teaser (S05) sequences use letter box. Cropping is required for derivation of objective metrics.

  - For StEM, the objective metrics must be computed in cropping window delimited by pixels (column=0, row=140) to (column=1919, row=939).

  - For ShowGirl2Teaser, the objective metrics must be computed in cropping window delimited by pixels (column=10, row=10) to (column=1909, row=1069).

Metrics configuration files HDRMetric_CfE_ShowGirl.cfg and HDRMetric_CfE_StEM.cfg are provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package for YCbCr and ICTCP.

- TIFF sequences must be converted, before encoding, to YCbCr 4:2:0 10b format, as described in section 4.2.2; or ICTCP 4:2:0 10b format, as described in section 5.2.2.

Comments:

Sequences with cuts/fading

- BikeSparklers : only 1st and 3rd cuts are kept ([023-128] + [216-340])

- Sequences with cuts are coded with constant qp

  - Bitrate for each cut is provided and has to be matched by proposals

  - Objective metrics are computed for each cut

  - Viewing done on the complete re-concatenated sequences

# Anchors coding Conditions

## Test configuration

Random Access (RA) coding constraint conditions shall be used, with the IntraPeriod in line with the intra period defined in the JCT-VC Common Test Conditions.

If the JCT-VC CTCs use a GOP size of 8, IntraPeriod in the HM configuration files is set to 16 for 20 fps, 24 for 24 fps, 24 for 25 fps, 32 for 30 fps, 48 for 50fps, 64 for 60fps, and 100 for 100fps sequences.

If the JCT-VC CTCs use a GOP size of 16, IntraPeriod in the HM configuration files is set to 16 for 20 fps, 32 for 24fps, 32 for 25fps, 32 for 30fps, 48 for 50fps, 64 for 60fps, and 96 for 100fps sequences.

For sequences S10 and S11, even if the frame rate of the source content is 100, the intra refresh period must be set to 48, and the sequences are displayed at 50fps.

## Bitrates

NCL YCbCr anchors are generated per the process described in section 4.1.2. The bitstreams are generated based on the HEVC Main 10 Profile, using BT.2020 colour primaries and NCL YCbCr colour space conversion, the ST 2084 Transfer Function, fixed QP values. The corresponding overall rates are in the Excel files provided by the AHG on HEVC HM and HDRTools software development and software technical evaluation.

ICTCP anchors are generated per the process described in section 5.1.2. The bitstreams are generated based on the HEVC Main 10 Profile, using BT.2020 colour primaries and ICTCP colour space conversion, the ST 2084 Transfer Function, fixed QP values.

Bitstreams provided by proponents must not exceed the exact bit rates achieved by the anchors. For sequences with scene cuts (S04, S06, S07), the sequence is first split in separate sub-sequences. For a given rate Ri (i=1..4), each sub-sequence is coded with the same QP, and the bitrate for each sub-sequence is considered. Proponents must perform independent encoding of each sub-sequence with bitrate targets given in the Excel files provided by the AHG on HEVC HM and HDRTools software development and software technical evaluation.

## Test Procedures

HDR anchors are compressed using HEVC Main 10 Profile. The HM software shall be used to generate the bit streams. Figure 1 shows the end-to-end coding and decoding chain used in generating HDR YCbCr 4:2:0 anchors and evaluating visual quality. Figure 2 shows the end-to-end coding and decoding chain used in generating HDR ICTCP 4:2:0 anchors and evaluating visual quality.

Figure 1. End to end coding and decoding chain for YCbCr 4:2:0 video.

A

B

E

F’

A’

B’

E’

C

C’

D

D’

Input

HDR

video

Coding

TF

L’M’S’

to

ICTCP

Quant

10

b

4

:

4

:

4

to

4

:

2

:

0

Encoding HM

4

:

2

:

0 10

bits

bitstream

bitstream

Decoding HM

4

:

2

:

0 10

bits

4

:

2

:

0

to

4

:

4

:

4

Inv

.

Quant

10

b

ICTCP

to

L’M’S’

Inv

.

Coding

TF

Output

HDR

video

RGB

to

LMS

LMS

to

RGB

Figure 2. End to end coding and decoding chain for ICTCP 4:2:0 video.

Proponents are required to submit bit streams at bit rates that do not exceed those indicated in the Excel files provided by the AHG on HEVC HM and HDRTools software development and software technical evaluation.

Objective visual quality evaluations

Comparisons will be made using objective measurement(s) from the decoded video provided by all proponents. The objective measurements will be made between the points EE′ at various bit rates.

For sequences with cuts, as mentioned in section 3, the encoding is performed on each cut (sub-sequence). The viewing is done with the re-concatenated of the sub-sequences.

Subjective visual quality evaluation

Formal viewing tests (at point F′ in Figure 1 and Figure 2) will be performed when necessary.

# Anchor generation process for YCbCr 420 video

## Software versions

### Conversion software HDRConvert

The different conversions required in the generation of anchors are done using the HDRConvert tool from the tagged version of HDRTools software package recommended in the JCT-VC Common Test Conditions.

This tool is accessible at the following location using login/password credentials sent to JCT-VC participants (qualified members of the JCT-VC may contact Gary Sullivan or Jens-Rainer Ohm to obtain access):

https://gitlab.com/standards/HDRTools/

### HDR anchor coding process

The HDR anchors are based on the HEVC Main 10 profile, which implies encodings using 4:2:0 subsampling and 10 bits per channel. The YCbCr colour space is used.

The encoding is performed with the tagged version of HM recommended in the JCT-VC Common Test Conditions.

The HM must be configured in 4:2:0 with the following settings:

- The macro RExt__HIGH_BIT_DEPTH_SUPPORT is set to 1, which results in

  - FULL_NBIT set to 1

  - RExt__HIGH_PRECISION_FORWARD_TRANSFORM set to 1

- Using Random Access (RA) configuration from HEVC common test conditions.

The following example configuration files are in the “cfg/” folder of the HM reference software package:

- encoder_randomaccess_main10_HDR_YCbCr_classAGH.cfg and

- encoder_randomaccess_main10_HDR_YCbCr_classBCD.cfg

## Conversion and inverse conversion processes

### RGB linear-light format with BT.709 or BT.2020 primaries for HDR anchor

If the input is in a half float 4:4:4 RGB linear-light format (e.g. OpenEXR), the bit streams shall be generated using the coding / decoding chain illustrated in Figure 3.

The conversion to 4:2:0 10 bits YCbCr is obtained with the HDRConvert tool using the example configuration file HDRConvertEXR2020ToYCbCr420.cfg (for the BT.2020 content container primaries) provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

This conversion consists of the following steps:

- Convert half precision floating point data to single precision floating point data (not illustrated).

- Map using the PQ transfer function (PQ-TF) from RGB (float) to R′G′B′ (float) by invoking Section 4.4.1.

- Convert from R′G′B′ (float) to YCbCr by invoking section 4.4.2 if the sequence is in BT.709 container, or section 4.4.3 if the sequence is in BT.2020 container.

- Quantize from YCbCr (float) into DY′DCbDCr (10bit) by invoking Section 4.5, with BitDepthY and BitDepthC set to 10.

- Downsample both chroma components from 4:4:4 DY′DCbDCr (10bit) to 4:2:0 DY′DCbDCr (10bit) by invoking Section 4.6.

- Perform luma adjustment using RGB and the subsampled DCbDCr signals as inputs to get the DY′ signal resulting in 4:2:0 DY′DCbDCr (10bit) by invoking Section 4.10.

Note: 	Dxx stands for Digitized version of signal xx. R′G′B′ or YCbCr stands for normalized value within [0,1] (R′G′B′ Y′) and [-0.5, 0.5] (CbCr).

The reverse conversion is obtained with the HDRConvert tool using the configuration file HDRConvertYCbCr420ToEXR2020.cfg (for the BT.2020 content container primaries) provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

The process consists of the following steps:

- Upsample both chroma components from 4:2:0 DY′DCbDCr (10bit) to 4:4:4 DY′DCbDCr (10bit) by invoking Section 4.7.

- Inverse quantize from DY′DCbDCr (10bit) into YCbCr (float) by invoking section 4.8, with BitDepthY and BitDepthC set to 10.

- Convert from Y′CbCr (float) to RGB (float) by invoking section 4.9.1 if the sequence is in a BT.709 container, or section 4.9.2 if the sequence is in a BT.2020 container.

- Inverse map using the inverse PQ-TF from RGB (float) to RGB (float) by invoking Section 4.9.3.

- Convert, if needed, the data from single precision floating point numbers to half precision floating point numbers using appropriate rounding operations (not illustrated).

Figure 3. Simplified encoding / decoding chains when input HDR video is RGB linear light.

### Input 4:4:4 DRDGDB PQ-TF 12bit for HDR Anchor

If the input is using a 4:4:4 PQ-TF 12bit DR′DG′DB′ format, the bit streams shall be generated using the coding / decoding chain illustrated in Figure 4.

#### Input 4:4:4 DRDGDB PQ-TF 12bit in BT.2020 container

The conversion to 4:2:0 10 bits Y′CbCr is obtained with the HDRConvert tool using the configuration file HDRConvertBT2020TiffToYCbCr420.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

The process consists of the following steps:

- Convert from 12bit DR′DG′DB′ into normalized RGB by invoking section 4.12.1.

- Convert from RGB (BT.2020) to linear RGB by applying the inverse transfer function (inversePQ-TF) as described in Section 4.9.3. The linear RGB will be used in the last step (luma adjustment).

- Convert from RGB (BT.2020) to Y′CbCr by invoking section 4.4.3.

- Quantize from YCbCr (float) into DY′DCbDCr (10bit) by invoking Section 4.5, with BitDepthY and BitDepthC set to 10.

- Downsample both chroma components from 4:4:4 DY′DCbDCr (10bit) to 4:2:0 DY′DCbDCr (10bit) by invoking Section 4.6.

- Perform luma adjustment using the linear RGB obtained in the second step and the subsampled DCbDCr signals obtained in the previous step, as described in Section 4.10. The output is a DY′ signal resulting in 4:2:0 DY′DCbDCr (10bit).

The reverse conversion is obtained by applying the following steps and the output is DR′DG′DB′ PQ-TF 12b in BT.2020 container. It can be obtained with the HDRConvert tool using the configuration file HDRConvertYCbCr420ToBT2020Tiff.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

- Upsample both chroma components from 4:2:0 DY′DCbDCr (10bit) to 4:4:4 DY′DCbDCr (10bit) by invoking Section 4.7.

- Inverse quantize from DY′DCbDCr (10bit) into YCbCr (float) by invoking section 4.8, with BitDepthY and BitDepthC set to 10.

- Convert from Y′CbCr (float) to RGB (float) (BT.2020) by invoking section 4.9.2.

- Quantize normalized R′G′B′ into 12bit DR′DG′DB′ in BT.2020 container by invoking section 4.12.2.

Figure 4. Simplified encoding / decoding chains when the input HDR video is in 4:4:4 R′G′B′ PQ-TF 12bit.

#### Input 4:4:4 DRDGDB PQ-TF 12bit in P3D65 container

The conversion to 4:2:0 10 bits Y′CbCr is obtained with the HDRConvert tool using the configuration file HDRConvertP3D65TiffToYCbCr420.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package. It consists of the following steps:

- Convert from PQ-TF P3D65 12bit DR′DG′DB′ into normalized R′G′B′ by invoking section 4.12.1.

- Convert RGB PQ-TF P3D65 to RGB P3D65 by invoking section 4.9.3.

- Convert from RGB P3D65 to RGB BT.2020 by invoking section 4.11.4.

- Convert from RGB BT.2020 to R′G′B′ PQ-TF BT.2020 by invoking section 4.4.1.

- Convert from RGB (BT.2020) to Y′CbCr by invoking section 4.4.3.

- Quantize from Y′CbCr (float) into DY′DCbDCr (10bit) by invoking Section 4.5, with BitDepthY and BitDepthC set to 10.

- Downsample both chroma components from 4:4:4 DY′DCbDCr (10bit) to 4:2:0 DY′DCbDCr (10bit) by invoking Section 4.6.

- Perform luma adjustment using the linear RGB P3D65 obtained in the second step and the subsampled DCbDCr signals obtained in the previous step, as described in Section 4.10. The output is a DY′ signal resulting in 4:2:0 DY′DCbDCr (10bit).

## Colour space conversion from RGB to XYZ

### RGB with BT.709 primaries to XYZ

  - X = 0.412391 * R + 0.357584 * G + 0.180481 * B

  - Y = 0.212639 * R + 0.715169 * G + 0.072192 * B

  - Z = 0.019331 * R + 0.119195 * G + 0.950532 * B

### RGB with BT.2020 primaries to XYZ

  - X = 0.636958 * R + 0.144617 * G + 0.168881 * B

  - Y = 0.262700 * R + 0.677998 * G + 0.059302 * B

  - Z = 0.000000 * R + 0.028073 * G + 1.060985 * B

### RGB with P3D65 primaries to XYZ

  - X = 0.486571 * R + 0.265668 * G + 0.198217 * B

  - Y = 0.228975 * R + 0.691739 * G + 0.079287 * B

  - Z = 0.000000 * R + 0.045113 * G + 1.043944 * B

## Colour transformation from RGB to YCbCr

### Conversion from RGB to RGB

  - R = PQ_TF(max(0, min(R/10000,1)) )

  - G = PQ_TF(max(0, min(G/10000,1)) )

  - B = PQ_TF(max(0, min(B/10000,1)) )

with		PQ_TF(L)=c1+c2Lm11+c3Lm1m2

m1=26104096×14=0.1593017578125

m2=25234096×128=78.84375

c1=c3-c2+1=34244096=0.8359375

c2=24134096×32=18.8515625

c3=23924096×32=18.6875

### RGB with BT.709 primaries to Y′CbCr

The ITU-R BT.709 standard specifies the following conversion process from RGB to Y′CbCr (non-constant luminance representation):

  - Y = 0.2126 * R′ + 0.7152 * G′ + 0.0722 * B′

  - Cb=B'-Y'1.8556

  - Cr= R'-Y'1.5748

The above can also be implemented using the following approximate conversion that avoids the division for the Cb and Cr components:

  - Y′ =  0.212600 * R + 0.715200 * G + 0.072200 * B

  - Cb = −0.114572 * R − 0.385428 * G + 0.500000 * B

  - Cr =  0.500000 * R − 0.454153 * G − 0.045847 * B

The above method, without division, is the method recommended to use for participation in this activity and for the generation of the current test anchors.

### RGB with BT.2020 primaries to YCbCr

The ITU-R BT.2020 standard specifies the following conversion process from RGB to YCbCr (non-constant luminance representation):

  - Y′ = 0.2627 * R + 0.6780 * G + 0.0593 * B

  - Cb=B'-Y'1.8814

  - Cr= R'-Y'1.4746

The above can also be implemented using the following approximate conversion that avoids the division for the Cb and Cr components:

  - Y′ =  0.262700 * R + 0.678000 * G + 0.059300 * B

  - Cb = −0.139630 * R − 0.360370 * G + 0.500000 * B

  - Cr =  0.500000 * R − 0.459786 * G − 0.040214 * B

The above method, without division, is the method recommended to use for participation in this activity and for the generation of the current test anchors.

## Quantization from YCbCr into DY′DCbDCr

This process quantizes the input Y′CbCr signal into a signal of bit-depth BitDepthY for the Y component and BitDepthC for the chroma components (Cb, Cr).

  - DY'=Clip1YRound1≪ BitDepthY-8* 219*Y'+16

  - DCb=Clip1CRound1≪ BitDepthC-8* 224*Cb+128

  - DCr=Clip1CRound1≪ BitDepthC-8* 224*Cr+128

with

Round( x ) = Sign( x ) * Floor( Abs( x ) + 0.5 )

Sign ( x ) = -1 if x < 0, 0 if x=0, 1 if x > 0

Floor( x )	the largest integer less than or equal to x

Abs( x ) = x if x>=0, -x if x<0

Clip1Y( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, x )

Clip1C( x ) = Clip3( 0, ( 1  <<  BitDepthC ) − 1, x )

Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

## Chroma downsampling from 4:4:4 to 4:2:0

The chroma samples alignment is as follows:

| Phase k | Coefs c1[k] 4:4:4 4:2:2 (phase=0) | Coefs c2[k] 4:2:2 4:2:0 (phase=0) |
| --- | --- | --- |
| −1 | 1 | 1 |
| 0 | 6 | 6 |
| 1 | 1 | 1 |

- Define shift = 6 and offset = 32.

- Let H and W be the input picture height and width in chroma samples. For i = 0..H−1, j = 0..W/2−1, the intermediate samples f[ i ][ j ] are derived from the input samples s[ i ][ j ] as follows:

fi[j]=k=-11c1k* siClip3(0,W-1,2*j+k)

with	Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

- For i = 0..H/2-1, j = 0..W/2-1, the output samples r[ i ][ j ] are derived from the intermediate samples f[ i ][ j ] as follows:

ri[j]=(k=-11c2k* fClip3⁡(0, H-1, 2*i+k) j+offset)≫shift

Comment: new filters could be used following first anchors generation steps

## Chroma upsampling from 4:2:0 to 4:4:4 (YCbCr domain)

The upsampling filter used is the same for both horizontal and vertical processes. First, vertical filtering is applied on the 4:2:0 picture, then horizontal filtering.

Filter coefficients values are as follows

| Phase | −2 | −1 | 0 | 1 |
| --- | --- | --- | --- | --- |
| Coef c[k] | −4 | 36 | 36 | −4 |

Define shift1 = 6, offset1 = 32, shift2 = 12, offset2 = 2048.

Let H and W be the input picture height and width in chroma samples. For i = 0..H-1, j = 0..W-1, the intermediate samples f[ i ][ j ] are derived from the input samples s[ i ][ j ] as follows:

f2*ij=sij*64

f2*i+1[j]=k=-21ck* sClip3(0, H-1,i+k+1)j

For i = 0..2*H-1, j = 0..W-1, the output samples r[ i ][ j ] are derived from the intermediate samples f[ i ][ j ] as follows:

ri2*j= fij+offset1 ≫shift1

ri2*j+1=k=-21ck* fiClip3(0,W-1,j+k+1)+offset2≫shift2

Comment: new filters could be used following first anchors generation steps

## Inverse Quantization from DY′ DCbDCr into YCbCr

This process dequantizes the input signal represented on BitDepthY bits for the Y component and BitDepthC bits for the chroma components (Cb, Cr) into a (float) signal Y′CbCr.

  - Y'=ClipY'(D'Y1 ≪BitDepthY-8-16)/219

  - Cb=ClipC(D'Cb1 ≪BitDepthC-8-128)/224

  - Cr=ClipC(D'Cr1 ≪BitDepthC-8-128)/224

with

ClipY′ (x) = Clip3 ( 0, 1.0, x)

ClipC (x) = Clip3 ( -0.5, 0.5, x)

Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

## Colour transformation from YCbCr to RGB

### YCbCr to RGB with BT.709 primaries

  - R = clipRGB(Y + 1.57480 * Cr)

  - G = clipRGB(Y – 0.18733 * Cb – 0.46813 * Cr)

  - B = clipRGB(Y + 1.85563 * Cb)

### YCbCr to RGB with BT.2020 primaries

  - R = clipRGB(Y + 1.47460 * Cr)

  - G = clipRGB(Y – 0.16455 * Cb – 0.57135 * Cr)

  - B = clipRGB(Y + 1.88140 * Cb)

with clipRGB( x ) = Clip3( 0, 1, x )

Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

### Conversion from RGB to RGB

  - R = 10000*inversePQ_TF(R)

  - G = 10000*inversePQ_TF(G)

  - B = 10000*inversePQ_TF(B)

with inversePQ_TF(N)=maxN1m2-c1,0c2-c3N1m21m1

m1=26104096×14=0.1593017578125
m2=25234096×128=78.84375
c1=c3-c2+1=34244096=0.8359375
c2=24134096×32=18.8515625
c3=23924096×32=18.6875

## Luma Adjustment

This section describes the process of calculating the luma code word DY′. The Y′ obtained in Section 4.4 (and hence the quantized DY' value obtained from it) can generate pixels for which the luminance deviates from the original pixel, giving rise to strong artifacts. The luma adjustment process selects a luma code word DY′ that makes the decoded pixel have a more correct luminance. Luminance is defined as the Y-component of the CIE1931 XYZ colour that is obtained by converting the original linear RGB value as described in section 4.3.

First the desired, or original, luminance Yo is calculated from the linear RGB values using Section 4.3:

Yo=wRR+wGG+wBB,

where wR, wG and wB depend on the input container. For instance, if the input container is Rec. BT.709, the values are wR= 0.212639, wG =0.715169 and wB=0.072192 in accordance with the middle equation of Section 4.3.1.

Second, the upsampled versions of Cb and Cr are obtained for each pixel. This is done by first upsampling the subsampled DCbDCr signals according to Section 4.7 followed by inverse quantization according to Section 4.8. Note that since these values have been downsampled and then upsampled, they differ from the ones obtained in Section 4.4.1, and hence hats are used to denote them according to Cb and Cr.

Third, interval halving is used to find the DY' value that results in a luminance value Y that best matches the original luminance value Yo. To that end, a starting interval [64, 940] is associated with each pixel. A candidate code word in the middle of this interval is then tried: DY'cand=64+940≫1 = 502. A floating point candidate Y'cand is obtained by using the first equation of Section 4.8:

Y'cand=ClipY'(DY'cand1 ≪BitDepthY-8-16)/219

This Y'cand is now used to calculate the R′, G′ and B′ values by applying Section 4.9.2, which is repeated here for the convenience of the reader:

  - R'cand = clipRGB(Y'cand + 1.47460 * Cr)

  - G'cand = clipRGB(Y'cand – 0.16455 * Cb – 0.57135 * Cr)

  - B'cand = clipRGB(Y'cand + 1.88140 * Cb)

These values are then converted to linear RGB by invoking Section 4.9.3, and then the candidate luminance value is calculated as

Ycand=0.262700 Rcand+0.677998 Gcand+0.059302 Bcand.

Now, if the candidate luminance value Ycand is too high, i.e., Ycand>Yo, the interval is updated to the lower half of the starting interval, i.e., [64, 502]. If on the other hand Ycand<Yo, the upper half [502, 940] is used. A new candidate DY'cand is now computed as the middle of the updated value, and the process continues until, after at most 10 steps, we end up with an interval [a, b] of size one, ie., b = a+1.

Finally, a selection between a and b is done by computing Ycand-a and Ycand-b according to the process above. DY'=a is selected if

PQ_TFYcand-a-PQ_TFYo2<PQ_TFYcand-b-PQ_TFYo2

and DY'=b is selected otherwise, where PQ_TF∙ is defined in Section 4.4.1

## Colour space container conversion

This section describes the process of converting RGB samples encoded in and limited by one particular colour space, e.g. ITU-R Recommendation BT.709, BT.2020, or P3D65, into another RGB colour space specified with different primaries. We are particularly interested in the conversion of RGB BT.709 as well as RGB P3D65 samples into RGB BT.2020 samples and vice versa.

### RGB conversion: from BT.2020 to BT.709

It is essential when displaying data on a display that can only operate with BT.709 primaries to appropriately convert them in that space before display. To do so, this can be done using either a two-step conversion process, from the current colour space of the data (i.e. ITU-R BT.2020) to XYZ, followed by a subsequent conversion to BT.709, or using a single step process. In particular, the two-step conversion process can be applied as follows:

- Conversion from R2020G2020B2020 to XYZ:

  - X = 0.636958 * R2020 + 0.144617 * G2020 + 0.168881 * B2020

  - Y = 0.262700 * R2020 + 0.677998 * G2020 + 0.059302 * B2020

  - Z = 0.000000 * R2020 + 0.028073 * G2020 + 1.060985 * B2020

- Conversion from XYZ to R709G709B709 (BT.709):

  - R709 = clipRGB( 3.240970 * X - 1.537383 * Y - 0.498611 * Z )

  - G709 = clipRGB( -0.969244 * X + 1.875968 * Y + 0.041555 * Z )

  - B709 = clipRGB( 0.055630 * X - 0.203977 * Y + 1.056972 * Z )

The above could be converted into a single step by combining the two matrix conversions above into a single matrix. This would result in the following, high precision matrix conversion:

  - R709 = clipRGB(	1.660490254890140 * R2020

− 0.587638564717282 * G2020 - 0.072851975229213 * B2020 )

  - G709 = clipRGB( −0.124550248621850 * R2020

+ 1.132898753013895 * G2020 − 0.008347895599309 * B2020 )

  - B709 = clipRGB(	−0.018151059958635 * R2020

− 0.100578696221493 * G2020 + 1.118729865913540 * B2020 )

We currently would recommend the single step approach, as above, for the conversion of RGB BT.2020 material to RGB BT.709.

### RGB conversion: from P3D65 to BT.709

This conversion can be done using either a two-step conversion process, from the current colour space of the data (i.e. P3D65) to XYZ, followed by a subsequent conversion to BT.709, or using a single step process. In particular, the two-step conversion process can be applied as follows:

- Conversion from RP3GP3BP3 (P3D65) to XYZ:

  - X = 0.486571 * RP3 + 0.265668 * GP3 + 0.198217 * BP3

  - Y = 0.228975 * RP3 + 0.691739 * GP3 + 0.079287 * BP3

  - Z = 0.000000 * RP3 + 0.045113 * GP3 + 1.043944 * BP3

- Conversion from XYZ to R709G709B709 (BT.709):

  - R709 = clipRGB( 3.240970 * X − 1.537383 * Y − 0.498611 * Z )

  - G709 = clipRGB( −0.969244 * X + 1.875968 * Y + 0.041555 * Z )

  - B709 = clipRGB( 0.055630 * X − 0.203977 * Y + 1.056972 * Z )

The above could be converted into a single step by combining the two matrix conversions above into a single matrix. This would result in the following, high precision matrix conversion:

  - R709 = clipRGB(	1.224939741445000 * RP3

- 0.224939599120000 * GP3 − 0.000001097215000 * BP3 )

  - G709 = clipRGB(	−0.042056249524000 * RP3

+ 1.042057784075000 * GP3 − 0.000000329788000 * BP3 )

  - B709 = clipRGB(	−0.019637688845000 * RP3

− 0.078636557327000 * GP3 + 1.098273664879000 * BP3 )

We currently would recommend the single step approach, as above, for the conversion of RGB P3D65 material to RGB BT.709.

### RGB conversion: from BT.709 to BT.2020

The process to convert RGB BT.709 samples to RGB BT.2020 samples is very similar to the process performed earlier for the inverse conversion. A two-step conversion involves first applying the RGB BT.709 to XYZ conversion process, followed by a conversion from XYZ to RGB BT.2020 using the appropriate conversion matrices. In particular, this is done as follows:

- Conversion from R709G709B709 (BT.709) to XYZ

  - X = 0.412391 * R709 + 0.357584 * G709 + 0.180481 * B709

  - Y = 0.212639 * R709 + 0.715169 * G709 + 0.072192 * B709

  - Z = 0.019331 * R709 + 0.119195 * G709 + 0.950532 * B709

- Conversion from XYZ to R2020G2020B2020 (BT.2020)

  - R2020 = clipRGB( 1.716651 * X – 0.355671 * Y - 0.253366 * Z )

  - G2020 = clipRGB( -0.666684 * X + 1.616481 * Y + 0.015768 * Z )

  - B2020 = clipRGB( 0.017640 * X - 0.042771 * Y + 0.942103 * Z )

Similarly, the single step and recommended method is as follows:

  - R2020 = clipRGB( 0.627404078626 * R709 + 0.329282097415 * G709 + 0.043313797587 * B709 )

  - G2020 = clipRGB( 0.069097233123 * R709 + 0.919541035593 * G709 + 0.011361189924 * B709 )

  - B2020 = clipRGB( 0.016391587664 * R709 + 0.088013255546 * G709 + 0.895595009604 * B709 )

### RGB conversion: from P3D65 to BT.2020

The conversion process to convert RGB P3D65 samples to RGB BT.2020 is very similar to the one described in the previous section. Again, this could be done using either a two-step or a single-step method. The two-step method is as follows:

- Conversion from RP3GP3BP3 (P3D65) to XYZ:

  - X = 0.486571 * RP3 + 0.265668 * GP3 + 0.198217 * BP3

  - Y = 0.228975 * RP3 + 0.691739 * GP3 + 0.079287 * BP3

  - Z = 0.000000 * RP3 + 0.045113 * GP3 + 1.043944 * BP3

- Conversion from XYZ to R2020G2020B2020 (BT.2020):

  - R2020 = clipRGB( 1.716651 * X − 0.355671 * Y − 0.253366 * Z )

  - G2020 = clipRGB( −0.666684 * X + 1.616481 * Y + 0.015768 * Z )

  - B2020 = clipRGB( 0.017640 * X − 0.042771 * Y + 0.942103 * Z )

Similarly, the single step and recommended method is as follows:

  - R2020 = clipRGB( 0.753832826496 * RP3 + 0.198597635641 * GP3 + 0.047569409186 * BP3 )

  - G2020 = clipRGB( 0.045744636411 * RP3 + 0.941777687331 * GP3 + 0.012478735611 * BP3 )

  - B2020 = clipRGB( -0.001210377285 * RP3 + 0.017601107390 * GP3 + 0.983608137835 * BP3 )

## Conversion of HDR TIFF Input files

HDR files provided in TIFF format use a 12 bit non-linearly quantized (using PQ) RGB signal representation. The data are provided using the SDI data range (code values from 16 up to 4076) and may use either the BT.2020 or P3D65 RGB colour space. Converting back and forth from non-linear quantized data to non-linear normalized data is specified in the following sections:

Note: The 12 bit R′G′B′ data is contained in 16 bit tiff container and the 12 most significant bits are used.

### Inverse Quantization from 12bit DR′DG′DB′ into normalized PQ R′G′B′

The inverse quantization from 12bit DR′DG′DB′ into normalized R′G′B′ for PQ is achieved as follows:

  - C′ = (DC′ – range_low) / (range_high - range_low) where C′ = R′, G′, B′

with range_low = 16 and range_high = 4076.

### Quantization from normalized RGB into 12bit PQ DRDGDB

The quantization from normalized RGB to 12bit DRDGDB for PQ is achieved as follows:

  - DC = Round((C + range_low) * (range_high - range_low)) where C = R, G, B

with range_low = 16 and range_high = 4076

## HM encoding

The HM encoding is performed with two changes compared to HM16.12:

- A specific chroma qp offset process is applied to better control the chroma shifting

- A specific control of the luma delta QP is applied to better balance the bitrate between dark and bright areas.

These two changes are described in the following sub-sections.

### Chroma QP offset

In order to get a good balance between bits spent on luma and chroma it may be needed to adjust the QP offsets for Cb and Cr, depending upon the bit rate, and the native colour space. A model is used that assigns Cb and Cr QP offsets based on the luma QP and a factor based on the capture colour space and the representation colour space. The model is expressed as

QPoffset_Cb = max(−12, round(c_cb*(k*QP+l)))

QPoffset_Cr = max(−12, round(c_cr*(k*QP+l))),

where c_cb = 1 if capture colour space is same as representation colour space, c_cb=1.04 if capture colour space is P3D65 and representation colour space is BT 2020, c_cb=1.14 if capture colour space is BT.709 and representation space is BT 2020.

Furthermore, c_cr = 1 if capture colour space is same as representation colour space, c_cr=1.39 if capture colour space is P3D65 and representation colour space is BT 2020, c_cr=1.78 if capture colour space is BT.709 and representation space is BT 2020.

Finally, k = −0.46 and l = 9.26.

#### Modifications of HM encoder

HM-16.12 has been modified to include several new configuration parameters to cover k, l, c_cb and c_cr in the above equations, see Table 2. Based on the luma QP, a Cb QP offset and a Cr QP offset are determined and used to set the PPS Cb QP and Cr QP offsets.

These parameters are used to calculate the two chroma QP offsets which are then signalled in the PPS. HM is also updated to signal multiple PPS units within each coded sequence. This feature is used to signal new chroma QP offsets when the base luma QP changes in order to match bit rate in a fixed QP setting. When a QP change according to the floating point QP occurs for a slice a new PPS is encoded with chroma QP offsets set according to the new base luma QP and referred by the slice. A control parameter WCGPPSEnable is used to control the on/off of these settings and shall be set to 1.

Table 2. New configuration parameters for HM encoder.

| WCGPPSChromaQpScale | This floating point value corresponds to the k parameter in the model |
| --- | --- |
| WCGPPSChromaQpOffset | This floating point value corresponds to the l parameter in the model |
| WCGPPSCbQpScale | This floating point value corresponds to the c_cb parameter |
| WCGPPSCrQpScale | This floating point value corresponds to the c_cr parameter |

To remain compatible with current behavior the two existing parameters (CbQpOffset and CrQpOffset) for setting chroma QP offsets in HM are kept. They are added afterwards to the calculated offsets when the scaling has been done and allows for setting a stronger or weaker chroma qp offset than that specified by the model.

### Average Luma Controlled Adaptive dQP

The combination of the ST 2084 transfer function and the BT.2020 colour space allocates relatively more bits to the darker areas than what is the case for SDR compression (BT.709 transfer function / BT.2020 colour space). In order to get a better balance between dark and bright areas in the image, the QP value is changed according to the average luma value in each 64x64 CTU pixel block.

#### Implementation

For every 64x64 CTU, the average luma value, Laverage, is calculated. An integer version intL of this is then calculated using intL = floor(Laverage + 0.5). The QP to use for a particular 64x64 CTU block is then obtained by adding the picture QP with the dQP obtained by using the look-up table shown in Table 3. The source code can be found in this contribution as a patch to HM 16.12 (svn revision 4690) marked with the SHARP_LUMA_DELTA_QP defines. This luma adaptive dQP method is enabled by the following parameter in config file.

LumaLevelToDeltaQPMode: 1           # Change luma delta QP based on average luma

Table 3. Look-up table of the dQP value from the average of the luma value. As an example, an average luma value of 503 in a 64x64 block would result in a dQP value of −1.

| luma intL range | dQP |
| --- | --- |
| intL < 301 | 3 |
| 301 ≤ intL < 367 | 2 |
| 367 ≤ intL < 434 | 1 |
| 434 ≤ intL < 501 | 0 |
| 501 ≤ intL < 567 | −1 |
| 567 ≤ intL < 634 | −2 |
| 634 <= intL < 701 | −3 |
| 701 <= intL < 767 | −4 |
| 767 <= intL < 834 | −5 |
| intL >= 834 | −6 |

The data can also be drawn as a diagram, which is shown in Figure 5.

Figure 5. Change in QP value as a function of the average luma value in the 64x64 pixel block.

# Anchor generation process for ICTCP 420 video

## Software versions

### Conversion software HDRConvert

HDRTools software as specified in section 4.1.1 is used. The conversion process and configuration files for different input HDR content format are as described in section 5.2.

### HDR anchor coding process

The HDR anchors are based on the HEVC Main 10 profile, which implies encodings using 4:2:0 subsampling and 10 bits per channel. The ICTCP colour space is used.

The encoding is performed with HM16.12.

The HM must be configured in 4:2:0 with the following settings:

- The macro RExt__HIGH_BIT_DEPTH_SUPPORT is set to 1, which results in

  - FULL_NBIT set to 1

  - RExt__HIGH_PRECISION_FORWARD_TRANSFORM set to 1

- Using Random Access (RA) configuration from HEVC common test conditions.

The following example configuration files are provided in the “cfg/” folder of the HM reference software package:

- encoder_randomaccess_main10_HDR_ICtCp_classAGH.cfg and

- encoder_randomaccess_main10_HDR_ICtCp_classBCD.cfg

## Conversion and inverse conversion processes

### RGB linear-light format with BT.2020 primaries for HDR anchor

If the input is in a half float 4:4:4 RGB linear-light format (e.g. OpenEXR), the bit streams shall be generated using the coding / decoding chain illustrated in Figure 6.

The conversion to 4:2:0 10 bits ICTCP is obtained with the HDRConvert tool using the configuration file HDRConvertEXR2020ToICtCp420.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

This conversion consists of the following steps:

- Convert half precision floating point data to single precision floating point data (not illustrated).

- Convert from RGB (BT.2020) to LMS by invoking section 5.3.1.

- Map using the PQ transfer function (PQ-TF) from LMS (float) to L′M′S′ (float) by invoking section 5.3.3.

- Convert from L′M′S′ (float) to ICTCP by invoking section 5.3.4.

- Quantize from ICTCP (float) into DIDCtDCp (10bit) by invoking section 5.4, with BitDepthY and BitDepthC set to 10.

- Downsample both chroma components from 4:4:4 DIDCtDCp (10bit) to 4:2:0 DIDCtDCp (10bit) by invoking Section 5.5.

Note: 	Dxx stands for Digitized version of signal xx. L′M′S′ or ICTCP stands for normalized value within [0,1] (R′G′B′ I) and [−0.5, 0.5] (CTCP).

The reverse conversion is obtained with the HDRConvert tool using the configuration file HDRConvertICtCp420ToEXR2020.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

The process consists of the following steps:

- Upsample both chroma components from 4:2:0 DIDCtDCp (10bit) to 4:4:4 DIDCtDCp (10bit) by invoking section 5.6.

- Inverse quantize from DIDCtDCp (10bit) into ICTCP (float) by invoking section 5.7, with BitDepthY and BitDepthC set to 10.

- Convert from ICTCP (float) to L′M′S′ (float) by invoking section 5.8.1.

- Inverse map using the inverse PQ-TF from L′M′S′ (float) to LMS (float) by invoking Section 5.8.2.

- Convert from LMS (float) to RGB (float) by invoking section 5.8.3.

- Convert, if needed, the data from single precision floating point numbers to half precision floating point numbers using appropriate rounding operations (not illustrated).

Figure 6. Simplified encoding / decoding chains when input HDR video is RGB linear light.

### Input 4:4:4 DRDGDB PQ-TF 12bit for HDR Anchor

If the input is using a 4:4:4 PQ-TF 12bit DR′DG′DB′ format, the bit streams shall be generated using the coding / decoding chain illustrated in Figure 7.

#### Input 4:4:4 DRDGDB PQ-TF 12bit in BT.2020 container

The conversion to 4:2:0 10 bits ICTCP is obtained with the HDRConvert tool using the configuration file HDRConvertBT2020TiffToICtCp420.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

The process consists of the following steps:

- Convert from 12bit DR′DG′DB′ into normalized R′G′B′ by invoking section 4.12.1.

- Convert from RGB (BT.2020) to linear RGB by applying the inverse transfer function (inversePQ-TF) as described in Section 4.9.3.

- Convert RGB (BT.2020) to LMS by invoking section 5.3.1.

- Map using the PQ transfer function (PQ-TF) from LMS (float) to L′M′S′ (float) by invoking section 5.3.3.

- Convert from LMS (float) to ICTCP by invoking section 5.3.4.

- Quantize from ICTCP (float) into DIDCtDCp (10bit) by invoking section 5.4, with BitDepthY and BitDepthC set to 10.

- Downsample both chroma components from 4:4:4 DIDCtDCp (10bit) to 4:2:0 DIDCtDCp (10bit) by invoking Section 5.5.

The reverse conversion is obtained by applying the following steps and the output is DR′DG′DB′ PQ-TF 12b in BT.2020 container. It can be obtained with the HDRConvert tool using the configuration file HDRConvertICtCp420ToBT2020Tiff.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package.

- Upsample both chroma components from 4:2:0 DIDCtDCp (10bit) to 4:4:4 DIDCtDCp (10bit) by invoking section 5.6.

- Inverse quantize from DIDCtDCp (10bit) into ICTCP (float) by invoking section 5.7, with BitDepthY and BitDepthC set to 10.

- Convert from ICTCP (float) to L′M′S′ (float) by invoking section 5.8.1.

- Inverse map using the inverse PQ-TF from L′M′S′ (float) to LMS (float) by invoking Section 5.8.2.

- Convert from LMS (float) to RGB (float) by invoking section 5.8.3.

- Map using the PQ transfer function (PQ-TF) from RGB (float) to R′G′B′ (float) by invoking Section 4.4.1.

- Quantize normalized R′G′B′ into 12bit DR′DG′DB′ in BT.2020 container by invoking section 4.12.2.

Figure 7. Simplified encoding / decoding chains when the input HDR video is in 4:4:4 R′G′B′ PQ-TF 12bit.

#### Input 4:4:4 DRDGDB PQ-TF 12bit in P3D65 container

The conversion to 4:2:0 10 bits ICTCP is obtained with the HDRConvert tool using the configuration file HDRConvertP3D65TiffToICtCp420.cfg provided in the “bin/cfg_JCTVC_CTC/” folder of the HDRTools reference software package. It consists of the following steps:

- Convert from PQ-TF P3D65 12bit DR′DG′DB′ into normalized RGB by invoking section 4.12.1.

- Convert from RGB PQ-TF P3D65 to linear RGB P3D65 by applying the inverse transfer function (inversePQ-TF) as described in section 4.9.3.

- Convert RGB P3D65 to LMS by invoking section 5.3.2.

- Map using the PQ transfer function (PQ-TF) from LMS (float) to L′M′S′ (float) by invoking Section 5.3.3.

- Convert from L′M′S′ (float) to ICTCP by invoking section 5.3.4.

- Quantize from ICTCP (float) into DIDCtDCp (10bit) by invoking Section 5.4, with BitDepthY and BitDepthC set to 10.

- Downsample both chroma components from 4:4:4 DIDCtDCp (10bit) to 4:2:0 DIDCtDCp (10bit) by invoking Section 5.5.

## Colour transformation from RGB to ICTCP

### Conversion from BT.2020 RGB to LMS

  - L = (1688.0 * R2020 + 2146.0 * G2020 + 262.0 * B2020) / 4096.0

  - M = (683.0 * R2020 + 2951.0 * G2020 + 462.0 * B2020) / 4096.0

  - S = (99.0 * R2020 + 309.0 * G2020 + 3688.0 * B2020) / 4096.0

### Conversion from P3D65 RGB to LMS

  - L = 0.334550623220059 * RP3 + 0.576391094387108 * GP3 + 0.089058282392833 * BP3

  - M = 0.158520235234873 * RP3 + 0.713612932646569 * GP3 + 0.127866832118557 * BP3

  - S = 0.020581197388829 * RP3 + 0.091695662564561 * GP3 + 0.88772314004661 * BP3

### Conversion from LMS to LMS

  - L = PQ_TF(max(0, min(L/10000,1)) )

  - M = PQ_TF(max(0, min(M/10000,1)) )

  - S = PQ_TF(max(0, min(S/10000,1)) )

with		PQ_TF(L)=c1+c2Lm11+c3Lm1m2

m1=26104096×14=0.1593017578125

m2=25234096×128=78.84375

c1=c3-c2+1=34244096=0.8359375

c2=24134096×32=18.8515625

c3=23924096×32=18.6875

### Conversion from LMS to ICTCP

  - I = (2048.0 * L + 2048.0 * M) / 4096.0

  - CT = (6610.0 * L − 13613.0 * M + 7003.0 * S) / 4096.0

  - CP = (17933.0 * L − 17390.0 * M – 543.0 * S) / 4096.0

## Quantization from ICTCP into DIDCtDCp

This process quantizes the input ICTCP signal into a signal of bit-depth BitDepthY for the I component and BitDepthC for the chroma components (Ct, Cp).

  - DI=Clip1YRound1≪ BitDepthY-8* 219*I+16

  - DCt=Clip1CRound1≪ BitDepthC-8* 224*Ct+128

  - DCp=Clip1CRound1≪ BitDepthC-8* 224*Cp+128

with

Round( x ) = Sign( x ) * Floor( Abs( x ) + 0.5 )

Sign ( x ) = -1 if x < 0, 0 if x=0, 1 if x > 0

Floor( x )	the largest integer less than or equal to x

Abs( x ) = x if x>=0, -x if x<0

Clip1Y( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, x )

Clip1C( x ) = Clip3( 0, ( 1  <<  BitDepthC ) − 1, x )

Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

## Chroma downsampling from 4:4:4 to 4:2:0

The chroma samples alignment is as follows:

| Phase k | Coefs c1[k] 4:4:4 4:2:2 (phase=0) | Coefs c2[k] 4:2:2 4:2:0 (phase=0) |
| --- | --- | --- |
| −1 | 1 | 1 |
| 0 | 6 | 6 |
| 1 | 1 | 1 |

- Define shift = 6 and offset = 32.

- Let H and W be the input picture height and width in chroma samples. For i = 0..H-1, j = 0..W/2-1, the intermediate samples f[ i ][ j ] are derived from the input samples s[ i ][ j ] as follows:

fi[j]=k=-11c1k* siClip3(0,W-1,2*j+k)

with	Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

- For i = 0..H/2-1, j = 0..W/2-1, the output samples r[ i ][ j ] are derived from the intermediate samples f[ i ][ j ] as follows:

ri[j]=(k=-11c2k* fClip3⁡(0, H-1, 2*i+k) j+offset)≫shift

Comment: new filters could be used following first anchors generation steps

## Chroma upsampling from 4:2:0 to 4:4:4

The upsampling filter used is the same for both horizontal and vertical processes. First, vertical filtering is applied on the 4:2:0 picture, then horizontal filtering.

Filter coefficients values are as follows:

| Phase | −2 | −1 | 0 | 1 |
| --- | --- | --- | --- | --- |
| Coef c[k] | −4 | 36 | 36 | −4 |

Define shift1 = 6, offset1 = 32, shift2 = 12, offset2 = 2048.

Let H and W be the input picture height and width in chroma samples. For i = 0..H−1, j = 0..W−1, the intermediate samples f[ i ][ j ] are derived from the input samples s[ i ][ j ] as follows:

f2*ij=sij*64

f2*i+1[j]=k=-21ck* sClip3(0, H-1,i+k+1)j

For i = 0..2*H−1, j = 0..W−1, the output samples r[ i ][ j ] are derived from the intermediate samples f[ i ][ j ] as follows:

ri2*j= fij+offset1 ≫shift1

ri2*j+1=k=-21ck* fiClip3(0,W-1,j+k+1)+offset2≫shift2

Comment: new filters could be used following first anchors generation steps

## Inverse Quantization from DIDCtDCp into ICTCP

This process dequantizes the input signal represented on BitDepthY bits for the I component and BitDepthC bits for the chroma components (Ct, Cp) into a (float) signal ICTCP.

  - I=ClipY'(DI1 ≪BitDepthY-8-16)/219

  - Ct=ClipC(DCt1 ≪BitDepthC-8-128)/224

  - Cp=ClipC(DCp1 ≪BitDepthC-8-128)/224

with

ClipY(x) = Clip3( 0, 1.0, x)

ClipC(x) = Clip3( −0.5, 0.5, x)

Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

## Colour transformation from ICTCP to RGB

### ICTCP to LMS

- L = clipRGB(I + 0.008609037037933 * Ct + 0.111029625003026 * Cp)

- M = clipRGB(I − 0.008609037037933 * Ct − 0.111029625003026 * Cp)

- S = clipRGB(I + 0.560031335710679 * Ct − 0.320627174987319 * Cp)

### Conversion from LMS to LMS

  - L = 10000*inversePQ_TF(L)

  - M = 10000*inversePQ_TF(M)

  - S = 10000*inversePQ_TF(S)

with inversePQ_TF(N)=maxN1m2-c1,0c2-c3N1m21m1

m1=26104096×14=0.1593017578125
m2=25234096×128=78.84375
c1=c3-c2+1=34244096=0.8359375
c2=24134096×32=18.8515625
c3=23924096×32=18.6875

### LMS to BT.2020 RGB

  - R = clipRGB(3.436606694333079 * L − 2.506452118656270 * M + 0.069845424323191 * S)

  - G = clipRGB(−0.791329555598929 * L + 1.983600451792291 * M − 0.192270896193362 * S)

  - B = clipRGB(−0.025949899690593 * L − 0.098913714711726* M + 1.124863614402319* S)

with clipRGB( x ) = Clip3( 0, 1, x )

Clip3( x,y,z ) = x if z<x, y if z>y, z otherwise

## HM encoding

The same HM encoding method, as described in section 4.13 is used. Compared to Y′CbCr anchor, the configuration parameters for luma delta QP is kept the same. The configuration parameters for chroma qp offset is listed as in Table 4.

Table 4. Chroma QP offset parameters for ICTCP PQ signal

| Type | WCGPPSChromaQpScale | WCGPPSChromaQpOffset | WCGPPSCbQpScale | WCGPPSCrQpScale | CbQPOffset | CrQPOffset |
| --- | --- | --- | --- | --- | --- | --- |
| BT.709 | −0.46 | 10 | 0.9 | 1.6 | 6 | 6 |
| P3 | −0.46 | 10 | 0.5 | 0.9 | 8 | 7 |

Another change of configuration parameters in HM encoder is the MatrixCoefficients setting in the VUI, as shown in Table 5.

Table 5. VUI parameters in HM encoding

|  | ICTCP PQ |
| --- | --- |
| MatrixCoefficients | 14 |

Access and copyright conditions statements for test sequences

Access to test sequences

Access information to sequences from classes A, A′ and G, AA and GG can be obtained from Edouard Francois (edouard.francois@technicolor.com).

Access information to sequence from classes B and BB can be obtained from Walt Husak (WJH@dolby.com).

Access information to sequences from classes C and CC can be obtained from Jan Fröhlich (jan@fr.oehli.ch).

Sequences from classes D and DD are clips extracted from content in its complete version. Details to access this content can be found at http://www.dcimovies.com/2014_StEM_Access/. Special care has to be made about the copyright license. The proponents are invited to perform on their own clips extraction from the complete content.

Copyright conditions statements for test sequences

Classes A sequences:

This video content (“Content”) may only be used for the purpose of developing, testing and promulgating technology standards developed by the MPEG, VCEG or JCTVC standardization groups (“Purpose”), under Technicolor R&D France SNC (“Technicolor”) owned or controlled copyrights, by individual(s) or organization(s) that participates, contributes or is part of such standardization groups (“User(s)”).

RESTRICTIONS: No license whatsoever is implied except from this expressed limited personal, non-sublicensable, non-transferrable authorization. In particular no right under any patent of Technicolor or its affiliates is granted. Any other use is strictly prohibited. This Content cannot be distributed or otherwise disposed of or made available (via internet or otherwise), broadcasted, disclosed to third parties (except to other User(s) who agreed with the present terms and conditions), used for providing any services to third parties or for any commercial use, nor copied (except as technically necessary for the Purpose), modified, adapted, translated, exchanged, publicly performed (even for demonstration or educational purposes), integrated, without limitation. This Content and all intellectual property rights, titles and interests therein, are and remain the exclusive property of Technicolor or its affiliates.

NO WARRANTY: This Content is supplied "as is" and without any warranty of any kind, expressed or implied, including but not limited to any warranty for accuracy or performance, fitness for a general or particular purposes, or any warranty arising by statute or by law, or non-infringement of any third parties’ rights. In no event will Technicolor be liable for damages of any kind, including without limitation, any special, indirect, incidental, or consequential damages even if Technicolor has been advised of the possibility of such damages.

This authorization is effective as of the date of use of the Content and shall expire on January, 1, 2020. Technicolor may nevertheless terminate it at any time for any reason upon notice to User(s). It is governed by the laws of France without regard to conflict of laws. It shall be exclusively submitted to the first instance civil court of Paris, France (TGI) in the event of a dispute not settled amicably.

BY USING THIS CONTENT USER(S) AGREES AND ACCEPTS THE ABOVE MENTIONED TERMS AND CONDITIONS. IF USER(S) DOES NOT AGREE TO THESE TERMS AND CONDITIONS, IT SHALL NOT ACCESS OR OTHERWISE USE THIS CONTENT. DO NOT REMOVE OR MODIFY TECHNICOLOR OR ITS AFFILIATES’ NAMES, SIGNS, LOGOS, NOTICES OF/IN THE CONTENT.

Copyright © 2012-2014 – Technicolor R&D France, SNC

All Rights Reserved

Classes B sequences:

The copyright conditions statement for this content is attached with this document (DOLBY LIMITED CLIP LICENSE AGREEMENT_112014.pdf).

Classes C sequences:

The copyright conditions statement for this content is attached with this document (Stuttgart_14_10_07_EULA_Unterschrift.pdf).

Classes D sequences:

The copyright conditions statement for this content can be found at:

http://www.dcimovies.com/2014_StEM_Access/.

Classes G sequences:

The copyright conditions statement for this content is attached with this document (CableLabs_Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International Public License.pdf).

Since standard range is used, where 0.0 is mapped to code word 64 and 1.0 is mapped to code word 940, the starting interval is [64, 940] rather than [0, 1023].