| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–24 Oct. 2017 | Document: JCTVC-AC0021 |
| --- | --- |

| Title: | Topics for coordination between OMAF and JCT-VC in Macau |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92122, USA | Tel: Email: | +1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2017/M41441

October 2017, Macau, China

| Source | Qualcomm Incorporated |
| --- | --- |
| Status | Input contribution |
| Title | Topics for coordination between OMAF and JCT-VC in Macau |
| Author | Ye-Kui Wang |

_____________________________

# Abstract

This contribution, submitted as both JCTVC-AC0021 and m41441, provides a list of topics that the author thinks should be coordinated between OMAF and JCT-VC, targeting at aligned designs between OMAF version 1 and the omnidirectional video related SEI messages that covering the same functionalities. For each topic, a suggestion was made.

# List of topics and suggestions

| Index | Topic | Suggestion |
| --- | --- | --- |
| 1 | OMAF clause 5.1 includes a detailed description of the global and local coordinate axes as well as the rotations for conversion between the two, using two figures (Figures 5-1 and Figure 5-2) with some discrepancy between them. In JCTVC-AB1005-v1, a simpler description without using a figure is included (in the semantics of both the equirectangular projection SEI message and the cubemap projection SEI message). | Due to that 1) there was some complaint about using a smiley face in standard specs being a bit creepy, and 2) there is some redundancy and discrepancy between the two figures in OMAF clause 5.1, it is suggested to remove the two figures from OMAF clause 5.1 and align the description therein with the description in JCTVC-AB1005-v1. |
| 2 | The equirectangular projection SEI message includes the signalling of rotation information as part of the SEI message. However, the cubemap projection SEI message does not include the signalling of rotation information. In OMAF, the rotation information is signalled in file format level using a separate structure than that for the projection, and that rotation information signalling applies to any projection type. | It is suggested to use a separate SEI message for signalling of rotation parameters such that the same syntax could be used for any type of projection. |
| 3 | The equirectangular projection SEI message includes signalling of the sphere coverage information as part of the SEI message. However, the cubemap projection SEI message does not include signalling of the sphere coverage information. In OMAF, the sphere coverage information is signalled in file format level using a separate structure than that for the projection, and that sphere coverage signalling applies to any projection type. | It is suggested to use a separate SEI message for signalling of sphere coverage information such that the same syntax could be used for any type of projection. See another aspect in item 4 below. |
| 4 | The equirectangular projection SEI message uses four parameters for signalling of the sphere coverage information: erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max. To allow signalling of coverage sphere region that spans across the left and right boundaries of the projected picture, the values of erp_aizmuth_min and erp_aizmuth_max can be outside the range of −180 to 180 degrees. This was previously also for support of ERP padding. However, the padded ERP can now be covered by repeated/overlapping projected regions using the region-wise packing syntax. In OMAF and in the omnidirectional viewport SEI message, a sphere region may be a tilted sphere region thanks to the signalling of the tilt angle of the sphere region, which is missing in the current coverage sphere region in JCTVC-AB1005-v1. | It is suggested to signal the sphere coverage information by specifying the center point of the coverage sphere region, the azimuth and elevation ranges, and the tilt angle of the sphere region. This would make the signalling aligned with sphere region signalling in the omnidirectional viewport SEI message as well as sphere coverage signalling in OMAF. This way, the azimuth and elevation ranges can also be specified to be in the range of −180 to 180 degrees and −90 to 90 degrees, respectively. |
| 5 | The semantics of the equirectangular projection SEI message and the cubemap projection SEI message allow the use of omnidirectional projection with frame packing types 3, 4, and 5, while in OMAF omnidirectional projection can be used only with frame packing types 3 and 4. | Because only frame packing types 3 and 4 are widely used and supported for omnidirectional video, it is suggested to disallow the use of omnidirectional projection with frame packing type 5, for both the equirectangular projection and the cubemap projection. |
| 6 | On guard band signalling, OMAF relies on the signalling that is part of the region-wise packing syntax, while in JCTVC-AB1005-v1, the syntax of the region-wise packing SEI message does not include guard band signalling, and the cubemap projection SEI message includes a guard band syntax (although it is called padding) that is different than that in OMAF. Also, it should be noted that there some are issues in JCTVC-AB1005-v1 regarding padding or guard band signalling for the cubemap projection, as described below. The pictureWdith and pictureHeight (of the monoscopic projected luma picture) in the cubemap projection equations should not count any padded samples. Therefore, when the region-wise packing signalling is not present, in which case the size of the projected picture is not signalled, the size of the projected picture needs to be derived based on the cubemap projection SEI message syntax. The width should be set equal to the width of the cropped output picture minus the total number of columns of padded samples, and the height should be set equal to the height of the cropped output picture minus the total number of row of padded samples, and there needs to be a constraint to require that all the remaining samples of the cropped output picture by excluding all the padded samples shall exactly form a rectangle, which is the projected picture. When the region-wise packing signalling is present, there needs to be a constraint that no packed region shall contain any padded sample. Specifications for the above derivation of the size of the projected picture and the constraints are currently missing. The above issues can be resolved by adding guard band padding signalling into region-wise packing syntax, same as in OMAF, and relying on that for providing support of guard band padding, (i.e., removing the padding signalling from the cubemap projection SEI message syntax). | It is suggested to adding guard band padding signalling into the syntax of the region-wise packing SEI message, same as in OMAF, and relying on that for providing support of guard band padding (i.e., and removing-e the padding signalling from the cubemap projection SEI message syntax). |
| 7 | In the semantics of the region-wise packing SEI message, currently (same as in the OMAF draft text before the Torino MPEG meeting in July 2017), the unit of the size of the projected picture and the size of projected and packed regions are either unspecified or specified as luma samples. In the latest OMAF draft text, these sizes are specified in relative units, to allow the use of the same region-wise packing syntax for multiple bitstreams representing the same source video content. For example, multiple bitstreams representing the same source video content may be generated for adaptive streaming purpose. | It is suggested to align the syntax and semantics of the region-wise packing SEI message with the region-wise packing syntax and semantics in the latest OMAF draft text, to signal packed picture sizes and the sizes of the projected and packed regions in relative units. |
| 8 | In JCTVC-AB1005-v1, recommended viewports, generated per director's cut or viewing frequency, may be signalled using the viewport omnidirectional viewport SEI message. In OMAF, the director's cut type of recommended viewport is supported, but not the type per viewing frequency. | It is suggested to align OMAF with the HEVC omnidirectional viewport SEI message by adding the recommended viewport type of most-viewed viewports by statistical measurements. |
| 9 | The overall sample location mapping processes are aligned between JCTVC-AB1005-v1 and OMAF. However, there is one significant discrepancy in the mapping equations for the equirectangular projection. In JCTVC-AB1005-v1, the mapping equations for the equirectangular projection involve the sphere coverage parameters, while this is not the case in OMAF. A study is needed on the impacts caused by this discrepancy. [Add more details after the study.] | It is suggested to align the mapping equations for the equirectangular projection between the two specifications. |
| 10 | Regarding the equations for the cubemap projection (in clause D.3.4.51.2 of JCTVC-AB1005-v1 and clause 5.2.2 of the OMAF SoDIS), there are the following discrepancies: - In JCTVC-AB1005-v1 there is a constraint that requires that pictureWidth shall be a multiple of 3 and pictureHeight shall be a multiple of 2, but not in the OMAF SoDIS. - There were a few switches between '/' and '÷' at the beginning of the equations (rows 1, 2, 5, 6). | It is suggested to discuss these discrepancies and align them. |
| 11 | In OMAF clauses 10.1.2.2 and 10.1.2.3, there are the following constraints: When the video does not provide full 360 coverage, for each picture, there shall be a region-wise packing SEI messages present in the bitstream that applies to the picture. When the video elementary stream contains a region-wise packing SEI message, RegionWisePackingBox shall be present. When present, RegionWisePackingBox shall signal the same information as in the region-wise packing SEI message(s). However, these constraints wouldn't be needed when the coverage information is present, since for the equirectangular projection the coverage in sphere domain is identical to the coverage in 2D picture domain, which the region-wise packing box provides in this scenario. In the context of JCTVC-AB1005-v1, there were also requests, e.g., from Minhua Zhou of Broadcom, to make the design of the OMAF-related SEI messages work for sub-360 coverage scenarios without mandating the presence of the region-wise packing SEI message when not necessary (i.e., when the projection type is the equirectangular projection and the sphere coverage information is present). | It is suggested to make aligned designs for OMAF and the OMAF-related SEI messages to address this issue. |
| 12 | There will most likely be OMAF and JCT-VC inputs to Macau that contain other topics than listed above for which coordination between OMAF and JCT-VC is necessary or desirable. | It is suggested to discuss as early as possible in OMAF meeting sessions (including Saturday and Sunday OMAF AHG meeting sessions) all OMAF inputs containing topics for which coordination between OMAF and JCT-VC is necessary or desirable, to allow joint discussions between OMAF and JCT-VC, knowing that JCT-VC will end by Tuesday. These topics should be identified early and considered in preparing the meeting agendas of both OMAF and JCT-VC. |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).