| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–24 Oct. 2017 | Document: JCTVC-AC0023 |
| --- | --- |

| Title: | On CMP padding and region-wise packing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92122, USA | Tel: Email: | +1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to align the semantics of the region-wise packing (RWP) SEI message with the RWP signalling in the Omnidirectional MediA Format (OMAF) that is being developed by the MPEG Systems subgroup, and remove the padding signalling from the cubemap projection (CMP) SEI message.

It is claimed that the proposal resolves existing issues of padding for CMP, and makes the designs of RWP and guard band padding aligned between OMAF-related SEI messages and OMAF.

It is further claimed that the proposal resolves issues #6 and #7 in JCTVC-AC0021, and fixes some minor bug fixes related to the presence of the RWP SEI message.

# Introduction

## On CMP padding

On guard band signalling, OMAF relies on the signalling that is part of the region-wise packing (RWP) syntax, while in JCTVC-AB1005-v1, the syntax of the RWP SEI message does not include guard band signalling, and the cubemap projection (CMP) SEI message includes a guard band syntax (although it is called padding) that is different from that in OMAF.

Also, it should be noted that there some are issues in JCTVC-AB1005-v1 regarding padding or guard band signalling for CMP, as described below.

The pictureWdith and pictureHeight (of the monoscopic projected luma picture) in the CMP equations should not count any padded samples. Therefore, when the RWP signalling is not present, in which case the size of the projected picture is not signalled, the size of the projected picture needs to be derived based on the CMP padding syntax. The width of the projected picture should be set equal to the width of the cropped output picture minus the total number of columns of padded samples, and the height should be set equal to the height of the cropped output picture minus the total number of row of padded samples, and there needs to be a constraint to require that all the remaining samples of the cropped output picture (i.e., excluding all the padded samples) shall exactly form a rectangle, which is the projected picture. When the RWP signalling is present for CMP, there needs to be a constraint that no packed region shall contain any padded sample.

Specifications for the above derivation of the size of the projected picture and the constraints are currently missing.

The above issues can be resolved by adding guard band padding signalling into the RWP syntax, same as in OMAF, and relying on that for providing support of guard band padding, i.e., remove the padding signalling from the CMP SEI message syntax. These are therefore proposed.

## On units of sizes in RWP

In the semantics of the RWP SEI message, currently (same as in the OMAF draft text before the Torino MPEG meeting in July 2017), the unit of the size of the projected picture and the size of projected and packed regions are either unspecified or specified as luma samples. In the latest OMAF draft text, these sizes are specified in relative units, to allow the use of the same RWP syntax for multiple bitstreams representing the same source video content. For example, multiple bitstreams representing the same source video content may be generated for adaptive streaming purpose.

It is therefore suggested to align the syntax and semantics of the RWP SEI message with the RWP syntax and semantics in the latest OMAF draft text, to signal packed picture sizes and the sizes of the projected and packed regions in relative units.

# Proposal

## Introduction

The proposal includes the following three parts:

- Alignment of the syntax and semantics of the RWP SEI message with the RWP signalling in OMAF, including addition of guard band signalling, and specifying the sizes of projected/packed pictures and regions in relative units (signalling of the width and height of the packed picture is part of this), and refined constraints of syntax element values (allowing overlapping packed regions is part of this).

- Removal of the syntax and semantics of cmp_padding_flag, cmp_padding_type, cmp_reserved_zero_6bits, and cmp_padding_chroma_sample_range_minus from the CMP SEI message.

- Some minor bug fixes related to the presence of the RWP SEI message.

Changes relative to JCTVC-AB1005-v1 are marked.

## Changes to RWP SEI message syntax and semantics

| regionwise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| rwp_persistence_flag | u(1) |
| constituent_picture_matching_flag | u(1) |
| rwp_reserved_zero_5bits | u(5) |
| num_packed_regions | u(8) |
| proj_picture_width | u(16) |
| proj_picture_height | u(16) |
| packed_picture_width | u(16) |
| packed_picture_height | u(16) |
| for( i = 0; i < num_packed_regions; i++ ) { |  |
| rwp_reserved_zero_3bits[ i ] | u(3) |
| guard_band_flag[ i ] | u(1) |
| packing_type[ i ] | u(4) |
| if( packing_type[ i ] = = 0 ) { |  |
| proj_region_width[ i ] | u(16) |
| proj_region_height[ i ] | u(16) |
| proj_region_top[ i ] | u(16) |
| proj_region_left[ i ] | u(16) |
| transform_type[ i ] | u(3) |
| rwp_reserved_zero_5bits[ i ] | u(5) |
| packed_region_width[ i ] | u(16) |
| packed_region_height[ i ] | u(16) |
| packed_region_top[ i ] | u(16) |
| packed_region_left[ i ] | u(16) |
| if( guard_band_flag[ i ] ) { |  |
| left_gb_width[ i ] | u(8) |
| right_gb_width[ i ] | u(8) |
| top_gb_height[ i ] | u(8) |
| bottom_gb_height[ i ] | u(8) |
| gb_not_used_for_pred_flag[ i ] | u(1) |
| for( j = 0; j < 4; j++ ) |  |
| gb_type[ i ][ j ] | u(3) |
| rwp_gb_reserved_zero_3bits[ i ] | u(3) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped decoded pictures onto projected pictures as well as information on the location and size of the guard bands, if any.

rwp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous region-wise packing SEI message in output order. rwp_cancel_flag equal to 0 indicates that region-wise packing information follows.

rwp_persistence_flag specifies the persistence of the region-wise packing SEI message for the current layer.

rwp_persistence_flag equal to 0 specifies that the region-wise packing SEI message applies to the current decoded picture only.

Let picA be the current picture. rwp_persistence_flag equal to 1 specifies that the region-wise packing SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a region-wise packing SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the region-wise packing SEI message in decoding order, a region-wise packing SEI message with rwp_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore region-wise packing SEI messages with rwp_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 in the CLVS that applies to the current picture. [Ed. (YK): Note that the changes in this paragraph are bug fixes that should be made independent of the issues described in section 1 of this document.]

If a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0, frame_packing_arrangement_type equal to 3, 4, or 5, and quincunx_sampling_flag equal to 0 is not present that applies to the current picture, the variables StereoFlag, TopBottomFlag, SideBySideFlag, and TempInterleavingFlag are all set equal to 0, the variables HorDiv1 and VerDiv1 are both set equal to 1. Otherwise the following applies:

–	StereoFlag is equal to 1.

–	When the frame_packing_arrangement_type is equal to 3, SideBySideFlag is set equal to 1, TopBottomFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 is set equal to 2 and VerDiv1 is set equal to 1.

–	When the frame_packing_arrangement_type is equal to 4, TopBottomFlag is set equal to 1, SideBySideFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 is set equal to 1 and VerDiv1 is set equal to 2.

–	When the frame_packing_arrangement_type is equal to 5, TempInterleavingFlag is set equal to 1, TopBottomFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 and VerDiv1 are both set equal to 1.

constituent_picture_matching_flag equal to 1 specifies that the projected region information, packed region information, and guard band region information in this SEI message apply individually to each constituent picture and that the packed picture and the projected picture have the same stereoscopic frame packing format indicated by the frame packing arrangement SEI message. constituent_picture_matching_flag equal to 0 specifies that the projected region information, packed region information, and guard band region information in this SEI message apply to the projected picture.

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0, frame_packing_arrangement_type equal to 3, 4, or 5, and quincunx_sampling_flag equal to 0 is not present that applies to the current picture, the value of constituent_picture_matching_flag shall be equal to 0.

rwp_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_5bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_5bits[ i ].

num_packed_regions specifies the number of packed regions when constituent_picture_matching_flag is equal to 0. The value of num_packed_regions shall be greater than 0. When constituent_picture_matching_flag is equal to 1, the number of packed regions is equal to num_regions * 2, and the information in each entry of the loop of num_packed_regions entries applies to each constituent picture of the projected picture and the packed picture.

proj_picture_width and proj_picture_height specify the width and height, respectively, of the projected picture, in relative projected picture sample units.

The values of proj_picture_width and proj_picture_height shall both be greater than 0.

packed_picture_width and packed_picture_height specify the width and height, respectively, of the packed picture, in relative packed picture sample units.

The values of packed_picture_width and packed_picture_height shall both be greater than 0.

It is a requirement of bitstream conformance that packed_picture_width and packed_picture_height shall have such values that packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ], represent integer horizontal and vertical coordinates of luma sample units within the cropped decoded pictures.

rwp_reserved_zero_3bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_3bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_3bits[ i ].

guard_band_flag[ i ] equal to 0 specifies that the i-th packed region does not have a guard band. guard_band_flag[ i ] equal to 1 specifies that the i-th packed region has a guard band.

packing_type[ i ] specifies the type of region-wise packing. packing_type[ i ] equal to 0 indicates rectangular region-wise packing. Other values are reserved. The value of packing_type[ i ] shall be equal to 0 in this version of this Specification. Decoders shall allow values of packing_type[ i ] greater than 0 and shall ignore all region-wise packing SEI messages with packing_type[ i ] greater than 0 for any value of i.

proj_region_width[ i ],

proj_region_height[ i ],

proj_region_top[ i ] and proj_region_left[ i ] specify the width, height, top sample row, and the left-most sample column, respectively, of the i-th projected region, either within the projected picture (when constituent_picture_matching_flag is equal to 0) or within the constituent picture of the projected picture (when constituent_picture_matching_flag is equal to 1).

proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ], and proj_region_left[ i ] are indicated in relative projected picture sample units.

NOTE 1 – Two projected regions may partially or entirely overlap with each other.

The values of proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ] and proj_region_left[ i ] are constrained as follows:

–	If SideBySideFlag is equal to 0, proj_region_width[ i ] shall be in the range of 1 to proj_picture_width, inclusive. Otherwise (SideBySideFlag is equal to 1), proj_region_width[ i ] shall be in the range of 1 to proj_picture_width / 2, inclusive.

–	proj_region_height[ i ] shall be greater than 0.

–	If constituent_picture_matching_flag is equal to 0, the values of proj_region_top[ i ] and proj_region_left[ i ] shall be in the range from 0, indicating the top-left corner of the projected picture, to proj_picture_height − 1, inclusive, and proj_picture_width − 1, inclusive, respectively. Otherwise (constituent_picture_matching_flag is equal to 1), the values of proj_region_top[ i ] and proj_region_left[ i ] shall be in the range from 0 to proj_picture_height / VerDiv1 − 1, inclusive, and proj_picture_width / HorDiv1 − 1, inclusive, respectively.

–	When StereoFlag is equal to 0 or constituent_picture_matching_flag is equal to 0, proj_region_height[ i ] and proj_region_top[ i ] shall be constrained such that proj_region_height[ i ] + proj_region_top[ i ] − 1 is less than proj_picture_height.

–	When StereoFlag is equal to 1, the following applies:

–	proj_region_width[ i ] shall be less than or equal to proj_picture_width / HorDiv1.

–	proj_region_height[ i ] shall be less than or equal to proj_picture_height / VerDiv1.

–	If proj_region_left[ i ] is less than proj_picture_width / HorDiv1, proj_region_width[ i ] and proj_region_left[ i ] shall be constrained such that proj_region_width[ i ] + proj_region_left[ i ] − 1 is less than proj_picture_width / HorDiv1. Otherwise (proj_region_left[ i ] is greater than or equal to proj_picture_width / HorDiv1), proj_region_width[ i ] and proj_region_left[ i ] shall be constrained such that proj_region_width[ i ] + proj_region_left[ i ] − proj_picture_width / HorDiv1 − 1 is less than proj_picture_width / HorDiv1.

–	If proj_region_top[ i ] is less than proj_picture_height / VerDiv1, proj_region_height[ i ] and proj_region_top[ i ] shall be constrained such that proj_region_height[ i ] + proj_region_top[ i ] − 1 is less than proj_picture_height / VerDiv1. Otherwise (proj_region_top[ i ] is greater than or equal to proj_picture_height / VerDiv1), proj_region_height[ i ] and proj_region_top[ i ] shall be constrained such that proj_region_height[ i ] + proj_region_top[ i ] − proj_picture_height / VerDiv1 − 1 is less than proj_picture_height / VerDiv1.

–	When constituent_picture_matching_flag is equal to 1, the following applies:

–	proj_region_width[ i ] and proj_region_left[ i ] shall be constrained such that proj_region_width[ i ] + proj_region_left[ i ] − 1 is less than proj_picture_width / HorDiv1.

–	proj_region_height[ i ] and proj_region_top[ i ] shall be constrained such that proj_region_height[ i ] + proj_region_top[ i ] − 1 is less than proj_picture_height / VerDiv1.

transform_type[ i ] specifies the rotation and mirroring to be applied to the i-th packed region to remap to the i-th projected region. When transform_type[ i ] specifies both rotation and mirroring, rotation applies before mirroring. The values of transform_type[ i ] are specified in Table D. X:

Table D.X – transform_type[ i ] values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (counter-clockwise) |
| 3 | rotation by 180 degrees (counter-clockwise) after mirroring horizontally |
| 4 | rotation by 90 degrees (counter-clockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (counter-clockwise) |
| 6 | rotation by 270 degrees (counter-clockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (counter-clockwise) |

rwp_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_5bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_5bits[ i ].

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] specify the width, height, the top luma sample row, and the left-most luma sample column, respectively, of the packed region, either within the region-wise packed picture (when constituent_picture_matching_flag is equal to 0) or within each constituent picture of the region-wise packed picture (when constituent_picture_matching_flag is equal to 1).

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] are indicated in relative region-wise packed picture sample units.

NOTE 2 – Two packed regions may partially or entirely overlap with each other.

The values of packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] are constrained as follows:

–	packed_region_width[ i ] and packed_region_height[ i ] shall both be greater than 0.

–	The values of packed_region_top[ i ] and packed_region_left[ i ] shall be in the range from 0, indicating the top-left corner luma sample of the region-wise packed picture, to packed_picture_height − 1, inclusive, and packed_picture_width − 1, inclusive, respectively.

–	If constituent_picture_matching_flag is equal to 0, the following applies:

–	The sum of packed_region_width[ i ] and packed_region_left[ i ] minus 1 shall be less than packed_picture_width.

–	The sum of packed_region_height[ i ] and packed_region_top[ i ] minus 1 shall be less than packed_picture_height.

–	Otherwise (constituent_picture_matching_flag is equal to 1), the following applies:

–	The sum of packed_region_width[ i ] and packed_region_left[ i ] minus 1 shall be less than packed_picture_width / HorDiv1.

–	The sum of packed_region_height[ i ] and packed_region_top[ i ] minus 1 shall be less than packed_picture_height / VerDiv1.

–	When the decoded picture has 4:2:0 or 4:2:2 chroma format, packed_region_left[ i ] shall correspond to an even horizontal coordinate value of luma sample units, and packed_region_width[ i ] shall correspond to an even number of luma samples, both within the cropped decoded picture.

–	When the decoded picture has 4:2:0 chroma format, packed_region_top[ i ] shall correspond to an even vertical coordinate value of luma sample units, and packed_region_height[ i ] shall correspond to an even number of luma samples, both within the cropped decoded picture.

left_gb_width[ i ] specifies the width of the guard band on the left side of the i-th packed region in relative region-wise packed picture sample units. When the decoded picture has 4:2:0 or 4:2:2 chroma format, left_gb_width[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

right_gb_width[ i ] specifies the width of the guard band on the right side of the i-th packed region in relative region-wise packed picture sample units. When the decoded picture has 4:2:0 or 4:2:2 chroma format, right_gb_width[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

top_gb_height[ i ] specifies the height of the guard band above the i-th packed region in relative region-wise packed picture sample units. When the decoded picture has 4:2:0 chroma format, top_gb_height[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

bottom_gb_height[ i ] specifies the height of the guard band below the i-th packed region in relative region-wise packed picture sample units. When the decoded picture has 4:2:0 chroma format, bottom_gb_height[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

When guard_band_flag[ i ] is equal to 1, left_gb_width[ i ], right_gb_width[ i ], top_gb_height[ i ], or bottom_gb_height[ i ] shall be greater than 0.

The i-th packed region as specified by this SEI message shall not overlap with any other packed region specified by the same SEI message or any guard band specified by the same SEI message.

The guard bands associated with the i-th packed region, if any, as specified by this SEI message shall not overlap with any packed region specified by the same SEI message or any other guard bands specified by the same SEI message.

gb_not_used_for_pred_flag[ i ] equal to 0 specifies that the guard bands may or may not be used in the inter prediction process. gb_not_used_for_pred_flag[ i ] equal to 1 specifies that the sample values of the guard bands are not used in the inter prediction process.

NOTE 3 – When gb_not_used_for_pred_flag[ i ] is equal to 1, the sample values within guard bands in cropped decoded pictures can be rewritten even if the cropped decoded pictures were used as references for inter prediction of subsequent pictures to be decoded. For example, the content of a packed region can be seamlessly expanded to its guard band with decoded and re-projected samples of another packed region.

gb_type[ i ][ j ] specifies the type of the guard bands for the i-th packed region as follows, with j equal to 0, 1, 2, or 3 indicating that the semantics below apply to the left, right, top, or bottom edge, respectively, of the packed region:

–	gb_type[ i ][ j ] equal to 0 specifies that the content of the guard bands in relation to the content of the packed regions is unspecified. When gb_not_used_for_pred_flag[i] is equal to 0, gb_type[ i ][ j ] shall not be equal to 0.

–	gb_type[ i ][ j ] equal to 1 specifies that the content of the guard bands suffices for interpolation of sample values at sub-pel sample fractional locations within the packed region and less than sample outside of the boundary of the packed region.

NOTE 4 – gb_type[ i ][ j ] equal to 1 can be used when the boundary samples of a packed region have been copied horizontally or vertically to the guard band.

–	gb_type[ i ][ j ] equal to 2 specifies that the content of the guard bands represents actual picture content at quality that gradually changes from the picture quality of the packed region to that of the spherically adjacent packed region.

–	gb_type[ i ][ j ] equal to 3 specifies that the content of the guard bands represents actual picture content at the picture quality of the packed region.

–	gb_type[ i ][ j ] values greater than 3 are reserved. Decoders shall ignore the value of gb_type[ i ][ j ] when the value is greater than 3.

rwp_gb_reserved_zero_3bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_gb_reserved_zero_3bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_gb_reserved_zero_3bits[ i ].

## Changes to the CMP SEI message syntax and semantics

| cubemap_projection( payloadSize ) { | Descriptor |
| --- | --- |
| cmp_cancel_flag | u(1) |
| if( !cmp_cancel_flag ) { |  |
| cmp_persistence_flag | u(1) |
| cmp_rotation_flag | u(1) |
| cmp_reserved_zero_5bits | u(5) |
| if( cmp_rotation_flag = = 1 ) { |  |
| cmp_yaw_rotation | i(32) |
| cmp_pitch_rotation | i(32) |
| cmp_roll_rotation | i(32) |
| } |  |
| } |  |
| } |  |

...

[Ed. (YK): The semantics of cmp_padding_flag is missing in JCTVC-AB1005-v1.]

cmp_reserved_zero_5bits when present, shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for cmp_reserved_zero_5bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of cmp_reserved_zero_5bits.

cmp_yaw_rotation specifies the value of the yaw rotation angle, in units of 2−16 degrees. The value of cmp_yaw_rotation shall be in the range of −180 * 216 (i.e., −11796480) to 180 * 216 − 1 (i.e., 11796479), inclusive. When not present, the value of cmp_yaw_rotation is inferred to be equal to 0.

...

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).