| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-Bxxx |
| --- | --- |

| Title: | Report of VCEG AHG1 on Coding Efficiency Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Madhukar Budagavi | Email: | martak@qti.qualcomm.com m.budagavi@samsung.com |
| Source: | VCEG AHG |  |  |

_____________________________

# Abstract

The following summarizes the Coding Efficiency Improvements AHG activities between Q.6/16 VCEG meeting in Geneva, Switzerland (October 2015) and the current meeting in San Diego, USA.

# JEM Status

First version of Joint Exploration Test Model Software (HM-16.6-JEM-1.0) was released 17th of December, 2016. The software can be downloaded at:

https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-1.0

The following tools were included on top of HM-14.0-KTA-2.0 software:

- Position dependent intra prediction combination (PDPC).

- Harmonization and improvements for Bi-Directional Optical Flow (BIO).

- Non-separable secondary transform.

- Affine motion prediction.

- Adaptive reference sample smoothing.

Signal dependent transform (SDT) is still being integrated. In addition configuration files were updated to reflect modifications to chroma QP offset.

Table 1 shows the BD-Rate reduction of JEM1 comparing to HM-14.0 for JCT-VC common test condition [1] for random access configuration.

Table 1: Coding performance improvement of JEM1.

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -20.81% | -29.93% | -23.77% |
| Class B | -21.27% | -13.17% | -9.24% |
| Class C | -20.65% | -14.78% | -18.20% |
| Class D | -20.55% | -9.84% | -12.06% |
| Class E |  |  |  |
| Overall | -20.84% | -16.71% | -15.43% |

# Contributions to this Meeting

The following contributions proposing new tools or the modifications of the existing tools have been registered for this meeting:

- JVET-B0023: Quadtree plus binary tree structure integration with JEM tools.

- JVET-B0028: Direction-dependent sub-TU scan order on intra prediction.

- JVET-B0033: Adaptive Multiple Transform for Chroma.

- JVET-B0038: Harmonization of AFFINE, OBMC and DBF.

- JVET-B0043: Polyphase subsampled signal for spatial scalability.

- JVET-B0047: Non Square TU Partitioning.

- JVET-B0048: Universal string matching for ultra high quality and ultra high efficiency SCC.

- JVET-B0051: Further improvement of intra coding tools.

- JVET-B0054: De-quantization and scaling for next generation containers.

- JVET-B0058: Modification of merge candidate derivation.

- JVET-B0058: TU-level non-separable secondary transform.

- JVET-B0059: Improvements on adaptive loop filter.

In addition there are two contributions proposing encoder only modifications to JEM:

- JVET-B0039: Non-normative JEM encoder improvements.

- JVET-B0041: Adaptive reference sample smoothing simplification.

There is also a number of contributions analyzing performance of tools already included in JEM1:

- JVET-B0022: Performance of JEM 1 tools analysis by Samsung.

- JVET-B0037: Performance analysis of affine inter prediction in JEM1.0.

- JVET-B0044: Coding Efficiency / Complexity Analysis of JEM 1.0 coding tools for the Random Access Configuration.

- JVET-B0045: Performance evaluation of JEM 1 tools by Qualcomm.

- JVET-B0057: Evaluation of some intra-coding tools of JEM1.

Three of the contributions provide analysis of all the tools currently included in JEM software (JVET-B0022, JVET-B0044 and JVET-B0045). Documents JVET-B0022 and JVET-B0045 include both tool-on and tool-off results, while the document JVET-B0044 only includes tool-on results. Tool-on results in documents JVET-B0022 and JVET-B0045 are the same for all the tools except for cross-component linear model (COM16_C806_LMCHROMA) prediction, secondary transform (COM16_C1044_NSST) and adaptive reference sample smoothing (COM16_C983_RSAF). For cross-component linear model prediction and secondary transform the mismatch is limited to single Class A sequence. The tool-off results in JVET-B0022 and JVET-B0045 differ for larger number of tools: sub-PU level motion vector prediction (COM16_C806_VCEG_AZ10_SUB_PU_TMVP), pattern matched motion vector derivation (VCEG_AZ07_FRUC_MERGE), position dependent intra prediction combination (COM16_C1044_PDPC), cross-component linear model prediction (COM16_C806_LMCHROMA), adaptive reference sample smoothing (COM16_C983_RSAF) and adaptive loop filter (ALF_HM3_REFACTOR).

# Recommendations

The AHG recommends

- To review all the related contribution.

- Discuss possibility of an extension/annex to HEVC based on the tool analysis submissions.

# References

[1] 	F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Doc. JCTVC-L1100, 12th Meeting.