| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0055 |
| --- | --- |

| Title: | Netflix Chimera test sequences evaluation report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maxim Sychev Huanbang Chen | Tel: Email: | +7 925 0081124 sychev.maxim@huawei.com chenhuanbang@huawei.com |
| Source: | Huawei Technologies Co., LTD. (Russia) |  |  |

_____________________________

# Abstract

This is an evaluation results report of 4K resolution test sequences (9 segments) taken from Chimera video provided by Netflix. Both objective and subjective evaluations were performed. For objective evaluation, the HM-16.6 and JEM-1.0 reference software were used. All the obtained results of objective evaluation can be found in the attached Excel table. In addition, some examples of subjective distortions are presented. Encoded streams are available can be provided by the authors on demand for evaluation or cross-check. The decoded YUV sequences are available as well.

# Test configuration

## Netflix set of sequences conversion

From the original “Chimera” sequence, nine 10-second sequences (each 600 frames) selected as suggested by Netflix. A script for converting from the source format to YUV 4:2:0 is as follows:

ffmpeg.exe  -f rawvideo -s 4096x2160 -pix_fmt yuv444p10le -i *_444.yuv -f rawvideo -pix_fmt yuv420p10le *_420.yuv

Later it was matched with uploaded converted ones.

| # | Sequence Name | Md5 sum |
| --- | --- | --- |
| 1 | Aerial | e5135189aae0a87a190a8a53e03c0eb5 |
| 2 | BarScene | 6832ad82b16f7fc6bbfc01184d979d56 |
| 3 | Dancers | 57edb8ef05c47678db51c9b7c8270631 |
| 4 | DinnerScene | 0444909099af36134a04bcfcaa8845b4 |
| 5 | DrivingPOV | 43f0c38cd43620c55290b54a0ef69300 |
| 6 | PierSeaside | 0dd24b7cf6553d1e4190262adc228423 |
| 7 | RollerCoaster | 6632b92cfa52a5bb43b01b82c231d951 |
| 8 | ToddlerFountain | 9aaf6e6a55488717513d15a003898523 |
| 9 | WindAndNature | 79f8164a38a5233733df67754675b3a0 |

We used ffmpeg software (Copyright (c) 2000-2015 the FFmpeg developers) of version N-71737-gb410c69 built with GCC 4.9.2 [3]

# Test sequence evaluation

## Objective evaluation

### Encoding condition

HM-16.6 [6] is used to obtain encode results with recommended in w15453 [1] adjustments.

A simulation is conducted under CTC AI, LD-B, LD-P, and RA settings [7]. There only 300 frames were used for evaluation according an agreement and recommendation of work plan in W15791/TD-WP3-212.

The default Main10 configurations provided in the HM16.6 software package are set for the 420 sequence.

Additionally, the JVET conditions were used with corresponded JVET10 configurations provided in the JEM1.0 software package are set for the 420 sequence.

Tested QP range is from 12 to 42 with 5 steps. It covers wider bitrate range than CTC that tests 22, 27, 32, and 37.

## Subjective evaluation

### Viewing equipment and methodology

Samsung 78" JS9500 9 Series SUHD 4K Nano Crystal Smart 3D TV was used for subjective evaluation of decoded video. Additionally ASUS UX501V laptop with 4K resolution display was used. Video sequences were viewed in two modes:

- real-time mode,

- frame-by-frame mode.

Several people took part in this experiment.

## Results

The attached Excel tables contain all the simulation results. In this document, the RD-curves obtained as a result of objective evaluation are presented sequence by sequence below. The curves obtained by averaging bit-rate and PSNR values are also shown on the graphs. Some visual artefacts found while performing subjective evaluation are shown after the RD-curves that characterize this sequence. In addition, we commented artefacts for each sequence.

Unfortunately, for this set of new sequences the averaged RD-curves are not very useful, because of some abnormal behaviour for some sequences. The decision will be made individually sequence by sequence.

### Netflix sequence by sequence

##### Aerial

The RD curves behaviour is quite normal and below the averages curves.

This sequence contains a global motion lot of textures (e.g., dry grass at the ground, leaves). The scene is almost static excluding full frame rotation and shaking. Below, some examples are presented. They were taken from decoded sequence (RA, JEM1.0, QP=37).  Subjectively, many distortions, blurring artefacts and color distortions with propagation through the sequence appear due to directional intra prediction.

Frame #1 – Color artefacts on the operator face and ringing around head, highly blurred region instead of texturised background, some diagonal artefacts from directional prediction

Frame #48 – color distortion propagation from temporal prediction at the left marked region and color artefacts at the leg kept from the beginning.

##### BarScene

This sequence contains textures at the background with softly shifting focal length with increasing sharpness of foreground objects. Subjectively, many blurring artefacts on texturised areas and color distortions with propagation through the sequence are observed.

Frame #1 – Color differ from original – due to color denoising and blurring (original color looks more brown), highly blurred region instead of texturised background. Artefacts propagated through a long time period.

##### Dancers

This most part of sequence contains dark black and noisy background. The scene is almost static. Below, some examples are presented. Pictures were taken from decoded sequence (RA, JEM1.0, QP=37,42).  Inspite of the large amount of dark areas in frames the ROI quality is very bad because of blurriness.

Frame #1 – More than 80% of frame is a dark black area, but the quality of ROI is very bad. (QP=37 at the left picture, QP=42 at the right picture)

Frame #35 – Bad quality propagation kept from the beginning. QP offset for inter frames does not improve quality

##### DinnerScene

The global horizontal camera rotation goes to translation motion of the whole frame. Some examples can be found below. They were taken from decoded sequence (RA, JEM1.0, QP=37 and 42).  Subjectively, some distortions appear due to overlapping of some background area of interest by foreground objects. Some object borders in low contrast areas are disappeared at all.

Frame #80 – (QP=37) The vase is not recognisable during long time period due to blurring distortions on the border. Occasionally blocking artefacts appear. Dynamically, there are a lot of distortions on the face behind the chair back.

##### DrivingPOV

This sequence contains a global perspective zooming, the highly texturized asphalt ground. Some examples of distortions are shown below. They were taken from decoded sequence (RA, JEM1.0, QP=37).  Subjectively, road is fully blurred and asphalt is not recognizable.

Frame #111 – Lights disappear for some period of time after trees moved in front of them

Frame #48 – (QP=37) Almost all ground surface not recognizable due to highly blurring.

##### PierSeaside

This sequence contains a global zooming, contains a lots of small details and the sea surface. The scene is almost static. Some examples are presented below. They were taken from decoded sequence with QP equal to 42, because for higher QP amount of distortion is low enough.  Subjectively, some blocking/blurring distortions appear at the water surface and some thin details disappeared occasionally.

Frame #299 – (QP=42) This sequence quality is pretty good in terms of subjective quality and bitrate reduction. There is only for QP=42 some blocking artefacts appears at water, and the robe is disappeared.

##### RollerCoaster

This sequence contains a wide range of different non translational global motion like zooming (with perspective), rotation, shaking and other warping (like on the flag). Some examples of typical distortions are presented below. They were taken from decoded sequence (RA, JEM1.0, QP=37).  Subjectively, some blocking artefacts appeared.

Frame #93 – (QP=37) There is a number of blocking artefacts on flag.

##### ToddlerFountain

This sequence contains a lot of high frequency details which are very dynamical changing. A lot of water drops and splashes in front of scene. Below, some typical examples are presented. They were taken from decoded sequence (RA, JEM1.0, QP=37).  Subjectively, many distortions can be observed especially due to highly dynamic content. Sometimes the quality of neighbour frames is very different due to inter QP offset.

Frame #96 and 97 – Two sequential frames have big enough difference due to QP difference.

##### WindAndNature

There is a static scene with large amount of small dynamic objects. Highly contrast foreground and very smooth background. Below, some examples are presented. They were taken from decoded sequence (RA, JEM1.0, QP=37).  Sometimes small objects are disappeared or not recognizable due to blurring and deblocking artefacts.

Frame #3 – (QP=37 for right picture) occasionally the objects disappear from frame.

Frame #3 – static and dynamic ringing artefacts mostly at low contrast areas. Sometimes objects are not recognisable.

# List of 4K sequences with descriptions

|  | Netflix_Aerial |
| --- | --- |
|  | Netflix_DinnerScene |
|  | Netflix_RollerCoaster |
|  | Netflix_BarScene |
|  | Netflix_DrivingPOV |
|  | Netflix_ToddlerFountain |
|  | Netflix_Dancers |
|  | Netflix_PierSeaside |
|  | Netflix_WindAndNature |

# Experimental results

Excel table with HM16.6 and JEM1.0 encoding results for the Netflix sequence “Chimera” is attached to this evaluation report.

The tables below present JEM1.0 encoding results vs. HM16.6.

# Sequence preselection

The table below summarize results and observations of this evaluation green marked cells in favor of choice:

| Sequence name | Bitrate max HM/JEM, Mbps | Subjective distortions | Widespread presence | Compression efficiency distribution | RD-behaviour | Potencies for improving |
| --- | --- | --- | --- | --- | --- | --- |
| Aerial | 54/50 | middle | widely/UGC | abnormal | normal | below normal |
| BarScene | 45/48 | specific | special effect | abnormal | abnormal | below normal |
| Dancers | 27/30 | typical | specific | abnormal | abnormal | normal |
| DinnerScene | 32/34 | typical and specific | quite widely | abnormal | abnormal | below normal |
| DrivingPOV | 55/50 | typical and specific | widely/surveilance | normal | normal | above normal |
| PierSeaside | 20/18 | low | rare | normal | normal | normal |
| RollerCoaster | 17/15 | typical | widely/UGC | normal | normal | above normal |
| ToddlerFountain | 235/# | very high | rare/special effect | normal | below normal | normal |
| WindAndNature | 19/17 | typical | quite widely | normal | normal | normal |

According to this evaluation report it is suggested to consider the following 4K sequences as candidates for new CTC.

Highly recommended fine chroma QP adjustment according to presented result for some of sequences.

Suggested to establish a special extension as an additional class of sequences in CTC with specific content which undoubtedly very complicated for vastly improving compression efficiency, but very reasonable to care about subjective quality neither than bit-rate reduction. There are several sequences from previous evaluation suggested to include to this class: Sparkler, BricksBushesStatic, BricksBushesStatic.

Suggestion for the further consideration:

- ToddlerFountain* - include to special set for subjective evaluation (as extension of CTC)

- DrivingPOV

- WindandNature (include shading part of sequence)

- RollerCoaster is a good alternative to Netflix Elfuente Tunnel sequence (not from the starting point – shaking part is not interested)

The following two sequences have quite interesting content but further study is required because of quite abnormal compression behaviour:

- Aerial

- BarScene

The rest set of sequences have enough small bit-rate and quite similar content with little bit higher bitrate can be encoded without big distortions:

- DinnerScene

- Dancers

- PierSeaside

# Reference

- N15791, "Work Plan for Investigation of Test Sequence Encoding,” MPEG, Oct. 2015

- JVET test sequence ftp site, ftp://ftp.ient.rwth-aachen.de/

- JCTVC-P1006, “Common test conditions and software reference configurations for HEVC range extensions,” Jan. 2014

- T13-SG16-151012-TD-WP3-0213!!MSW JEM Draft

- T13-SG16-151012-TD-WP3-0212!R1!MSW-E_test_sequence_selection

# Patent rights declaration(s)

Huawei does not have any current or pending patent rights relating to the technology described in this contribution.