| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0057 |
| --- | --- |

| Title: | Evaluation of some intra-coding tools of JEM1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy Huawei Research Center, 1/7, Altufievskoe shosse, Moscow, Russia | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution presents an evaluation of some of JEM1.0 intra-coding tools, specifically: 4-tap interpolation filter for intra prediction, position dependent intra prediction combination, adaptive reference sample smoothing and MPI. Simulations include “off-tests” as provided in [1] as well as a brief tools efficiency analysis. Tools efficiency is estimated by calculating a ration of coding gain increase to encoder complexity.

# Introduction

The proposal studies some of JEM1.0 intra-coding tools (see Table 1). “Off-tests” have been performed within the same conditions as in [1] using the same macro values given in table 2.

Table 1. Studied JEM1.0 intra-coding tools.

| Acronym | Macro name | Tool |
| --- | --- | --- |
| 4TAP | VCEG_AZ07_INTRA_4TAP_FILTER | 4-tap interpolation filter for intra prediction |
| PDPC | COM16_C1046_PDPC_INTRA | Position dependent intra prediction combination |
| ARSS | COM16_C983_RSAF | Adaptive reference sample smoothing |
| MPI | VCEG_AZ05_INTRA_MPI | Multi-parameter intra prediction |

Table 2. Macro values used in “tool-off” simulations as specified in [1].

| MACRO VALUES | Variations | 4TAP | PDPC & MPI | ARSS | w/o PREVENT_ OVERSMOOTHING |
| --- | --- | --- | --- | --- | --- |
| ALF_HM3_REFACTOR |  | 1 | 1 | 1 | 1 |
| COM16_C806_VCEG_AZ10_SUB_PU_TMVP |  | 1 | 1 | 1 | 1 |
|  | COM16_C806_HEVC_MOTION_CONSTRAINT_REMOVAL | 1 | 1 | 1 | 1 |
|  | COM16_C806_DISABLE_4X4_PU | 1 | 1 | 1 | 1 |
|  | COM16_C806_GEN_MRG_IMPROVEMENT | 1 | 1 | 1 | 1 |
| COM16_C806_OBMC |  | 1 | 1 | 1 | 1 |
| COM16_C806_EMT |  | 1 | 1 | 1 | 1 |
| COM16_C806_T64 |  | 1 | 1 | 1 | 1 |
| COM16_C806_LARGE_CTU |  | 1 | 1 | 1 | 1 |
| COM16_C806_LMCHROMA | (ChromaQPOffset = 0) | 1 | 1 | 1 | 1 |
|  | COM16_C806_CR_FROM_CB_LAMBDA_ADJUSTMENT | 1 | 1 | 1 | 1 |
| VCEG_AZ07_IMV |  | 1 | 1 | 1 | 1 |
| VCEG_AZ07_FRUC_MERGE |  | 1 | 1 | 1 | 1 |
| VCEG_AZ06_IC |  | 1 | 1 | 1 | 1 |
| VCEG_AZ07_INTRA_4TAP_FILTER |  | 0 | 1 | 1 | 1 |
| VCEG_AZ07_INTRA_BOUNDARY_FILTER |  | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_INTRA_BOUNDARY_FILTER_MULTI_LINE | 1 | 1 | 1 | 1 |
| VCEG_AZ07_INTRA_65ANG_MODES |  | 1 | 1 | 1 | 1 |
| VCEG_AZ07_ECABAC |  | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_CTX_RESIDUALCODING | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_BAC_ADAPT_WDOW | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_INIT_PREVFRAME | 1 | 1 | 1 | 1 |
| VCEG_AZ05_MULTI_PARAM_CABAC |  | 1 | 1 | 1 | 1 |
| VCEG_AZ05_BIO |  | 1 | 1 | 1 | 1 |
|  | COM16_C1045_BIO_HARMO_IMPROV | 1 | 1 | 1 | 1 |
| COM16_C1016_AFFINE |  | 1 | 1 | 1 | 1 |
| COM16_C983_RSAF |  | 1 | 1 | 0 | 1 |
|  | COM16_C983_RSAF_PREVENT_OVERSMOOTHING | 1 | 1 | 0 | 0 |
|  | COM16_C983_RSAF_ESTIMATION_MODE_FULL | 1 | 1 | 1 | 1 |
| COM16_C1044_NSST |  | 1 | 1 | 1 | 1 |
|  | VCEG_AZ05_ROT_TR | 0 | 0 | 0 | 0 |
| COM16_C1046_PDPC_INTRA |  | 1 | 0 | 1 | 1 |
|  | VCEG_AZ05_INTRA_MPI | 0 | 0 | 0 | 0 |
|  | COM16_C1046_PDPC_RSAF_HARMONIZATION | 1 | 1 | 1 | 1 |

# Technical description

Simulations for “off-tests” were performed for CTC, full-length test sequences. Results are given in Tables 3-6. Reference is JEM1.0, second reference is HM16.6.

Table 3. JEM1 without 4-tap inta interpolation filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 0.12% | 0.18% | 0.27% | 99% | 110% |
| Class B | 0.53% | 0.67% | 0.77% | 101% | 106% |
| Class C | 0.74% | 1.02% | 0.95% | 99% | 100% |
| Class D | 0.48% | 0.77% | 0.78% | 99% | 96% |
| Class E | 1.00% | 1.41% | 1.67% | 99% | 107% |
| Class F | 0.39% | 0.73% | 0.67% | 99% | 102% |
| Overall (Ref) | 0.52% | 0.77% | 0.81% | 99% | 103% |
| Overall (Ref-2) | -13.55% | -11.79% | -11.73% | 1940% | 155% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | 0.02% | 0.37% | 0.17% | 101% | 105% |
| Class B | 0.25% | 0.37% | 0.54% | 102% | 106% |
| Class C | 0.30% | 0.52% | 0.26% | 100% | 104% |
| Class D | 0.21% | 0.49% | 0.24% | 99% | 103% |
| Class E |  |  |  |  |  |
| Class F | 0.13% | 0.09% | -0.02% | 101% | 109% |
| Overall (Ref) | 0.19% | 0.37% | 0.25% | 101% | 106% |
| Overall (Ref-2) | -19.86% | -16.09% | -15.06% | 547% | 678% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.10% | 0.41% | 0.51% | 101% | 107% |
| Class C | 0.13% | 0.62% | 0.46% | 99% | 103% |
| Class D | 0.08% | 0.27% | 0.27% | 99% | 98% |
| Class E | 0.13% | 1.10% | 0.38% | 99% | 104% |
| Class F | 0.06% | -0.20% | -0.19% | 100% | 106% |
| Overall (Ref) | 0.10% | 0.41% | 0.29% | 100% | 104% |
| Overall (Ref-2) | -16.53% | -21.79% | -22.32% | 385% | 466% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.09% | 0.30% | 0.34% | 101% | 110% |
| Class C | 0.14% | 0.38% | -0.14% | 99% | 102% |
| Class D | 0.15% | 0.74% | 0.07% | 99% | 105% |
| Class E | 0.13% | 1.14% | 1.82% | 99% | 115% |
| Class F | 0.03% | 0.47% | 0.19% | 101% | 108% |
| Overall (Ref) | 0.11% | 0.56% | 0.38% | 100% | 108% |
| Overall (Ref-2) | -19.24% | -23.70% | -23.93% | 349% | 292% |

Table 4. JEM1 without PDPC and MPI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 1.40% | 0.78% | 1.43% | 85% | 111% |
| Class B | 0.94% | 0.62% | 0.75% | 86% | 107% |
| Class C | 0.63% | 0.63% | 0.69% | 84% | 102% |
| Class D | 0.58% | 0.40% | 0.28% | 84% | 97% |
| Class E | 1.04% | 0.61% | 0.17% | 86% | 104% |
| Class F | -0.13% | -0.07% | -0.08% | 84% | 102% |
| Overall (Ref) | 0.74% | 0.50% | 0.56% | 85% | 104% |
| Overall (Ref-2) | -13.37% | -12.03% | -11.94% | 1653% | 156% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | 0.48% | -0.27% | 0.52% | 99% | 103% |
| Class B | 0.57% | 0.13% | 0.29% | 100% | 105% |
| Class C | 0.38% | 0.17% | 0.34% | 98% | 104% |
| Class D | 0.29% | 0.11% | 0.16% | 97% | 100% |
| Class E |  |  |  |  |  |
| Class F | 0.00% | -0.20% | -0.11% | 99% | 109% |
| Overall (Ref) | 0.35% | -0.01% | 0.24% | 99% | 104% |
| Overall (Ref-2) | -19.73% | -16.33% | -15.05% | 535% | 670% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.11% | 0.28% | 0.29% | 100% | 108% |
| Class C | 0.12% | 0.35% | 0.42% | 97% | 103% |
| Class D | 0.11% | -0.38% | 0.30% | 97% | 101% |
| Class E | 0.04% | 0.13% | -0.34% | 99% | 104% |
| Class F | 0.03% | -0.06% | -0.32% | 99% | 103% |
| Overall (Ref) | 0.08% | 0.07% | 0.10% | 98% | 104% |
| Overall (Ref-2) | -16.55% | -22.03% | -22.43% | 379% | 466% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.12% | 0.32% | 0.38% | 98% | 110% |
| Class C | 0.13% | 0.48% | -0.13% | 96% | 99% |
| Class D | 0.16% | 0.50% | 1.27% | 96% | 103% |
| Class E | 0.21% | 0.55% | 0.43% | 98% | 109% |
| Class F | 0.16% | 0.17% | -0.12% | 99% | 106% |
| Overall (Ref) | 0.15% | 0.39% | 0.36% | 97% | 105% |
| Overall (Ref-2) | -19.20% | -23.82% | -23.89% | 341% | 285% |

Table 5. JEM1 without ARSS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 0.52% | 1.05% | 0.82% | 54% | 114% |
| Class B | 0.81% | 1.55% | 1.67% | 58% | 110% |
| Class C | 0.80% | 1.21% | 1.20% | 57% | 101% |
| Class D | 0.61% | 0.95% | 0.81% | 56% | 98% |
| Class E | 0.83% | 0.85% | 0.63% | 58% | 102% |
| Class F | 0.41% | 0.65% | 0.44% | 60% | 101% |
| Overall (Ref) | 0.66% | 1.07% | 0.97% | 57% | 105% |
| Overall (Ref-2) | -13.42% | -11.52% | -11.57% | 1115% | 157% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | 0.11% | 0.57% | 0.42% | 92% | 106% |
| Class B | 0.34% | 1.06% | 1.02% | 94% | 104% |
| Class C | 0.29% | 0.58% | 0.70% | 91% | 105% |
| Class D | 0.20% | 0.59% | 0.97% | 90% | 102% |
| Class E |  |  |  |  |  |
| Class F | 0.31% | 0.28% | 0.49% | 92% | 108% |
| Overall (Ref) | 0.25% | 0.64% | 0.73% | 92% | 105% |
| Overall (Ref-2) | -19.80% | -15.82% | -14.63% | 498% | 676% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.08% | 0.60% | 0.62% | 94% | 110% |
| Class C | 0.16% | 0.51% | 0.93% | 90% | 100% |
| Class D | 0.08% | 0.23% | 0.31% | 90% | 98% |
| Class E | 0.28% | 1.49% | 0.24% | 97% | 104% |
| Class F | 0.24% | 0.59% | 0.42% | 94% | 103% |
| Overall (Ref) | 0.16% | 0.64% | 0.52% | 93% | 103% |
| Overall (Ref-2) | -16.48% | -21.61% | -22.16% | 358% | 464% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.04% | 0.46% | 0.70% | 89% | 113% |
| Class C | 0.10% | 0.69% | 0.22% | 86% | 105% |
| Class D | 0.08% | 0.29% | 0.58% | 86% | 100% |
| Class E | 0.28% | 1.37% | 0.87% | 94% | 108% |
| Class F | 0.14% | 0.67% | 0.12% | 91% | 103% |
| Overall (Ref) | 0.12% | 0.65% | 0.49% | 89% | 106% |
| Overall (Ref-2) | -19.23% | -23.62% | -23.83% | 311% | 287% |

Table 6. JEM1 without COM16_C983_RSAF_PREVENT_OVERSMOOTHING macro

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.03% | 0.00% | 0.05% | 101% | 106% |
| Class B | 0.18% | 0.61% | 0.51% | 102% | 107% |
| Class C | 0.12% | 0.34% | 0.34% | 100% | 102% |
| Class D | 0.07% | 0.22% | 0.13% | 100% | 98% |
| Class E | 0.23% | 0.96% | 0.18% | 101% | 104% |
| Class F | 0.08% | 0.25% | 0.23% | 100% | 103% |
| Overall (Ref) | 0.11% | 0.38% | 0.25% | 101% | 103% |
| Overall (Ref-2) | -13.90% | -12.11% | -12.21% | 1963% | 155% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.06% | -0.22% | -0.20% | 102% | 101% |
| Class B | 0.08% | 0.18% | 0.01% | 103% | 104% |
| Class C | -0.01% | -0.18% | -0.24% | 101% | 106% |
| Class D | -0.04% | 0.02% | 0.29% | 100% | 99% |
| Class E |  |  |  |  |  |
| Class F | 0.05% | -0.13% | -0.24% | 102% | 112% |
| Overall (Ref) | 0.01% | -0.05% | -0.07% | 102% | 104% |
| Overall (Ref-2) | -20.00% | -16.38% | -15.31% | 551% | 671% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.02% | 0.34% | 0.16% | 101% | 104% |
| Class C | -0.02% | -0.15% | 0.28% | 100% | 100% |
| Class D | 0.00% | 0.45% | 0.53% | 100% | 97% |
| Class E | 0.19% | 1.35% | 0.19% | 100% | 104% |
| Class F | 0.12% | -0.05% | -0.22% | 101% | 106% |
| Overall (Ref) | 0.05% | 0.34% | 0.19% | 100% | 102% |
| Overall (Ref-2) | -16.57% | -21.83% | -22.41% | 386% | 458% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.02% | 0.10% | 0.06% | 101% | 110% |
| Class C | -0.04% | 0.13% | -0.36% | 100% | 103% |
| Class D | 0.03% | 0.75% | 0.45% | 100% | 100% |
| Class E | 0.11% | 1.38% | 1.21% | 100% | 105% |
| Class F | 0.13% | -0.07% | -0.31% | 102% | 108% |
| Overall (Ref) | 0.05% | 0.39% | 0.15% | 101% | 105% |
| Overall (Ref-2) | -19.29% | -23.82% | -24.08% | 352% | 286% |

Tools efficiency analysis is performed as described in [2]. The basic idea behind this evaluation is to normalize coding gains of different tools by their complexity. We perform similar analysis, but for All intra mode and both “tool-off” (see Table 7) and “tool-on” simulations (see Table 8).

However, in Table 7 and Table 8 weighting parameter w is different from the one used in [2], because all intra mode and random access have different encoder to decoder complexity ratios.

For Class B simulation for “on-tests” we obtained the following ratios:

;.

Evidently , and hence importance factor w should be shifted to encoder complexity. For all intra modes, we found value of w equal to 3 to be a reasonable balance to estimate improvement of encoding and decoding times.

Evaluation results presented in Table 7 include two MPI experiments, named “MPI2” and “MPI4”. The difference between MPI2 and MPI4 tools is analyzed in details in [1].

Table 7. Evaluation of tools efficiency, AI tool-on tests, first 30 frames simulations

| Tool test | BD-rate |  |  |  | Complexity |  |  | Slope |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | BDYUV | Enc | Dec | C | tan(Angle) | Angle (degrees) |
| MPI2 | -0.98% | -1.06% | -1.21% | 1.01% | 176.58% | 97.91% | 117.57% | 0.0578 | 3.3053 |
| MPI4 | -1.41% | -1.55% | -1.70% | 1.46% | 328.16% | 97.37% | 155.07% | 0.0266 | 1.5212 |
| PDPC | -1.36% | -1.33% | -1.39% | 1.36% | 191.03% | 101.23% | 123.68% | 0.0574 | 3.2867 |
| ARSS | -0.90% | -1.14% | -1.03% | 0.95% | 160.34% | 98.30% | 113.81% | 0.0687 | 3.9313 |
| ARSS w/o full RQT based estimation | -0.38% | -0.55% | -0.55% | 0.43% | 127.61% | 99.04% | 106.18% | 0.0688 | 3.9342 |

# Conclusions

Based on the simulation results, we conclude as follows:

- Our experimental results are close to JVET-B0022

- The efficiency of some intra-coding tools of JEM-1.0 is estimated using the methodology proposed in JVET-B0044 but for the AI configuration

- We recommend launching exploration experiments for the intra-coding tools to study the integration issues between them in JEM-1.0

# References

[1] E.Alshina, A.Alshin, K. Choi and M. Park, “Performance of JEM1.0 tools analysis by Samsung,” JVET-B0022, February 2016.

[2] H. Schwarz, C. Rudat, M. Siekmann, B. Bross, D. Marpe and T. Wiegand, “Coding Efficiency / Complexity Analysis of JEM 1.0 coding tools for the Random Access Configuration,” JVET-B0044, February 2016.

# Patent rights declaration

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).