| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0037 |
| --- | --- |

| Title: | Sequential/Parallel bistreams unification for JVET CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruslan Mullakhmetov Igor Sharonov Maxim Sychev | Email: | mullakhmetov.ruslan@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution provides method for exact comparing sequential and parallel simulations introduced by JVET-B0036 accurate to floating-point error and tool to concatenate output bitstreams from parallel test conditions introduced by JVET-B0036. Output of this tool is bitexact with output of sequential test conditions. With the help of this tool it is possible to measure accurate sequential decoding complexity, provide and transfer single output bitstream, playback it while using parallel encoding from JVET-B0036.

# Rationale

As it was stated in JVET-B0036 reference software simulations take significant amount of time, up to 1 month on new 4k sequences. A way to speed up simulation and parallelise it was proposed which gives quite similar coding gain numbers relative to sequential simulation. Unfortunately, using this method it is impossible to get accurate decoding complexity and single output bitstream for playback and transferring. A discussion and call for such concatenation tool took place in JVET mailing lists. Summarizing all, having such tool is useful for simulation process.

Also, original method has some deviations from corresponding sequential simulation due to rounding in logs, most of real values are shown with just 4 digit after point. This contribution provides additional full precision representation of PSNR, bitrate in logs and adopts aggregation scripts from JVET-B0036 to use this metrics, what removes any rounding steps in parallel simulation results calculation.

# Implementation of cat tool

Before release of JEM 2.0 is was impossible to implement such tool because of dependencies across random access points. This dependencies are removed in JEM 2.0 and concatenation is done by straightforward one-pass bit manipulations (filtering) on each random access segment (RAS) in terms of JVET-B0036 with final concatenation of filtered segments.

During filtering duplicated NALU (VPS, SPS, PPS, IDR, some SEI) is removed, for each frame POC is recalculated to reflect it actual position in bitstream.

Output of the tool taking encoded RAS is bitexact with output of corresponding sequential simulation. Full tests to check bitexact matching on all CTC sequences is in progress, the results will be provided in a future revision as soon as possible.

The tool is implemented as one-file C++ program with no external dependencies. Sources are included. Currently only JEM 2.0 bitstreams are supported. It is possible to adopt this tool for HM 16.9 bitstreams.

# Implementation of precise aggregation of parallel results

New information was added to JEM logs under summary sections containing PSNR, MSE, bitrate in full hexadecimal representation of IEEE 754 double floating point format, this is how JEM internally store this metrics, so no rounding take place. Example of this precise metrics see below.

SUMMARY --------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

49    a     100.3837   32.3969   38.8336   37.9851   33.6056

I Slices--------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

2    i    1267.2000   34.9092   39.0088   38.9925   35.8911

P Slices--------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

0    p      -1.#IND   -1.#IND   -1.#IND   -1.#IND   -1.#IND

B Slices--------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

47    b      50.7319   32.2900   38.8262   37.9422   33.5307

Precise All --------------------------------------------------------

Total Frames |          Bitrate           Y-PSNR           U-PSNR           V-PSNR          YUV-MSE MAXVAL

49    x 4059188e1b2ad740 404032ce018ba25b 40436ab4f07358f9 4042fe17e70da657 407c58f9e1c2ae75   1020

Precise Excluding First --------------------------------------------------------

Total Frames |          Bitrate           Y-PSNR           U-PSNR           V-PSNR          YUV-MSE MAXVAL

48    x 40528c4444444444 40402cb4a8432814 40436ad719958b38 4042fbf1896f6edc 407c924037237238   1020

There are two types of metrics, one for all frames and one for all but first, to take into account segment intersection on IDR-frames with exception on first segment (see JVET-B0036). Aggregation scripts are modified to use this precise values and calculate overall metrics with full precision. The only possible deviation with sequential results can come from non-associative nature of IEEE 754 arithmetic, i.e. (a + b) + c != a + (b + c), which is negligible.

It should be noted that methods of calculation per-component and total YUV sequence PSNR are different in JEM 2.0. For per-component PSNR each frame PSNRs are calculated and averaged over the entire sequence,

PSNRC= 1Ni=1N10logMAX2MSEiC,

where N is number of frames.

YUV-PSNR is calculated as PSNR of entire sequence as single indivisible signal, see below. Therefore it is impossible to calculate YUV-PSNR of whole sequence from YUV-PSNR’s of segments, to overcome this problem YUV-MSE is output to log, which is averaged easily, final PSNR is calculated in averaging scripts.

PSNRyuv= 10logMAX21Ni=1NMSEiyuv.

Parallel and sequential simulation results are enclosed as csv tables, both original and stripped from unnecessary data to use any text comparison tool. YUV-metrics differ because of original using YUV-PSNR as source of aggregation, which is not working. Simulations using MSE metric as source of YUV-PSNR are in progress.

# Conclusion

This proposal provides the following:

- Tool for concatenation of stream encoded in parallel into stream exactly the same as sequential

- Way to evaluate sequential decoding complexity in parallel test condition

- Method to get single stream for playback, transferring etc. after parallel simulations.

- Method for aggregating parallel simulation results exact with sequential accurate to floating-point error.

This contribution is proposed to be used in JVET-CTC for JEM 2.0 and above.

# References

[1] JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 10 (HM10) Encoder Description”, JCTVC-L1002, JCT-VC Meeting, Geneva, Jan 2013

[2] ITU-T H. 265: High efficiency video coding[S]. ITU-T Recommendation H.265, Jan.2013, p68, p79

[3] Karsten Suehring, Xiang Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 2nd Meeting: San Diego, USA, 20–26 February 2016.

[4] X. Ma, H. Chen, H. Yang “Simplification of the common test condition for fast simulation”, JVET-B0036, San Diego, USA, 20–26 February 2016.

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).