| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0040 |
| --- | --- |

| Title: | Adaptive Clipping in JEM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, P. Bordes, F. Le Léannec 975 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 31 98 {franck.galpin, philippe.bordes fabrice.leleannec}@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In this contribution, an improved clipping process in the whole JEM2.0 coding and decoding process is proposed.

The clipping bounds are determined from the original signal characteristics and encoded in the bitstream.

BD-rate savings of 0.35% is reported in Random Access Configuration, 0.17% in All-Intra and 0.23% in low-delay B. No encoding or decoding runtimes increase is obtained compared to the JEM2.0 anchor.

# Introduction

In JEM-2.0 [1], the clipping bounds depend on the internal bit depth used in the coding and decoding processes. Depending on the video content characteristics, these default clipping bounds may be significantly different from the actual minimum and maximum sample values of the pictures.

In this contribution, it is proposed to use clipping bounds that are determined at the encoder side and are explicitly coded in the bit-stream.

Clipping bounds are signaled in a way that minimizes the bitrate overhead.

# Technology description

In JEM2.0 [1], the clipping operation that is used in the various steps of the coding/decoding process is the following:

Tclip =ClipBDT, bitdepth=Clip3 0 , (1<<bitdepth)-1 , T

Where Tclip is the resulting clipped value, the function Clip3(min,max,T) clips the value T between the bounds min and max, and bitdepth corresponds to the internal bit depth used for the current component type.

Here it is proposed to replace the above clipping function by the following component-wise adaptive clipping:

Tclip = ClipBDT, bitdepth,C=Clip3  minC, maxC , T

Where:

- C is the component ID (typically Y, Cb or Cr)

- minC is the lower clipping bound used in current slice for component ID C

- maxC is the upper clipping bound used in current slice for component ID C

The clipping bounds may change from slice to slice. In every step of the codec where a clipping is performed between 0 and (1<<bitdepth-1), the proposed adaptive clipping process is used instead.

Adaptive clipping is computed at the encoder side for each component of each slice, as the minimum and maximum sample values of the current input picture.

The coding of the clipping bounds is done as follows. Clipping bounds are either intra coded (they are called reference clipping bounds in the following) or are coded in a predictive way based on the last intra coded clipping bounds. The intra or predictive mode for bounds coding is decided on the encoder side, and is signaled with a flag. Typically, intra coding of clipping bounds occurs in intra pictures.

- The intra coding of clipping bounds employs a fixed length code. The upper bound is coded related to the value(1≪internalBitDeth). Reference clipping bounds can be coded either in the PPS or slice header, and include 6 clipping bounds: minY, maxY, minCb, maxCb, minCr, maxCr.

- Predictive-coded clipping bounds are coded in the slice header, with a signed VLC code. The coded values correspond to the difference of the current clipping bounds with the reference clipping bounds.

In this encoder implementation, the intra coding of clipping bounds is used for the slices coded in Intra and the reference clipping bounds are the last clipping values coded in intra. However it would be probably more efficient to store the reconstructed clipping bounds with the reference pictures in the DPB and use the clipping bounds corresponding to the reference picture with closest POC as reference clipping values.

To reduce the bitrate overhead of clipping bounds signaling, bounds can be quantized and the range of the clipping bounds values can be limited.

To further benefit from the proposed content adaptive clipping mechanism, the encoding process is modified. A residual can be modified to take into account the reconstructed signal is likely to be clipped. Practically, for a residual block to transform, quantize and encode, if the corresponding original block contains some samples equal to the clipping bounds, then the residual block undergoes a smoothing process in the pixel domain. This leads to slightly improved coding efficiency.

Also, to limit the bitrate overhead of clipping bounds signaling, the proposed adaptive clipping is de-activated in slices where only a small amount of original samples are close to the minimum or maximum in the considered slice.

# Simulation results

The proposed tool has been implemented in the JEM2.0. Table 1summarizes the performances obtained with the proposed tool, combined with JEM2.0 tools activated by default as specified in [2].

BD rate savings are observed in all configurations, in all classes. In particular, the proposed method provides 0.35% gain over the anchor in RA configuration. Full results are provided in the aside Excel file. One notices strong coding gain on some sequences.

In terms of encoding and decoding runtime, the impact of the proposed method onto the JEM2.0 is negligible. Here JEM2.0 anchors have been run on the same platform, which makes the runtime comparison reliable.

Table 1: performances of the adaptive clipping process

# Conclusions

An adaptive clipping method is proposed. The clipping between 0 and ((1<<internalBitDepth)-1) is replaced by the clipping between the actual original signal bounds, respectively for each component. Each component’s clipping bounds are signalled in the slice header, and can be intra- or predictively coded.

Compression efficiency is improved for all classes and all configurations, with some coding gains higher than 2% on some sequences.

In terms of coding/decoding runtime, a negligible impact on the JEM2.0 is obtained.

Based on the trade-off obtained between coding efficiency and coding/decoding runtime, it is proposed to integrate the proposed tool into the main branch of JEM2.0.

# References

- JEM-2.0, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0/

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 3rd Meeting

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).