| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET- C0050 |
| --- | --- |

| Title: | Test sequence formats for virtual reality video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kwang Pyo Choi Zakharchenko Vladyslav Minseok Choi Elena Alshina | Email: Email: Email: Email: | kp5.choi@samsung.com vlad.zak@samsuung.com minseok48.choi@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution provides s an informative comparison on different Virtual Reality (VR) video sequences for the Future Video Coding Standardization activity. As the rendering process for VR compression is device specific it is proposed to revise most popular 360-degree video sequence formats and add comparison information on other possible solutions for this approach. It is suggested that this comparison would help in development of video codec for 360-degree VR content.

Introduction

Capturing and rendering process for the 360-degree VR video is significantly different from the conventional video content processing pipeline [1]. In case of conventional video one camera is used to capture the content that will be displayed to user, however in case of 360-degree video for virtual reality content is usually captured by two or more cameras, each facing in its own direction with a following stitching and rendering to a video codec conformant format, sometimes mentioned as sphere-to-2D image projection.

The major difference of 360-degree video format is that in most cases user can see only a small part of the entire image using HMD (for instance Oculus Rift or GearVR) or specialized application as video player (Samsung Milk VR [2], GoPro VR App) and controls a view by directly moving head or using others specialized controls provided by the environment. Display resolution in modern HMD devices exceeds FullHD resolution and tends to grow further. As a result image representing the whole 360 degree space often may exceed 6K or even 12K resolution.

As a result there is a demand for more efficient way to represent and compress 360-degree VR video content which allows people to experience VR in high resolution.

Representation of 360-degree VR video

Equirectangular projection (ERP)

The most common equirectangular projection method for 360-degree video applications is similar to a solution used in cartography to describe earth surface in a rectangular format on a plane. This type of projection has been widely used in computer graphics applications to represent textures for spherical objects and has gained recognition in gaming industry. Although it perfectly compatible with a synthetic content in case of natural images this format is facing several problems. Equirectangular projection is known for simple transformation process; however different latitude lines have different stretching due to transformation process. In this rendering method the equator line has minimal distortions or is free of distortions while poles area has a maximum stretching and suffers from maximal distortions.

It’s obvious that resolution preserving translation of an image from a spherical surface, which natively represents 360-degree video content, to the plane using equirectangular projection method results in pixel count increase.

| # of vertexes : 2000 ~ 9000 | Equirectangular Projection |
| --- | --- |

In terms of rendering process equal area patches on a plane should be mapped to variable area patches on the sphere model, subdivided into a number of variable size triangle elements. Generally if the sphere has less than 2000 triangles projection distortions result in a significant viewing discomfort, especially in polar areas. Although creating a highly detailed subdivided sphere does not create difficulties for a personal computer as there is no dependency on power required for graphics processing unit operation; mobile devises such as tablets and smartphones consume a lot of battery power. It is considered that different rendering methods utilizing constant geometry and simple mapping process allow reducing energy consumption. Needless to say that simple transformation functionality is friendly to be applied as a hardware implementation.

Platonic solid projection (PSP)

Recently Facebook Company proposed to use cubemap method of platonic solids [3] and provided a conversion tool to represent 360 degree VR contents [4].

We offer comparison of the different projection methods using platonic solids to represent each face. Platonic solid is a convex polyhedron that is constructed of regular polygonal faces, which means that in every vertex the same number of equal polygons meet. Only five solids meet this criterion.

Process of building environment for 360-degree space has to be conformance rendering method. From a graphics processing device perspective the environment consists of triangular primitives that make platonic solids a natural selection candidate in general for building VR environment. The fact that each face is equal helps to minimize scaling distortions in rendering process because the projected part in image comply with a 3D model geometry and can be directly mapped without additional transformations such as image warping or interpolation

Therefore, we can consider a following constrain: platonic solid has to produce a virtual environment, equivalent to a sphere with a unit radius to eliminate possible distortions as the following table.

| Projections | 3D Model | 2D Projection | Vertexes | Area Ratio |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | vs. Sphere | vs. ERP |  |
| Tetrahedron (4 faces) |  |  | 4 | 3.31x | 2.11x |  |
| Cube (6 faces) |  |  | 8 | 1.91x | 1.22x |  |
| Octahedron (8 faces) |  |  | 6 | 1.65x | 1.05x |  |
| Dodecahedron (12 faces) |  |  | 20 | 1.32x | 0.84x |  |
| Icosahedron (20 faces) |  |  | 12 | 1.21x | 0.77x |  |

Especially, among all the platonic solids, cube and icosahedron have some preferable characteristics.

Using cube for rendering mixed content that comprises of CG environment and real-life content for different faces can significantly reduce rendering complexity even though it has higher resolution than equirectangular projection. At the same time 6 camera 360-degree video capturing platform can naturally be mapped to a cube with a significantly lower stitching and mapping complexity.

According to the table provided above, icosahedron could lead to high compression ratio because it has the smallest surface area among all platonic solids, compared to a sphere, and at the same time provides lower resolution of the same pixel density, compared to equirectangular projection.

Proposed test sequences for VR video

There are 3-types actual serviced is used for sharing and editing 360-degree image and video contents. Equirectangular is widely used by services as YouTube to deliver content VR video content. Different variations of a cube projection recently had been promoted by Facebook for social network integration. Icosahedral projection has been recently introduced by Samsung Milk VR service as an alternative rendering method for high quality content distribution over mobile networks to smartphone devices.

In this proposal, we propose to use test sequences in 3-types of VR video formats, known as equirectangular, cubic, and icosahedron for. Coding tools in existing video compression standards, such as HEVC and AVC, will work well regardless of the formats. The areas marked in black region would be not-coded or skip blocks and barely increase the bitstream size or complexity. Tetrahedron, octahedron, and icosahedron would have the same coding property because the element of these projections is triangle. Bit-depth of the proposed test sequences are 8-bit for now, it could be increased to 10-bit by bit padding

Equirectangular format

An example image of equirectangular format is shown below:

Compression efficiency of the equirectangular projection can be increased by tuning rate control algorithm to reduce redundancy in polar areas [7].

Cubic format

An example image of cubic format is shown below:

It was reported in [5] that cubic format can be converted from equirectangular format. They did this by transforming the top 25 percent of the equirectangular video to one cube face and the bottom 25 percent to another, dividing the middle 50 percent into the four remaining cube faces, and then later aligning them in two rows.

As a result of [5], a bitrate savings of up to 25% (compared to equirectangular) was reported, but image degradation caused by reduced pixel density has to be considered. A simple conversion tool [4] was shared that transforms video from equirectangular layout to cubic layout. The layout generated by this tool outputs six faces of the cube in the following format:

Icosahedron format

An example image of icosahedron format is shown below:

In similar to cubic format, icosahedron format can be converted from equirectangular format. Equal quality video compared to equirectangular projection 23% bitrate savings can be achieved due to reduced pixel area.

With 360-degree stitched sequences provided by [3] in equirectangular format, we converted these videos to icosahedral format and the following sequences are being provided for testing video coding performance.

| Sequence | Resolution | # Frames | FPS | Duration |
| --- | --- | --- | --- | --- |
| abyss_great_shark | 3840x1816 | 1145 | 30 | 38s |
| ballooning | 3840x1816 | 1501 | 25 | 60s |
| bicyclist | 3840x1816 | 1500 | 25 | 60s |
| glacier | 3840x1816 | 1440 | 24 | 60s |
| paramotor_training | 3840x1816 | 1279 | 25 | 51s |
| skateboarding | 4096x1936 | 1372 | 60 | 22s |
| ski_downhill | 3840x1816 | 1799 | 30 | 60s |
| timelapse_basejump | 3840x1816 | 330 | 25 | 13s |
| timelapse_building | 3840x1816 | 327 | 25 | 13s |

Copyright information

Since the proposed content in this proposal has been provided by [3], the contents follow copyright information of [3].

For icosahedron format sequence generated from [3]’s test sequences;

The following uses are allowed for the proposed sequences for icosahedron format sequence:

- Can be published in technical papers, played at technology research and development events.

- Can be used by Standards committees. (e.g., ITU, MPEG, VQEG,).

The following uses are not allowed for the proposed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

Conclusion

In this contribution, we overview three rendering formats for VR test sequences. Considering the fact that the three formats are being used widely in VR streaming services, it would be desirable to use VR test sequences in those three formats for identification benefits of different rendering processes and potential development VR specific coding technologies during the development of next-generation video coding standard.

References

- J. Ridge, M.M. Hannuksela, “Virtual reality requirements for future video coding” 53rd Meeting: 20–26 February 2016, San Diego, US (VCEG-BA07)

- Samsung VR distribution platform (https://samsungmilkvr.com)

- A. Abbas, “GoPro test sequences for Virtual Reality video coding”, ” 54th Meeting: May 2016, Geneva (JVET-C0021)

- ERP to Cube conversion tool by Facebook. (https://github.com/facebook/transform)

- “Next-generation video encoding techniques for 360 video and VR” (https://code.facebook.com/posts/1126354007399553/next-generation-video-encoding-techniques-for-360-video-and-vr/)

- “N15452: Call for Test Materials for Future Video Coding Standardization“, Warszawa, 2015.

- M. Budagavi et. al., “ 360 degrees video coding using region adaptive smoothing”, The International Conference on Image Processing, pp750-754, 2015

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).