| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0051 |
| --- | --- |

| Title: | The cross-check for proposed in JVET-C0022 improvements to the Adaptive multiple Core transform |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | E. Alshina | Email: | elena_a.alshina@samsung.com |
| Source: |  | Samsung Electronics, Ltd. |  |  |

____________________________

Abstract

This contribution verifies the performance of new multiple core transforms proposed by Orange in JVCT-C0022. Two more transform kernels are added to the transforms dictionary. Additionally transform selection based on Intra prediction direction is modified on a way to provide wider variety of combinations. At the same time the number of transforms per prediction mode is kept unchanged. Both modifications together provide 0.4% average gain in AI w/o additional search complexity.

1. Introduction

Adaptive multiple core transforms was proposed by Qualcomm and adopted to the HEVC-KTA at Geneva VCEG meeting [1], and later was became the part of JEM [2].

Verified contribution proposes modification of Adaptive multiple core transform for Intra residual only. There are five core transforms in JEM, they compose 3 transform sets for Intra residual coding. Selection of transform set is based on Intra Prediction Mode information.

Three pre-defined transform candidate sets in JEM2.0

| Transform Set | Transform Candidates |
| --- | --- |
| 0 | DST-VII, DCT-VIII |
| 1 | DST-VII, DST-I |
| 2 | DST-VII, DCT-VIII |

In verified contribution 2 more transform cores are added:

- DST-IV transform;

- ID transform.

The later one is just scaling, which is interesting to see.

Transform dictionary selection in proposed version depends on both Intra Prediction direction and TU size, overall variety of transforms combinations has been increased significantly. At the same time the number of transforms per prediction mode is kept unchanged. It allows keeping syntax and encoder search the same as in JEM2.0.

We can confirm that code modifications are minor. The biggest change is LUT table for mapping between Intra Prediction Mode, TU size and Transform Dictionary which goes from 136 (JEM2.0) to 2720 bytes. But this mapping table size is negligible compared to memory required to the full size multiple core transform support.

It is highly desirable to specify the rules for mapping between Intra Prediction Direction Mode and TU size to the Transforms set instead of using LUT.

Test results

We ran simulations under JVET common test condition [3]. Test results are summarized in Table below. We don’t have reliable time measurement in RA test since we use parallel encoding. But from code analysis we can confirm that encoding and decoding time should not increase compare to the JEM2.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM2.0 (sequential) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,15% | -0,20% | -0,06% | 101% | 97% |
| Class A2 | -0,31% | 0,01% | 0,02% | 103% | 94% |
| Class B | -0,33% | -0,28% | -0,21% | 103% | 104% |
| Class C | -0,45% | -0,19% | -0,18% | 102% | 104% |
| Class D | -0,48% | -0,29% | -0,23% | 102% | 102% |
| Class E | -0,53% | -0,31% | -0,34% | 104% | 110% |
| Overall | -0,37% | -0,21% | -0,16% | 102% | 101% |
| Class F (optional) | -1,32% | -0,54% | -0,51% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over JEM2.0 (sequential) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,10% | 0,20% | -0,02% | #NUM! | #NUM! |
| Class A2 | -0,39% | -0,17% | -0,03% | #NUM! | #NUM! |
| Class B | -0,17% | 0,19% | 0,11% | #NUM! | #NUM! |
| Class C | -0,24% | 0,04% | -0,07% | #NUM! | #NUM! |
| Class D | -0,25% | -0,21% | -0,27% | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0,23% | 0,02% | -0,05% | #NUM! | #NUM! |
| Class F (optional) | -1,09% | -0,34% | -0,61% | #NUM! | #VALUE! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over JEM2.0 (sequential) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | -0,14% | 0,12% | 100% | 100% |
| Class C | -0,11% | 0,02% | -0,24% | 102% | 100% |
| Class D | -0,02% | -0,90% | -0,55% | 101% | 102% |
| Class E | -0,14% | -0,23% | -0,65% | 102% | 99% |
| Overall (Ref) | -0,07% | -0,31% | -0,28% | 101% | 100% |
| Class F (optional) | -0,67% | 0,07% | -0,32% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over JEM2.0 (sequential) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | -0,04% | -0,27% | 102% | 99% |
| Class C | -0,08% | -0,14% | -0,15% | 103% | 105% |
| Class D | -0,11% | -0,29% | 0,48% | 101% | 103% |
| Class E | -0,05% | -0,27% | -0,60% | 104% | 110% |
| Overall (Ref) | -0,07% | -0,17% | -0,11% | 102% | 103% |
| Class F (optional) | -0,63% | 0,08% | -0,17% | 102% | 105% |

Conclusion

Verified proposal on adaptive core transform proposal modifications provides 0,4% (AI) / 0,2%(RA) / 0,1%(LDB) / 0,1% (LDP) performance gain over JEM2.0 w/o increment encoding and decoding run time. Proposal is natural extension of JEM2.0 tools set.

References

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding”, ITU-T SG16 Doc. COM16–C806, Feb. 2015.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm description of Joint Exploration Test Model 2”, JVET-B1001, San-Diego, Feb. 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

Patent rights declaration(s)

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).