| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0099 |
| --- | --- |

| Title: | Further JVET CTC simplification for RA |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Sychev Maxim | Email: | Sychev.Maxim@huawei.com |
| Source: |  | Huawei |  |  |

____________________________

# Abstract

This contribution suggests to apply one modification to current JVET CTC for further simplification of RA simulations.

# Introduction

In contribution JVET-B0036 the parallel simulations of test sequence segments was proposed for RA configuration. It allows to reduce encoding simulations ~5 times. Unfortunately, we still have a bottleneck in RA simulations related to hardly predictable content (Class A1 / ”ToddlerFountain”). Table 1 demonstrates RA segments with highest encoding time.

Table 1. Top encoding time of RA segments for JVET CTC.

As can be observed that most segments with top encoding time belong to one sequence: ToddlerFountain from Class A1 (marked with red color).

Examples of this disadvantage were provided by most of the companies:

- JVET-C0039, Technicolor:

RA configuration is still running at the time the contribution is uploaded, and will be provided in a revised version of the contribution.

- JVET-C0042, Sharp:

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | ## | ## | ## |  |  |
| Class A2 | -0.47% | -0.66% | -0.89% | 113% | 106% |
| Class B | -0.21% | -0.29% | -0.56% | 111% | 103% |
| Class C | -0.32% | -0.43% | -0.47% | 113% | 102% |
| Class D | -0.25% | -0.46% | -0.38% | 113% | 103% |
| Overall | ## | ## | ## | ## | ## |
| Class F (optional) | -0.34% | -0.52% | -0.69% |  |  |

- JVET-C0045, SONY:

- JVET-C0047, InterDigital:

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -1.00% | -0.97% | -0.99% |
| Class C | -0.79% | -0.62% | -0.81% |
| Class D | -0.79% | -0.64% | -0.66% |
| Class E |  |  |  |
| Avg. |  |  |  |
| Enc. Time [%] |  |  |  |
| Dec. Time [%] |  |  |  |

- JVET-C0062, Qualcomm:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.15% | -0.18% | -0.05% | 109% | 100% |
| Class C | -0.13% | -0.13% | -0.21% | 107% | 102% |
| Class D | -0.12% | -0.28% | -0.25% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall (Ref) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F (optional) | -0.61% | -0.70% | -0.50% | 108% | 102% |

- JVET-C0063, Qualcomm:

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.58% | 1.49% | 1.75% | 92% | 100% |
| Class C | -0.93% | 1.22% | 1.37% | 93% | 101% |
| Class D | -0.48% | 1.50% | 1.41% | 93% | 101% |
| Class E |  |  |  |  |  |
| Overall (Ref) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | -0.75% | 0.25% | 0.11% | 94% | 101% |

- JVET-C0068, Ericsson:

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -0.7% | -0.7% | -0.6% |
| Class C | -0.6% | -0.6% | -0.7% |
| Class D | -0.6% | -0.6% | -0.7% |
| Class E |  |  |  |
| Avg. |  |  |  |
| Enc. Time [%] |  |  |  |
| Dec. Time [%] |  |  |  |

- JVET-C0069, NHK:

| Class | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | V | U | V |
| ClassA1 | TBP | TBP | TBP |
| ClassA2 | TBP | TBP | TBP |
| ClassA | -0.13% | -0.35% | -0.37% |
| ClassB | -0.12% | -0.15% | -0.21% |
| ClassC | -0.06% | -0.13% | -0.14% |
| ClassD | -0.05% | 0.19% | -0.32% |
| ClassE | -0.24% | -0.50% | -0.48% |
| ClassF | -0.06% | -0.17% | -0.21% |
| Average | -0.11% | -0.17% | -0.28% |
| Enc Time | 100% |  |  |
| Dec Time | 103% |  |  |

# Proposed solution

Proposed to change Intra period (Ip) only for one sequence in Class A1 – “ToddlerFountain” from 64 frames to 48 (at least). The actual framerate of this sequence is 60. To adjust Intra period for one second and to multiple of 16 (new GOP size) the 64 frames was used. It is exceed one second for ~7%. Suggested Ip is less than one second on 20%. But less Ip even better for quality of service (for RA application scenario). Amount of segments for ToddlerFountain increased from 5 to 7 for each QP simulation.

After applying lower Ip to ToddlerFountain we have the following distribution of highest encoding time:

Table 2. Top encoding time of RA segments for JVET CTC.

The longest encoding time was reduced from 4 days to 3 days as was expected according to proposed changes. More reduction is possible if needed (Ip = 32 for example).

The new Ip was simulated and compared with current:

Table 3.

# Conclusion

Actually, the proposed change for Intra period does not break the rule for RA – smaller period is better for video service. Reduction of Intra period allows to reduce the encoding simulation time from 4 to 3 days (25% of speedup) with parallelization. It helps to provide results (including averages) quicker. Lower intra periods for other sequences also can be considered.

This contribution is proposed to be applied to JVET-CTC.

# References

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 10 (HM10) Encoder Description”, JCTVC-L1002, JCT-VC Meeting, Geneva, Jan 2013

- ITU-T H. 265: High efficiency video coding[S]. ITU-T Recommendation H.265, Jan.2013, p68, p79

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 2nd Meeting: San Diego, USA, 20–26 February 2016.

- X. Ma, H. Chen, H. Yang “Simplification of the common test condition for fast simulation”, JVET-B0036, San Diego, USA, 20–26 February 2016.

# Patent rights declaration(s)

Huawei have no current or pending patent rights relating to the technology described in this contribution.