| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0010 | Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0010 |
| --- | --- | --- | --- |

| Title: | Exploration Experiments on Coding Tools Report. |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | E. Alshina, J. Boyce, Xiaoyu Xiu, Yao Jen Chang, Vadim Seregin, S.-H. Kim | Email: | elena_a.alshina@samsung.com jill.boyce@intel.com xiaoyu.xiu@interdigital.com yjchang@itri.com vseregin@qti.qualcomm.com kimse@sharplabs.com |
| Source: |  | EE coordinators |  |  |

____________________________

# Abstract

Nine experiments on coding tools were agreed to carry out between JVET-C and JVET-D meetings in order to get better understanding of technologies considered for inclusion to the next version of JEM, analyse and verify their performance, complexity and interaction with existing JEM tools. This report summarises the status of each experiment.

# Introduction

JVET group coordinated experiments were conducted following principles were agreed in [1].

Key announcement related to EE activities were done using JVET reflector. Mainly discussion was carried on EE mail list which has 66 subscribers from 18 companies.

Exploration experiments were conducted according to the agreed during JVET-C time-line:

T1= 3 weeks after JVET-C meeting = June 30: EE description was finalized.

T2 = JEM3.0 SW release + 4 weeks= July, 28: Initial integration for all tools into separate EE branch of JEM is available.

T3: JVET-D meeting start – 3 weeks = September 26: Final version for each tool is available, start of official cross-check.

Since EE9 was JEM3.1 (not JEM3.0) based and test materials for EE9 need to be defined by AhG7 only preliminary results are available for EE9 and it was decided not to request cross-check for those test at this stage, considering EE9 results for information only.

Table 1. Summary of Exploration Experiments.

| # | Tests and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 1 | Secondary transform (HyGT) and combination of PDPC and NSST - HyGT kernel for 44 and 88 2nd transforms | JVET-D0120 (Qualcomm) | AI: -1.1% (ET 0.99, DT 1.00) RA: -0.6% (ET 1.00, DT 1.00) | JVET-D0055 Samsung JVET-D0116 Sony |
|  | - Decoupling PDPC and NSST | JVET-D0168 (Sharp) |  | Not needed |
| 2 | Adaptive primary transform - 2 new transforms (DST-IV and ID) | JVET-D0065 (Orange) | AI: -0.3% (ET 1.01, DT 0.99) RA: -0.1% (ET 1.01, DT 0.96) | JVET-D0066 Technicolor JVET-D0096 Samsung JVET-D0091 Sony |
|  | - Transforms flag is signaled per area | JVET-D0112 (Qualcomm) | AI: -0.2% (ET 1.04, DT 0.97) RA: -0.1% (ET 1.0, DT 1.0) |  |
| 3 | Generalized bi-prediction (*) | JVET-D0102 (Inter Digital) | RA: -0.8% (ET1.70, DT1.02) LD: -0.9% (ET2.00, DT1.00) | JVET-D0136 Sharp JVET-D0141 Samsung |
| 4 | Improved affine motion prediction | JVET-D0121 (Qualcomm) | RA: -0.3% (ET 1.06, DT 1.00) LD: -0.3% (ET 1.11, DT 1.00) LDP: -0.3%(ET 1.10, DT 1.00) | JVET-D0057 Samsung |
| 5 | Improved MV coding (*) - MV sign info hiding - MV candidates list modification for bi-pred - Int-pel and half-pel MV rounding | JVET-D0062 (Ericsson) | RA: -0.4% (ET 1.20, DT 1.0) LD: -0.1% (ET 1.28, DT 1.0) LDP: -0.1% (ET 1.00, DT 1.0) | JVET-D0062 Samsung JVET-D0084 Qualcomm |
| 6 | Extended intra prediction reference - ARTIP (prediction from one of the multiple tiers which is adaptively selected) | JVET-D0099 (ITRI) | Adaptive selection between 4 tiers AI: -0.9% (ET 2.03, DT 1.03) RA: -0.6% (ET 1.25, DT 0.99) | JVET-D0139 InterDigital JVET-D0106 Sony JVET-D0137 Sharp JVET-D0147 Qualcomm |
|  | - MLIP (prediction from multiple tiers) (*) | JVET-D0149 (Microsoft) | Prediction from 4 tiers AI: -0.3% (ET 2.6, DT 1.0) RA: -0.0% (ET 1.3, DT 1.0) |  |
| 7 | Adaptive clipping - With and w/o prediction for clipping values - As post-filter only | JVET-D0033 (Technicolor) | AI: -0.2% (ET 0.97, DT 1.05) RA:-0.4% (ET 0.99, DT 1.01) LD: -0.3% (ET 1.00, DT 0.95) LDP:-0.4% (ET 0.99, DT 1.00) | JVET-D0063 Orange JVET-D0094 Samsung |
| 8 | Decoder side intra mode derivation - Different sizes of template for matching (L) - Restriction of the tool for specific block sizes | JVET-D0097 (InterDigital) | AI: -0.8% (ET 1.64, DT 1.21) RA: -0.5% (ET 1.15, DT 1.01) | JVET-DXXXX Qualcomm JVET-D0059 Samsung JVET-D0041 Huawei JVET-DXXXX Sharp JVET-D0130 ITRI |
| 9 | Adaptive Scaling for Extended Colour Volume Material | JVET-D0118 Qualcomm JVET-D0124 Sharp |  |  |

Comments:

.(*) still partial results only are available (Oct. 12), to be up-dated

# List of Experiments

## EE1: Secondary transform (HyGT) and combination of PDPC and NSST

This experiment is targeting exploration following modification of secondary transforms proposed in 3 contributions:

- New secondary transform kernel, so-called Hypercubic Givens transform (HyGT) as a replacement of the current JEM secondary transforms. This includes:

  - new 44 secondary transforms;

  - new 88 secondary transforms;

  - Secondary transform is not applied to LM chroma mode and secondary transform index is not signaled.

  - 2 or 4 secondary transform candidates are used

- Decoupling PDPC and NSST by removing the bit-stream restriction currently precluding enabling NSST and PDPC at the same time.

Questions recommended to be answered during EE tests.

[Q]: What is the performance effect of 44 HyGT vs existing JEM secondary transforms?

[A]: 0.0%(AI) / 0.0% (RA) (EE1-T1), similar performance for Chroma.

[Q]: What is the memory usage for storage of 44 HyGT compared to existing JEM secondary transforms?

[A]: Existing JEM secondary transforms (all are 44 ) require 10368  bytes. HyGT 44 implemented using 8264 bytes.  Please note JEM 2nd transforms are implemented as matrix multiplication, HyGT is series of permutations and Civen’s rotations (so memory size is smaller).

[Q]: What is the performance effect from replacement of 44 HyGT by 88 HyGT in large blocks?

[A]: 1.1%(AI) / 0.6% (RA) (EE1-T1), similar gain is observed for Chroma.

[Q]:What is the memory usage for storage of 88 HyGT compared 44 HyGT?

[A]: HyGT 88 implemented using 85680 bytes

Conclusion: replacement of existing secondary transform design by combination of 44 HyGT for small blocks and 88 HyGT for larger blocks provides 1.1% (AI ) and 0.6% (RA) gain w/o noticeable run-time increment. Memory usage for secondary transform matrices storage increases from 10368  bytes (in JEM3.0) up to 93944 bytes.

## EE2: Adaptive primary transform

This experiment is targeting to the exploration of following modification of Adaptive multiple core transforms suggested:

- Two additional core-transforms kernels:

  - DST-IV (which is similar to DST-VII but has a fast algorithm);

  - ID (no transform, but not TS as it can be selected separately per direction).

- Adaptive Multiple Transforms flag signaling per area, size of which can be configured and sent in slice header.

Questions recommended to be answered during EE tests.

[Q]: What would happen when just replacing DST-VII by DST-IV in current configuration?

[A]: EE2-T1 gives an answer in case of AI configuration: 0.1% Luma gain is observed is DST-VII is replaces by DST-IV for Intra (for Inter DST-VII still used).

[Q]: What is the logic of the mapping between intra prediction mode, transform size and transform kernel (is it possible to make it symmetric)?

[A]: It is symmetrical in EE test.

[Q]: What is the performance impact of block-size dependency?

[A]: Difference between EE2-T4 and EE2-T5 gives an answer. W/o block size dependency gain is 0.1%(AI)/0.1%(RA). With block size dependency gain is 0.3%(AI)/0.1%(RA).

Conclusion: addition of 2 primary transform to the set of adaptive transforms of JEM3.0 provides 0.3% (AI) and 0.1% (RA) gain w/o noticeable run-time increment. Memory usage for primary transform matrices storage increases from 54585bytes (in JEM3.0) up to 61405bytes.*

*based on discussion among EE mail list subscribers.

## EE3: Generalized bi-prediction

This experiment is targeting to the exploration of generalized bi-prediction (GBi) technology. GBi relaxes the restriction that weights used for bi-prediction should be 1/2.  It allows bi-prediction weights to be signaled at the block level for bi-predicted blocks:

P[x] = (1 - w) * P0[x + v0] + w * P1[x + v1].

The candidate set of weights in the GBi mode comprises 7 weights in total (including 0.5 which corresponds to the conventional bi-prediction mode). These weights are arranged from the lowest to the highest in the candidate set,  W={-1/4, 1/4, 3/8, 1/2, 5/8, 3/4, 5/4}. The binarization of syntax element for the weigh depends on mvd_l1_zero_flag. The combination with Local Illumination Compensation and BIO and the advanced temporal prediction techniques in JEM is provided. Adjustment to BIO is likely needed.

Questions recommended to be answered during EE tests.

[Q]: What is the meaning of the negative weight in bi-prediction?

[A]: Difference between EE3-T3 and EE3-T2 gives an answer about performance impact of negative weights in generalized bi-prediction. In LD, coding gain almost doubles when negative weight is used. Performance impact is about 0.1% (RA) and 0.4% (LD).

[Q]: How does gain grow with additional new weights (one-by-one)?

[A]: Sequence of tests EE3-T1, EE3-T2 and EE3-T3 gives an answer. For RA 0.6% (add 1 pair of weights)    0.7% (add 2 pairs of weights)   0.8% (add 3 pairs of weights). For LD, 0.4% (add 1 pair of weights)  0.5% (add 2 pairs of weights)  0.9% (add 3 pairs of weights).

[Q]: Is it possible to reduce encoder complexity by using fewer weights or restricting some of those at encoder side?

[A]: Encoder run time in original contribution (3 additional pairs of weights) was 3.2, in EE version Enc. run time is 1.7.

Conclusion: Adding 1 pair of weights for bi-prediction 0.5% (RA) gain and increases Enc. run time by roughly 20%. Next additional pairs of weights increases gain roughly by 0.1% with 20% Enc. run time increment.  Negative weight almost doubles the coding gain (from 0.5% to 0.9%) n LD.

## EE4: Improved affine motion prediction

The 6-parameter affine motion model is used in addition to the existing 4-parameter affine motion model. Order of candidates in affine merge mode has been modified in original proposal but discarded during this EE.

Questions recommended to be answered during EE tests.

[Q]: What if we just replace 4-parameters affine model by 6-parameters model?

[A]: There is no performance improvement.

[Q]: What is the impact of change the order of candidates in affine merge?

[A]: There is no performance improvement.

[Q]: What is the performance of proposed affine MC changes on affine motion friendly content (with zooming and so on)?

[A]: ????????.

Conclusion: Affine merge candidates’ modification is not useful. Gain can be achieved only by adaptive selection between 4-parameters and 6-parameters Affine MC on CU level. It provides 0.3%(RA)/ 0.3%(LD)/ 0.3%(LDP) gain with ~10% increment of Enc. run time.

## EE5: Improved MV coding

This experiment is targeting to the exploration of following changes proposed for motion vector coding improvement:

- Derive the sign of one of the motion vector component from other coded information

- Allow one of the motion vectors in bi-prediction to be used as predictor for the other,

- Use new rounding criteria for full-pixel motion vectors that depends on the mvp index selected

Questions recommended to be answered during EE tests.

[Q]: What is performance gain of each modifications separately in QTBT framework?

[A]: EE5-T1, T2 and T3 give an answer.

- MV sign information hiding gain is 0.1%(RA)/ 0.1%(LD)/ 0.1%(LDP)

- MV prediction candidates list up-date  is 0.3%(RA)

- Full and half pel candidate rounding gain is 0.0%(RA)/ 0.0%(LD)/ 0.0%(LDP) gain with around 5% increment of Enc. run time.

[Q]: Motion vector hiding should be modified for QTBT (since there will be no PU any more).

[A]: Done.

Conclusion: Whole package of 3 modifications provides 0.4%(RA)/ 0.1%(LD)/ 0.1%(LDP) gain with around 20% increment of Enc. run time.

## EE6: Extended intra prediction reference

This experiment is targeting to the exploration of extended Intra prediction references to multiple lines, in single line increments, in the following ways:

- Reference tiers are additional lines/columns from non-boundary samples of reference blocks. These may be used instead of the boundary samples.

- Not only the nearest reference line but also further reference lines are utilized in intra prediction. The prediction generated from further reference line will compete with the prediction generated from the nearest reference line in order to choose the best prediction for each block. When further reference line is used, a residue compensation procedure is introduced to further refine the prediction.

Questions recommended to be answered during EE tests.

[Q]: What is the performance of proposed technologies compared to the JEM with the same encoder complexity (full search of 67 Intra prediction modes, for example)?

[A]: EE6-T7 and T8 were designed to give an answer. It is observed that the coding gains out of the farther reference tiers (T2) and the encoder modification of the better encoder (T7) are additive. The coding gain of the combined T2 + T7 (T8) is 0.9% as compared with the better encoder (T7), the same as that of T2 as compared with JEM 3.0.

[Q]: Which N (number of lines) (in JVET-C0043) provides the best tradeoff between performance and complexity?

[A]: EE6-T1, T2 and supplementary test in JVET-D0099 give an answer. According to the results, N = 4 gives a better trade-off on the performance.

- 2 additional lines 0.7% (AI) gain with 1.8 Enc. run time increment

- 3 additional lines 0.9% (AI) gain with 2.0 Enc. run time increment

[Q]: What are the benefits of residual compensation?

[A]: Difference between EE6-T5 and T3 gives an answer. Residual compensation provides 0.2%(AI) gain.

Conclusion: 0.9% gain in (AI) test with 2.0 increment of Enc. run time could be achieved by adaptive selection from 4 tiers.

## EE7: Adaptive clipping

This experiment is targeting to the exploration of modification of clipping process with signaled values. The clipping bounds are determined from the original signal characteristics and encoded in the slice-header using prediction reference frame clipping bounds (in non-Intra pictures). Residuals smoothing for clipped samples is applied (encoder only modification).

Questions recommended to be answered during EE tests.

[Q]: What would be the visual quality impact from proposed clipping modification?

[A]: Demo in a meeting place is required.

[Q]: What would be an effect of proposed clipping modification as post processing, not in the loop (which could be done by SEI)?

[A]: EE7-T3 gives an answer: 0.1%(AI)/ 0.3%(RA)/ 0.2%(LD)/ 0.2%(LDP) gain from post-filter can be achieved.

Conclusion: 0.4% gain in (RA) w/o Enc. run time increment could be achieved.

## EE8: Decoder side intra mode derivation

This experiment is targeting to the exploration of decoder-side intra mode derivation (DIMD) approach. The DIMD derives the intra prediction mode and so reduces the overhead of intra mode signaling. Further, because no mode signaling is needed in DIMD, intra prediction can be performed at higher granularity than in JEM. A one-line wide reference is used to determine the best direction for predicting an L-shaped template which has a width of 2 or 4 samples, depending on block size. The usage of DIMD is signaled by an additional flag.

Questions recommended to be answered during EE tests.

[Q]: What is the best tradeoff complexity vs. compression benefit?

[A]: EE8-T1,2,3,4 designed to answer this question. The proponents consider T3  (0.8% AI gain and 1.4x Enc time and 1.1x Dec time) to be the best option.

[Q]: Is it possible to restrict this technology and not use in small blocks?

[A]: Comparison between EE8-T4 and T3 gives an answer. Restriction of this tool for blocks smaller than 64 samples reduces gain in AI test  0.8%  0.5%.

[Q]: Is it possible to restrict the number of search candidates further?

[A]: Additional testing results are provide in JVET-D0097 to answer this question, which reduces the number of search candidates in the worst case from 19 to 11. The gain in AI is reduced from 0.8%  0.5%.

[Q]: Is it possible to remove parsing dependencies (shall not rely on reconstructed samples)?

[A]: JVET-D0098 provides solution to remove the parsing dependency of DIMD which reduces the gain in AI from 0.8%  0.7%.

[Q]: How this tool can be aligned with QTBT; if it is intended to still retain two different ways of treating for larger and smaller blocks (as currently for 2Nx2N and NxN)?

[A]: Tool was tests on top of QTBT framework.

[Q]: The comparison against anchor with similar encoder complexity (using other than the fast search for intra mode) was requested (and also identify if decoder-side derivation still gives similar gains when the encoder would check more modes).

[A]: The comparison between T8 and T7 gives the answer, which shows that most of the coding gains from DIMD can be preserved when compared to the encoder-only option with comparable complexity (0.7% AI gain).

Conclusion: 0.8% (AI) / 0.5% (RA) gain with 1.4 Enc. run time increment (AI) and 10% Dec. run time increment could be achieved.

## EE 9 Adaptive Scaling for Extended Colour Volume Material

This experiment studies an alternative to the residual scaling proposed in JVET-B0054. The motivation in JVET-B0054 is to remove the overhead of delta QP that is signaled by the encoder to compensate for the reshaping of quantization noise as a function of brightness in next generation containers such as ST 2084.

Questions recommended to be answered during EE tests.

[Q]: What is performance of JVET-C0066, JVET-C0095 and JVET-C0102 using the content and test conditions provided by the AhG 7 on JEM coding of extended colour material with:

- Results reported with the weighted BD-Rate and BD-PSNR metrics, where the weighting corresponds to the target luma-to-QP mapping function.

- Results reported with BD-Rate and BD-PSNR for the tPNSRY and deltaE metrics, where the metrics are computed from linear RGB

- Results reported for visual quality assessment

[A]: Only partial data available.

[Q]: What is the benefit of combining different aspects of the JVET-C0066, JVET-C0095 and JVET-C0102?

[A]: Not answered yet.

Conclusion: Test procedure should be agreed first. AhG7 is discussing testing procedure.

# Recommendation

- Review ALL EE tests’ contribution and cross-checks.

- Make decisions on each part on technology under consideration by reviewing EE results.

# Acknowledgement

- Thanks all proponents for providing EE s/w in time with proper announcement and explanation about modification in JVET reflector.

- Thanks all volunteers provided comprehensive cross-check.

# References

- E. Alshina, J. Boyce, Xiaoyu Xiu, Yao Jen Chang, Vadim Seregin, S.-H. Kim (EE coordinators) Description of Exploration Experiments on Coding ToolsJVET-C1010, 3nd Meeting, May. 2016.