| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0064 |
| --- | --- |

| Title: | Asymmetric Coding Units in QTBT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, T. Poirier, F. Urban 75 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 31 98 {fabrice.leleannec, tangi.poirier, fabrice.urban}@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a so-called Asymmetric CU coding tool on top of the Quad-Tree plus Binary Tree (QTBT) technology. The set of Binary Tree (BT) split modes of QTBT is extended, to support asymmetric binary splitting of a coding unit. As in QTBT, exactly one transform block and one prediction block is associated to each coding block.

Each coding stage (BT split signaling, prediction, transform, entropy coding, de-blocking filter) is adapted to new rectangular block sizes introduced in the JEM-3.0. These sizes correspond to a CU width or height multiple of 3, resulting from the division of a CU into 2 sub-CU with respective size ¼ and ¾ the size of the parent CU. In practice, this leads to a CU width or height potentially equal to 12 or 24.

To activate the Asymmetric CU tool in Chroma components, the MaxBTSizeISliceC parameter is set to 64, instead of 16 in the test3 QTBT configuration chosen in 3rd JVET meeting. Around 6.5% average bitrate reduction in Chroma reportedly results from this modified MaxBTSizeISliceC, without any coding time increase.

For the asymmetric CU proposed tool, average coding gains of 1.54% in Luma and around 8% in Chroma are reported, compared to the unchanged JEM-3.0, with 400% encoding runtime ratio. The decoding runtime impact is reported to be negligible. A faster version of the proposed tool is proposed with reported 1.16% gain in Luma, around 7.5% in Chroma, and 217% coding time ratio. Compared to the (JEM-3.0 + MaxBTSizeISliceC=64) configuration, 1.46% and 1.08% coding gains in Luma are reported, respectively with the slow and fast configurations of proposed tool.

Finally, it is suggested that the Asymmetric CU tool is integrated into a dedicated EE, to be further investigated in JVET.

# Introduction

The goal of this contribution is to investigate if the spatial topology provided by QTBT [1] can be further improved for better coding efficiency. More precisely, QTBT provides a high degree of flexibility in the representation of a coded picture in the compressed domain. This takes the form of multiple possible Coding Unit rectangular sizes. These sizes are always equal to a power of two, as a result of the dyadic spatial division that is performed.

On the other side, HEVC [2] includes the so-called AMP (Asymmetric Motion Partition) feature, which allows dividing a coding unit into two prediction units in an asymmetric way.

In this contribution, the QTBT coding tree representation is being extended for Intra slices. Asymmetric splitting of a CU into 2 sub-CUs is introduced as an additional Binary Tree (BT) split mode.  This involves the adaptation of each coding stage of the JEM3.0 to new block sizes, expressed as 3∙2n.

Section 2 describes the technology developed to support such enriched QTBT coding tree. BD-rate performance of the proposed tool is given in section 3, and is discussed in section 4.

Finally, section 5 concludes the contribution, and recommends to further evaluate the presented tool in a JVET Exploration Experiment.

# Technology Description

The QTBT introduces flexibility in the Coding Unit (CU) structure used to divide the picture in rectangular blocks. Coding Units are no more sub-divided into Prediction Units and/or Transform Units, as was the case in HEVC [2], then until JEM2 [3].

Figure 1: Quad-Tree Plus Binary-Tree (QTBT) CTU representation adopted in JVET-C0024[1]

This contribution explores the potentiality to further improve the coding efficiency provided by QTBT, through an extended set of available Coding Unit sizes.

To do so, 4 new Binary Tree splitting modes have been introduced into the QTBT framework, so as to allow new splitting configurations. So-called asymmetric splitting modes are proposed in addition to the splitting modes already available in QTBT, as shown by Figure 2.

According to the added splitting, a coding unit with size S is divided into 2 sub-CU with sizes S/4 and 3.S/4, either in the horizontal or in the vertical direction.  In practice the added available CU sizes are 12 and 24. In a further extended version of the tool CU sizes 6 and 48 may be allowed.

Figure 2: Set of CU splitting modes available in the proposed coding scheme

Furthermore, a CU that is asymmetrically divided can be split in a binary fashion, in the same way as in the already existing binary tree structure. This typically leads to the CU topology illustrated on Figure 3.

Figure 3: coding structures chosen to encode (intra) an exemplary BasketballDrill picture. Red squares represent the Quad-Tree decomposition of CTUs. Blue rectangles represent the Binary-Tree embedded in the quad-tree.

## Binary-Tree Split Syntax extension

Figure 4 shows the BT split mode syntax extension brought to the QTBT framework, so as to signal asymmetric binary splitting. This extension corresponds to dashed lines on Figure 4.

As can be seen, modifications are brought in case CU is split in a binary way. The BT split mode flag that was already present in QTBT is now interpreted as a BT split orientation (horizontal or vertical).

Next two additional flags are introduced. First flag signals if current BT split mode is asymmetric or not. If not, the BT split mode is derived as HOR or VER, according to the known BT split orientation. If yes, the second new flag indicates the type of asymmetric split mode used, for the considered BT split orientation.

Figure 4: BT-split mode syntax extended to support the signaling of proposed asymmetric binary splitting modes

## Transform

The EMT [4] set of transforms of JEM3 has been extended to the support of transform sizes 12 and 24. Thus, the transforms DCT-II, DCT-V, DCT-VIII, DST-I and DST-VII are defined for sizes 12 and 24, in addition to EMT transform sizes 2n,n=1,2,…,7.

A butterfly version of the DCT-II is provided for the added transform sizes.

## Entropy coding of transform coefficients

The transform coefficient entropy coding is slightly adapted to support the coding of blocks with width or height equal to 12 or 24.

The coefficient coding scheme is unchanged, which is easy since proposed asymmetric block sizes are a superset of 4x4 blocks, i.e. the Coding Group (CG) size.

Some block size dependent contexts associated to already existing block sizes 2n are re-used. This concerns the coding of the last significant coefficient XY position in a transform block and the GreaterThan0 coefficient significant flag.

## Encoder speed-ups

A few encoder speed-ups are used so as to limit the combinatory of the Rate Distortion Optimization.

# Simulation Results

In the simulation results provided here, the Asymmetric CU tool is activated in Luma and Chroma components of Intra slices. Thus only All-Intra results are provided. First, note that in the QTBT test3 configuration chosen during the Geneva BoG on QTBT [6], CU size up to 16 is allowed in Intra Slices (parameter MaxBTSizeISliceC). This leads to an effective maximum CU size equal to 8 in Chroma components.

To allow the use of asymmetric CU in Chroma components, MaxBTSizeISliceC is set to 64 in the results reported here. Table 1 shows the coding gain obtained in Chroma through this adjusted QTBT configuration for Intra Chroma components. These results are obtained without any significant change in encoding time. The encoding time reduction obtained on Table 1 is mainly due to some speed-ups and source code improvements brought to the JEM-3.0 software.

This is confirmed by Table 2, which shows results for the default test3 configuration, with encoding accelerated by about 12% without any change in the results.

Table 1: Impact of settings the maximum BT size to 64 in Chroma component, Intra slice

Table 2: Encoder runtime decrease obtained through a number of source code cleaning and speed-ups

Table 3 shows the BD-rate gain obtained over JEM-3.0 thanks to the proposed Asymmetric CU tool, used in Luma and Chroma intra slices. 1.54% average bitrate reduction is obtained in Luma, and respectively 8.17% and 7.48% in Chroma, compared to JEM3.0. The additional combinatory brought to the RD optimization leads to about 400% coding time ratio compared to JEM3.0.

Table 3: Coding gain obtained with the proposed tool, and with maximum BT size 64 in Chroma

Table 4 shows the coding gain and runtime obtained with the encoder speed-ups presented in section 2.4. As can be seen, the coding time ratio over JEM3.0 is decreased to around 200%, while the BD-rate gain of 1.16% is obtained in Luma, and respectively 7.78% and 7.06% in Chroma.

Table 4: coding gain with alternative trade-off between coding time and BD-rate performance

Table 5 and Table 6 show the BD-rate results of the Asymmetric CU tool, compared to the speeded-up JEM-3.0 + MaxBTSizeISliceC=64 configuration, i.e. the JEM-3.0 where max BT Chroma size is set to 64 in intra slices. One sees that in larger RD search configuration, 1.46% gain is obtained in Luma, and 1.55% / 1.24% gain is obtained in Chroma. In the faster RD search configuration, 1.08% gain is obtained in Luma, and 1.08%/0.78% gain is obtained in Chroma.

Note that the coding time ratios on Table 5 and Table 6 differ from those of Table 3 and Table 4. This is because in those experiments, the source code speed-ups of the encoder, whose impact is reported by Table 2, are activated in the (JEM-3.0 + MaxBTSizeISliceC=64) codec, which is used as the anchor here.

Table 5: Coding gain provided by the Asymmetric CU tool compared to (JEM3.0 + MaxBTSizeISliceC=64)

Table 6: Comparative gain and coding time of proposed Asymmetric CU in fast mode, vs (JEM3.0 + MaxBTSizeISliceC=64)

# Discussion

Several QTBT configurations have been evaluated in [6], and the test3 trade-off was adopted during 3rd JVET meeting.

In this section, we discuss the trade-off between compression performance improvement and encoder runtime increase. Figure 9 shows several points obtained when evaluating various configurations of the QTBT tool on one side, and of the proposed QTBT extension on the other side.

It can be seen that configuring QTBT at a higher level of complexity than the test3 of [6] allows improving the coding efficiency of JEM-3.0, at the cost of increased encoding time.

However, Figure 9 shows that compression efficiency provided by the Asymmetric CU tool brings higher BD-rate improvement than QTBT, at equivalent coding time increase.

This demonstrates the proposed tool increases the potential of QTBT in terms of compression efficiency.

Figure 9: comparison of Luma BD-rate improvement=f(Coding time) for various configurations of unchanged QTBT and proposed Asymmetric CU tool. Red points correspond to the QTBT with different maximum BT depths for Luma. Blue points correspond to the proposed tool with different speed-ups.

# Conclusion

In this proposal, the coding topologies proposed in QTBT are enriched, in order to offer increased flexibility in the representation of the coded picture in the compressed domain.

This is achieved through asymmetric CU split modes added to QTBT, and JEM3.0 codec adaptation to support the coding/decoding of coding units with size equal to 3∙2n in width or height.

It has been shown that interesting trade-offs between coding time and coding gain over JEM-3.0 can be obtained with the proposed tool, when activated in intra slices.

Further work will consist in extending the Asymmetric CU to inter slices. It is recommended that the JVET group integrates this proposed Asymmetric-CU coding tool into a dedicated EE, so that further experiments and investigation can be conducted around this concept.

# References

[1] 	JVET-C0024, “EE2.1: Quadtree plus binary tree structure integration with JEM tools”, Han Huang, Kai Zhang, Yu-Wen Huang, Shawmin Lei, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016

[2] 	ITU-T H.265 TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU (10/2014), SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS, Infrastructure of audiovisual services – Coding of moving video, High efficiency video coding, Recommendation ITU-T H.265

[3] 	JVET-B1001_v3, ”Algorithm Description of Joint Exploration Test Model 2”, Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, Jill Boyce,  Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016

[4] 	JVET-C1001_v3, ”Algorithm Description of Joint Exploration Test Model 3”, Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, Jill Boyce,  Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 3rd Meeting: Geneva, CH, 26 May – 1 June 2016

[5] 	JVET-B0041, ” Adaptive reference sample smoothing simplification”, A. Filippov, V. Rufitskiy, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 2nd Meeting: San Diego, USA, 20–26 February 2016.

[6] 	JVET-C0101, “Report of BoG on QTBT configuration setting”, K. Choi, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 3rd Meeting: Geneva, CH, 26 May – 1 June 2016

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).