| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0090 |
| --- | --- |

| Title: | AHG8: Algorithm description of InterDigital’s projection format conversion tool (PCT360) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Bharath Vishwanath, Xiaoyu Xiu, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | 858-210-4819 Yuwen.He@interdigital.com Xiaoyu.Xiu@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

At the 3rd JVET meeting, AHG8 on 360 video test conditions was established. One of the mandates of the AHG8 is to “study software tools for conversion and rendering of 360 video content, and make it available to JVET as appropriate”. In response to AHG8’s mandate, InterDigital provided a 360 video projection format conversion tool (PCT360) available to JVET for experimentation before this meeting. This document describes the algorithms implemented in PCT360.

# Introduction

PCT360 is written in C++ and is integrated into HM-16.12 and JEM-3.0. Five projection formats listed in Table 1 are implemented.

Table 1. Projection formats

| Index | Projection format |
| --- | --- |
| 0 | Equirectangular [2] (ERP) |
| 1 | Cubemap [3] (CMP) |
| 2 | Equal-area [4] (EAP) |
| 3 | Octahedron [5] (OHP) |
| 4 | Icosahedron (ISP) |

We will introduce the representation of each projection format at first. Then we discuss the conversion design between any two projection formats. We also talked about the viewport generation using rectilinear projection.

In order to describe the 3D geometry of each projection format representation, 3D XYZ coordination system in PCT360 is defined as Figure 1. X axis points toward the front view of the sphere from the center of sphere. Y axis points toward the top view of the sphere from the center of sphere. Z axis points toward the right view of the sphere from the center of sphere.

The sphere can be sampled with longitude (ϕ) and latitude θ. The longitude ϕ in the range [-π, π] is known as yaw, and latitude θ in the range [-π/2, π/2] is known as pitch in aviation, where π is the ratio of a circle's circumference to its diameter. In PCT360, the longitude is defined by the angle starting from X axis in counter-clockwise as shown in Figure 1. The latitude is defined by the angle from equator plane toward Y axis as shown in Figure 1. The (X, Y, Z) coordinates on unit sphere can be presented by (ϕ, θ) with Equation (1) (2) (3).

| X = cos(θ)cos(ϕ) | (1) |
| --- | --- |
| Y = sin(θ) | (2) |
| Z = -cos(θ)sin(ϕ) | (3) |

Inversely, the longitude and latitude (ϕ, θ) can be evaluated with (X, Y, Z) coordinates with Equation (4)(5).

| ϕ = tan-1(-Z/X) | (4) |
| --- | --- |
| θ = sin-1(Y/(X2+Y2+Z2)1/2) | (5) |

Another 2D plane coordinate system is defined for each face of geometry projection. Each face is mapped to 2D plane, which is referred as uv plane associated with one face index. The image sampling grid is defined in 2D uv plane, and we refer the sampling point position as (m, n), where m and n are the column and row coordinates of the sampling position. Figure 2 shows an example for the sampling coordinates defined in uv plane for ERP projection. Those orange circles are sampling points. In PCT360, all sampling points are arranged in a symmetric way in each face in both two directions. Therefore there is a shift between the origin of (u, v) coordinates and the origin of (m, n) coordinates.

Figure 1. XYZ coordinate definition in PCT360, A3 is the equator

Figure 2. Sampling coordinate definition in (u, v) plane in PCT360

# Conversion of 2D and 3D coordinates for each projection format

This section describes the conversion between sampling point (m, n) on face f and 3D point position (X, Y, Z) for each projection formats. The (m, n) on face f is referred as (f, m, n). In PCT360, the conversion from (f, m, n) to (X, Y, Z) is implemented by the function map2Dto3D(), and the conversion from (X, Y, Z) to (f, m, n) is implemented by the function map3Dto2D(). Assume W and H are the width and height of face, respectively.

## Equirectangular projection format

There is only one face for ERP. Therefore, the face index for ERP is always set to 0 in PCT360. In uv plane, u and v are in the range [0, 1]. Given the positon (m, n), we can calculate (u, v) first with Equation (6)(7).

| u = (m + 0.5)/W, 0≤ m <W | (6) |
| --- | --- |
| v = n + 0.5)/H, 0 ≤ n < H | (7) |

Then longitude and latitude (ϕ, θ) in the sphere can be calculated from (u, v) as:

| ϕ = (u - 0.5)*(2* π) | (8) |
| --- | --- |
| θ = (0.5 - v)*π | (9) |

Then (X, Y, Z) can be calculated from Equation (1)(2)(3).

Given (X, Y, Z), (ϕ, θ) is calculated with Equation (4)(5) at first. (u, v) is calculated by solving Equation (8)(9). Then (m, n) is calculated by solving Equation (6)(7).

## Cubemap projection format

Figure 3 shows the (u, v) definition for each face of cubemap in PCT360. Table 2 specifies the face index values corresponding to each of the six cubemap faces used in PCT360. Each face in uv plane is a 2x2 square. u and v are defined in the range of [-1, 1].

(a)                 (b)

Figure 3. Coordinates definition for cubemap

Table 2. Face index of cubemap

| Face index | Face label | Notes |
| --- | --- | --- |
| 0 | PX | Front face with positive X axis value |
| 1 | NX | Back face with negative X axis value |
| 2 | PY | Top face with positive Y axis value |
| 3 | NY | Bottom face with negative Y axis value |
| 4 | PZ | Right face with positive Z axis value |
| 5 | NZ | Left face with negative Z axis value |

Given the positon (m, n) on face f, (u, v) is calculated as:

| u = (m + 0.5)*2/W - 1, 0≤ m <W | (10) |
| --- | --- |
| v = (n + 0.5)*2/H - 1, 0 ≤ n < H | (11) |

(X, Y, Z) are derived with Table 3 given the position (u, v) on face f.

Table 3. (X, Y, Z) derivation given (u, v) and face index f

| f | X | Y | Z |
| --- | --- | --- | --- |
| 0 | 1.0 | -v | -u |
| 1 | -1.0 | -v | U |
| 2 | U | 1.0 | V |
| 3 | U | -1.0 | -v |
| 4 | U | -v | 1.0 |
| 5 | -u | -v | -1.0 |

Given (X, Y, Z), the (u, v) and face index f is calculated according to Table 4.

Table 4. Derivation of (u, v) and face index f given (X, Y, Z)

| Condition | f | u | v |
| --- | --- | --- | --- |
| ¦X¦ ≥ ¦Y¦ and ¦X¦ ≥ ¦Z¦ and X >0 | 0 | -Z/¦X¦ | -Y/¦X¦ |
| ¦X¦ ≥ ¦Y¦ and ¦X¦ ≥ ¦Z¦ and X <0 | 1 | Z/¦X¦ | -Y/¦X¦ |
| ¦Y¦ ≥ ¦X¦ and ¦Y¦ ≥ ¦Z¦ and Y >0 | 2 | X/¦Y¦ | Z/¦Y¦ |
| ¦Y¦ ≥ ¦X¦ and ¦Y¦ ≥ ¦Z¦ and Y <0 | 3 | X/¦Y¦ | -Z/¦Y¦ |
| ¦Z¦ ≥ ¦X¦ and ¦Z¦ ≥ ¦X¦ and Z >0 | 4 | X/¦Z¦ | -Y/¦Z¦ |
| ¦Z¦ ≥ ¦X¦ and ¦Z¦ ≥ ¦Y¦ and Z <0 | 5 | -X/¦Z¦ | -Y/¦Z¦ |

(m, n) is calculated by solving the Equation (10)(11).

## Equal-area projection format

[to be updated]

## Octahedron and icosahedron projection formats

[to be updated]

# Conversion between two projection formats

Assume we have source projection format and destination projection format, and we want convert source projection format to destination projection format. In PCT360, the picture of certain projection format is stored face by face. The conversion is also performed face by face.

Denote (md, nd) as a point on face fd in sampling grid space of destination projection format, and (X, Y, Z) as the corresponding coordinates in XYZ space. Denote (ms, ns) as a point on face fs in in sampling grid space of source projection format. The conversion is performed in following steps:

- Map the 2D sampling point (md, nd) on face fd to 3D space coordinates (X, Y, Z) according to destination projection format;

- Map (X, Y, Z) to 2D point (ms, ns) on face fs according to source projection format;

- Calculate sample at position (ms, ns) by interpolating from neighboring samples at integer positions on face fs in source projection picture, store at (md, nd) on face fd in destination projection format.

The step (1) is done by function map2Dto3D() in the class of destination projection geometry in PCT360. The step (2) is done by function map3Dto2D() in the class of source projection geometry in PCT360.

# Spherical objective quality metrics

Three spherical quality metrics are implemented in PCT360 for 360 video quality evaluation: Sphere based PSNR (SPSNR), SPSNR weighted by latitude access frequency (L-SPSNR), and area weighted spherical PSNR (AW-SPSNR). SPSNR and L-SPSNR were originally proposed in [8]. In PCT360, there is no additional interpolation filter introduced in SPSNR L-SPSNR calculation to avoid the impact of interpolation filter in metric calculation. The AW-SPSNR metric is developed by InterDigital [9].

Table 3. Quality metrics supported

| Metric name | Characteristics |
| --- | --- |
| Area weighted Spherical PSNR (AW-SPSNR) | Calculate PSNR based on all samples; the distortion is weighted by sample area on the unit sphere |
| Spherical PSNR (S-PSNR) | Calculate PSNR based on a point set evenly sampled on the unit sphere |
| Latitude Spherical PSNR (L-PSNR) | Calculate PSNR based on a point set evenly sampled on the unit sphere; the distortion is weighted by viewing probability |
| PSNR | Calculate PSNR based on all samples with equal weight |

# Viewport generation

[to be updated]

Figure 4. Viewport generation with rectilinear projection

# References

- Y. He, B. Vishwanath, X. Xiu, Y. Ye, “AHG8: InterDigital’s projection format conversion tool”, JVET-D0021, Oct. 2016, Chengdu, China.

- Equirectangular projection, https://en.wikipedia.org/wiki/Equirectangular_projection

- Cubemap, https://en.wikipedia.org/wiki/Cube_mapping

- Lambert cylindrical equal-area projection, https://en.wikipedia.org/wiki/Lambert_cylindrical_equal-area_projection

- Octahedron, https://en.wikipedia.org/wiki/Octahedron

- Icosahedron, https://en.wikipedia.org/wiki/Regular_icosahedron

- Rectilinear projection, http://wiki.panotools.org/Rectilinear_Projection

- M. Yu, H. Lakshman, B. Girod, “A Framework to Evaluate Omnidirectional Video Coding Schemes”, IEEE International Symposium on Mixed and Augmented Reality, 2015.

- B. Vishwanath, Y. He, Y. Ye, “AHG8: Area Weighted Spherical PSNR for 360 video quality evaluation”, JVET-D0072, Oct. 2016, Chengdu, China.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).