| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0094 |
| --- | --- |

| Title: | EE7: Cross-check for adaptive clipping signalled w/o prediction (tests 1) and adaptive clipping as (post-filter 3). |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Seoul R&D Campus F-10 A Zone, Seongchon-gil 34, Seocho-gu, Seoul, Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution reports verification test results for EE7 (an adaptive clipping) tests 1 and 3. In first test clipping values are encoded w/o prediction from previously encoded frames. Performance gain achieved in EE7-T1 is 0.2%(AI)/0.3%(RA)/0.3%(LD)/0.4%(LDP). In EE7 test 3 adaptive clipping is applied as post-filter only, in-loop non-modified clipping (according to the bit-depth) is used. Performance gain achieved in EE7-T3 is 0.1%(AI)/0.3%(RA)/0.2%(LD)/0.2%(LDP).

# Introduction

Adaptive clipping was proposed to JVET in [1] and studied in EE7. Per each picture actual Min and Max values are measured on encoder side, signalled to the decoder and use in clipping process.

Signalling for adaptive clipping values include following parameters:

- PPS

  - Adaptive clipping on/off flag (1 bit in PPS)

  - Q-Quantization parameters for clipping values (2 bits in PPS)

- Slice Header

  - Adaptive clipping on/off flag (1 bit)

  - Adaptive clipping on/off flag for Chroma (1 bit)

  - Prediction of straightforward encoding of clipping values flag (for non Intra slices)

  - Fixed length (BitDepth-Q bits) code for quantized Min and Max values (if encoded w/o prediction )

  - Signed variable length code for quantized Min and Max values (if encoded w/o prediction )

In EE7 test 1 no prediction for clipping values was used (last line in syntax description is not needed). In EE7 test 3 prediction was used (all lines in syntax description are not needed), but application of adaptive clipping was restricted to operate as post-filter only (in-loop clipping values based on BitDepth were used).

# Experimental Results

Code for verified EE7 tests 1 and 3 was provided in time. Performance was tests vs JEM3.0 under JVET common test conditions [2]. Performance matches the data reported by proponent (except 2 byte minor mismatch due to the different settings for level in cfg). SW implementation matches EE description. Test results are summarized in Table1.

We have not found minor run time increment compared to JEM3.0

Table 1. Test results for EE7-T1 and EE7-T3.

Some additional study was also performed additionally to the test 1. In order to further simplify design we additionally also tested variant w/o quantization for clipping values (second line in syntax description is not needed). Min and Max values can be quantized only is source bit-depth is lower than internal bit-depth. It was found that w/o quantizing clipping values almost the same performance can be achieved (Table 2).

Table 2. Test results for EE7-T1 and EE7-T1 w/o quantizing clipping values.

Results for this additional study are provided for information.

# Conclusions

Test results for EE7 (an adaptive clipping) tests 1 and 3 are verified. In first test clipping values are encoded w/o prediction from previously encoded frames. Performance gain achieved in EE7-T1 is 0.2%(AI)/0.3%(RA)/0.3%(LD)/0.4%(LDP). In EE7 test 3 adaptive clipping is applied as post-filter only, in-loop non-modified clipping (according to the bit-depth) is used. Performance gain achieved in EE7-T3 is 0.1%(AI)/0.3%(RA)/0.2%(LD)/0.2%(LDP).

# References

- F. Galpin, P. Bordes, F. Le Léannec “P Adaptive Clipping in JEM2.0”, JVET-C0040

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).